3GPP TSG RAN WG1 Meeting #100b-e	R1-2002707

e-Meeting, April 24th-30th 2020

Agenda Item:	7.2.4.7

Source:	Moderator (FUTUREWEI)

Title:	FL summary #1 on NR Uu scheduling LTE sidelink

Document for:	Discussion and decision

# Introduction

A WID on NR-V2X was approved at RAN#83, with one of the objectives as follows:

2. Specify support for NR Uu to provide control for LTE sidelink

- Sidelink mode 4 as per the study outcome [RAN2, RAN1]; and

- Sidelink mode 3-like RRC-configured SPS scheduling with either RRC-based activation/deactivation as per the study outcome or DCI-based activation/deactivation [RAN1, RAN2].

  - RAN1 to make a decision on which option is supported until RAN#84.

- 6 companies submitted contributions for this AI ([1]-[6]), although [6] appears to be a misplaced contribution. This summary aims to collect issues brought in submitted contributions.

# Summary after preparation work

After discussion on the reflection, the proposal is:

- Have an email discussion to address Issue 5 for DL path loss only: whether open loop power control should be supported on the sidelink when NR Uu schedules LTE sidelink

- No email discussion for other issues, but:

  - Issues 1, 2, 3, to be addressed in May if needed

  - Issue 4: comment is passed to the Editor

# Issues listed by companies

The list of issues brought by companies is summarized here. The issues are summarized in Table 1, along with the proposals for each of them. Then, in following subsections, each issue is discussed in more detail.

Table 1. Summary of issues.

| Issue | Description | FL Proposal |
| --- | --- | --- |
| Issue 1 | When a DCI format 3_1 is sent: whether a DCI 3_01 can be sent at the same time as an NR Uu DCI | No email discussion |
| Issue 2 | SPS configuration index and activation fields: whether the description in format 3_1` should refer to 36.213 instead of 36.212 | No email discussion |
| Issue 3 | SL index: whether SL index should always be present in DCI 3_1 | No email discussion |
| Issue 4 | Units for timing of the grant: whether the formula indicating the timing of a grant should be in seconds or milliseconds | No email discussion |
| Issue 5 | Power control: whether open loop power control should be supported on the sidelink when NR Uu schedules LTE sidelink | Email discussion |

## Issue 1: When a DCI format 3_1 is sent

This issue and the proposal are listed in [1]. The proposal is as follows:

Proposal 1: Since size alignment of format 3_0 to a Uu DCI size is agreed to be not specified, size alignment of format 3_1 to a Uu DCI (when 3_0 is not configured) has also been agreed to be not specified.

- This means a UE does not expect to receive in the same PDCCH monitoring occasion and the same scheduling cell PDCCH carrying a DL grant and PDCCH carrying DCI format 3_1.

Essentially, this proposal would mirror the following proposal for DCI 3_0:

    - A UE does not expect to receive in the same PDCCH monitoring occasion and the same scheduling cell PDCCH carrying a DL grant and PDCCH carrying a SL DG.

The issue of which Uu DCI to match DCI 3_0 is listed as a remaining issue in 7.2.4.2.1 (mode 1).

FL’s proposal: given that this proposal is linked to the size alignment of DCI format 3_0 and Uu DCI size, which is an item under discussion in 7.2.4.2.1, RAN1 should let the discussion on DCI size alignment conclude before deciding on taking this discussion

## Issue 2: SL SPS configuration index

This issue is listed in [2]. DCI format 3_1 refers to DCI format 5A for the fields SL SPS configuration index and activation/release. In [2], it is stated that “ clause 5.3.3.1.9A of DCI format 5A in which the SPS configuration index and activation/release indication exist only when an LTE V2X SPS RNTI (i.e., SL-SPS-V-RNTI) is used to scramble to the DCI. Consequently, these fields will be absent in DCI format 3_1 as there is no such RNTI specified in NR.”

In the Feature Lead’s opinion, there is no ambiguity. DCI format 3_1 refers to DCI format 5A for the description of the fields. Thus, the fact that they are only present in 5A when the CRC is scrambled by SL-SPS-V-RNTI is irrelevant. Thus,  no correction is needed.

FL’s proposal: No action taken since this is not an essential correction

## Issue 3: SL index

This issue is brought up in [3] and [4]. SL index is listed in DCI format 3_1:

-	SL index – 2 bits as defined in 5.3.3.1.9A of [11, TS 36.212]

TS 36.212 in turn specifies the following for  “SL index”:

-	SL index – 2 bits as defined in clause 14.2.1 of [3] (this field is present only for cases with TDD operation with uplink-downlink configuration 0-6).

This brings the question of whether SL index is always present or only for case with TDD configurations 0-6.

In the Feature Lead’s opinion, there is no ambiguity. At RAN1#97, the following was agreed:

- DCI-based activation/deactivation is supported

  - Support of LTE PC5 scheduling by NR Uu (mode 3-like ) is based on UE capability

  - NR DCI provides the fields of DCI 5A in LTE-V that are related to SPS scheduling

Given that the DCI 5A does not always contain the SL index, the same applies for DCI 3_1. Furthermore, at RAN1#100b, the following was agreed:

- For NR Uu scheduling LTE sidelink, the subframe of the first sidelink transmission is the first SL subframe of the corresponding resource pool that starts not earlier than , where TDCI is the start timing of the slot carrying DCI format 3_1, NTA and Tc are defined in TS 38.211, X is the value indicated by Timing offset field in DCI format 3_1, m is the value indicated by SL index in DCI format 3_1 if SL index is present, otherwise m=0.

The highlighted text seems to unambiguously indicate that SL index is not always present.

FL’s proposal:

- No action taken as specs follow the agreements and no essential correction proposed.

## Issue 4: Units for timing of the grant

This issue is brought in [4]. In the formula: , ZTE argues that since Tc is defined in second, the formula should be expressed in seconds.

This is not an essential correction since there is no ambiguity. The formula above is expressed in ms, thus Tc has to be in ms.

FL’s suggestion: Not an essential correction. No need to discuss

## Issue 5: Power control

This issue is brought in [5]. In the physical procedure AI, the following decision was taken:

- For the SL open-loop power control, a UE can be configured to use DL pathloss (between TX UE and gNB) only, SL pathloss (between TX UE and RX UE) only, or both DL pathloss and SL pathloss.

[5] argues that open-loop power control for SL is supported for cross-RAT scheduling and thus, the above agreement should be implemented as well.

Given that there is open loop power control for LTE V2X, having an email discussion to see if and how to support would be beneficial

FL’s suggestion: have an email discussion to decide if and how to support SL open-loop power control in case of NR Uu scheduling LTE sidelink

# Conclusion

Remaining issues on cross-RAT scheduling were listed and discussed.

# Decisions at previous meetings

## RAN1#96

Agreements:

- Scheduling by gNB using RRC for LTE sidelink scheduled mode is supported from RAN1 perspective under the premise that there is sufficient time for coordination between the NR and LTE modules. No DCI to activate/release

  - RRC message delivers the SPS grant configuration and releases the SPS configuration.

  - Support of this scheduling mode is subject to UE capability (may or may not have capability for both LTE & NR)

  - Note: some specification LTE change is needed to support the reception of a grant through RRC

    - RRC message contains mode 3 grant content and timing

    - Up to the Editor to capture it as mode 3 or new LTE sidelink mode

  - No intention to have additional NR & LTE specification change (other than those described above) for this function in Rel-16

- RAN1 studied the feasibility of SPS scheduling by gNB for LTE sidelink with DCI activation/release, but there is no consensus to support it

## RAN1#96bis

Agreements:

Regarding RRC-based versus DCI-based activation/release of LTE sidelink SPS, RAN1 agrees to make the choice on the basis of at least:

- Spec impact

- Flexibility

- Performance, including latency

- Implementation complexity

- Timing of the activation/deactivation

## RAN2#105bis

Agreements:

For scheduling LTE SL UEs, the gNB uses RRC messages to deliver the SPS grant configuration.

Separate system information block should be designed to support LTE resource pool configuration via NR Uu. It will be defined as a container (OCTET STRING) and actual information follows what defined in LTE RRC.

gNB should be able to configure the LTE V2X mode 4 sidelink resource pool via dedicated signalling. In addition, gNB should be able to configure mode3 SL resources via dedicated signaling. It will be defined as a container (OCTET STRING) and actual information follows what defined in LTE RRC.

## RAN1#97

Agreements:

- DCI-based activation/deactivation is supported

  - Support of LTE PC5 scheduling by NR Uu (mode 3-like ) is based on UE capability

  - NR DCI provides the fields of DCI 5A in LTE-V that are related to SPS scheduling

  - The size of DCI for activation/deactivation is one of the DCI size(s) that will be defined for NR Uu scheduling NR V2V

    - FFS whether the DCI format is the same as one of the DCI formats that will be defined for NR Uu scheduling NR V2V

  - Activation/deactivation applies to the first LTE subframe after Z+X ms after receiving the DCI

    - Z is the same timing offset in current LTE V2X specs

    - X>0. FFS value(s) of X, and if one or multiple values of X are possible

## RAN1#98

Agreements:

- A new RNTI is introduced to scramble the NR DCI used for scheduling LTE PC5.

- X is dynamically indicated using a field in the DCI

  - FFS whether the DCI field provides an index to a table or the value of X

  - The minimum value of X is subject to UE capability

    - UE reports a single value subject to UE capability

## RAN1#98b

Agreements:

- The NR DCI field to indicate X provides an index to a table of values

  - The table of values is configurable, and has 8 values

  - The size of the DCI field is fixed at 3 bits

## RAN1#99

Agreements:

- Use a separate PDCCH monitoring configuration (as configured in Rel-15) for NR DCI scheduling LTE SL

  - The per-CC and across-CC blind decoding budget and the maximum number of non-overlapped CCEs for channel estimation are not increased.

  - The per-CC and across-CC maximum number of search spaces is not increased.

  - The per-CC and across-CC maximum number of CORESETs is not increased

    - When in the same slot, there is both PDCCH monitoring for Uu SL and PDCCH monitoring for SL for the same CC, the search space(s) for LTE SL is configured to be the same or a subset of those for Uu for the same CC or vice versa

Agreements:

- The minimum value of X signalled in the UE capability is one of the values (excluding spare values) that can be signalled in DCI 3_1

Agreements:

The supported values of X signaled in the DCI are:

- 0.75ms, 1ms, [1.25ms], [1.5ms], 2ms, 4ms, 5ms, 8ms, 10ms, 20 ms

  - Additional value(s) can be discussed during the Feb. meeting

- Spare values are reserved for future deployments

## RAN1#100

Agreements

- For NR Uu scheduling LTE sidelink, the subframe of the first sidelink transmission is the first SL subframe of the corresponding resource pool that starts not earlier than , where TDCI is the start timing of the slot carrying DCI format 3_1, NTA and Tc are defined in TS 38.211, X is the value indicated by Timing offset field in DCI format 3_1, m is the value indicated by SL index in DCI format 3_1 if SL index is present, otherwise m=0.

Agreements

- If UE is configured to monitor DCI 3_0, the sizes of DCI 3_1 and DCI 3_0 are aligned by zero padding.

- If UE is not configured to monitor DCI 3_0, the mechanism for size alignment between DCI 3_0 and Uu DCI is reused for size alignment between DCI 3_1 and Uu DCI

Agreements:

- The set of possible values for X is: 0ms, 0.25ms, 0.5ms, 0.625ms, 0.75ms, 1ms, 1.25ms, 1.5ms,1.75ms, 2ms, 2.5ms, 3ms, 4ms, 5ms, 6ms, 8ms, 10ms, 20 ms

# References

| No. | Tdoc | Type | Source | Title | Status |
| --- | --- | --- | --- | --- | --- |
| 1 | R1-2001557 | discussion | Huawei, HiSilicon | Remaining details of NR Uu control for LTE sidelink | present |
| 2 | R1-2001667 | discussion | vivo | Remaining issues on support of NR Uu controlling LTE sidelink | present |
| 3 | R1-2001810 | discussion | Nokia, Nokia Shanghai Bell | Remaining details of Support of NR Uu controlling LTE sidelink | present |
| 4 | R1-2001900 | discussion | ZTE, sanechips | Remaining issues on NR Uu control LTE sidelink | present |
| 5 | R1-2002240 | decision | Ericsson | NR UU controlling LTE sidelink transmissions | present |
| 6 | R1-2002543 | discussion | Qualcomm | Syncronisation details for NR-V2X | present |