3GPP TSG RAN WG1 #100bis-e			R1-2002894

e-Meeting, April 20th – 30th, 2020

Source:	NTT DOCOMO, INC.

Title:	Summary on Email discussion [100b-e-NR-TEIs-02]

Agenda Item:	7.2.12

Document for: 	Discussion and Decision

- Introduction

This contribution summarizes the NR Rel-16 TEI related and CLI/RIM related discussions and proposals in AI 7.2.12.

[100b-e-NR-TEIs-02]: Email discussion/approval of TPs on remaining issues for half-duplex operation in CA

- Whether/how to capture the agreement which is not correctly reflected in TS38.213

  - TPs in R1-2002074 and R1-2002229 are starting points for the discussion

- Whether/how to cover mixed numerology case

  - TP in R1-2002229 is a starting point for the discussion

till 4/23 (DCM, Hiroki)

- Remaining issue for half duplex operation in CA

In [4], the following remaining issues regarding half duplex operation in CA are identified.

- The agreement that half-duplex CA UE determines reference cell per symbol as a cell with the lowest ID among multiple serving cells in a band or band combination having direction determined by RRC D/U or semi SFI D/U is not correctly reflected and the meaning of reference cell has changed.

- Only one “and” exists in the “if” conditions, and the last condition should be satisfied for all the multiple serving cells.

- When transmission directions of multiple other cells are not aligned, UE cannot determine to follow the transmission direction of which cell.

- The agreements for inter-band CA case are not correctly captured in the specification.

According to the agreement, half-duplex CA UE determines reference cell per symbol as a cell with the lowest ID among multiple serving cells in a band or band combination having direction determined by RRC D/U or semi SFI D/U. However, this is not correctly reflected in the CR for TEI, the reference cell is defined as a cell with the smallest cell index among the multiple serving cells in the CR and the meaning of reference cell has changed compared with the agreement above. According to the email discussion after RAN1#99, to avoid the need of resolving the conflict among other cells, the reference cell should always have a direction determined by RRC D/U or semi SFI D/U. In addition, the four conditions for reference cell determination should be satisfied at the same time but only one ‘and’ exists in the if conditions. Meanwhile, the last condition for reference cell determination should be satisfied for all the multiple serving cells. The same issue also exist in other if conditions for half-duplex operation in CA. Hence, a text proposal is provided for 38.213 section 11.1 as below. Proposal 1: Adopt the following corrections for reference cell determination to the half-duplex operation in CA. -------------------------------------------------- Start of text proposal ------------------------------------------------------ ## 11.1 Slot configuration *** Unchanged text is omitted *** If a UE - is configured with multiple serving cells and is provided half-duplex-behavior-r16 = 'enable', and - is not capable of simultaneous transmission and reception on any of the multiple serving cells, and - indicates support of capability for half-duplex operation in CA with unpaired spectrum, and - is not configured to monitor PDCCH for detection of DCI format 2_0 on any of the multiple serving cells, for a set of symbols of a slot that are indicated to the UE for reception of SS/PBCH blocks in any of multiple serving cells by ssb-PositionsInBurst in SystemInformationBlockType1 or by ssb-PositionsInBurst in ServingCellConfigCommon, when provided to the UE, the UE does not transmit PUSCH, PUCCH, or PRACH in the slot if a transmission would overlap with any symbol from the set of symbols, and the UE does not transmit SRS in the set of symbols of the slot in any of multiple serving cells. *** Unchanged text is omitted *** If a UE - is configured with multiple serving cells and is provided half-duplex-behavior-r16 = 'enable', and - is not capable of simultaneous transmission and reception on any of the multiple serving cells, and - indicates support of capability for half-duplex operation in CA with unpaired spectrum, and - is not configured to monitor PDCCH for detection of DCI format 2-0 on any of the multiple serving cells, the UE determines per symbol a reference cell as a cell with the smallest cell index among the multiple serving cells having direction determined to be - downlink, or uplink as indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated - uplink, if the symbol is flexible and the UE is configured to transmit SRS, PUCCH, PUSCH, or PRACH on the symbol - downlink, if the symbol is flexible and the UE is configured to receive PDCCH, PDSCH or CSI-RS on the symbol If a UE - is configured with multiple serving cells in a frequency band and is provided half-duplex-behavior-r16 = 'enable', and - is not capable of simultaneous transmission and reception on any of the multiple serving cells, and - indicates support of capability for half-duplex operation in CA with unpaired spectrum, and - is not configured to monitor PDCCH for detection of DCI format 2_0 on any of the multiple serving cells, the UE does not expect - a symbol to be indicated as downlink or uplink on the reference cell and as uplink or downlink on another cell, respectively, by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated, - tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigDedicated to indicate a symbol as downlink on the reference cell and to detect a DCI format scheduling a transmission on the symbol on another cell, and - to be configured by higher layers to receive PDCCH, PDSCH, or CSI-RS on a flexible symbol on the reference cell and to detect a DCI format scheduling a transmission on the symbol on another cell. If the reference cell and another cell for a UE operate in different frequency bands and if the UE - is configured with multiple serving cells and is provided half-duplex-behavior-r16 = 'enable', and - is not capable of simultaneous transmission and reception on any of the multiple serving cells, and - indicates support of capability for half-duplex operation in CA with unpaired spectrum, and - is not configured to monitor PDCCH for detection of DCI format 2-0 on any of the multiple serving cells, the UE - UE assumes symbol as flexible, is not required to receive higher layer configured PDCCH, PDSCH, or CSI-RS and not expected to transmit higher layers configured SRS, PUCCH, PUSCH, or PRACH, when tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated indicates symbol as downlink or uplink on the other cell and as uplink or downlink for the reference cell, respectively, - transmits a signal/channel scheduled by a DCI format on a symbol of the other cell when the symbol is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigDedicated for the reference cell, - is not required to receive a higher layer configured PDCCH, PDSCH, or CSI-RS on flexible symbols on the reference cell in a set of symbols, if the UE detects a DCI format scheduling a transmission on one or more symbols in the set of symbols on the other cell. If a UE - is configured with multiple serving cells and is provided half-duplex-behavior-r16 = 'enable', and - is not capable of simultaneous transmission and reception on any cell from the multiple serving cells, and - indicates support of capability for half-duplex operation in CA with unpaired spectrum, and - is not configured to monitor PDCCH for detection of DCI format 2-0 on any of the multiple serving cells, the UE - does not expect tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated for the reference cell to indicate a symbol as uplink and to detect a DCI format scheduling a reception on the symbol on another cell - does not expect to be configured by higher layers to transmit SRS, PUCCH, PUSCH, or PRACH on a flexible symbol on the reference cell and to detect a DCI format scheduling a reception on the symbol on another cell - does not transmit a PUCCH, PUSCH or PRACH that is configured by higher layers on a set of symbols on another cell if at least one symbol from the set of symbols is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or is a symbol corresponding to a PDCCH, PDSCH, or CSI-RS reception that is configured by higher layers on the reference cell - does not transmit a SRS that is configured by higher layers on a set of symbols on another cell if the set of symbols is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or corresponds to a PDCCH, PDSCH or CSI-RS reception that is configured by higher layers on the reference cell - does not receive a PDCCH, PDSCH or CSI-RS that is configured by higher layers on a set of symbols on another cell if at least one symbol from the set of symbols is indicated as uplink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or is a symbol corresponding to a SRS, PUCCH, PUSCH, or PRACH transmission that is configured by higher layers on the reference cell - assumes a symbol indicated as downlink or uplink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated on another cell to be flexible, if the UE is respectively configured by higher layers to transmit SRS, PUCCH, PUSCH, or PRACH or to receive PDCCH, PDSCH, or CSI-RS on the reference cell - does not expect to detect a first DCI format scheduling a transmission or reception on a symbol on a first cell and a second DCI format scheduling a reception or transmission on the symbol on a second cell, respectively ----------------------------------------------------- End of text proposal ------------------------------------------------------ According to the agreement for half-duplex UE operation, UE does not always follow reference cell transmission direction. For example, in case 3 and case 16 as shown in Table 1, UE will drop the transmission on reference cell for inter-band CA, the issue is when transmission directions of multiple other cells are not aligned, UE cannot determine to follow the transmission direction of which cell. Table 1 Half-duplex UE behavior in different cases No; Ref cell; Other cell; UE behavior; Note. 3; Semi SFI D; Dynamic U; Alt 1: Allowed to drop D for inter-band Error case in intra-band; Overriding semi SFI D to F on reference cell for the UE. 16; RRC D; Dynamic U; Alt 1: Allowed to drop D for inter-band Error case in intra-band. As an example shown in Table 2, if UE is configured with three cells, Pcell is configured with semi SFI D or RRC D, a dynamic U is scheduled on Scell 1 and RRC D is configured on Scell2. Then, according to the agreement, Pcell is the reference cell, UE should drop D on reference cell and transmit dynamic U on Scell1, but there is no conflict between Pcell and Scell2, it is not clear whether a half-duplex UE should drop dynamic U on Scell 1 or drop RRC D on Scell 2. From our perspective, UE should prioritize dynamic U on Scell1 in this case according to the principle of case 3 and case 16. Table 2 Conflict direction on Scells Pcell (Reference cell); Scell1; Scell2; UE behavior. Semi SFI D; Dynamic U; RRC D; drop D or drop U?. RRC D; Dynamic U; RRC D; drop D or drop U?. Proposal 2: For a half-duplex CA UE, if reference cell is semi SFI D or RRC D, UE should drop high layer configured D on other cells if there is dynamic U on one of the other cells. In addition, the agreements for inter-band CA case are not correctly captured in the specification. Hence, a text proposal is provided below for half-duplex operation in CA in 38.213 section 11.1. -------------------------------------------------- Start of text proposal ------------------------------------------------------ If the reference cell and another cell for a UE operate in different frequency bands and if the UE - is configured with multiple serving cells and is provided half-duplex-behavior-r16 = 'enable', - is not capable of simultaneous transmission and reception on any of the multiple serving cells, - indicates support of capability for half-duplex operation in CA with unpaired spectrum, and - is not configured to monitor PDCCH for detection of DCI format 2-0, the UE - UE assumes symbol on the other cell as flexible, is not required to receive higher layer configured PDCCH, PDSCH, or CSI-RS and not expected to transmit higher layers configured SRS, PUCCH, PUSCH, or PRACH, when tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated indicates symbol as downlink or uplink on the other cell and as uplink or downlink for the reference cell, respectively, - transmits a signal/channel on a symbol of the other cell and is not required to receive a higher layer configured PDCCH, PDSCH, or CSI-RS on the symbol on the reference cell and any of the other cells when the symbol is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigDedicated for the reference cell and if the UE detects a DCI format scheduling the transmission on the symbol on the other cell, - transmits a signal/channel on a symbol of the other cell and is not required to receive a higher layer configured PDCCH, PDSCH, or CSI-RS on the symbol on the reference cell and any of the other cells, if the symbol on the reference cell is flexible and the UE is configured by higher layers to receive PDCCH, PDSCH, or CSI-RS on the symbol on the reference cell and the UE detects a DCI format scheduling the transmission on the symbol on the other cell. ----------------------------------------------------- End of text proposal ------------------------------------------------------

In [5], the following remaining issues regarding half duplex operation in CA are identified.

- The agreement that half-duplex CA UE determines reference cell per symbol as a cell with the lowest ID among multiple serving cells in a band or band combination having direction determined by RRC D/U or semi SFI D/U is not correctly implemented.

- If the above agreement is correctly implemented, then mixed numerology case (which remained open after RAN1#100e) can be handled by defining the reference cell as cell with lowest SCS among cells for which the symbols (configured as RRC D/U or semi SFI D/U) are overlapping

In RAN1#100e TP for this feature has been approved. Firstly, it seemed that the following agreement (highlighted in yellow) on determination of reference cell has not been correctly implemented in the TP Agreement: - Half-duplex CA UE determines reference cell per symbol as a cell with the lowest cell ID among multiple serving cells in a band or band combination having direction determined by RRC D/U or semi SFI D/U - Note: this overrides earlier agreement ”Reference (Ref) cell is the cell with the lowest cell ID among cells: (i) within the band or band combination and (ii) with conflicting directions, and “Other cell” is any cell within the band or band combination other than the Ref cell. “ - Note: Agreed cases 12, 14, 17 and 18 are not needed anymore - Note: Agreed cases 9 and 10 should apply to collisions between two cells irrespective of a cell being reference or other The cell configured with flexible symbol by semi SFI F should not be considered as reference cell, based on the above agreement. Otherwise, it would be necessecary to re-introduce following cases No; Ref cell; Other cell; UE behavior; Note. 12; Dynamic U; Semi SFI D; Allowed to drop D; Dropping on other cell. 14; Dynamic D; Semi SFI U; Allowed to drop U; Dropping on other cell. 17; Dynamic U; RRC D; Allowed to drop D; Dropping on other cell. 18; Dynamic D; RRC U; Allowed to drop U; Dropping on other cell. Proposal 2: Adopt the following TP for half-duplex feature to align specification with agreement. ## 11.1 Slot configuration <unchanged text omitted > If a UE - is configured with multiple serving cells and is provided half-duplex-behavior-r16 = ‘enable’, - is not capable of simultaneous transmission and reception on any of the multiple serving cells, - indicates support of capability for half-duplex operation in CA with unpaired spectrum, and - is not configured to monitor PDCCH for detection of DCI format 2-0, the UE determines per symbol a reference cell as a cell with the smallest cell index among the multiple serving cells and determines a symbol on the reference cell to be - downlink, uplink, or flexible as indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated - flexible if tdd-UL-DL-ConfigurationCommon is not provided - uplink, if the symbol is flexible and the UE is configured to transmit SRS, PUCCH, PUSCH, or PRACH on the symbol - downlink, if the symbol is flexible and the UE is configured to receive PDCCH, PDSCH or CSI-RS on the symbol the UE determines a reference cell for a symbol, among serving cells where the symbol is configured as - downlink, uplink as indicated by tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationDedicated, or tdd-UL-DL-ConfigDedicated-IAB-MT - uplink, if the symbol is flexible and the UE is configured to transmit SRS, PUCCH, PUSCH, or PRACH on the symbol, or - downlink, if the symbol is flexible and the UE is configured to receive PDCCH, PDSCH or CSI-RS on the symbol, as a cell with the smallest cell index.. If the above TP is accepted, then mixed numerology case (which remained open after RAN1#100e) can be handled by defining the reference cell as cell with lowest SCS among cells for which the symbols (configured as RRC D/U or semi SFI D/U) are overlapping. Proposal 3: To support mixed numerology scenario for half-duplex feature, adopt the following TP (in magenta) ## 11.1 Slot configuration <unchanged text omitted > If a UE - is configured with multiple serving cells and is provided half-duplex-behavior-r16 = ‘enable’, - is not capable of simultaneous transmission and reception on any of the multiple serving cells, - indicates support of capability for half-duplex operation in CA with unpaired spectrum, and - is not configured to monitor PDCCH for detection of DCI format 2-0, the UE determines per symbol a reference cell as a cell with the smallest cell index among the multiple serving cells and determines a symbol on the reference cell to be - downlink, uplink, or flexible as indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated - flexible if tdd-UL-DL-ConfigurationCommon is not provided - uplink, if the symbol is flexible and the UE is configured to transmit SRS, PUCCH, PUSCH, or PRACH on the symbol - downlink, if the symbol is flexible and the UE is configured to receive PDCCH, PDSCH or CSI-RS on the symbol the UE determines a reference cell for a symbol of the lowest sub-carrier spacing among multiple serving cells, where the symbol or at least one of overlapping symbols is configured as - downlink, uplink as indicated by tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationDedicated, or tdd-UL-DL-ConfigDedicated-IAB-MT - uplink, if the symbol is flexible and the UE is configured to transmit SRS, PUCCH, PUSCH, or PRACH on the symbol, or - downlink, if the symbol is flexible and the UE is configured to receive PDCCH, PDSCH or CSI-RS on the symbol, as a cell with the smallest sub-carrier spacing first and the smallest cell index second..

Based on above, following remaining issues for half-duplex operation in CA should be discussed in RAN1#100bis-e meeting.

- Whether/how to capture the agreement which is not correctly reflected in TS38.213

- Whether/how to cover mixed numerology case

2.1	Discussion 1

Companies are encouraged to provide views if there is a concern or comment on the following proposals.

Proposal: Capture the agreement which is not correctly reflected in TS38.213

Alt.1: TP in [4]

Alt.2: TP in [5]

| Company | Comment |
| --- | --- |
| ZTE | Regarding the definition of reference cell, both [4] and [5] have similar updates. We are open to the two updates. Regarding the second issue pointed out in [4], i.e., the case where more than two cells are configured, there could be some conflictions and the current spec is not clear about this confliction. More discussion is needed for this issue. |
| Nokia, NSB | I suppose reference [5] should be [6], but that is a minor detail. Issue1 (Reference cell correction): I think both CATT and our TP are towards right direction, we just need to agree on exact wording. Issue 2 (Case 3 and 16 with 3 and more cells): We believe that cases 3 and 16 are the only cases where other cell direction is followed. And cancelling higher layer configured DL transmissions on all configured cells seems to be the way to solve it. We could agree on principle and then discuss next week corresponding TP. |
| CATT | - For reference cell definition, we agree that the TP in [4] and [6] are both feasible, the exact wording can be further discussed; - For case 3 and 16 with more than 2 cells, we propose that UE should prioritize dynamic U and cancel all high layer configured DL transmissions according to the principle of case 3 and case 16, and this should be clarified in the specification; - The description of UE behavior for inter-band CA case are unclear, the TP in [4] can be considered; - The if conditions for half-duplex operations in CA needs to be modified as proposed in [4]. |
| Qualcomm | Regarding Issue 1: We think there is a more fundamental problem in that the reference cell is determined based on ‘logical time’ but due to Rx and Tx timing differences, the overlap can occur between other symbols, not considered in the reference cell determination. So the reference cell is determined based on a set of symbols but the rules apply to a potentially different set of symbols. Our understanding is that the agreement was that the UE can handle this and similar problems based on implementation. However, due to this fundamental ambiguity, it is unclear why other small changes would be necessary. Regarding Issue 2: Our understanding is that for more than 2 CCs, the UE first determines pair-wise application of the rules for the reference cell and each other cell. After that, it is expected that there are no directional conflicts in the resulting operation among all cells. The scheduling has to ensure that this happens. If this condition is not satisfied, there is no specified requirement, and operation is up to UE implementation. |
| Ericsson | Regarding Issue 1 on reference cell definition: I agree with Qualcomm it is important to first address the fundamental understanding instead of directly jumping into updating the spec text. Regarding Issue 2 on Case 3 and Case 16 with more cells: It appears there exists some unclarity in this regard. We are open to discuss this further. |
| vivo | - For reference cell definition, we are fine with the TP in [4] and [6] in principle, the wording can be further discussed. - For case 3 and case 16 with conflictions among other cells, solutions to handle this issue can be further discussed. We agree with Nokia and CATT that cancelling higher layer configured DL transmissions on all configured cells seems reasonable solution. - In addition to the issues mentioned above, we find that the conflict direction is determined based on RRC configured serving cells in current specification. Further clarification is needed on whether the deactivated Scell(s) are considered in the serving cells for reference cell determination, and how to handle the deactivated Scells if considered. |

2.2	Discussion 2

Companies are encouraged to provide views if there is a concern or comment on the following proposals.

Proposal: Cover mixed numerology case.

Alt.1: TP in [5]

| Company | Comment |
| --- | --- |
| ZTE | The current TP is not clear how to handle the case of BWP switching, i.e., whether the SCS of the previous BWP or the SCS of the target BWP should be for the determination of reference cell. Some further discussion may be beneficial. |
| Nokia, NSB | sub-carrier spacing of serving cells is given by their active BWP, this can be clarified. |
| CATT | For mixed numerology case, we agree the principle of defining the reference cell as cell with lowest SCS among cells for which the symbols (configured as RRC D/U or semi SFI D/U) are overlapping, the exact wording can be further discussed. |
| Qualcomm | We prefer not to add support of different SCS. Half-duplex restriction can only occur between TDD cells in the same FR. Considering this restriction, there is no practical use of different SCS. |
| vivo | We agree that the mix numerology scenario can be discuss but the current spec (always using the lowest cell index as reference cell) seems to work even with mixed numerology case. Maybe some minor spec update would be needed to clarify the handling the partial overlapping symbol across different cell due to mixed numerology and do not need to introduce additional rule by considering lowest SCS for determine reference cell. |

- Conclusion

Based on the discussion, following agreements and conclusion were made.

Agreement:

- Develop TP for Issue 1: Agreements on determination of reference cell has not been correctly captured in spec

Conclusion:

The reference cell is determined with excluding the effects of UL TA and DL and UL timing differences.
The existence of directional conflict between cells is determined with including the effects of UL TA and DL and UL timing differences.
The error cases resulting from the discrepancy between the methods of reference cell determination and duplex direction conflict determination will not be discussed further in Rel-16. These cases include at least:
    - duplex direction conflict between cells where both cells are reference
    - duplex direction conflict between cells where there is no reference
    - conflict between Dynamic D and Dynamic U

References

[1]	R1-2001589	Discussion on CLI and TBS ambiguity	ZTE

[2]	R1-2001957	Remaining details of CLI measurement and reporting at a UE	LG Electronics

[3]	R1-2002027	Maintenance of aperiodic CSI-RS triggering with beam switching timing of 224 and 336	Intel Corporation

[4]	R1-2002074	Remaining issues of half-duplex operation in CA	CATT

[5]	R1-2002170	On TRS muting for NR coexistence with a narrow band system	MediaTek Inc.

[6]	R1-2002229	On remaining NR TEI issues	Nokia, Nokia Shanghai Bell

[7]	R1-2002282	Remaining issues for Rel-16 maintenance and TEI	Ericsson

[8]	R1-2002355	Considerations on HARQ/CSI enhancements	Apple

[9]	R1-2002679	Discussion on conditions of rate matching pattern overlapping with PDSCH DMRS symbols	Huawei, HiSilicon

[10]	R1-2001724	Discussion on UE TEI feature 14-7	vivo

[11]	R1-2001834	Views on Rel-16 UE features for NR TEIs	MediaTek Inc.

[12]	R1-2002025	UE features for NR TEI	Intel Corporation

[13]	R1-2002280	UE features for TEIs	Ericsson

[14]	R1-2002597	Rel-16 UE features for TEIs	Huawei, HiSilicon