3GPP TSG RAN WG1 #100bis-e			R1-2002895

e-Meeting, April 20th – 30th, 2020

Source:	NTT DOCOMO, INC.

Title:	Summary on Email discussion [100b-e-NR-TEIs-03]

Agenda Item:	7.2.12

Document for: 	Discussion and Decision

- Introduction

This contribution summarizes the NR Rel-16 TEI related and CLI/RIM related discussions and proposals in AI 7.2.12.

100b-e-NR-TEIs-03]: Email discussion/approval on whether or not (and if yes, how) the text below in UE feature 6-1 needs to be updated:

- This feature should be mandatory without capability signaling for at least BWPs which is the same as the set of specified channel BW. RAN4 may discuss other BW requirements.

Note: this is due to the motivation that an operator may need to deploy NR with reduced BW between 5MHz and 10MHz due to necessity to transmit TRS within whole 10MHz BWP, i.e. 52PRBs.

Till 4/24 – Hiroki (DCM)

- Issue for TRS bandwidth

In [6], the following issues regarding TRS bandwidth is identified.

- An operator may not deploy R16 NR with reduced BW between 5MHz and 10MHz due to necessesity to transmit TRS within whole 10MHz BWP, i.e. 52PRB.

- If it is agreed to reduce the TRS RB size minimum requirement for 15 kHz carrier smaller or equal to 52 RB, the exact reduced size of TRS should be defined.

On a carrier smaller than 52 PRB, TRS need to span across whole BWP. In addition, R15 and also R16 UEs are mandated to support only BWP sizes corresponding to nominal channel BW, e.g. 5MHz or 10MHz. An operator may not deploy R16 NR with reduced BW between 5MHz and 10MHz due to necessesity to transmit TRS within whole 10MHz BWP, i.e. 52PRB. Figure 1 Illustration of intended operation To solve this issue, the following options were discussed in RAN1#99 - Alt1: Reduce the TRS RB size minimum requirement for 15kHz carrier smaller or equal to 52 RB - Alt1b: Reduce the TRS RB size minimum requirement for 15kHz carrier smaller or equal to 52 RB and UE is not expected to receive on RBs of a BWP not containing TRS resources - Alt2: Introduce R16 capability 14-x, which indicates the support of 34 PRB BWP size - Alt3: Send LS to RAN5 and CC RAN4 and ask RAN5 to add a demodulation test case for 34 PRB BWP size at least for Rel-16 UEs. - No further RF requirement is added compared with Rel-15 No discussion was allowed in RAN1#100e to resolve this critical deployment case. Unless BWPs with non-nominal size (i.e. other than nominal channel BW) become mandatory for all R16 UEs, we think that Alt1 is the cleanest and simplest option to solve the above issue with the following TP ##### TP for 38.214 ##### 5.1.6.1.1 CSI-RS for tracking <unchanged text omitted > Each CSI-RS resource, defined in Subclause 7.4.1.5.3 of [4, TS 38.211], is configured by the higher layer parameter NZP-CSI-RS-Resource with the following restrictions: - the time-domain locations of the two CSI-RS resources in a slot, or of the four CSI-RS resources in two consecutive slots (which are the same across two consecutive slots), as defined by higher layer parameter CSI-RS-resourceMapping, is given by one of - , , or for frequency range 1 and frequency range 2, - , , , , , or for frequency range 2. - a single port CSI-RS resource with density given by Table 7.4.1.5.3-1 from [4, TS 38.211] and higher layer parameter density configured by CSI-RS-ResourceMapping. - if carrier Ngridsize,μ≤52 and μ=0, the bandwidth of the CSI-RS resource, as given by the higher layer parameter freqBand configured by CSI-RS-ResourceMapping, is the minimum of [32] and NBWP,isize resource blocks, or is equal to NBWP,isize resource blocks, otherwise, the bandwidth of the CSI-RS resource, as given by the higher layer parameter freqBand configured by CSI-RS-ResourceMapping, is the minimum of 52 and NBWP,isize resource blocks, or is equal to NBWP,isize resource blocks. For operation with shared spectrum channel access, freqBand configured by CSI-RS-ResourceMapping, is the minimum of 48 and NBWP,isize resource blocks, or is equal to NBWP,isize resource blocks. <unchanged text omitted >. What remains open is the exact reduced size of TRS. Size of 32 RBs should be able to accommodate the case above, however it would not be sufficient anymore for deployments with available BW smaller than 7MHz. Proposal 1: To resolve critical NR deployment issue, Adopt Alt1 and corresponding enclosed Draft CR1 (including TP) co-sourced by several companies.

In [7], same view as in [6] is provided as below.

In RAN1#99, the issue of blanking the TRS bandwidth was discussed [1]. On carriers smaller than or equal to 52 resource blocks, the tracking reference signal (TRS) spans across whole bandwidth part bandwidth. With Rel-15 UE supporting only bandwidth part bandwidths equal to the nominal channel bandwidths as defined by RAN4, e.g. 5MHz and 10MHz, the TRS bandwidth is the same as the channel bandwidth. This means that an operator cannot deploy Rel-15 NR with a reduced bandwidth between 5MHz and 10MHz without the TRS interfering with transmissions outside the frequency part allocated to NR. In order to avoid this, it was proposed to allow an additional signaling value of 32 RB as a valid bandwidth for carriers with 52 RBs or less. Additionally, in order to have maximum bandwidth available for scheduling of other channels, Alt1 from [1] should be selected. Since the configuration of TRS is using the generic signaling for CSI-IM, no new signaling needs to be defined in Rel-16. No new signaling, also means that it would be possible for Rel-15 UEs to implement the change early allowing the deployment scenario in question to happen as soon as possible. From this perspective, it is also important that the agreement is made already in the RAN1#100bis-e and is not delayed to a later meeting. A joint company draft CR has been submitted in R1-12002229[2]. It is therefore proposed: Introduce a new allowed TRS bandwidths for carriers with less than or equal to 52 RBs as proposed in draft CR in R1-12002229

In [5], different view from [6] and [7] on how to solve the issue is provided as below.

As pointed in a RAN1 LS to RAN2/4 (R1-1909900), RAN1 specifications have the flexibility to support any BWP size from 1 PRB to 275 PRBs, although currently Rel-15 does not support BWP sizes smaller than the RBG size or the PRG size. Therefore, we don’t see any problem in RAN1 specifications to support the deployment shown in Figure 1. However, we understand that companies have no consensus on the support of non-nominal BWP sizes in Rel-15 according to the LS (R1-1909900). Therefore, we proposed the following options for companies to discuss to resolve the issues. - Option #1: All Rel-16 UEs are required to support BWP sizes from 2 to 273 PRBs in FR1 and from 2 to 264 PRBs in FR2 - No new RF requirements for BWP sizes other than RAN4-defined UE channel bandwidths are needed - Option #2: For Rel-16, TRS bandwidth is given by the higher layer parameter freqBand configured by CSI-RS-ResourceMapping when a UE is configured with a carrier bandwidth ≤ 10MHz using 15KHz SCS - UE reports via capability signalling whether it supports or not Our preference is Option #1 because it’s a more clean & natural way to support the deployment in Figure 1. However, Option #2 is acceptable if companies still don’t have consensus on Option #1. In order to avoid potential comebacks for other carrier bandwidth sizes, we slightly modify the proposal in R1-2001121 as Option #2. Note that above options don’t fully resolve the issues for the deployment in Figure 1. Due to the lack of UE RF requirements for 7MHz, network has to guarantee there is no excessive out of band emission between two neighbouring systems; otherwise, it doesn’t work. Proposal #1: Adopt one of the following options. - Option #1: All Rel-16 UEs are required to support BWP sizes from 2 to 273 PRBs in FR1 and from 2 to 264 PRBs in FR2 - No new RF requirements for BWP sizes other than RAN4-defined UE channel bandwidths are needed - Option #2: For Rel-16, TRS bandwidth is given by the higher layer parameter freqBand configured by CSI-RS-ResourceMapping when a UE is configured with a carrier bandwidth ≤ 10MHz using 15KHz SCS - UE reports via capability signalling whether it supports or not Proposal #2: If Option #1 is adopted, adopt one of the following alternatives. - Alt. #1: Capture the following conclusion in RAN1 chairman notes - All Rel-16 UEs are required to support BWP sizes from 2 to 273 PRBs in FR1 and from 2 to 264 PRBs in FR2 - Alt. #2: Adopt the following TP for TS38.213. -----------------Begin of Text Proposal------------------------------- 12 Bandwidth part operation <Omitted> For each DL BWP or UL BWP in a set of DL BWPs or UL BWPs, respectively, the UE is provided the following parameters for the serving cell as defined in [4, TS 38.211] or [6, TS 38.214]: - a SCS by subcarrierSpacing - a cyclic prefix by cyclicPrefix - a common RB and a number of contiguous RBs provided by locationAndBandwidth that indicates an offset and a length as RIV according to [6, TS 38.214], setting , and a value provided by offsetToCarrier for the subcarrierSpacing, where is any integer value between 1 and 275. - an index in the set of DL BWPs or UL BWPs by respective BWP-Id - a set of BWP-common and a set of BWP-dedicated parameters by BWP-DownlinkCommon and BWP-DownlinkDedicated for the DL BWP, or BWP-UplinkCommon and BWP-UplinkDedicated for the UL BWP [12, TS 38.331] For unpaired spectrum operation, a DL BWP from the set of configured DL BWPs with index provided by BWP-Id is linked with an UL BWP from the set of configured UL BWPs with index provided by BWP-Id when the DL BWP index and the UL BWP index are same. For unpaired spectrum operation, a UE does not expect to receive a configuration where the center frequency for a DL BWP is different than the center frequency for an UL BWP when the BWP-Id of the DL BWP is same as the BWP-Id of the UL BWP. <Omitted> -----------------End of Text Proposal------------------------------- Proposal #3: If Option #2 is adopted, adopt the following TP for TS38.214. -----------------Begin of Text Proposal------------------------------- ##### 5.1.6.1.1 CSI-RS for tracking A UE in RRC connected mode is expected to receive the higher layer UE specific configuration of a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info. For a NZP-CSI-RS-ResourceSet configured with the higher layer parameter trs-Info, the UE shall assume the antenna port with the same port index of the configured NZP CSI-RS resources in the NZP-CSI-RS-ResourceSet is the same. - For frequency range 1, the UE may be configured with one or more NZP CSI-RS set(s), where a NZP-CSI-RS-ResourceSet consists of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot. If no two consecutive slots are indicated as downlink slots by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigDedicated, then the UE may be configured with one or more NZP CSI-RS set(s), where a NZP-CSI-RS-ResourceSet consists of two periodic NZP CSI-RS resources in one slot. - For frequency range 2 the UE may be configured with one or more NZP CSI-RS set(s), where a NZP-CSI-RS-ResourceSet consists of two periodic CSI-RS resources in one slot or with a NZP-CSI-RS-ResourceSet of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot. A UE configured with NZP-CSI-RS-ResourceSet(s) configured with higher layer parameter trs-Info may have the CSI-RS resources configured as: - Periodic, with the CSI-RS resources in the NZP-CSI-RS-ResourceSet configured with same periodicity, bandwidth and subcarrier location - Periodic CSI-RS resource in one set and aperiodic CSI-RS resources in a second set, with the aperiodic CSI-RS and periodic CSI-RS resource having the same bandwidth (with same RB location)and the aperiodic CSI-RS being 'QCL-Type-A' and 'QCL-TypeD', where applicable, with the periodic CSI-RS resources. For frequency range 2, the UE does not expect that the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources is smaller than the UE reported ThresholdSched-Offset. The UE shall expect that the periodic CSI-RS resource set and aperiodic CSI-RS resource set are configured with the same number of CSI-RS resources and with the same number of CSI-RS resources in a slot. For the aperiodic CSI-RS resource set if triggered, and if the associated periodic CSI-RS resource set is configured with four periodic CSI-RS resources with two consecutive slots with two periodic CSI-RS resources in each slot, the higher layer parameter aperiodicTriggeringOffset indicates the triggering offset for the first slot for the first two CSI-RS resources in the set. A UE does not expect to be configured with a CSI-ReportConfig that is linked to a CSI-ResourceConfig containing an NZP-CSI-RS-ResourceSet configured with trs-Info and with the CSI-ReportConfig configured with the higher layer parameter timeRestrictionForChannelMeasurements set to 'configured'. A UE does not expect to be configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to other than 'none' for aperiodic NZP CSI-RS resource set configured with trs-Info. A UE does not expect to be configured with a CSI-ReportConfig for periodic NZP CSI-RS resource set configured with trs-Info. A UE does not expect to be configured with a NZP-CSI-RS-ResourceSet configured both with trs-Info and repetition. Each CSI-RS resource, defined in Subclause 7.4.1.5.3 of [4, TS 38.211], is configured by the higher layer parameter NZP-CSI-RS-Resource with the following restrictions: - the time-domain locations of the two CSI-RS resources in a slot, or of the four CSI-RS resources in two consecutive slots (which are the same across two consecutive slots), as defined by higher layer parameter CSI-RS-resourceMapping, is given by one of - , , or for frequency range 1 and frequency range 2, - , , , , , or for frequency range 2. - a single port CSI-RS resource with density given by Table 7.4.1.5.3-1 from [4, TS 38.211] and higher layer parameter density configured by CSI-RS-ResourceMapping. - if carrier Ngridsize,μ≤52 and μ=0, the bandwidth of the CSI-RS resource is given by the higher layer parameter freqBand configured by CSI-RS-ResourceMapping, otherwise, the bandwidth of the CSI-RS resource, as given by the higher layer parameter freqBand configured by CSI-RS-ResourceMapping, is the minimum of 52 and NBWP,isize resource blocks, or is equal to NBWP,isize resource blocks. For operation with shared spectrum channel access, freqBand configured by CSI-RS-ResourceMapping, is the minimum of 48 and NBWP,isize resource blocks, or is equal to NBWP,isize resource blocks. - the UE is not expected to be configured with the periodicity of slots if the bandwidth of CSI-RS resource is larger than 52 resource blocks. - the periodicity and slot offset for periodic NZP CSI-RS resources, as given by the higher layer parameter periodicityAndOffset configured by NZP-CSI-RS-Resource, is one of slots where 10, 20, 40, or 80 and where µ is defined in Subclause 4.3 of [4, TS 38.211]. - same powerControlOffset and powerControlOffsetSS given by NZP-CSI-RS-Resource value across all resources. <Omitted> -----------------End of Text Proposal-------------------------------

During the preparation phase discussion, following views were provided.

| Company | Comment |
| --- | --- |
| Nokia | Our opinion is that TRS bandwidth issue is of high priority and should have an email thread in RAN1#100be. It is clear that R16 UEs are mandated to support only BWPs corresponding to nominal channel BW. Issue is clear. Moreover, this is critical deployment scenario, therefore, question is not “whether to solve” but “how to solve” and for that technical discussion is needed. |
| ZTE | We are okay with the priority in general. From our perspective, 2nd priority issues can be discussed in next meeting instead, depending on the overall email thread situation in next meeting. Regarding the 2nd issue of the first priority issues, it is related to UE capability. Hence it is suggested to be treated under UE feature AI to avoid duplicated discussion. |
| Ericsson | Share the same opinion as Nokia on TRS bandwidth. This topic falls under 3GPP obligation and commitment to in Rel-16 address learnings from early deployments and is more important, and should have higher priority, than any of the issues listed above as high priority. |
| Samsung | On the priority of TRS bandwidth, we share the same view with Nokia and Ericsson. |
| Huawei, HiSilicon | At first, we support Feature lead’s guideline to handle Rel-16 endorsed TEIs first but not to treat or open other TEIs. For the issue of TRS bandwidth, we do not think it need to be discussed. In the current spec, it is clear that the TRS bandwidth (TS38.213) can be min(52, NBWP) or NBWP size. In the case that 7MHz (5MHz~10MHz), the TRS bandwidth can be equal to NBWP size. So, there is no issue at all. Then, for the concern that bandwidth configuration is nominal or non-nominal, I copied the agreement in RAN4 (R4-1910522) as follows: RAN4 agreed that “UE can access the cell if UE supports a channel bandwidth which is equal to or narrower than the channel bandwidth in SIB1 and is equal to or wider than the initial BWP-bandwidth.” BWP-bandwidth can be configured with any number of RBs equal to or narrower than RB size of the supported channel bandwidths. Although the core requirement is applied to any RB configuration, it is noted that only the set of supported channel bandwidths are included in conformance tests due to the test coverage limitation. From the agreement, it is clear that NBWP can be any number of RBs, not restricted as nominal cases. So, we do not see any problems on current specs. |
| CATT | On supported BWP size, RAN1 has reached consensus which is captured in agreed LS to RAN2 (R1-1909900). I copied related part below RAN1 view: – As per the RAN1 UE feature 6-1, all UEs are required to support “at least BWPs which is the same as the set of specified channel BW” without any capability indication. – Specifically, for initial DL BWP, all possible sizes of CORESET 0 (24, 48, 96 PRBs) need to be supported by all UEs. – RAN1 specifications have the flexibility to support any BWP size from 1 PRB to 275 PRBs, although currently Rel-15 does not support BWP sizes smaller than the RBG size or the PRG size. Our understanding is that RAN1 specs support configuration of BWP of any size. Since there is no capability reporting on supported BWP size, all UEs are mandated to support BWPs with non-nominal size. |
| MediaTek | As Qiubin pointed out below, RAN1 did conclude that RAN1 specifications have the flexibility to support any BWP size from 1 PRB to 275 PRBs in the LS to RAN2 (R1-1909900). But I also agree with Karol that RAN1 didn’t reach consensus on whether all Rel-15 UEs shall support any BWP size other than RAN4-defined channel bandwidths (given a SCS) and CORESET0 size. So the fundamental issue is the UE capability to support non-nominal BWP size, rather than RAN1 spec problem. From our perspective, we support to resolve the issue in TEI this meeting though we don’t think RAN1 spec has any problem to support the scenario mentioned in Nokia’s Tdoc. |
| Verizon | We second Ericsson's opinion that we should try to seek a pragmatic solution during RAN1#100b-e. We have a critical situation where we have to keep some legacy narrowband systems around for a long time, due to our past long-term customer commitment. As of today, it looks like we may have to sacrifice a wide NR band just to do that - which is inconceivable, but unfortunately, the reality of the state of industry. We can argue that RAN1 supports all non-nominal BWPs, and we can just ask RAN4 to specify their requirements accordingly. But RAN4 needs to do the bandwidth one by one, delivering the whole set of the requirements for a proposed BW in the same way as it did for nominal BW. Not only does it take time, but if operators don't have consensus on.one or a few bandwidths, their work is just not going to get started. Ideally it would be very desirable to seek a relatively easier-to-manage RAN1 solution that keeps it in the "signaling/control" domain and does not need RAN4 to go the full length to redefine requirements, e.g., rx performance of a 7MHz control channel, and then the set for maybe 8.5MHz and so on. So we are looking for a pragmatic approach, not just from RAN1 point of view but from end user point of view. As a user, we are neutral to all technical solutions but we want one of them that we can implement. Another reason why we can't delay it further is that without a solution, there will be more and more NR UEs coming to the market that will become "legacy" when eventually a solution to this is found. And our experience is that such "legacy" UEs are a headache to the operation, as we are bound by our commitment to customers. Thank you all, and please kindly at least allow this topic to be discussed next week - let's give it a try please. This is indeed an urgent need and we appreciate the support from the vendor community. |
| Qualcomm | We support Alt. 1 or 1b with preference towards 1b. No need to update FG 6-1. |

Based on above, whether or not (and if yes, how) the text below in UE feature 6-1 needs to be updated:

- This feature should be mandatory without capability signaling for at least BWPs which is the same as the set of specified channel BW. RAN4 may discuss other BW requirements.

2.1	Discussion 1

Companies are encouraged to provide views whether or not the text below in UE feature 6-1 needs to be updated. If a company thinks the text needs to be updated, the company is also encouraged to provide how to update the text.

- This feature should be mandatory without capability signaling for at least BWPs which is the same as the set of specified channel BW. RAN4 may discuss other BW requirements.

Alt.1: The text is updated to e.g., “This feature should be mandatory without capability signaling for at leastall BWPs from 1 to 275 PRBs which is the same as the set of specified channel BW. RAN4 may discuss other BW requirements”

Alt.2: The text is not updated

| Company | Comment |
| --- | --- |
| Nokia, NSB | While Alt1 would be the best option, we are not sure how we could change R15 capability captured in TR that has been deemed as not to be maintained by RAN2. Therefore, we are left with Alt2, and we see the following options for R16 (Opt1): introduce capability for support of BWP sized other than specified channel BW (Opt2): introduce TRS BW relaxation for 10MHz carrier with 15kHz sub-carrier spacing, potential subject to capability. We prefer Opt2, as it allows for simpler implementation at both UE and gNB. |
| MediaTek | We support Alt. 1 with one additional sentence highlighted with blue color as follows to clarify that BWP size is mainly UE baseband capability, not bundled with UE RF capability. Alt.1: The text is updated to e.g., “This feature should be mandatory without capability signaling for at leastall BWPs from 1 to 275 PRBs which is the same as the set of specified channel BW. RAN4 may discuss other BW requirements. No RF requirements are required for the BWP sizes” |
| Samsung | We share the similar view with Nokia. Alt 1 would be the best choice but not sure whether it is really feasible solution or not. If Alt 1 is selected, we prefer the modified version suggested by MeidaTek rather than the original version. If Alt 2 is selected, we are okay to either Opt 1 or Opt 2 suggested by Nokia. |
| Huawei, HiSilicon | Although we do not think there is any ambiguity in current spec on this issue, but we are fine with Alt.1 to clarify FG6-1 that all bandwidth for BWP are mandatory without capability signaling. And, obviously, we are not fine for Nokia’s proposal to open other discussion, which is not in the scope of the email discussion. In our understanding, if no update, the non-nominal bandwidth for BWP are also mandatory to be supported by all UEs (since there is no dedicated UE capability reporting for the feature). However, Nokia have the concern on whether mandatory for non-nominal bandwidth for BWP due to unclear statement in the notation of FG6-1. Now, with clarification in Alt.1, it is clear that the bandwidth should be mandatory feature to address concerns. |
| ZTE | We support Alt 1 and we are okay with MediaTek’s update. As indicated by RAN1 chair, the scope of this discussion is to focus on the text in UE FG 6-1, and no RAN1 spec should be impacted. Any other solutions beyond FG 6-1 are not in the scope of this email discussion. Further, we think Alt 1 itself is also a better solution considering future proof. If in the future, another operator has similar requirement but with another bandwidth, we don’t need to revisit the TRS bandwidth again. |
| Ericsson | It is unclear to us how changing the non-normative text in a TR can be a solution to this problem. Hence, we do not see Alt 1 as an alternative worth discussing. Our understanding is that the original text that include “for at least BWPs which is the same as the set of specified channel BW” reflects the market situation and a solution is needed based on the original text. Based on this, Nokia’s option 2 and the corresponding CR is really the only solution that will allow fast deployment of this case. |
| CATT | We support Alt.1. We understand this update as a clarification that all UEs are mandated to support all BWP sizes. The two options proposed by Nokia/NSB for Alt.2 cannot support deploying NR with reduced BW between 5MHz and 10MHz if the channel bandwidth is not defined in RAN4. Assume the available bandwidth is 7 MHz, and gNB signals 10MHz channel bandwidth in SIB. A UE accessing the system may set its Rx filter according to 10MHz channel bandwidth. Even the TRS and other scheduled transmission are confined within 7MHz bandwidth, UE may suffer unwanted interference from those 3MHz bandwidth on which other system may be running, because UE Rx filter is set according to 10MHz channel bandwidth. We suggest proponent companies to bring proposal to RAN4 to define corresponding channel bandwidth. |
| OPPO | We support Alt.1 with MTK’s revision. The update is only to clarify the FG and all the UE should support all possible BWP bandwidth. No other signaling / CR is needed. |
| Verizon | Thanks for all the constructive comments. Like we said before, in an ideal world, the ecosystem jumps on the concept of mandatory support of every possible BWP and we will be happy. But it is far from the case in reality. So realistically, we have to agree with Nokia. We like it from the view of practicality. BTW, we are also unsure if Alt1indeed has the binding power to make the support of every BWP a mandatory requirement. |
| Qualcomm | Qualcomm objects to Alt.1. This issue has been discussed a couple of times before, LSs were sent with the agreements, we do not see a point in revisiting those. For every BWP, there is a different set of DCI sizes, etc. that need to be tested, which was part of the original reasoning and this has not changed. |
| Vodafone | I intend to elaborate the network operator’s perspective on behalf of Vodafone. Many thanks to all companies for all their contributions so far during these difficult circumstances. Firstly, let us agree there is ambiguity in the specs and uncertainty regarding UE vendors’ ability to implement the desired capabilities correctly. In theory, I support the notion of strengthening the wording of the specs to accommodate all BWPs between 1 RB and 275 PRBs. However, if we consider the second version of Alt. 1, there is even disagreement between UE/chipset vendors as to whether or not new supported BWPs should necessitate new RF requirements. Therefore, I’m somewhat uncomfortable rushing to support Alt. 1, as it stands, given that it doesn’t even have consensus amongst UE vendors, let alone the rest of the industry. This does not inspire confidence that UE vendors have sufficient clarity to successfully support all the BWPs that an operator may wish to exploit. I agree that currently, a UE can support at least 10 MHz BW for TRS (other smaller BWPs not precluded) and it is mandatory without capability signalling which by definition means mandatory. There is also a fallback mechanism relying on non-nominal BWPs to re-establish the RRC connection in case the UE is not built to support a BWP < 10MHz, 52 PRBs. However, one must consider, the limitations of this current approach. If we rely on this fallback for every occasion that a UE does not support a lower BW (e.g. 7 MHz) and falls back to 5 MHz, this means we are effectively losing 2 MHz of valuable spectrum for each RRC connection re-establishment attempt. It may be that the NW impact is marginal, but if a solution can be optimised to flexibly allow BWPs < 10 MHz, 52 PRBs that reduces the amount of spectrum non-usage, and we can reach consensus before Rel-16 closes, that would be highly desirable for network operators. It would also be beneficial for the gNB to know which UEs support relaxed TRS BWs or not for the network to operate more smoothly. Therefore, let’s not make a rash decision to preclude Alt. 2 and consider Nokia’s approach laid out in Options 1 and 2. Either the addition of capability signalling and the relaxation of TRS BW both would be desirable from the network deployment perspective. Let’s work together to ensure which version of this approach can lead to the smallest possible specification impact and also minimise TRS estimation loss as much as we can. |

2.2	Discussion 2

Discussion on following alternative solutions

  Alt1:  Reduce the TRS RB size minimum requirement for 15kHz carrier smaller or equal to 52 RB

  Alt1b: Reduce the TRS RB size minimum requirement for 15kHz carrier smaller or equal to 52 RB and UE is not expected to receive on RBs of a BWP not containing TRS resources

  Alt2:  Introduce R16 capability 14-x, which indicates the support of 34 PRB BWP size

  Alt3: Send LS to RAN5 and CC RAN4 andask RAN5 to add a demodulation test case for 34 PRB BWP size at least for Rel-16 UEs.

- No further RF requirement is added compared with Rel-15

  Alt4:The text in FG6-1 is updated to e.g., “This feature should be mandatory without capability signaling forat leastall BWPsfrom 1 to 275 PRBs which is the same as the set of specified channel BW.RAN4 may discuss other BW requirements.NoRF requirements are required for the BWP sizes”

| Company | Comment |
| --- | --- |
| MediaTek | As mentioned before, we don’t see any RAN1 spec problem to support the scenario so we can’t agree with Alt1 & Alt1b. For Alt2, I don’t think it’s a good way to resolve the deployment issue because it can’t cover all irregular cases. If there are deployment requirements other than 7MHz using 15KHz, do we continue to add new UE feature for those cases? For Alt3, without Alt4 clarification, not sure how to ask RAN5 to define the a demodulation test case for 34 PRB BWP size for Rel-16 UEs. So we think Alt4 is the best to go. However, to resolve concerns from other companies on demodulation test cases, we can revise Alt4 as follows. Alt4a:The text in FG6-1 is updated to e.g., “This feature should be mandatory without capability signaling forat leastall BWPsfrom 1 to 275 PRBs which is the same as the set of specified channel BW.RAN4 may discuss other BW requirements.NoRF requirements are required for the BWP sizes. If needed, related demodulation requirements and test cases are defined in RAN4/5” - Send LS to RAN5 and CC RAN4 and ask RAN5 to add a demodulation test case for 34 PRB BWP size at least for Rel-16 UEs. |
| Ericsson | Of alternatives that were already on the table in November, we still prefer alt 1 which is the CR submitted by Nokia to this meeting and cosigned by us. |
| ZTE | We support Alt 4a updated by MediaTek. We believe this is the best solution for a general demand from operators on non-nominal bandwidth, and it's pratical as we can see several UE vendors consider it feasible. |
| CATT | The discussion now is clearly out the scope of this email discussion. Can proponents of Alt1/Alt1b/Alt2 elaborate how can these alternatives support an operator operating on a 7MHz channel without defining 7MHz channel bandwidth in RAN4? What is the signaled channel bandwidth in SIB? If the channel bandwidth is signaled as 10MHz, how does a UE set its Rx filter? |
| Huawei | At first, we object to open the discussion out of the scope from FL’s proposal, otherwise it is meaningless to discuss any of scope. So, please following the scope for the email discussion which is only focus on whether to update the notation of FG6-1. We share the similar view as MTK, ZTE, CATT that there is no problem on current specs, the feature for non-nominal bandwidth BWP is mandatory without capability signaling, since it is really no dedicated signaling at all and the supportive are clarified in RAN1/2/4 already (R1-1909900, R2-1915488, R4-1910522). Then, the concern and argument for spec from some companies during last week discussion is the wording “This feature should be mandatory without capability signaling for at least BWPs which is the same as the set of specified channel BW.” then the direct way to address this issue is to clarify it. But not to revert the feature groups or defined some other solutions. Then, if a UE feature is defined without any capability signaling, is the feature understood as optional feature? Someone may need to explain it, since in our understanding it should be mandatory and it is basic assumption for UE feature discussion. If without the common understanding, I am not sure whether all of the other feature groups (without capability signaling) should be re-open to discuss as well. Some companies said the worry that some UEs may not implement the mandatory features, I want to know which UE vendor did not implement it correctly in Rel-15/16? And whose mistake but ask RAN1 to discuss the UE feature group again? We do have concerns to open the discussion of the UE feature for non-nominal bandwidth BWP, which was already defined as mandatory feature in Rel-15, any change means non-backward compatibility. Frankly speaking, even there is some UEs not follow the specs, i.e., not support 7MHz bandwidth, the impact to NW is still small. NW can configure 7MHz BWP, the most UEs (correctly implemented) works well. Then, the incorrect UE may RRC re-establishment to other nominal bandwidth, which these UEs support. |
| Nokia | For 4A, unfortunately this is evidently something that the group won't be able to agree, because this would revert 6-1 also for R15 UEs, and there is objection on that. To address CATT: On RF, even if UE supports BWP = 7MHz (4A), RF requirements are defined per channel BW, which is 10MHz, operator either owns the full band and operates different RATs there or agrees with neighbor operator. In this respect, 4A is not any better. To address Huawei: FG6-1 is basic feature group for BWP in R15, if it says that UE is mandated to support X, there should be also optional capability X+Y, but it is not there. Not sure how it could have happened. |
| Qualcomm | Given that the alternatives have been renumbered, we would like to reiterate that we object to Alt.4 and Alt. 4a. This issue have been discussed a few times times and the reasoning, the way we see it, has not changed. |

2.3	Discussion 3

Further discussion on following alternative solutions

  Alt1:  Reduce the TRS RB size minimum requirement for 15kHz carrier smaller or equal to 52 RB

- Introduce R16 capability 14-x for above

  Alt2:  Introduce R16 capability 14-x, which indicates the support of 34 PRB BWP size

- Send LS to RAN5 and CC RAN4 and ask RAN5 to add a demodulation test case for 34 PRB BWP size at least for Rel-16 UEs.

  Alt3:  Introduce R16 capability 14-x, which indicates the support of BWP sizes from 1 to 275 PRBs

- No RF requirements are required for the BWP sizes

| Company | Comment |
| --- | --- |
| Qualcomm | we are ok with Alt.1 or Alt.2 but not with Alt.3. |
| MediaTek | To make progress, it’s acceptable to us if Alt. 2 is revised as follows. - Introduce R16 capability 14-x, which indicates the support of 1-275 PRB BWP sizes or only 34 PRB BWP size - Send LS to RAN5 and CC RAN4 on this new FG and ask RAN5 to add a demodulation test case for 34 PRB BWP size at least for Rel-16 UEsRAN5 to decide whether a demodulation test case is needed for the supported BWP sizes or not for Rel-16 UEs. |
| Nokia | could we still check whether the following compromise (proposed by Pei-Kai) would be acceptable? - Introduce R16 capability 14-x, which indicates the support of 1-275 PRB BWP sizes or only 34 PRB BWP size - Send LS to RAN5 and CC RAN4 on this new FG and ask RAN5 to add a demodulation test case for 34 PRB BWP size at least for Rel-16 UEsRAN5 to decide whether a demodulation test case is needed for the supported BWP sizes or not for Rel-16 UEs. Supported: Nokia, NSB, Mediatek |
| Telefonica | As a network operator, we highly appreciate the efforts you have been done to find a solution for this real market situation that we are facing in order to take advantage of legacy bands in 5G deployments. We believe that Alt1 brings more flexibility and is the ideal solution for operators, however, being realistic and as it has been stated before, we also will appreciate more flexibility for considering those alternatives based on optional feature. |
| Qualcomm | We are not supportive of this proposal, since support of 1-275 PRB BWP sizes seems meaningless. But we can accept it as a compromise, as long as the bits are separated and both are agreed to be optional and per band: - Introduce optional per band Rel-16 capability 14-x, which indicates the support of 34 PRB BWP size in addition to the BWP sizes that match the channel BW cases defined in Rel-15 - Send LS to RAN5 and CC RAN4 on this new FG and RAN5 to decide whether a demodulation test case is needed for the supported BWP size or not for Rel-16 UEs. - Introduce optional per band Rel-16 capability 14-y, which indicates the support of 1-275 PRB BWP sizes |
| Huawei | We OBJECT the proposal. Introducing the FG of non-nominal bandwidth for BWPs as an optional features in Rel-16, it will confuse RAN1 (including us) that the feature of non-nominal bandwidth BWPs in Rel-15 is not supported. However, following current spec and also from RAN1/2/4’s clarification, it is clear that the FG of non-nominal bandwidth BWP is mandatory without signaling. The proposed change will make the features difficult to be used in Rel-15 NW and UEs, who have implemented the feature. Actually, with mandatory feature for Non-nominal bandwidth BWPs following current spec, the raised scenarios (7MHz) can be supported well. Please note that these discussion is already out of the scope for the discussion, which is only focus on the discussion on clarification of the notation in FG6-1. We are NOT fine to extend the discussion like this. If look at the discussion under the scope in the summary, most of companies are supportive for Alt.1 from feature lead in the summary (draft_R1-20xxxxx Summary on 100b-e-NR-TEIs-03_v10_QC_Vodafone.docx), we also supportive for it. Alt.1: The text is updated to e.g., “This feature should be mandatory without capability signaling for at leastall BWPs from 1 to 275 PRBs which is the same as the set of specified channel BW. RAN4 may discuss other BW requirements” |
| Samsung | “Although it would be RAN5 who would implement the test case if it is there, wouldn’t it be RAN4 who is supposed to decide whether the test is necessary or not and design it if necessary?” Also, something like “No further RF requirement is added compared with Rel-15” is missing in the latest proposal. Can this be clarified? |
| MediaTek | Clarify the FG scope for the second FG. Add points raised by Samsung. - Introduce optional per band Rel-16 capability 14-x, which indicates the support of 34 [32] PRB BWP size in addition to the BWP sizes that match the channel BW cases defined in Rel-15 - Send LS to RAN5 and CC RAN4 on this new FG and RAN5 to decide whether a demodulation test case is needed for the supported BWP size or not for Rel-16 UEs. - Introduce optional per band Rel-16 capability 14-y, which indicates the support of all other1-275 PRB BWP sizes in addition to [32] PRB BWP size and the BWP sizes that match the channel BW cases defined in Rel-15 - For the above new FGs, no further RF requirement is added compared with Rel-15 - Send LS to RAN2/4/5 on the above new FGs and RAN4/5 to decide whether a demodulation test case is needed for the supported BWP size or not for Rel-16 UEs. Though our top reference is the one proposed by Leiming, we can accept the above proposal as compromise. |
| Moderator (NTT DOCOMO) | Based on the discussion, I see majority companies kindly being flexible to accept following compromised proposal. - Introduce optional per band Rel-16 capability (14-x), which indicates the support of [32] PRB BWP size in addition to the BWP sizes that match the channel BW cases defined in Rel-15. - Introduce optional per band Rel-16 capability (14-y), which indicates the support of all other BWP sizes in addition to [32] PRB BWP size and the BWP sizes that match the channel BW cases defined in Rel-15. - For the above new FGs, no further RF requirement is added compared with Rel-15. - Send LS to RAN4/5 on the above new FGs and RAN4/5 to decide whether a demodulation test case is needed for the supported BWP size or not for Rel-16 UEs. I also see strong objection from at least a few companies. So, my suggested proposal as moderator is to take above as working assumption to give companies to check if there is any significant problem with above proposal, as we usually take such approach to make a progress based on extensive discussion and majority supports. Since it is working assumption if agreed, proposed FGs may not be included in the UE features list to be sent to RAN2 after this e-meeting, and in the LS to RAN4/5, we may need to clarify the situation in RAN1. |
| Huawei | 1. The non-nominal BPWs are already mandatory without capability signaling, each UE should implement the feature. There is no argued issue at all. Even, if some UEs (not sure whether there is) are real incorrect implement the feature, or do not want to implement the issue in Rel-16. It is still small impact to NW, only these UEs without supporting non-nominal BWPs will be impact, i.e., RRC re-establishment is required. There is no impact NW configuration/scheduling, and also no impact the UEs with non-nominal bandwidth BWPs. (Since some companies clarified “reality” and very sure some UEs did not implement in market, these companies also should be clear whose UEs’ did/do not implement feature. Then, it will be very easy to RRC re-establishment for these specific UEs). 2. With companies said the “reality”, I want to know which UE vendor did not implement the mandatory feature in Rel-15, and also do not want to implement in R16? Who now force all the NW vendors and UE vendors follow himself’s implementation with changing the current specs? If such behaviors are allowed in 3GPP, whether any company is also allowed to not implement features which is without UE capability signaling and kick off RAN1 further discussion? 3. I also curious that how to understand the UE feature that without any capability signaling following some companies’ reply. If some companies think it can be optional, and can be not implemented by UE side, then, how to understand other feature groups in R15 and also in LTE systems, which are also without UE capability signaling. Before this discussion, I am sure it is a basic assumption that any UE features defined without capability signaling is a mandatory FG. 4. For companies’ concerns for the feature of non-nominal bandwidth BWPs, one concern is the notation of FG 6-1 is not clear enough; another is RAN4/5 is without test case for other bandwidth of BWPs. We have clarified them already: for the first concern, it is only a notation during the discussion stage, and also marked as “at least”, in the late LS/CR of RAN1/2/4 (R1-1909900, R2-1915488, R4-1910522) already clarified that non-nominal BWPs are supported. But we are fine to clarified it again as Alt.1 shown again. Then, for the second concern, I cannot understand the argument that if no test case designed, then the feature can be not supported?? which agreement in RAN1 said that? Actually, in RAN4’s clarification (R4-1910522), already clarify that non-nominal BWPs are supported, no test case is only due to the limitation resources to test every case. 5. For the changes on UE capability in Hiroki’s proposal, the FG is without signaling, but will be optional with UE capability signaling in R16, it seems change the FG of R15 is not mandatory, even cannot be used. These changes will make the features difficult to be used in Rel-15 NW and UEs, who have implemented the feature already. Please note that it is a non-BC change, which will impact the Rel-15 UEs. It is unfair for the NW vendors and UE vendors who already implemented the features, who also want to use the feature. The current specs have already supported non-nominal BWPs as mandatory without capability signaling, we only need to clarify that (since some companies think it is not clear enough for the notation for FG6-1). From the reply, we can see supporting non-nominal BWPs as a mandatory feature is considered as the best solution from almost all companies. If the concern is for R16 UEs, why cannot accept non-nominal BWPs as mandatory for R16 UEs which are still open to implement I guess? What we should do is only to clarify the feature is mandatories, NW vendors and Operators together require to implement this feature (it is also what I try to do in the email discussion). If the UE is not implemented the feature, actually, only these UEs may be impacted (small punished) with RRC re-establishment as I clarified before. Then, every UE vendor will try to ensure the correct implementation of the mandatory feature, and 7MHz will be perfectly supported from R15 and R16. |
| Vodafone | From Vodafone’s perspective we are fine with the Working Assumption in Hiroki’s summary below. More importantly, at this point, I see the Working Assumption as an essential way to make progress on an issue that is of great concern to at least 3 network operators who participated in this discussion. However, I would like to make two small amendments to the Working Assumption as currently stated. 1. In the second bullet, where it says “indicates the support of all other BWP sizes”, this should be changed to “indicates the support of all BWP sizes from 1 PRB…..275 PRBs” since this is what “all other BWP sizes” actually means. The danger of leaving it as “all other BWP sizes” is that it can be interpreted in English as if there is a specific but unclarified “other”. 2. The final part of the email where it says, “proposed FGs may not be included in the UE features list to be sent to RAN2 after this e-meeting” should be changed to “the proposed FGs should be included in the UE Features list to be sent to RAN2 after this e-meeting, with the clarification that those FGs form part of the above Working Assumption”. This qualification is necessary because otherwise, RAN2 will be prohibited from interpreting this as a true Working Assumption since they won’t be able to start any of the signaling work. |

- Conclusion

There is no consensus and no conclusion at this meeting.

References

[1]	R1-2001589	Discussion on CLI and TBS ambiguity	ZTE

[2]	R1-2001957	Remaining details of CLI measurement and reporting at a UE	LG Electronics

[3]	R1-2002027	Maintenance of aperiodic CSI-RS triggering with beam switching timing of 224 and 336	Intel Corporation

[4]	R1-2002074	Remaining issues of half-duplex operation in CA	CATT

[5]	R1-2002170	On TRS muting for NR coexistence with a narrow band system	MediaTek Inc.

[6]	R1-2002229	On remaining NR TEI issues	Nokia, Nokia Shanghai Bell

[7]	R1-2002282	Remaining issues for Rel-16 maintenance and TEI	Ericsson

[8]	R1-2002355	Considerations on HARQ/CSI enhancements	Apple

[9]	R1-2002679	Discussion on conditions of rate matching pattern overlapping with PDSCH DMRS symbols	Huawei, HiSilicon

[10]	R1-2001724	Discussion on UE TEI feature 14-7	vivo

[11]	R1-2001834	Views on Rel-16 UE features for NR TEIs	MediaTek Inc.

[12]	R1-2002025	UE features for NR TEI	Intel Corporation

[13]	R1-2002280	UE features for TEIs	Ericsson

[14]	R1-2002597	Rel-16 UE features for TEIs	Huawei, HiSilicon