3GPP TSG-RAN WG1 Meeting #100bis-e	R1-2002974

e-Meeting, April 20th – 30th,2020

Source: 	Moderator (ZTE)

Title: 	Text proposal for email discussion on [100b-e-LTE-NB_IoTenh3-Multi-TB-01]

Agenda item:	6.2.2.3

Document for:	Discussion/Decision

Introduction

This contribution provides discussion on the following issue:

[100b-e-LTE-NB_IoTenh3-Multi-TB-01] Issue #1 in R1-2002741 by 4/24 and corresponding TP (if any) by 4/30 – Shupeng (ZTE)

This document also includes:  “reasons for change”, “summary of changes”, “Specs/Sections impacted”, and “consequences if not approved” for each text proposal.

Discussion

Reasons for change:  If an UE has received DCI scheduling multiple SC-MTCH TBs, it is not required to monitor Type2A-CSS during the scheduling gap or the processing gap. Current spec is not clear about this.

Summary of changes:  Specify that a UE is not required to monitor Type2A CSS during the scheduling gap and the processing gap

Specs/Sections impacted: TS 36.213 section 16.6

Consequences if not approved:  Some UE may not be able to monitor Type2-CSS during the scheduling gap or the processing gap but the eNB may have different assumptions

Text proposal:

-------------------------------------------- Unchanged parts omitted -----------------------------------------

An NB-IoT UE is not required to monitor NPDCCH candidates of an NPDCCH search space during an NPUSCH UL gap.

An NB-IoT UE is not required to monitor NPDCCH candidates of a Type2A-NPDCCH common search space during the scheduling gap or the processing gap.

-------------------------------------------- Unchanged parts omitted -----------------------------------------

----------------------------------------------- End of Text Proposal ------------------------------------------

Reference

R1-2001571, ‘Corrections on scheduling of multiple DL/UL transport blocks’, Huawei, HiSilicon

R1-2001853, ‘Remaining issues on scheduling enhancement for NB-IoT’, ZTE

R1-2002186, ‘Scheduling of multiple DL/UL transport blocks’, Qualcomm

R1-2002508, ‘Corrections for Multi-TB scheduling for NB-IoT’, Ericsson

R1-2002646, 'Remaining issues for scheduling of multiple TBs', Nokia, Nokia Shanghai Bell