3GPP TSG RAN WG1 Meeting #100bis	R1-2003115

# e-Meeting, 20th – 30th April 2020

Agenda Item:	6.2.1.4

Source:	Moderator (Ericsson)

Title:	TP for 36.211 on TDD special subframe handling for LTE-MTC resource reservation

Document for:	Discussion, Decision

# Introduction

RAN1#100bis-e discussed TDD special subframe handling for Rel-16 LTE-MTC resource reservation in the email discussion “[100b-e-LTE-eMTC5-Coex-NR-01]”. For more background information, see “Issue #2” in the email discussion summary in R1-2002797. This contribution presents a 36.211 TP based on the outcome of the discussion.

| Reason for change: | RAN1#100bis-e made the following agreement for Rel-16 LTE-MTC which needs to be captured in the specification: - Symbol-level granularity resource reservation is not applied in special subframes. |
| --- | --- |
| Summary of change: | The agreement is implemented in the specification. |
| Consequences if not approved: | Resource reservation may be incorrectly implemented for special subframes. |
| Clauses affected: | 4.2 |

# Text proposal for 36.211

## 4.2	Frame structure type 2

<Unchanged parts are omitted>

For frame structure type 2, the higher-layer parameters for symbol-level resource reservation for BL/CE UEs (symbolBitmap1 and symbolBitmap2) do not apply to special subframes.

<Unchanged parts are omitted>