3GPP TSG RAN WG1 #100bis	R1-200xxxx

e-Meeting, April 20th – 30th, 2020

Source:	moderator (vivo)

Title:	Feature lead summary NRU-CG-01

Agenda Item:	7.2.2.2.4

Document for:	Discussion and Decision

Introduction

Following email thread is assigned:

[100b-e-NR-unlic-NRU-CG-01] Email discussion/approval on following issues by 4/23; if necessary, followed by endorsing the corresponding TPs by 4/28 – Rakesh (Vivo)

- RRC value ranges for RRC parameters

- Correction related to semiPersistentOnPUSCH

- Clarification on offset-r16

- TP on CG-UCI transmission

Discussion on topic#1

Issue 1: RRC value ranges for multiple RRC parameters

## 2.1.1	a) Values for CP extension

The values supported for duration of the CP extension prior to a CG-PUSCH are 0, 11, 20, 29, 38, 47, and 56 us.

or

the set of starting offset can be {16, 25, 34, 43, 52, 61, OS #1} for 15 kHz SCS and the specific starting position and the duration of CP extension for 30 kHz and 60 kHz should be further studied

or

the set of starting offset is Toffset = {16, 25, 34, 43, 52, 61, k=1CTsymb, (l-k)mod 7∙2μμ}, and the CP extension is calculated as

Text=k=1CTsymb,(l-k)mod 7∙2μμ - Toffset

where C=1/2/4 for μ=0/1/2.

Please provide your views/comments in the table below

| Company/organization | comments |
| --- | --- |
| ZTE | We are fine with the proposal |

## 2.1.2	b) Value range of X, D, O

RAN1 to agree on value range of X for the case of ULtoDL-CO-SharingED-Threshold-r16 is not configured, and number of combinations of D, O, CAPC for the case of ULtoDL-CO-SharingED-Threshold-r16 is configured.

- X is integer multiple of 14 or any positive integer.

- The maximum value of  X  and D as well as O is limited to the duration of MCOT

----------------

For the RRC parameters duration-r16 and offset-r16, the maximum configurable value is (the total number of slots in the MCOT corresponding to the maximum p and μ)-1=23 slots.

duration-r16                      INTEGER (1..23),

offset-r16                        INTEGER (1..23),

-------------------

To generate one row in the configured table indicating that the COT sharing information is not available, channelAccessPriority-r16=0 is provided by the corresponding parameter CG-COT-Sharing-r16 and the gNB disregards any values of duration-r16 and offset-r16 provided in that row.

- The value range of the parameter channelAccessPriority-r16 should be changed from        INTEGER (1..4) to INTEGER (0..4).

-------------------

Accounting for the maximum number of combinations of (O, D) per CAPC for µ=2,

The Value range for the parameter cg-COT-SharingList-r16 should be changed as follows:

cg-COT-SharingList-r16                 SEQUENCE (SIZE (1..701)) OF CG-COT-Sharing-r16

If configuring additional rows in the table to indicate possible UL-burst-end points in a slot is supported (See Issue 3):

- cg-COT-SharingList-r16                 SEQUENCE (SEQUENCE (SIZE (1..701)) OF CG-COT-Sharing-r16, SEQUENCE (SIZE (1..11)) OF ulBurstEnd-r16)

-------------------

For the RRC parameter cg-COT-SharingOffset-r16, the value of X should support 4ms offset duration in symbols assuming the largest SCS, i.e., X= 4 * 4 * 14 = 224 symbols

Please provide your views/comments in the table below

| Company/organization | comments |
| --- | --- |
| ZTE | The suggested fffValues (23 and 701) are based on the maximum duration of MCOT as 6ms. But we noticed that in the CAPC table for UL, there are two notes which may introduce even larger duration. So we think the values should be revisited based on 10ms duration of MCOT. NOTE1: For p=3,4, Tulmcot,p=10ms if the higher layer parameter absenceOfAnyOtherTechnology-r14 or absenceOfAnyOtherTechnology-r16 is provided , otherwise, Tulmcot,p=6ms. NOTE 2: When Tulmcot,p=6ms it may be increased to 8ms by inserting one or more gaps. The minimum duration of a gap shall be 100us. The maximum duration before including any such gap shall be 6ms. For the indication of the COT sharing information that is not available, rather than changing the value range of CAPC, we prefer to use a specific row index (e.g. the first row or the last row) in cg-COT-SharingList-r16 to indicate that the COT sharing is not available. Maybe we can list all the alternatives and ask RAN2 to decide. |

Issue 10: Correction related to semiPersistentOnPUSCH (Editorial/clarification)

### 2.10.1 TP1

---------------------------------------------------------------------------------------------------------------------

9	UE procedure for reporting control information

< Unchanged parts are omitted >

If a UE would multiplex HARQ-ACK information in a PUSCH transmission that is configured by a ConfiguredGrantConfig, or in an activated PUSCH transmission configured by semiPersistentOnPUSCH, and includes CG-UCI [5, TS 38.212], the UE multiplexes the HARQ-ACK information in the PUSCH transmission if the UE is provided cg-CG-UCI-Multiplexing; otherwise, the UE does not transmit the PUSCH and multiplexes the HARQ-ACK information in a PUCCH transmission or in another PUSCH transmission.

< Unchanged parts are omitted >

9.3	UCI reporting in physical uplink shared channel

< Unchanged parts are omitted >

For a PUSCH transmission that is configured by a ConfiguredGrantConfig, or for an activated PUSCH transmission that is configured by semiPersistentOnPUSCH, and includes CG-UCI, the UE is provided by betaOffsetCG-UCI-r16 a IoffsetCG-UCI value, from a set of values, with the mapping defined in Table 9.3-1. If the UE multiplexes HARQ-ACK information in the PUSCH transmission, as described in Clause 9.2.5, the UE jointly encodes the HARQ-ACK information and the CG-UCI [5, TS 38.212] and determines a number of resources for multiplexing the combined information in a PUSCH using βoffsetHARQ-ACK.

< Unchanged parts are omitted >

10.5  HARQ-ACK information for PUSCH transmissions

A UE can be configured a number of search space sets to monitor PDCCH for detecting a DCI format 0_1 with CRC scrambled with a CS-RNTI provided by cs-RNTI. The UE determines that the DCI format provides HARQ-ACK information for PUSCH transmissions based on an XYZ field value, as described in [5, TS 38.212], if a PUSCH transmission is configured by ConfiguredGrantConfig or is an activated PUSCH transmission configured by semiPersistentOnPUSCH.

The HARQ-ACK information corresponds to transport blocks in PUSCH transmissions for a number of HARQ processes provided by nrofHARQ-Processes for a serving cell of a PDCCH reception that provides DCI format 1_0 or, if DCI format 1_0 includes a carrier indicator field, for a serving cell indicated by a value of the carrier indicator field.

For a PUSCH transmission configured by ConfiguredGrantConfig or for activated PUSCH transmissions configured by semiPersistentOnPUSCH, HARQ-ACK information for a transport block of a corresponding HARQ process number is valid if a first symbol of the PDCCH reception is after a last symbol of the PUSCH transmission, or of any repetition of the PUSCH transmission, by a number of symbols provided by cg-minDFIDelay-r16.

< Unchanged parts are omitted >

---------------------------------------------------------------------------------------------------------------------

Please provide your views/comments in the table below

| Company/organization | comments |
| --- | --- |
| ZTE | We support the proposal |

Issue 11: Clarification on offset-r16 (Editorial/clarification)

Determining the DL starting slot might be misinterpreted as DL starting in slot n+O+1, e.g. when O=1, which is inconsistent with the use of other COT sharing offsets (n+O)

### 2.11.1 TP1

TP for 37.213 section 4.1.3

------------------------------------------------------------------------------------------------

If the 'COT sharing information' in CG-UCI detected in slot n indicates a row index that corresponds to a CG-COT-Sharing-r16 that provides channel occupancy sharing information, the gNB can share the UE channel occupancy assuming a channel access priority class p=channelAccessPriority-r16, starting from slot n+O where O=offset-r16 slots, for a duration of D=duration-r16 slots where duration-r16, offset-r16 and channelAccessPriority-r16 are higher layer parameters provided by CG-COT-Sharing-r16.

------------------------------------------------------------------------------------------------

Please provide your views/comments in the table below

| Company/organization | comments |
| --- | --- |
| ZTE | We are fine with the proposal |

Issue 13: TP on CG-UCI transmission (Editorial/clarification)

### 2.13.1 TP1

TP for 38.213

9.3	UCI reporting in physical uplink shared channel

- -------------------------------------------------------------------------------------------------

- <unchanged text omitted>

For a PUSCH transmission that is configured by a ConfiguredGrantConfig, or for an activated PUSCH transmission that is configured by semiPersistentOnPUSCH, and includes CG-UCI, the UE includes CG-UCI in the PUSCH if is provided by betaOffsetCG-UCI-r16 a IoffsetCG-UCI value, from a set of values, with the mapping defined in Table 9.3-1. If the UE multiplexes HARQ-ACK information in the PUSCH transmission, as described in Clause 9.2.5, the UE jointly encodes the HARQ-ACK information and the CG-UCI [5, TS 38.212] and determines a number of resources for multiplexing the combined information in a PUSCH using βoffsetHARQ-ACK.

- -------------------------------------------------------------------------------------------------

Please provide your views/comments in the table below

| Company/organization | comments |
| --- | --- |
| ZTE | TP1, TP2 and TP4 are addressing the same issue, i.e. every PUSCH transmission for NR-U CG should include CG-UCI. We slightly prefer TP4. |

### 2.13.2 TP2

- -------------------------------------------------------------------------------------------------

beta_offset indicator field indicates a  value, a  value and a  value from the respective sets of values, with the mapping defined in Table 9.3-3.

For operation with shared spectrum channel access, for CG-PUSCH transmissions configured by ConfiguredGrantConfig, or for activated PUSCH transmissions configured by semiPersistentOnPUSCH, each CG-PUSCH transmission has an associated CG-UCI and the associated CG-UCI is multiplexed on the CG-PUSCH.

For a PUSCH transmission that is configured by a ConfiguredGrantConfig, or for an activated PUSCH transmission that is configured by semiPersistentOnPUSCH, and includes CG-UCI, the UE is provided by betaOffsetCG-UCI-r16 a IoffsetCG-UCI value, from a set of values, with the mapping defined in Table 9.3-4. If the UE multiplexes HARQ-ACK information in the PUSCH transmission, as described in Subclause 9.2.5, the UE jointly encodes the HARQ-ACK information and the CG-UCI [5, TS 38.212] and determines a corresponding IoffsetHARQ-ACK/CG-UCI value as TBD.

- -------------------------------------------------------------------------------------------------

Please provide your views/comments in the table below

| Company/organization | comments |
| --- | --- |
| ZTE | TP1, TP2 and TP4 are addressing the same issue, i.e. every PUSCH transmission for NR-U CG should include CG-UCI. We slightly prefer TP4. |

### 2.13.3 TP3

- -------------------------------------------------------------------------------------------------

<omitted>

For a PUSCH transmission that is configured by a ConfiguredGrantConfig, or for an activated PUSCH transmission that is configured by semiPersistentOnPUSCH, and includes CG-UCI, the UE is provided by betaOffsetCG-UCI-r16 a IoffsetCG-UCI value, from a set of values, with the mapping defined in Table 9.3-1. If the UE multiplexes HARQ-ACK information in the PUSCH transmission, as described in Clause 9.2.5, the UE jointly encodes the HARQ-ACK information and the CG-UCI [5, TS 38.212] and determines a number of resources for multiplexing the combined information in a PUSCH using βoffsetHARQ-ACK where indexes  Ioffset,0HARQ-ACK ,Ioffset,1HARQ-ACK, and Ioffset,2HARQ-ACK are used for up to 2, more than 2 and up to 11, and more than 11 combined information bits, respectively.

<Omitted>

- -------------------------------------------------------------------------------------------------

Please provide your views/comments in the table below

| Company/organization | comments |
| --- | --- |
| ZTE | Fine with the proposal |

### 2.13.4 TP4

----------------------------------------------------------------

< Unchanged parts are omitted >

For a PUSCH transmission that is configured by a ConfiguredGrantConfig, and includes CG-UCI, the UE is provided by betaOffsetCG-UCI-r16 a IoffsetCG-UCI value, from a set of values, with the mapping defined in Table 9.3-1. If the cg-RetransmissionTimer is provided, every PUSCH transmission that is configured by a ConfiguredGrantConfig includes CG-UCI. If the UE multiplexes HARQ-ACK information in the PUSCH transmission, as described in Clause 9.2.5, the UE jointly encodes the HARQ-ACK information and the CG-UCI [5, TS 38.212] and determines a number of resources for multiplexing the combined information in a PUSCH using βoffsetHARQ-ACK.

< Unchanged parts are omitted >

----------------------------------------------------------------

Please provide your views/comments in the table below

| Company/organization | comments |
| --- | --- |
| ZTE | TP1, TP2 and TP4 are addressing the same issue, i.e. every PUSCH transmission for NR-U CG should include CG-UCI. We slightly prefer TP4. |

# References

[1] R1-2002737, Feature lead summary#2 on NRU configured grant enhancement, RAN1#100b-e

[2] R1-2002745, Summary of prep email discussion on NRU-CG, RAN1#100b-e