3GPP TSG RAN WG1 #100bis	   	                      R1-2001940

e-Meeting, April 20th – 30th, 2020

Agenda Item:	7.2.2.2.5

Source: 	Moderator (LG Electronics)

Title: 	Summary on maintenance of wide-band operation for NR-U

Document for:	Discussion and decision

- Introduction

This is the summary document for 7.2.2.2.5 on remaining issues of wide-band operation for NR-U, based on the contributions listed in reference section. The identified topics are categorized into three issues, as follows:

- Issue A: RB set configuration

- Issue B: CORESET and search space configuration

- Issue C: DL reception or UL transmission on resource overlapped with intra-cell guard band

Further details for the above issues and preliminary views on the priority for each sub-issue are provided in Sections 2 to 4. The priority for each specific issue is summarized in Section 5. Text proposals corresponding to sub-issues are collected in Appendix A.

- Issue A: RB set configuration

Issue A1: Determination of FFS values for RRC parameters

One company (LG Electronics [6]) addressed an issue to resolve FFS values for RRC parameters regarding RB set configuration. As captured below from running CR for TS 38.331, it is observed that there are still FFS points for RRC parameters related to intra-cell guard band configuration.

IntraCellGuardBand-r16 ::=         SEQUENCE (SIZE (1..ffsValue)) OF GuardBand-r16 -- FFS upper size 4, assuming 100Mhz cell

GuardBand-r16       ::= SEQUENCE {

startCRB-r16         INTEGER (0..ffsValue), --FFS upper range 275

nrofCRBs-r16         INTEGER (1..ffsValue)

}

For IntraCellGuardBand-r16,

- The number of entries of GuardBand-r16 can be up to 4 considering 100 MHz is the maximum carrier bandwidth for FR1.

For GuardBand-r16,

- The value range of startCRB-r16 is from 0 to 2474 (=2199+275), considering that

  - The value range of offsetToCarrier RRC parameter indicating offset in frequency domain between Point A (lowest subcarrier of common RB 0) and the lowest usable subcarrier on the carrier in number of PRBs (using the subcarrierSpacing defined for this carrier) is defined from 0 to 2199 (=275*8-1), and

  - The value range of carrierBandwidth RRC parameter indicating the width of a carrier in number of PRBs is defined from 1 to 275

- The value range of nrofCRBs-r16 is from 1 to 10, considering the table below captured from RAN4 agreement made in RAN4#93.

| 15 kHz |  | 30 kHz |  | 60 kHz |  |
| --- | --- | --- | --- | --- | --- |
| Usable RBs | Guard RBs | Usable RBs | Guard RBs | Usable RBs | Guard RBs |
| 104, 105 | 6, 7 | 49, 50, 51 | 5, 6, 7 | [22, 23, 24] | [3, 4, 5] |

Therefore, above RRC parameters can be updated as follows:

IntraCellGuardBand-r16 ::=         SEQUENCE (SIZE (1..4)) OF GuardBand-r16 -- FFS upper size 4, assuming 100Mhz cell

GuardBand-r16       ::= SEQUENCE {

startCRB-r16         INTEGER (0..2474), --FFS upper range 275

nrofCRBs-r16         INTEGER (1..10)

}

Comments for priority

| Company | Priority (High or Low) | Comments |
| --- | --- | --- |
| LG Electronics | High | Detailed value ranges can be changed based on discussion. It would be preferable that RAN1 determine value ranges for those RRC parameters and recommend to RAN2 what RAN1 decide. |
| Huawei, HiSilicon | Low | We can wait for more agreement in RAN4. For example, nrofCRB may not be necessary to start from 1. |
| Nokia, NSB | High (but could wait) | Value ranges are typically in RAN1 scope, but agree with HW, RAN2 may introduce, e.g. nrofCRB=0 for zero-GB |
| DOCOMO | Low | Agree with Huawei and Nokia |
| Ericsson | Low | Can wait for further agreements from RAN2, RAN4 |
| Intel | Low | Wait for further inputs from RAN2/4 |
| Qualcomm | Low | Can wait for RAN2/4 |
| Sharp | Low |  |
| Samsung | Low | We can wait RAN2 and RAN4 decisions |
| Apple | Low | Can wait for RAN2/4 further progress. |
| vivo | Low |  |

Issue A2: Corrections based on RAN1 and RAN2 agreements

Several companies suggested corrections on RB set, considering followings:

- RAN1 to reflect RAN2 agreement that intra-cell guard band is configured with starting CRB index and size (instead of starting/ending CRB index)

  - Pointed out by Huawei [1], vivo [2], ZTE [3], OPPO [4], LG Electronics [6], Intel [7], Ericsson [8], Nokia [11]

- Separation of RB set configuration for DL and UL (as per previous RAN1 agreement)

  - Pointed out by Huawei [1], ZTE [3], LG Electronics [6], Ericsson [8], Nokia [11]

- Error in computing RBNRB-set-1end,μ=Ngridstart,μ+Ngridsize,μ-1, instead of RBNRB-set-1end,μ=Ngridstart,μ+Ngridsize,μ

  - Pointed out by ZTE [3], LG Electronics [6], Ericsson [8], Qualcomm [15]

- BWP configured not to include parts of a RB set (as per previous RAN1 agreement)

  - Pointed out by Qualcomm [15]

Comments for priority

| Company | Priority (High or Low) | Comments |
| --- | --- | --- |
| LG Electronics | High | Corrections reflecting all of above 4 points should be adopted. |
| Huawei, HiSilicon | High |  |
| MediaTek | High | Agree with FL |
| Nokia, NSB | High |  |
| DOCOMO | High | Agree with FL |
| Ericsson | High |  |
| Intel | High |  |
| Qualcomm | High | Though priority is high, this is mostly editorial and may not need a lot of discussion |
| Sharp | High |  |
| Samsung | High | Agree with FL |
| Apple | High | Simple editorial issue and easily approved. |
| vivo | High |  |

Issue A3: RB set index within a BWP

One company (Ericsson [8]) addressed an issue that RB set is configured per cell but RB set indexing within a BWP is necessary at least for frequency domain resource allocation in the uplink BWP.

Comments for priority

| Company | Priority (High or Low) | Comments |
| --- | --- | --- |
| LG Electronics | High |  |
| Huawei, HiSilicon | High | It is necessary to complete PUCCH design |
| MediaTek | High |  |
| Nokia, NSB | High | Could be clarified |
| DOCOMO | High |  |
| Ericsson | High | So that frequency domain resource allocation for PUCCH, PUSCH is complete |
| Intel | High |  |
| Qualcomm | High |  |
| Sharp | Low | UL signals/channels agenda item (7.2.2.1.3) may be better to discuss. |
| Samsung | High |  |
| Apple | High | This should be addressed to avoid any mis-alignment across spec and mis-interpretation |
| vivo | High |  |

Issue A4: No guard band configuration

Several companies discussed the issue on no guard band configuration, as follows:

- Huawei [1] proposed to revisit previous RAN1 agreement (intra-cell guard band defined in RAN4 specification by default), and to assume no guard band when intraCellGuardBandDL-r16 or intraCellGuardBandUL-r16 is absent

- OPPO [4] proposed that when active UL BWP bandwidth is more than 20 MHz, the zero guard band should not be configured for uplink when interlace is enabled, since UL BWP configured with no guard band contradicts to interlaced PUCCH configured to be confined within a LBT subband.

- MediaTek [5], Ericsson [8], and Sharp [14] proposed that a single RB set is defined for a serving cell configured without intra-cell guard band.

- Nokia [11] proposed to set both of startCRB-r16 and nrofCRBs-r16 to 0 for a serving cell without intra-cell guard band.

Comments for priority

| Company | Priority (High or Low) | Comments |
| --- | --- | --- |
| LG Electronics | High |  |
| Huawei, HiSilicon | Low | It is going to be discussed in RAN2 meeting next week. We can wait for their agreement. |
| MediaTek | High | How to indicate no intra-cell guard band for a serving cell could be up to RAN2 decision. However, a single RB set still has to be defined for a serving cell configured without intra-cell guard band in RAN1 specification, and it is critical. |
| Nokia, NSB | Low (except for sub-bullet 2 and 3) | for WB carrier, zero-intra-GB is left up to RAN2, 20MHz carrier has no intra-GBs as per default configuration. We could discuss how to handle carrier without GBs in the RAN1 specification (i.e. sub-bullet 2 and 3) |
| DOCOMO | High | Agree with MediaTek and Nokia |
| Ericsson | High (2nd and 3rd sub-bullets) | High: 2nd and 3rd sub-bullets about handling of single RB set for the case of no guard bands should be discussed so frequency domain resource allocation for PUCCH, PUSCH is complete Low: Signaling of no guard bands being decided by RAN2 (1st and 4th sub-bullets) |
| Intel | Low | Wait for discussion in RAN2 |
| Qualcomm | Low | Already ask RAN2 to handle zero guard band signalling design. Don’t think we should have a single RB set to cover multiple 20MHz, or the PUCCH definition may be messed up |
| Sharp | High | It impacts on UL scheduling behaviour when no intra-cell guard bands are configured for a UL carrier. |
| Samsung | High | It seems necessary to define single RB set |
| Apple | Low (Except 3rd sub bullet) | Except 3rd bullet, we can leave it to RAN2 for detailed designs. |
| vivo | Low |  |

Issue A5: Relationship between RB set and LBT channel defined in 37.213

One company (OPPO [4]) proposed to clarify that the configured RB set shall not be partially overlapped with a LBT subband in unlicensed spectrum.

Comments for priority

| Company | Priority (High or Low) | Comments |
| --- | --- | --- |
| LG Electronics | Low |  |
| Huawei, HiSilicon | High | It should be common understanding. It worth clarifying in spec |
| MediaTek | Low | Not necessary to clarify it in the spec. If gNB chooses to do so, following operations in the RB set partially overlapped with a LBT BW will violate regulation. |
| Nokia, NSB | Low | RAN4 requirement on GBs and RB-sets locations will handle this |
| DOCOMO | Low | Agree with MediaTek and Nokia |
| Ericsson | Low |  |
| Intel | Low |  |
| Qualcomm | High | Agree with HW. Good to clarify to avoid unnecessary confusions |
| Sharp | Low |  |
| Samsung | Low |  |
| Apple | Low |  |
| vivo | Low |  |

Issue A6: Reduced nominal channel bandwidth

One company (Nokia [11]) proposed that a reduction of nominal channel bandwidth by 1 PRB can be configured in intraCellGuardBandDL-r16 or intraCellGuardBandUL-r16 with reducedNominalChannelBW-R16, considering that RAN4 is discussing possibility to reduce nominal channel BW from 51 to 50 for 20 MHz carrier to enable better coexistence of wideband operation with CA

Comments for priority

| Company | Priority (High or Low) | Comments |
| --- | --- | --- |
| LG Electronics | Low | Further discussion is needed to identify the issue itself. |
| Huawei, HiSilicon | Low | Need more information from RAN4 |
| MediaTek | Low |  |
| Nokia, NSB | High | RAN4 usable PRBs for 30kHz for e.g. 20MHz are 50, 51 (49 is FFS). So if UE determines RB-set in 20MHz carrier, it does not know whether to assume nominal BW (51RB) or the reduced BW (50RB). For that purpose, we should add parameter to inform UE about reduced channel BW. |
| DOCOMO | Low |  |
| Ericsson | Low |  |
| Intel | Low |  |
| Qualcomm | Low | Not clear why we need to know the reduced nominal channel bandwidth information if we have guard band configuration |
| Sharp | Low | Can be discussed when RAN4 provides information on agreement. |
| Samsung | Low |  |
| Apple | Low |  |
| vivo | Low |  |

Issue A7: Relocation of RB set definition from 38.214 to 38.211

One company (Ericsson [8]) proposed to move text in Section 38.214 Section 7 to 38.211 Section 4.4.6 and to update references in 38.212, 38.213, 38.214, and 38.215 as needed to point to the new section in 38.211.

Comments for priority

| Company | Priority (High or Low) | Comments |
| --- | --- | --- |
| LG Electronics | Low |  |
| Huawei, HiSilicon | Low | It can be determined by Editors |
| MediaTek | Low |  |
| Nokia, NSB | Low | No need. |
| DOCOMO | Low |  |
| Ericsson |  | Spec editors can discuss |
| Intel | Low |  |
| Qualcomm | Low | Editors can handle |
| Sharp | Low |  |
| Samsung | Low |  |
| Apple | Low |  |
| vivo | Low |  |

- Issue B: CORESET and search space configuration

Issue B1: PDCCH candidate and CCE mapping for search space configured with freqMonitorLocations-r16

Several companies (Huawei [1], vivo [2], LG Electronics [6], Panasonic [10], and Sharp [14]) proposed that the number of PDCCH candidates to be monitored (i.e., nrofCandidates or nrofCandidates-SFI) is assigned per each of RB sets for a search space set configured with freqMonitorLocations-r16. It is noted that nrofCandidates or nrofCandidates-SFI is assigned per CORESET associated with a search space set configured without freqMonitorLocations-r16.

In addition, Huawei [1] and Panasonic [10] suggested the enhancement of dropping rule for the case of overbooking of the number of PDCCH candidates and non-overlapping CCEs in a slot, per monitoring location for a search space set configured with freqMonitorLocations-r16, instead of per search space set as in Rel-15.

Comments for priority

| Company | Priority (High or Low) | Comments |
| --- | --- | --- |
| LG Electronics | High | Prioritize to clarify that nrofCandidates or nrofCandidates-SFI is assigned per each of RB sets for a search space set configured with freqMonitorLocations-r16. |
| Huawei, HiSilicon | High | It should be clarified in spec to complete the design of search space with multiple monitoring location in frequency domain |
| MediaTek | High |  |
| Nokia, NSB | High |  |
| DOCOMO | High |  |
| Ericsson | High | Agree with FL on prioritization of the first issue. The 2nd issue is not essential. |
| Intel | High |  |
| Qualcomm | High | Good to clarify the first issue for nrofCandidates. But for nrofCandidates-SFI, if this is per RB-set, given we allow 1 or 2 for the value, we may end up with a lot of decoding for DCI 2_0 already. For overbooking, we don’t believe any changes are needed. |
| Sharp | High | Agree with FL |
| Samsung | High |  |
| Apple | High |  |
| vivo | High |  |

Issue B2: Corrections for TS 38.213

Several companies suggested corrections on CORESET and search space configuration, considering followings:

- Resolution of potential misunderstanding that frequency domain resource of CORESET always starts from RB set 0 even if freqMonitorLocation-r16 in associated search space does not indicate RB set 0

  - Supported by MediaTek [5], LG Electronics [6], Samsung [9], Nokia [11], Spreadtrum [12]

  - Objected by Huawei [1] (stating that no need to change since the current text on frequencyDomainResources in CORESET configuration is clear)

- Terminology/index alignment between specifications

  - vivo [2] (freqMonitorLocations-r16), ZTE [3] (freqMonitorLocations-r16 and rb-offset-r16), LG Electronics [6] (rb-Offset-r16, RB kstart), Nokia [11] (RBk,DLstart,μ)

Comments for priority

| Company | Priority (High or Low) | Comments |
| --- | --- | --- |
| LG Electronics | High | Corrections reflecting all of above 2 points should be adopted. |
| Huawei, HiSilicon | Low | We think the current spec is clear enough. |
| MediaTek | High | Potential misunderstanding should be clarified in the spec. |
| Nokia, NSB | High | For 1) Disagree with Huawei assessment, it should be clarified. 2) is editorial |
| DOCOMO | High |  |
| Ericsson | Low | Agree with Huawei that the current text is clear, and there is not an ambiguity about the CORESET and SearchSpace configurations. We do agree that there are some editorial issue with notation, and these can be fixed, but this is low priority. |
| Intel | Low | Editorial issue |
| Qualcomm | Low | Editorial |
| Sharp | High | Agree with FL |
| Samsung | High | Need to be clarified to avoid ambiguity |
| Apple | Low | Editorial |
| vivo | Low | Editorial issue and could be handled in TP phase. |

- Issue C: DL reception or UL transmission on resource overlapped with intra-cell guard band

Issue C1: PDSCH reception

| Company | Views |
| --- | --- |
| Huawei [1] | Proposal 5: When the intra-cell guard band overlapped with frequency domain resource allocation for a PDSCH is determined as Type-2 intra-cell guard band, the PDSCH should not map on these resource. |
| Nokia [11] | For carrier larger than 40MHz, in DL, gNB may configure dynamic rate-matching where rate-matching resource is configured to overlap with intra-cell GBs. However, as per current specification such operation is precluded, see highlighted part of spec below. ### 5.1.4 PDSCH resource mapping When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 0, the UE shall assume that no SS/PBCH block is transmitted in REs used by the UE for a reception of the PDSCH. When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 1, RA-RNTI, MsgB-RNTI, P-RNTI or TC-RNTI, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst, and if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted. A UE expects a configuration provided by ssb-PositionsInBurst in ServingCellConfigCommon to be same as a configuration provided by ssb-PositionsInBurst in SIB1. When receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCHs with SPS, the REs corresponding to the configured or dynamically indicated resources in Clauses 5.1.4.1, 5.1.4.2 are not available for PDSCH. Furthermore, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources, the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted. A UE is not expected to handle the case where PDSCH DM-RS REs are overlapping, even partially, with any RE(s) not available for PDSCH.. |
| Apple [13] | Proposal 1: - If available RB Sets indictor is provided in a detected DCI format 2_0 and DL Type 0 resource allocation is used for PDSCH resource allocation, - If precoding granularity is equal to the values among {2,4}, the UE shall assume the PDSCH and DMRS are not mapped to any PRG that is partially overlapped with a Type 2 intra-CC guard band. - If precoding granularity is determined as “wideband”, the UE shall assume the PDSCH and DMRS are not mapped to any RB(s) that is partially overlapped with a Type 2 intra-CC guard band. Proposal 2: - If Available RB set Indicator is not provided and DL Type 0 resource allocation is used for PDSCH resource allocation, - If precoding granularity is equal to the values among {2,4}, UE shall assume the PDSCH and DMRS is not mapped to any PRG that is overlapped with any intra-CC guard band. - If precoding granularity is determined as “wideband”, the UE shall assume the PDSCH and DMRS are not mapped to any RB(s) that is partially overlapped with a Type 2 intra-CC guard band. |

Comments for priority

| Company | Priority (High or Low) | Comments |
| --- | --- | --- |
| LG Electronics | Low | It would be preferable to discuss under DL signal/channel agenda item (7.2.2.1.2), if deemed necessary. |
| Huawei, HiSilicon | High | The behaviour should be clarified when intra cell guard is configured. We are fine to discuss it in DL agenda item. |
| MediaTek | Low | Agree with FL |
| Nokia, NSB | High | DL signals are overloaded, would be better to discuss here. Spec change is needed to enable R15 rate-matching to work |
| DOCOMO | High | We are fine to discuss it in either DL agenda item or here, but the issue itself should be resolved |
| Ericsson | Low/High | We are okay to discuss; however, the core issue needs to be clarified. Is it rate matching, or is it issues with Type-0/Type-1 DL resource allocation. |
| Intel | High | Clarification is needed. It can be discussed under wideband agenda or DL agenda |
| Qualcomm | High | For Type 2 intra cell guard band, we don’t see why it cannot be handled by configuring cell-specific rate matching resource set. For [13] proposal 1, need discussion to understand the behaviour. |
| Sharp | Low | Agree with FL |
| Samsung | Low | We think Rel-15 rate matching behaviour can be applied without spec. change. Need some more clarification. In addition, DL reception in intra-cell guard band can be handled by gNB scheduling based on the below conclusion. Further optimization is not necessary at this stage. Conclusion(RAN1#97): A UE can receive a PDSCH scheduled within an LBT bandwidth or over multiple LBT bandwidths as per Rel-15 and current agreements in Rel-16. |
| Apple | High | We shared Nokia view and suggest discussing under WB agenda since clearly DL is much overloaded. |
| vivo | Low |  |

Issue C2: CSI-RS configuration

One company (OPPO [4]) proposed that the configured CSI-RS resource for tracking in unlicensed band shall be restricted within a RB set.

Comments for priority

| Company | Priority (High or Low) | Comments |
| --- | --- | --- |
| LG Electronics | Low | Up to gNB configuration |
| MediaTek | Low | Agree with FL |
| Nokia, NSB | Low | Agree with LG |
| DOCOMO | Low | Agree with FL |
| Ericsson | Low |  |
| Intel | Low |  |
| Qualcomm | Low |  |
| Sharp | Low | Agree with FL |
| Samsung | Low | Agree with FL |
| Apple | Low |  |
| vivo | Low |  |

Issue C3: PUSCH transmission

| Company | Views |
| --- | --- |
| Huawei [1] | Proposal 5: When the intra-cell guard band overlapped with frequency domain resource allocation for a PUSCH is determined as Type-2 intra-cell guard band, the PUSCH should not map on these resource. Observation 3: The intersection of the resource blocks of the indicated interlaces and the indicated set of RB sets and intra-cell guard bands defined in Clause 7 between the indicated RB sets will result zero frequency resource allocation. Proposal 6: For resource allocation type 0, the UE shall determine the resource allocation in frequency domain as an intersection of the resource blocks of the indicated RBG(s) and union of RB set(s) overlapping with the indicated RBG(s) and intra cell guard bands between the adjacent RB sets overlapping with the indicated RBG(s), if any. The corresponding text proposal is in TP#2 in the appendix. |

Comments for priority

| Company | Priority (High or Low) | Comments |
| --- | --- | --- |
| LG Electronics | Low | It would be preferable to discuss under UL signal/channel agenda item (7.2.2.1.3), if deemed necessary. |
| Huawei, HiSilicon | High | The behaviour should be clarified when intra cell guard is configured. We are fine to discuss it in UL agenda item |
| MediaTek | Low | Agree with FL |
| Nokia, NSB | Low | TYPE1 RA is typically used in UL, since allocation must be contiguous, it has 1RB granularity. NO big issue here. |
| DOCOMO | High | We are fine to discuss it in either UL agenda item or here, but the issue itself should be resolved |
| Ericsson | Low | Agree with Nokia |
| Intel | Low | No spec impact is needed |
| Qualcomm | Low | We don’t have RAN1 agreement to support type2 intra-cell guard band in the beginning. |
| Sharp | Low | Our contribution (R1-2002382) under UL signals/channels agenda item (7.2.2.1.3) discusses the issue. |
| Samsung | Low | Agree with Nokia |
| Apple | Low |  |
| vivo | Low |  |

- Summary on the priority for the remaining issues

| Issue | HIGH priority | LOW priority |
| --- | --- | --- |
| Issue A: RB set configuration |  |  |
| Issue A1: Determination of FFS values for RRC parameters | LG Electronics Nokia, NSB (but could be postponed) | Huawei, HiSilicon, DOCOMO, Ericsson, Intel, Qualcomm, Sharp, Samsung, Apple, vivo |
| Issue A2: Corrections based on RAN1 and RAN2 agreements | LG Electronics, Huawei, HiSilicon, MediaTek, Nokia, NSB, DOCOMO, Ericsson, Intel, Qualcomm, Sharp, Samsung, Apple, vivo |  |
| Issue A3: RB set index within a BWP | LG Electronics, Huawei, HiSilicon, MediaTek, Nokia, NSB, DOCOMO, Ericsson, Intel, Qualcomm, Samsung, Apple, vivo | Sharp |
| Issue A4: No guard band configuration | LG Electronics, MediaTek, Nokia, NSB (sub-bullets 2 and 3), DOCOMO, Ericsson (sub-bullets 2,3), Sharp, Samsung | Huawei, HiSilicon, Nokia, NSB (1 and 4), Ericsson (sub-bullets 1,4) , Intel, Qualcomm, Apple (Except 2nd sub bullet), vivo |
| Issue A5: Relationship between RB set and LBT channel defined in 37.213 | Huawei, HiSilicon,Qualcomm | LG Electronics, MediaTek, Nokia, NSB , DOCOMO, Ericsson, Intel, Sharp, Samsung, Apple, vivo |
| Issue A6: Reduced nominal channel bandwidth | Nokia, NSB | LG Electronics, Huawei, HiSilicon, MediaTek, DOCOMO, Ericsson, Intel, Qualcomm, Sharp, Samsung, Apple, vivo |
| Issue A7: Relocation of RB set definition from 38.214 to 38.211 |  | LG Electronics, Huawei, HiSilicon, MediaTek, Nokia, NSB (1 and 4), DOCOMO, Intel, Qualcomm, Sharp, Samsung, Apple, vivo |
| Issue B: CORESET and search space configuration |  |  |
| Issue B1: PDCCH candidate and CCE mapping for search space configured with freqMonitorLocations-r16 | LG Electronics Huawei, HiSilicon, MediaTek, Nokia, NSB, DOCOMO, Ericsson (1st issue), Intel, Qualcomm, Sharp, Samsung, Apple, vivo | Ericsson (2nd issue) |
| Issue B2: Corrections for TS 38.213 | LG Electronics, MediaTek Nokia, NSB, DOCOMO, Sharp, Samsung | Huawei, HiSilicon, Ericsson, Intel, Qualcomm, Apple, vivo |
| Issue C: DL reception or UL transmission on resource overlapped with intra-cell guard band |  |  |
| Issue C1: PDSCH reception | Huawei, HiSilicon Nokia, NSB, DOCOMO, Ericsson (if core issue is clarified), Intel, Qualcomm, Apple | LG Electronics, MediaTek, Ericsson (if core issue not clarified), Sharp, Samsung, vivo |
| Issue C2: CSI-RS configuration |  | LG Electronics, MediaTek Nokia, NSB, DOCOMO, Ericsson, Intel, Qualcomm, Sharp, Samsung, Apple, vivo |
| Issue C3: PUSCH transmission | Huawei, HiSilicon, DOCOMO | LG Electronics, MediaTek Nokia, NSB, Ericsson, Intel, Qualcomm, Sharp, Samsung, Apple, vivo |

- Reference

- R1-2001538	Maintainance on the wideband operation procedures	Huawei, HiSilicon

- R1-2001656	Remaining issues on wideband operation in NR-U	vivo

- R1-2001709	Remaining issues on the wideband operation for NR-U	ZTE, Sanechips

- R1-2001763	Discussion on the remaining issues of wide-band operations	OPPO

- R1-2001905	Remaining issues on wideband operation for NR-U	MediaTek Inc.

- R1-2001939	Remaining issues of wide-band operation for NR-U	LG Electronics

- R1-2001991	Wideband operation for NR-unlicensed	Intel Corporation

- R1-2002035	Wideband operation	Ericsson

- R1-2002121	Wide-band operation for NR-U	Samsung

- R1-2002198	Remaining issues on Rel-16 NR-U wideband operations 	Panasonic

- R1-2002226	Remaining issues on Wideband operation in NR-U	Nokia, Nokia Shanghai Bell

- R1-2002277	Remaining issues in wide-band operation	Spreadtrum Communications

- R1-2002322	Remaining issues of wideband operation	Apple

- R1-2002385	Remaining issues on wide-band operation for NR-U	Sharp

- R1-2002534	TP for Wideband operation for NR-U operation	Qualcomm Incorporated

Appendix A: Text proposals corresponding to sub-issues

Issue A2

From vivo [2],

--------------------------------------------- Start TP1 for Section 7 of TS 38.214 ---------------------------------------- For operation with shared spectrum channel access, when the UE is configured with any of intraCellGuardBandUL-r16 for UL carrier and intraCellGuardBandDL-r16 for DL carrier, the UE is provided with NRB-set-1 intra-cell guard bands on a carrier, each defined by start CRB and end number of CRBs, GB sstart,μ and NGB sμ , respectively. The intra-cell guard bands separate NRB-set RB-sets, each defined by start and end CRB, RB sstart,μ and RB send,μ, respectively. UE determines RB 0start,μ=Ngridstart,μ, RBNRB-set-1end,μ=Ngridstart,μ+Ngridsize,μ , and the remaining start and end CRBs as RB send,μ=GB sstart,μ-1 and RB s+1start,μ=GB send,μ+1GB sstart,μ+NGB sμ. When the UE is not configured with intraCellGuardBandUL-r16, the UE determines intra-cell guard band and corresponding RB-set according to the [default intra-cell GB pattern from 38.101 corresponding to μ and carrier size Ngridsize,μ]. When the UE is not configured with intraCellGuardBandDL-r16, the UE determines intra-cell guard band and corresponding RB-set according to the [default intra-cell GB pattern from 38.101 corresponding to μ and carrier size Ngridsize,μ]. ---------------------------------------------- End TP1 for Section 7 of TS 38.214 ----------------------------------------

From ZTE [3],

---------------------------------------------- < Start of text proposal for 38.214 [1]> -------------------------------------------- 7 UE procedures for transmitting and receiving on a carrier with intra-cell guard bands For operation with shared spectrum channel access, when the UE is configured with any of intraCellGuardBandUL-r16 for UL carrier and intraCellGuardBandDL-r16 for DL carrier, the UE is provided with NRB-set, x-1 intra-cell guard bands on a carrier, each defined by start CRB index and size in number of CRBs, GB s,xstart,μ and GB s,xsizeend,μ , respectively, with the subscript x set to DL and UL for downlink and uplink, respectively. The intra-cell guard bands separate NRB-set,x RB-sets, each defined by start and end CRB, RB s,xstart,μ and RB s,xend,μ, respectively. UE determines RB 0,xstart,μ=Ngrid,xstart,μ, RBNRB-set-1,xend,μ=Ngrid,xstart,μ+Ngrid,xsize,μ-1 , and the remaining start and end CRBs as RB s,xend,μ=GB s,xstart,μ-1 and RB s+1,xstart,μ=GB s,xstartend,μ+GB s,xsize,μ1. When the UE is not configured with intraCellGuardBandUL-r16, the UE determines intra-cell guard band and corresponding RB-set according to the [default intra-cell GB pattern from 38.101 corresponding to μ and carrier size Ngridsize,μ Ngrid,xsize,μNgrid,xsize,μ]. When the UE is not configured with intraCellGuardBandDL-r16, the UE determines intra-cell guard band and corresponding RB-set according to the [default intra-cell GB pattern from 38.101 corresponding to μ and carrier size Ngridsize,μ Ngrid,xsize,μNgrid,xsize,μ]. For a carrier with intra-carrier guard bands, the UE does not expect to receive a BWP configuration by BWP-Downlink or BWP-Uplink partially overlapping with a RB-set. RB-sets within BWP form a set SRB-sets,x of cardinality NRB-set,xBWP. [The configuration of intraCellGuardBandDL-r16 and intraCellGuardBandUL-r16 can indicate to the UE that no intra-cell guard-bands are configured.] ----------------------------------------------------- < End of text proposal> -------------------------------------------------------

From LG Electronics [6],

<TS 38.213> # 12 Bandwidth part operation <Unchanged parts are omitted> If a bandwidth part indicator field is configured in DCI format 0_1 and indicates an active UL BWP with different SCS configuration μ, or with different number NRB-set,ULBWP of RB sets, than a current active UL BWP, the UE determines an uplink frequency domain resource allocation Type 2 based on X' bits and Y' bits that are generated by independently truncating or padding the X MSBs and the Y LSBs [6, TS 38.214] of the frequency domain resource assignment field of DCI format 0_1, where truncation starts from the MSBs of the X bits or the Y bits, zero-padding prepends zeros to the X bits or the Y bits, and - if the indicated active UL BWP has SCS configuration μ=1 and the current active BWP has SCS configuration μ=0, the X MSBs are truncated to X'=X-1 bits, or - if the indicated active UL BWP has SCS configuration μ=0 and the current active BWP has SCS configuration μ=1, the X MSBs are zero-padded to X'=X+1 bits - otherwise, the X MSBs are unchanged and - the Y LSBs are truncated or zero-padded to Y'=log2NN+12 bits where N is a number of RB sets configured for the indicated active UL BWP <TS 38.214> ##### 6.1.2.2.3 Uplink resource allocation type 2 In uplink resource allocation of type 2, the resource block assignment information defined in [5, TS 38.212] indicates to a UE a set of up to M interlace indices, and for DCI 0_1 a set of up to NRB-set,ULBWP contiguous RB sets, where M and interlace indexing are defined in Clause 4.4.4.6 in [4, TS 38.211]. The UE shall determine the resource allocation in frequency domain as an intersection of the resource blocks of the indicated interlaces and the indicated set of RB sets and intra-cell guard bands defined in Clause 7 between the indicated RB sets, if any. For µ=0, the X MSBs of the resource block assignment information indicates to a UE a set of allocated interlace indices m0+l, where the indication consists of a resource indication value (RIV). For 0≤RIV<M(M+1)/2 , l=0,1,⋯L-1 the resource indication value corresponds to the starting interlace index m0 and the number of contiguous interlace indices (). The resource indication value is defined by: if (L-1)≤M/2 then RIV=M(L-1)+m0 else RIV=M(M-L+1)+(M-1-m0) For RIV≥M(M+1)/2 , the resource indication value corresponds to the starting interlace index m0 and the set of values according to Table 6.1.2.2.3-1. Table 6.1.2.2.3-1: m0 and for RIV≥M(M+1)/2. RIV-M(M+1)/2; m0. 0; 0; {0, 5}. 1; 0; {0, 1, 5, 6}. 2; 1; {0, 5}. 3; 1; {0, 1, 2, 3, 5, 6, 7, 8}. 4; 2; {0, 5}. 5; 2; {0, 1, 2, 5, 6, 7}. 6; 3; {0, 5}. 7; 4; {0, 5}. For µ=1, the X MSBs of the resource block assignment information comprise a bitmap indicating the interlaces that are allocated to the scheduled UE. The bitmap is of size M bits with one bitmap bit per interlace such that each interlace is addressable, where M and interlace indexing is defined in Clause 4.4.4.6 in [4, TS 38.211]. The order of interlace bitmap is such that interlace 0 to interlace M-1 are mapped from MSB to LSB of the bitmap. An interlace is allocated to the UE if the corresponding bit value in the bitmap is 1; otherwise the interlace is not allocated to the UE. For both µ=0 and µ=1, the Y=log2NRB-set,ULBWPNRB-set,ULBWP+12LSBs of the resource block assignment information indicate to a UE a set of contiguously allocated RB sets for PUSCH scheduled by DCI 0_1 and Type 1 and Type 2 configured grant. The resource allocation field consists of a resource indication value (RIVRBset). For 0≤RIVRBset<NRB-set,ULBWP(NRB-set,ULBWP+1)/2 , l=0,1,⋯LRBset-1 the resource indication value corresponds to the starting RB set (RBsetSTART) and the number of contiguous RB sets LRBset (LRBset≥1). The resource indication value is defined by; if (LRBset-1)≤NRB-set,ULBWP/2 then RIVRBset=NRB-set,ULBWP(LRBset-1)+RBsetSTART else RIVRBset=NRB-set,ULBWP(NRB-set,ULBWP-LRBset+1)+(NRB-set,ULBWP-1-RBsetSTART) If transform precoding is enabled according to the procedure in Clause 6.1.3, then the UE transmits PUSCH on the lowest-indexed MRBPUSCH PRBs indicated by the frequency domain resource assignment information. MRBPUSCH is the largest integer not greater than the number of RBs indicated by the frequency domain resource assignment information that fulfils the conditions in [4, TS 38.211 Clause 6.3.1.4]. <Unchanged parts are omitted> # 7 UE procedures for transmitting and receiving on a carrier with intra-cell guard bands For operation with shared spectrum channel access, when the UE is configured with any of intraCellGuardBandUL-r16 for UL carrier and intraCellGuardBandDL-r16 for DL carrier, the UE is provided with NRB-set, x-1 intra-cell guard bands on a carrier, each defined by start CRB and size in number of CRBs, GB s,xstart,μ and GB s,xsizeend,μ , respectively, with the subscript x set to DL and UL for downlink and uplink, respectively. The intra-cell guard bands separate NRB-set,x RB-sets, each defined by start and end CRB, RB s,xstart,μ and RB s,xend,μ, respectively. UE determines RB 0,xstart,μ=Ngrid,xstart,μ, RBNRB-set-1,xend,μ=Ngrid,xstart,μ+Ngrid,xsize,μ-1, and the remaining start and end CRBs as RB s,xend,μ=GB s,xstart,μ-1 and RB s+1,xstart,μ=GB s,xstartend,μ+GB s,xsize,μ1. When the UE is not configured with intraCellGuardBandUL-r16, the UE determines intra-cell guard band and corresponding RB-set according to the [default intra-cell GB pattern from 38.101 corresponding to μ and carrier size Ngridsize,μ]. When the UE is not configured with intraCellGuardBandDL-r16, the UE determines intra-cell guard band and corresponding RB-set according to the [default intra-cell GB pattern from 38.101 corresponding to μ and carrier size Ngridsize,μ]. For a carrier with intra-carrier guard bands, the UE does not expect to receive a BWP configuration by BWP-Downlink or BWP-Uplink partially overlapping with a RB-set. RB-sets within BWP form a set SRB-sets,x of cardinality NRB-set,xBWP. [The configuration of intraCellGuardBandDL-r16 and intraCellGuardBandUL-r16 can indicate to the UE that no intra-cell guard-bands are configured.]

From Intel [7],

Text proposal for TS 38.214 # 7 UE procedures for transmitting and receiving on a carrier with intra-cell guard bands For operation with shared spectrum channel access, when the UE is configured with any of intraCellGuardBandUL-r16 for UL carrier and intraCellGuardBandDL-r16 for DL carrier, the UE is provided with NRB-set-1 intra-cell guard bands on a carrier, each defined by start CRB, GB sstart,μ and the number of CRBs GB ssizeend,μ , provided by higher layer parameters startCRB-r16 and nrofCRBs-r16 respectively. The intra-cell guard bands separate NRB-set RB-sets, each defined by start and end CRB, RB sstart,μ and RB send,μ, respectively. UE determines RB 0start,μ=Ngridstart,μ, RBNRB-set-1end,μ=Ngridstart,μ+Ngridsize,μ , and the remaining start and end CRBs as RB send,μ=GB sstart,μ-1 and RB s+1start,μ=GB sendstart,μ+GB ssize,μ+1. When the UE is not configured with intraCellGuardBandUL-r16, the UE determines intra-cell guard band and corresponding RB-set according to the [default intra-cell GB pattern from 38.101 corresponding to μ and carrier size Ngridsize,μ]. When the UE is not configured with intraCellGuardBandDL-r16, the UE determines intra-cell guard band and corresponding RB-set according to the [default intra-cell GB pattern from 38.101 corresponding to μ and carrier size Ngridsize,μ]. For a carrier with intra-carrier guard bands, the UE does not expect to receive a BWP configuration by BWP-Downlink or BWP-Uplink partially overlapping with a RB-set. RB-sets within BWP form a set SRB-sets of cardinality NRB-setBWP. [The configuration of intraCellGuardBandDL-r16 and intraCellGuardBandUL-r16 can indicate to the UE that no intra-cell guard-bands are configured.]

From Ericsson [8]

----------------------------------------- Text Proposal for 38.214, Section 7 ------------------------------------------ *** Unchanged text omitted *** For operation with shared spectrum channel access, when the UE is configured with any of intraCellGuardBandUL-r16 for UL carrier and intraCellGuardBandDL-r16 for DL carrier, the UE is provided with NRB-set,x-1 intra-cell guard bands on a carrier, each defined by start and end CRB, GB s,xstart,μ and GB s,xend,μ , respectively. The subscript x is set to DL and UL for the downlink and uplink, respectively. The intra-cell guard bands separate NRB-set,x RB-sets, each defined by start and end CRB, RB s,xstart,μ and RB s,xend,μ, respectively. UE determines RB 0,xstart,μ=Ngrid,xstart,μ, RBNRB-set,x-1end,μ=Ngrid,xstart,μ+Ngrid,xsize,μ, and the remaining start and end CRBs as RB s,xend,μ=GB s,xstart,μ-1 and RB s+1,xstart,μ=GB s,xend,μ+1. When the UE is not configured with intraCellGuardBandUL-r16, the UE determines intra-cell guard band and corresponding RB-set according to the [default intra-cell GB pattern from 38.101 corresponding to μ and carrier size Ngrid,ULsize,μ]. When the UE is not configured with intraCellGuardBandDL-r16, the UE determines intra-cell guard band and corresponding RB-set according to the [default intra-cell GB pattern from 38.101 corresponding to μ and carrier size Ngrid,DLsize,μ]. For a carrier with intra-carrier guard bands, the UE does not expect to receive a BWP configuration by BWP-Downlink or BWP-Uplink partially overlapping with a RB-set. RB-sets within BWP form a set SRB-sets,x of cardinality NRB-set,xBWP. [The configuration of intraCellGuardBandDL-r16 and intraCellGuardBandUL-r16 can indicate to the UE that no intra-cell guard-bands are configured.] *** Unchanged text omitted *** ---------------------------------------------------- End Text Proposal ----------------------------------------------------- ------------------------------------ Text Proposal for 38.214, Section 6.1.2.2.3 ------------------------------------- *** Unchanged text omitted *** For both µ=0 and µ=1, log2NRBset,ULBWPNRBset,ULBWP+12 bits in the resource block assignment information indicate to a UE a set of contiguously allocated RB sets for PUSCH scheduled by DCI 0_1 and Type 1 and Type 2 configured grant. The resource allocation field consists of a resource indication value (RIVRBset). For 0≤RIVRBset<NRBset,ULBWP(NRBset,ULBWP+1)/2 , l=0,1,⋯LRBset-1 the resource indication value corresponds to the starting RB set (RBsetSTART∈0,1,…,NRBset,ULBWP-1) and the number of contiguous RB sets LRBset (LRBset≥1∈1,2,…,NRBset,ULBWP). The resource indication value is defined by; if (LRBset-1)≤NRBset,ULBWP/2 then RIVRBset=NRBset,ULBWP(LRBset-1)+RBsetSTART else RIVRBset=NRBset,ULBWP(NRBset,ULBWP-LRBset+1)+(NRBset,ULBWP-1-RBsetSTART) *** Unchanged text omitted *** ---------------------------------------------------- End Text Proposal ----------------------------------------------------- ----------------------------------------- Text Proposal for 38.xxx, Section y ------------------------------------------ *** Unchanged text omitted *** For operation with shared spectrum channel access, when the UE is configured with any of intraCellGuardBandUL-r16 for UL carrier and intraCellGuardBandDL-r16 for DL carrier, the UE is provided with NRB-set-1 intra-cell guard bands on a carrier, each defined by start CRB and end a number of CRBs, GB sstart,μ and GB send,μ GB ssize,μ, respectively. The intra-cell guard bands separate NRB-set RB-sets, each defined by start and end CRB, RB sstart,μ and RB send,μ, respectively. UE determines RB 0start,μ=Ngridstart,μ, RBNRB-set-1end,μ=Ngridstart,μ+Ngridsize,μ-1, and the remaining start and end CRBs as RB send,μ=GB sstart,μ-1 and RB s+1start,μ=GB send,μ+1 RB s+1start,μ=GB sstart,μ+GB ssize,μ. When the UE is not configured with intraCellGuardBandUL-r16, the UE determines intra-cell guard band and corresponding RB-set according to the [default intra-cell GB pattern from 38.101 corresponding to μ and carrier size Ngridsize,μ]. When the UE is not configured with intraCellGuardBandDL-r16, the UE determines intra-cell guard band and corresponding RB-set according to the [default intra-cell GB pattern from 38.101 corresponding to μ and carrier size Ngridsize,μ]. For a carrier with intra-carrier guard bands, the UE does not expect to receive a BWP configuration by BWP-Downlink or BWP-Uplink partially overlapping with a RB-set. RB-sets within BWP form a set SRB-sets of cardinality NRB-setBWP. [The configuration of intraCellGuardBandDL-r16 and intraCellGuardBandUL-r16 can indicate to the UE that no intra-cell guard-bands are configured.] *** Unchanged text omitted *** ---------------------------------------------------- End Text Proposal -----------------------------------------------------

From Nokia [11],

TP for 38.214 <unchanged text omitted> - 6.1.2.2.3 Uplink resource allocation type 2 <omitted text> For µ=1, the resource block assignment information includes a bitmap indicating the interlaces that are allocated to the scheduled UE. The bitmap is of size M bits with one bitmap bit per interlace such that each interlace is addressable, where M and interlace indexing is defined in Section 4.4.4.6 in [4, TS 38.211]. The order of interlace bitmap is such that interlace 0 to interlace M-1 are mapped from MSB to LSB of the bitmap. An interlace is allocated to the UE if the corresponding bit value in the bitmap is 1; otherwise the interlace is not allocated to the UE. For both µ=0 and µ=1, log2NRB-set,ULBWPNRB-set,ULBWP+12 bits in the resource block assignment information indicate to a UE a set of contiguously allocated RB sets for PUSCH scheduled by DCI 0_1 and Type 1 and Type 2 configured grant. The resource allocation field consists of a resource indication value (RIVRBset). For 0≤RIVRBset<NRB-set,ULBWP(NRB-set,ULBWP+1)/2 , l=0,1,⋯LRBset-1 the resource indication value corresponds to the starting RB set (RBsetSTART) and the number of contiguous RB sets LRBset (LRBset≥1). The resource indication value is defined by; if (LRBset-1)≤NRB-set,ULBWP/2 then RIVRBset=NRB-set,ULBWP(LRBset-1)+RBsetSTART else RIVRBset=NRB-set,ULBWP(NRB-set,ULBWP-LRBset+1)+(NRB-set,ULBWP-1-RBsetSTART) <unchanged text omitted> # 7 UE procedures for transmitting and receiving on a carrier with intra-cell guard bands For operation with shared spectrum channel access, when the UE is configured with any of intraCellGuardBandUL-r16 for x=UL carrier and intraCellGuardBandDL-r16 for x=DL carrier, the UE is provided with NRB-set,x-1 intra-cell guard bands on a carrier, each defined by start and end CRB, GB s,xstart,μ and GB s,xsize,μ , respectively, with the subscript x set to DL and UL for downlink and uplink, respectively. The intra-cell guard bands separate NRB-set,xgrid RB-sets, each defined by start and end CRB, RB s,xstart,μ and RB s,xend,μ, respectively. UE determines RB 0,xstart,μ=Ngrid,xstart,μ, RBNRB-set-1,xend,μ=Ngrid,xstart,μ+Ngrid,xsize,μNgrid,xsize,μ , and the remaining start and end CRBs as RB s,xend,μ=GB s,xstart,μ-1 and RB s+1,xstart,μ=GB s,xendsize,μ+1. RB-set consists of RBs,xsize,μ resource blocks. When the UE is not configured with intraCellGuardBandUL-r16, the UE determines intra-cell guard band and corresponding RB-set according to the [default intra-cell GB pattern from 38.101 corresponding to μ and carrier size Ngrid,xsize,μ]. When the UE is not configured with intraCellGuardBandDL-r16, the UE determines intra-cell guard band and corresponding RB-set according to the [default intra-cell GB pattern from 38.101 corresponding to μ and carrier size Ngrid,xsize,μ]. For a carrier with intra-carrier guard bands, the UE does not expect to receive a BWP configuration by BWP-Downlink or BWP-Uplink partially overlapping with a RB-set. RB-sets within BWP form a set SRB-sets,x of cardinality NRB-set,xBWP.

From Sharp [14],

Text proposal #2 --------- beginning of text proposal for TS 38.214 # 7 UE procedures for transmitting and receiving on a carrier with intra-cell guard bands For operation with shared spectrum channel access, when the UE is configured with any of intraCellGuardBandUL-r16 for UL carrier and intraCellGuardBandDL-r16 for DL carrier, the UE is provided with NRB-set,x-1 intra-cell guard bands on a carrier, each defined by start and end CRB, GB s,xstart,μ and GB s,xend,μ , respectively. The intra-cell guard bands separate NRB-set,x RB-sets, each defined by start and end CRB, RB s,xstart,μ and RB s,xend,μ, respectively. UE determines RB 0,xstart,μ=Ngrid,xstart,μ, RBNRB-set,x-1end,μ=Ngrid,xstart,μ+Ngrid,xsize,μ , and the remaining start and end CRBs as RB s,xend,μ=GB s,xstart,μ-1 and RB s+1,xstart,μ=GB s,xend,μ+1. When the UE is not configured with intraCellGuardBandUL-r16, the UE determines intra-cell guard band and corresponding RB-set according to the [default intra-cell GB pattern from 38.101 corresponding to μ and carrier size Ngrid,xsize,μ]. When the UE is not configured with intraCellGuardBandDL-r16, the UE determines intra-cell guard band and corresponding RB-set according to the [default intra-cell GB pattern from 38.101 corresponding to μ and carrier size Ngrid,xsize,μ]. For a carrier with intra-carrier guard bands, the UE does not expect to receive a BWP configuration by BWP-Downlink or BWP-Uplink partially overlapping with a RB-set. RB-sets within BWP form a set SRB-sets,x of cardinality NRB-set,xBWP. The subscript x for NRB-set,x, GB s,xstart,μ, GB s,xend,μ, RB s,xstart,μ, RB s,xend,μ and SRB-sets,x are set to DL for downlink, and set to UL for uplink. When there is no risk for confusion, the subscript x may be dropped. [The configuration of intraCellGuardBandDL-r16 and intraCellGuardBandUL-r16 can indicate to the UE that no intra-cell guard-bands are configured.] -------- Unchanged contents are omitted --------- end of text proposal

From Qualcomm [15],

====TP for 38.214 7============== 7 UE procedures for transmitting and receiving on a carrier with intra-cell guard bands For operation with shared spectrum channel access, when the UE is configured with any of intraCellGuardBandUL-r16 for UL carrier and intraCellGuardBandDL-r16 for DL carrier, the UE is provided with NRB-set-1 intra-cell guard bands on a carrier, each defined by start and end CRB, GB sstart,μ and GB send,μ , respectively. The intra-cell guard bands separate NRB-set RB-sets, each defined by start and end CRB, RB sstart,μ and RB send,μ, respectively. UE determines RB 0start,μ=Ngridstart,μ, RBNRB-set-1end,μ=Ngridstart,μ+Ngridsize,μ-1 , and the remaining start and end CRBs as RB send,μ=GB sstart,μ-1 and RB s+1start,μ=GB send,μ+1. When the UE is not configured with intraCellGuardBandUL-r16, the UE determines intra-cell guard band and corresponding RB-set according to the [default intra-cell GB pattern from 38.101 corresponding to μ and carrier size Ngridsize,μ]. When the UE is not configured with intraCellGuardBandDL-r16, the UE determines intra-cell guard band and corresponding RB-set according to the [default intra-cell GB pattern from 38.101 corresponding to μ and carrier size Ngridsize,μ]. or a carrier with intra-carrier guard bands, the UE expects RBBWP,istart, μ=RBs0start,μ , and RBBWP,isize, μ=RBs1end,μ-RBs0start,μ+1 for 0≤s0≤s1≤NRB-set-1.SRB-setsNRB-setBWP [The configuration of intraCellGuardBandDL-r16 and intraCellGuardBandUL-r16 can indicate to the UE that no intra-cell guard-bands are configured.] ========================

Issue A3

From Ericsson [8]

------------------------------------ Text Proposal for 38.214, Section 6.1.2.2.3 ------------------------------------- *** Unchanged text omitted *** For both µ=0 and µ=1, log2NRBset,ULBWPNRBset,ULBWP+12 bits in the resource block assignment information indicate to a UE a set of contiguously allocated RB sets for PUSCH scheduled by DCI 0_1 and Type 1 and Type 2 configured grant. The resource allocation field consists of a resource indication value (RIVRBset). For 0≤RIVRBset<NRBset,ULBWP(NRBset,ULBWP+1)/2 , l=0,1,⋯LRBset-1 the resource indication value corresponds to the starting RB set (RBsetSTART∈0,1,…,NRBset,ULBWP-1) and the number of contiguous RB sets LRBset (LRBset≥1∈1,2,…,NRBset,ULBWP). The resource indication value is defined by; if (LRBset-1)≤NRBset,ULBWP/2 then RIVRBset=NRBset,ULBWP(LRBset-1)+RBsetSTART else RIVRBset=NRBset,ULBWP(NRBset,ULBWP-LRBset+1)+(NRBset,ULBWP-1-RBsetSTART) *** Unchanged text omitted *** ---------------------------------------------------- End Text Proposal ----------------------------------------------------- ----------------------------------------- Text Proposal for 38.214, Section 7 ------------------------------------------ *** Unchanged text omitted *** For operation with shared spectrum channel access, when the UE is configured with any of intraCellGuardBandUL-r16 for UL carrier and intraCellGuardBandDL-r16 for DL carrier, the UE is provided with NRB-set-1 intra-cell guard bands on a carrier, each defined by start and end CRB, GB sstart,μ and GB send,μ , respectively. The intra-cell guard bands separate NRB-set RB-sets, each defined by start and end CRB, RB sstart,μ and RB send,μ, respectively. UE determines RB 0start,μ=Ngridstart,μ, RBNRB-set-1end,μ=Ngridstart,μ+Ngridsize,μ, and the remaining start and end CRBs as RB send,μ=GB sstart,μ-1 and RB s+1start,μ=GB send,μ+1. When the UE is not configured with intraCellGuardBandUL-r16, the UE determines intra-cell guard band and corresponding RB-set according to the [default intra-cell GB pattern from 38.101 corresponding to μ and carrier size Ngridsize,μ]. When the UE is not configured with intraCellGuardBandDL-r16, the UE determines intra-cell guard band and corresponding RB-set according to the [default intra-cell GB pattern from 38.101 corresponding to μ and carrier size Ngridsize,μ]. For a carrier with intra-carrier guard band(s), the UE does not expect to receive a BWP configuration by BWP-Downlink or BWP-Uplink partially overlapping with an RB-set. RB-sets within a BWP form a set SRB-sets of cardinality NRB-setBWP. A BWP contains a set of contiguous RB-sets indexed by r∈0,1,…,NRB-setBWP-1 where NRB-setBWP is the number of RB sets contained in the BWP, and NRB-setBWP,start is the common RB set index of the first RB set in the BWP. The common RB set index u∈0,1,…,NRB-set-1 within a carrier and RB set index within a BWP are related by u=r+ NRBset,xBWP,start. [The configuration of intraCellGuardBandDL-r16 and intraCellGuardBandUL-r16 can indicate to the UE that no intra-cell guard-bands are configured.] *** Unchanged text omitted *** ---------------------------------------------------- End Text Proposal -----------------------------------------------------

Issue A4

From MediaTek [5],

==============================Text Proposal 2 Starts================================ 7. UE procedures for transmitting and receiving on a carrier with intra-cell guard bands For operation with shared spectrum channel access, when the UE is configured with any of intraCellGuardBandUL-r16 for UL carrier and intraCellGuardBandDL-r16 for DL carrier, the UE is provided with NRB-set-1 intra-cell guard bands on a carrier, each defined by start and end CRB, GB sstart,μ and GB send,μ , respectively. The intra-cell guard bands separate NRB-set RB-sets, each defined by start and end CRB, RB sstart,μ and RB send,μ, respectively. UE determines RB 0start,μ=Ngridstart,μ, RBNRB-set-1end,μ=Ngridstart,μ+Ngridsize,μ , and the remaining start and end CRBs as RB send,μ=GB sstart,μ-1 and RB s+1start,μ=GB send,μ+1. When the UE is not configured with intraCellGuardBandUL-r16, the UE determines intra-cell guard band and corresponding RB-set according to the [default intra-cell GB pattern from 38.101 corresponding to μ and carrier size Ngridsize,μ]. When the UE is not configured with intraCellGuardBandDL-r16, the UE determines intra-cell guard band and corresponding RB-set according to the [default intra-cell GB pattern from 38.101 corresponding to μ and carrier size Ngridsize,μ]. For a carrier with intra-carrier guard bands, the UE does not expect to receive a BWP configuration by BWP-Downlink or BWP-Uplink partially overlapping with a RB-set. RB-sets within BWP form a set SRB-sets of cardinality NRB-setBWP. [The configuration of intraCellGuardBandDL-r16 and intraCellGuardBandUL-r16 can indicate to the UE that no intra-cell guard-bands are configured.] If a UE is provided with intraCellGuardBandDL-r16 or intraCellGuardBandUL-r16 to indicate no intra-cell guard-bands are configured for a DL or UL carrier, the UE determines a single RB-set with RB 0start,μ=Ngridstart,μ and RB0end,μ=Ngridstart,μ+Ngridsize,μ for the carrier and assumes NRB-set=1. ============================== Text Proposal 2 Ends==================================

From Ericsson [8],

----------------------------------------- Text Proposal for 38.214, Section 7 ------------------------------------------ *** Unchanged text omitted *** For operation with shared spectrum channel access, when the UE is configured with any of intraCellGuardBandUL-r16 for UL carrier and intraCellGuardBandDL-r16 for DL carrier, the UE is provided with NRB-set-1 intra-cell guard bands on a carrier, each defined by start and end CRB, GB sstart,μ and GB send,μ , respectively. The intra-cell guard bands separate NRB-set RB-sets, each defined by start and end CRB, RB sstart,μ and RB send,μ, respectively. UE determines RB 0start,μ=Ngridstart,μ, RBNRB-set-1end,μ=Ngridstart,μ+Ngridsize,μ-1, and the remaining start and end CRBs as RB send,μ=GB sstart,μ-1 and RB s+1start,μ=GB send,μ+1. When the UE is not configured with intraCellGuardBandUL-r16, the UE determines intra-cell guard band and corresponding RB-set according to the [default intra-cell GB pattern from 38.101 corresponding to μ and carrier size Ngridsize,μ]. When the UE is not configured with intraCellGuardBandDL-r16, the UE determines intra-cell guard band and corresponding RB-set according to the [default intra-cell GB pattern from 38.101 corresponding to μ and carrier size Ngridsize,μ]. For a carrier with intra-carrier guard bands, the UE does not expect to receive a BWP configuration by BWP-Downlink or BWP-Uplink partially overlapping with a RB-set. RB-sets within BWP form a set SRB-sets of cardinality NRB-setBWP. [The configuration of intraCellGuardBandDL-r16 and intraCellGuardBandUL-r16 can indicate to the UE that no intra-cell guard-bands are configured.] For a carrier without intra-cell guard bands, the carrier contains a single RB set with start and end CRB indices given by the above expressions with RB set index 0 and number of RB sets NRB-set=1. *** Unchanged text omitted *** ---------------------------------------------------- End Text Proposal -----------------------------------------------------

From Nokia [11],

TP for 38.214 # 7 UE procedures for transmitting and receiving on a carrier with intra-cell guard bands <unchanged text omitted> [The configuration of intraCellGuardBandDL-r16 and intraCellGuardBandUL-r16 can indicate to the UE that no intra-cell guard-bands are configured.] When intraCellGuardBandDL-r16 or intraCellGuardBandUL-r16 indicates startCRB-r16=0 and nrofCRBs-r16=0, UE assumes there are no intra-cell guard-bands present on the corresponding carrier.

From Sharp [14],

Text proposal #1 --------- beginning of text proposal for TS 38.214 # 7 UE procedures for transmitting and receiving on a carrier with intra-cell guard bands For operation with shared spectrum channel access, when the UE is configured with any of intraCellGuardBandUL-r16 for UL carrier and intraCellGuardBandDL-r16 for DL carrier, the UE is provided with NRB-set-1 intra-cell guard bands on a carrier, each defined by a starting and an ending CRB, GB sstart,μ and GB send,μ , respectively. The UE determines NRB-set RB-sets, each defined by a starting and an ending CRB, RB sstart,μ and RB send,μ, respectively. UE determines RB 0start,μ=Ngridstart,μ, RBNRB-set-1end,μ=Ngridstart,μ+Ngridsize,μ, and the remaining starting and ending CRBs as RB send,μ=GB sstart,μ-1 and RB s+1start,μ=GB send,μ+1. When the UE is not configured with intraCellGuardBandUL-r16, the UE determines intra-cell guard bands and corresponding RB-sets according to the [default intra-cell GB pattern from 38.101 corresponding to μ and carrier size Ngridsize,μ]. When the UE is not configured with intraCellGuardBandDL-r16, the UE determines intra-cell guard bands and corresponding RB-sets according to the [default intra-cell GB pattern from 38.101 corresponding to μ and carrier size Ngridsize,μ]. For a carrier with intra-cell guard bands, the UE does not expect to receive a BWP configuration by BWP-Downlink or BWP-Uplink partially overlapping with an RB-set. RB-sets within a BWP form a set SRB-sets of cardinality NRB-setBWP. -------- Unchanged contents are omitted --------- end of text proposal

Issue A5

From OPPO [4],

----------------------------------------TP1: Start of 38.214 section 7 -------------------------------------- 7 UE procedures for transmitting and receiving on a carrier with intra-cell guard bands For operation with shared spectrum channel access, when the UE is configured with any of intraCellGuardBandUL-r16 for UL carrier and intraCellGuardBandDL-r16 for DL carrier, the UE is provided with NRB-set-1 intra-cell guard bands on a carrier, each defined by start and end CRB, GB sstart,μ and GB send,μ , respectively. The intra-cell guard bands separate NRB-set RB-sets, each defined by start and end CRB, RB sstart,μ and RB send,μ, respectively. UE determines RB 0start,μ=Ngridstart,μ, RBNRB-set-1end,μ=Ngridstart,μ+Ngridsize,μ , and the remaining start and end CRBs as RB send,μ=GB sstart,μ-1 and RB s+1start,μ=GB send,μ+1. When the UE is not configured with intraCellGuardBandUL-r16, the UE determines intra-cell guard band and corresponding RB-set according to the [default intra-cell GB pattern from 38.101 corresponding to μ and carrier size Ngridsize,μ]. When the UE is not configured with intraCellGuardBandDL-r16, the UE determines intra-cell guard band and corresponding RB-set according to the [default intra-cell GB pattern from 38.101 corresponding to μ and carrier size Ngridsize,μ]. For a carrier with intra-carrier guard bands, the UE does not expect to receive a RB set configuration by intraCellGuardBandDL-r16 or intraCellGuardBandUL-r16 partially overlapping with a channel on which a channel access procedure is performed in shared spectrum [TS 37.213]. For a carrier with intra-carrier guard bands, the UE does not expect to receive a BWP configuration by BWP-Downlink or BWP-Uplink partially overlapping with a RB-set. RB-sets within BWP form a set SRB-sets of cardinality NRB-setBWP. [The configuration of intraCellGuardBandDL-r16 and intraCellGuardBandUL-r16 can indicate to the UE that no intra-cell guard-bands are configured.] ----------------------------------------End of 38.214 section 7 --------------------------------------

Issue A6

From Nokia [11],

TP for 38.214 # 7 UE procedures for transmitting and receiving on a carrier with intra-cell guard bands For operation with shared spectrum channel access, when the UE is configured with any of intraCellGuardBandUL-r16 for x=UL carrier and intraCellGuardBandDL-r16 for x=DL carrier, the UE is provided with NRB-set,x-1 intra-cell guard bands on a carrier, each defined by start and end CRB, GB s,xstart,μ and GB s,xsize,μ , respectively, with the subscript x set to DL and UL for downlink and uplink, respectively. The intra-cell guard bands separate NRB-set,xgrid RB-sets, each defined by start and end CRB, RB s,xstart,μ and RB s,xend,μ, respectively. UE determines RB 0,xstart,μ=Ngrid,xstart,μ, RBNRB-set-1,xend,μ=Ngrid,xstart,μ+Ngrid,xsize,μNgrid,xsize,μ , and the remaining start and end CRBs as RB s,xend,μ=GB s,xstart,μ-1 and RB s+1,xstart,μ=GB s,xendsize,μ+1. RB-set consists of RBs,xsize,μ resource blocks. When the UE is not configured with intraCellGuardBandUL-r16, the UE determines intra-cell guard band and corresponding RB-set according to the [default intra-cell GB pattern from 38.101 corresponding to μ and carrier size Ngrid,xsize,μ]. When the UE is not configured with intraCellGuardBandDL-r16, the UE determines intra-cell guard band and corresponding RB-set according to the [default intra-cell GB pattern from 38.101 corresponding to μ and carrier size Ngrid,xsize,μ]. For a carrier with intra-carrier guard bands, the UE does not expect to receive a BWP configuration by BWP-Downlink or BWP-Uplink partially overlapping with a RB-set. RB-sets within BWP form a set SRB-sets,x of cardinality NRB-set,xBWP. [The configuration of intraCellGuardBandDL-r16 and intraCellGuardBandUL-r16 can indicate to the UE that no intra-cell guard-bands are configured.] When intraCellGuardBandDL-r16 or intraCellGuardBandUL-r16 indicates startCRB-r16=0 and nrofCRBs-r16=0, UE assumes there are no intra-cell guard-bands present on the corresponding carrier. When UE is provided with parameter reducedNominalChannelBW-R16 in intraCellGuardBandDL-r16 or intraCellGuardBandUL-r16 , UE sets Ngrid,xsize,μ=Ngrid,xsize,μ-1, otherwise Ngrid,xsize,μ=Ngrid,xsize,μ.

Issue B1

From Huawei [1],

### 10.1 UE procedure for determining physical downlink control channel assignment *** Unchanged text is omitted *** For all search space sets within a slot , denote by a set of CSS sets with cardinality of and by a set of USS sets with cardinality of . The location of USS sets , , in is according to an ascending order of the search space set index. Denote by , , the number of counted PDCCH candidates for monitoring for CSS set and by , , the number of counted PDCCH candidates for monitoring for USS set . Denote by MSussj L, 0≤j<Juss, , the number of counted PDCCH candidates for each monitoring location for USS set Sussj, if freqMonitorLocations-r16 is configured. For the CSS sets, a UE monitors PDCCH candidates requiring a total of non-overlapping CCEs in a slot. The UE allocates PDCCH candidates for monitoring to USS sets for the primary cell having an active DL BWP with SCS configuration in a slot if the UE is not provided PDCCHMonitoringCapabilityConfig for the primary cell or if the UE is provided PDCCHMonitoringCapabilityConfig = R15 PDCCH monitoring capability for all serving cells, or in a span if the UE is provided PDCCHMonitoringCapabilityConfig = R16 PDCCH monitoring capability for the primary cell, according to the following pseudocode. If for the USS sets for scheduling on the primary cell the UE is not provided CORESETPoolIndex for first CORESETs, or is provided value 0 for first CORESETs, and is provided value 1 for second CORESETs, and if minγ∙MPDCCHmax,slot,μ,MPDCCHtotal,slot,μ>minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ or minγ∙CPDCCHmax,slot,μ,CPDCCHtotal,slot,μ>minCPDCCHmax,slot,μ,CPDCCHtotal,slot,μ, the following pseudocode applies only to USS sets associated with the first CORESETs. A UE does not expect to monitor PDCCH in a USS set without allocated PDCCH candidates for monitoring. Denote by the set of non-overlapping CCEs for search space set and by the cardinality of where the non-overlapping CCEs for search space set are determined considering the allocated PDCCH candidates for monitoring for the CSS sets and the allocated PDCCH candidates for monitoring for all search space sets , . Set Set Set while AND allocate PDCCH candidates for monitoring to USS set ; ; ; end while if freqMonitorLocations-r16 is configured, there are KMLmonitoring locations in frequency domain in the search space set Sussj. Denote by VCCESussj the set of non-overlapping CCEs in each monitoring location of search space set Sussj and by CVCCESussj the cardinality of VCCESussj where the non-overlapping CCEs in each monitoring location of search space set Sussj are determined considering the allocated PDCCH candidates for monitoring for the CSS sets and the allocated PDCCH candidates for monitoring for all search space sets Sussj, 0≤k≤j . Set ; Set ; Set ; While LMSussj L≤MPDCCHuss AND CVCCESussj≤CPDCCHuss Set l=0; While l+1∙LMSussj L≤MPDCCHuss AND l+1∙CVCCESussj≤CPDCCHuss AND l<KML k=k+1; end while allocate k∙LMSussj L PDCCH candidates for monitoring to USS set MPDCCHuss=MPDCCHuss-k∙LMSussj L; CPDCCHuss=CPDCCHuss-k∙CVCCESussj; j=j+1; end while

From vivo [2],

------------------------------------------ Start TP2 for Section 10.1 of TS 38.213 ---------------------------------------- For a search space set associated with CORESET and configured with freqMonitorLocations-r16, it comprises of K sub search space set where K is the number of RB sets indicated as ‘1’ by freqMonitorLocations-r16. For each sub search space set, CCE is indexed according to CORESET p configuration in each RB set respectively. For a search space set s associated with CORESET p without configuration of freqMonitorLocations-r16, or a sub search space set as defined above, the CCE indexes for aggregation level corresponding to PDCCH candidate of the search space set in slot for an active DL BWP of a serving cell corresponding to carrier indicator field value are given by where for any CSS, ; for a USS, , , for , for , for , and ; ; is the number of CCEs, numbered from 0 to , in CORESET ; is the carrier indicator field value if the UE is configured with a carrier indicator field by CrossCarrierSchedulingConfig for the serving cell on which PDCCH is monitored; otherwise, including for any CSS, ; , where is the number of PDCCH candidates the UE is configured to monitor for aggregation level of a search space set for a serving cell corresponding to ; for any CSS, ; for a USS, is the maximum of over all configured values for a CCE aggregation level of search space set ; the RNTI value used for is the C-RNTI. ------------------------------------------ End TP2 for Section 10.1 of TS 38.213 ----------------------------------------

From Panasonic [10],

… If the monitoringSymbolsWithinSlot indicates to a UE to monitor PDCCH in a subset of up to three consecutive symbols that are same in every slot where the UE monitors PDCCH for all search space sets, the UE does not expect to be configured with a PDCCH SCS other than 15 kHz if the subset includes at least one symbol after the third symbol. If freqMonitorLocations-r16 indicates to a UE to monitor the search space set over multiple RB sets, the number of PDCCH candidates is assigned to each RB set. A UE does not expect to be provided a first symbol and a number of consecutive symbols for a CORESET that results to a PDCCH candidate mapping to symbols of different slots. …

From Sharp [14],

Text proposal #3 --------- beginning of text proposal for TS 38.213 ## 10.1 UE procedure for determining physical downlink control channel assignment -------- Unchanged contents are omitted For a search space set associated with CORESET , - if the UE is not provided with freqMonitorLocations-r16, the CCE indexes for aggregation level corresponding to PDCCH candidate of the search space set in slot for an active DL BWP of a serving cell corresponding to carrier indicator field value - if the UE is provided with freqMonitorLocations-r16 and if the (k+1)’th bit in the bitmap in freqMonitorLocations-r16 is ‘1’, in RB set k, the CCE indexes for aggregation level corresponding to PDCCH candidate of the search space set in slot for an active DL BWP of a serving cell corresponding to carrier indicator field value are given by where for any CSS, ; for a USS, , , for , for , for , and ; ; is the number of CCEs, numbered from 0 to , in CORESET ; is the carrier indicator field value if the UE is configured with a carrier indicator field by CrossCarrierSchedulingConfig for the serving cell on which PDCCH is monitored; otherwise, including for any CSS, ; , where is the number of PDCCH candidates the UE is configured to monitor for aggregation level of a search space set for a serving cell corresponding to ; for any CSS, ; for a USS, is the maximum of over all configured values for a CCE aggregation level of search space set ; the RNTI value used for is the C-RNTI. -------- Unchanged contents are omitted --------- end of text proposal

Issue B2

From vivo [2],

------------------------------------------ Start TP3 for Section 10.1 of TS 38.213 ---------------------------------------- For each CORESET in a DL BWP of a serving cell, a respective frequencyDomainResources provides a bitmap. - if a CORESET is not associated with any search space set configured with freqMonitorLocations-r16, the bits of the bitmap have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP bandwidth of NRBBWP PRBs with starting common RB position NBWPstart, where the first common RB of the first group of 6 PRBs has common RB index 6⋅NBWPstart/6 if rb-offset is not provided, or the first common RB of the first group of 6 PRBs has common RB index NBWPstart+NRBoffset where NRBoffset is provided by rb-offset. - if a CORESET is associated with at least one search space set configured with freqMonitorLocations-r16, the first NRBG,set0size bits of the bitmap have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP bandwidth of NRBBWP PRBs with starting common RB position NBWPstart, where the first common RB of the first group of 6 PRBs has common RB index NBWPstart+NRBoffset. NRBG,set0size=(NRB,set0size-NRBoffset)/6, NRB,set0size is a number of available PRBs in the RB set 0 for the DL BWP, and NRBoffset is provided by rb-offset or NRBoffset=0 if rb-offset is not provided. … … - a bitmap by freqMonitorLocations-r16, if provided, to indicate one or more RB sets for the search space set s, where the MSB k in the bitmap corresponds to RB set k-1 in the DL BWP. For RB set k indicated in the bitmap, the first PRB of the frequency domain monitoring location confined within the RB set is given by NRB, set kstart+NRBoffset, where NRB,set kstart is the index of first PRB of the RB set k, and NRBoffset is provided by rb-offset or NRBoffset=0 if rb-offset is not provided. The frequency domain resource allocation pattern for each monitoring location is determined based on the first NRBG, set 0size bits in frequencyDomainResources provided by the associated CORESET configuration. ------------------------------------------ End TP3 for Section 10.1 of TS 38.213 ----------------------------------------

From ZTE [3],

---------------------------------------------- < Start of text proposal for 38.213 [2]> -------------------------------------------- 10.1 UE procedure for determining physical downlink control channel assignment < Unchanged parts are omitted > For each CORESET in a DL BWP of a serving cell, a respective frequencyDomainResources provides a bitmap. - if a CORESET is not associated with any search space set configured with freqMonitorLocations-r16, the bits of the bitmap have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP bandwidth of NRBBWP PRBs with starting common RB position NBWPstart, where the first common RB of the first group of 6 PRBs has common RB index 6⋅NBWPstart/6 if rb-offset-r16 is not provided, or the first common RB of the first group of 6 PRBs has common RB index NBWPstart+NRBoffset where NRBoffset is provided by rb-offset-r16. - if a CORESET is associated with at least one search space set configured with freqMonitorLocations-r16, the first NRBG,set0size bits of the bitmap have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP bandwidth of NRBBWP PRBs with starting common RB position NBWPstart, where the first common RB of the first group of 6 PRBs has common RB index NBWPstart+NRBoffset. NRBG,set0size=(NRB,set0size-NRBoffset)/6, NRB,set0size is a number of available PRBs in the RB set 0 for the DL BWP, and NRBoffset is provided by rb-offset-r16 or NRBoffset=0 if rb-offset-r16 is not provided. < Unchanged parts are omitted > - a bitmap by freqMonitorLocations-r16, if provided, to indicate one or more RB sets for the search space set s, where the MSB k in the bitmap corresponds to RB set k-1 in the DL BWP. For RB set k indicated in the bitmap, the first PRB of the frequency domain monitoring location confined within the RB set is given by NRB, set kstart+NRBoffset, where NRB,set kstart is the index of first PRB of the RB set k, and NRBoffset is provided by rb-offset-r16 or NRBoffset=0 if rb-offset-r16 is not provided. The frequency domain resource allocation pattern for each monitoring location is determined based on the first NRBG, set 0size bits in frequencyDomainResources provided by the associated CORESET configuration. < Unchanged parts are omitted > -------------------------------------------------- < End of text proposal> ----------------------------------------------------

From MediaTek [5],

=============================Text Proposal 1 Starts================================== 10.1 UE procedure for determining physical downlink control channel assignment *** Unchanged text is omitted *** For each CORESET in a DL BWP of a serving cell, a respective frequencyDomainResources provides a bitmap. - if a CORESET is not associated with any search space set configured with freqMonitorLocation-r16, the bits of the bitmap have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP bandwidth of NRBBWP PRBs with starting common RB position NBWPstart, where the first common RB of the first group of 6 PRBs has common RB index 6⋅NBWPstart/6 if rb-offset is not provided, or the first common RB of the first group of 6 PRBs has common RB index NBWPstart+NRBoffset where NRBoffset is provided by rb-offset. - if a CORESET is associated with at least one search space set configured with freqMonitorLocation-r16, the first NRBG,set0size bits of the bitmap have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP bandwidth of NRBBWP PRBs with starting common RB position NBWPstart, where the first common RB of the first group of 6 PRBs has common RB index NBWPstart+NRBoffset. NRBG,set0size=(NRB,set0size-NRBoffset)/6, NRB,set0size is a number of available PRBs in the RB set 0 for the DL BWP, and NRBoffset is provided by rb-offset or NRBoffset=0 if rb-offset is not provided. *** Unchanged text is omitted *** For each DL BWP configured to a UE in a serving cell, the UE is provided by higher layers with search space sets where, for each search space set from the search space sets, the UE is provided the following by SearchSpace: - a search space set index , , by searchSpaceId *** Unchanged text is omitted *** - a bitmap by freqMonitorLocation-r16, if provided, to indicate one or more RB sets for the search space set s, where the MSB k in the bitmap corresponds to RB set k-1 in the DL BWP. For RB set k indicated in the bitmap, the first PRB of the frequency domain monitoring location confined within the RB set is given by NRB, set kstart+NRBoffset, where NRB,set kstart is the index of first PRB of the RB set k, and NRBoffset is provided by rb-offset or NRBoffset=0 if rb-offset is not provided. The frequency domain resource allocation pattern for each monitoring location is determined based on the first NRBG, set 0size bits in frequencyDomainResources provided by the associated CORESET configuration, whereNRBG,set0size=(NRB,set0size-NRBoffset)/6, NRB,set0size is a number of available PRBs in the RB set 0 for the DL BWP. If search-space s is not provided with freqMonitorLocation-r16 and the associated CORESET is associated with at least one another search space set configured with freqMonitorLocation-r16, the first NRBG, set 0size bits in frequencyDomainResources provided by the associated CORESET configuration have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs in the RB set 0 for the DL BWP, in ascending order of the PRB index in the DL BWP bandwidth of NRBBWP PRBs with starting common RB index NBWPstart, where the first common RB of the first group of 6 PRBs has common RB index NBWPstart+NRBoffset. *** Unchanged text is omitted *** ============================== Text Proposal 1 Ends==================================

From LG Electronics [6],

## 10.1 UE procedure for determining physical downlink control channel assignment <Unchanged parts are omitted> For each CORESET in a DL BWP of a serving cell, a respective frequencyDomainResources provides a bitmap. - if a CORESET is not associated with any search space set configured with freqMonitorLocation-r16, the bits of the bitmap have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP bandwidth of NRBBWP PRBs with starting common RB position NBWPstart, where the first common RB of the first group of 6 PRBs has common RB index 6⋅NBWPstart/6 if rb-Offset-r16 is not provided, or the first common RB of the first group of 6 PRBs has common RB index NBWPstart+NRBoffset where NRBoffset is provided by rb-Offset-r16. - if a CORESET is associated with at least one search space set configured with freqMonitorLocation-r16, the first NRBG,set0size bits of the bitmap have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in each RB set kNRBBWP with starting common RB position RB kstartNBWPstart as described in [6, TS 38.214], where the first common RB of the first group of 6 PRBs has common RB index RB kstartNBWPstart+NRBoffset and RB set k is indicated in freqMonitoringLocation-r16 if provided with associated search space, otherwise, k=0. NRBG,set0size=(NRB,set0size-NRBoffset)/6, NRB,set0size is a number of available PRBs in the RB set 0 for the DL BWP, and NRBoffset is provided by rb-Offset-r16 or NRBoffset=0 if rb-Offset-r16 is not provided. <Unchanged parts are omitted> - a bitmap by freqMonitorLocation-r16, if provided, to indicate one or more RB sets for the search space set s, where the MSB k in the bitmap corresponds to RB set k-1 in the DL BWP. For RB set k indicated in the bitmap, the first PRB of the frequency domain monitoring location confined within the RB set is given by RB kstartNRB, set kstart+NRBoffset, where RB kstartNRB,set kstart is the index of first PRB of the RB set k as described in [6, TS 38.214], and NRBoffset is provided by rb-Offset-r16 or NRBoffset=0 if rb-Offset-r16 is not provided. The frequency domain resource allocation pattern for each monitoring location is determined based on the first NRBG, set 0size bits in frequencyDomainResources provided by the associated CORESET configuration.

From Samsung [9],

============================== Start of TP for TS 38.213 ============================== 10.1 UE procedure for determining physical downlink control channel assignment ============================= Unchanged Texts Omitted ============================== For each CORESET in a DL BWP of a serving cell, a respective frequencyDomainResources provides a bitmap. - if a CORESET is not associated with any search space set configured with freqMonitorLocation-r16, the bits of the bitmap have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP bandwidth of NRBBWP PRBs with starting common RB position NBWPstart, where the first common RB of the first group of 6 PRBs has common RB index 6⋅NBWPstart/6 if rb-offset is not provided, or the first common RB of the first group of 6 PRBs has common RB index NBWPstart+NRBoffset where NRBoffset is provided by rb-offset. - if a CORESET is associated with at least one search space set configured with freqMonitorLocation-r16, the first NRBG,set0size bits of the bitmap have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in each RB-set s indicated in freqMonitorLocation-r16 with starting common RB sstart,μ [7, TS 38.214], where the first common RB of the first group of 6 PRBs has common RB index RB sstart,μ+NRBoffset. NRBG,set0size=(NRB,set0size-NRBoffset)/6, NRB,set0size is a number of available PRBs in the RB set 0 for the DL BWP, and NRBoffset is provided by rb-offset or NRBoffset=0 if rb-offset is not provided. ============================= Unchanged Texts Omitted =============================== ============================== End of TP for TS 38.213 ==============================

From Nokia [11],

## TP to TS38.213 ## 10.1 UE procedure for determining physical downlink control channel assignment <unchanged text omitted > For each CORESET in a DL BWP of a serving cell, a respective frequencyDomainResources provides a bitmap. - if a CORESET is not associated with any search space set configured with freqMonitorLocation-r16, the bits of the bitmap have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP bandwidth of NRBBWP PRBs with starting common RB position NBWPstart, where the first common RB of the first group of 6 PRBs has common RB index 6⋅NBWPstart/6 if rb-offset is not provided, or the first common RB of the first group of 6 PRBs has common RB index NBWPstart+NRBoffset where NRBoffset is provided by rb-offset. - if a CORESET is associated with at least one search space set configured with freqMonitorLocation-r16, the first NRBG,set0size bits of the bitmap have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index each RB-set x indicated in freqMonitorLocation-r16 with starting common RBs,DLstart, μ the DL BWP bandwidth of NRBBWP PRBs with starting common RB position NBWPstart, where the first common RB of the first group of 6 PRBs has common RB index RBs,DLstart, μNBWPstart+NRBoffset. NRBG,set0size=NRB,set0sizeRB0,DLsize,μ-NRBoffset/6, RBk,DLsize,μNRB,ssize is a number of available PRBs in the RB set k=0 for the DL BWP, and NRBoffset is provided by rb-offset or NRBoffset=0 if rb-offset is not provided. For a CORESET other than a CORESET with index 0, - if a UE has not been provided a configuration of TCI state(s) by tci-StatesPDCCH-ToAddList and tci-StatesPDCCH-ToReleaseList for the CORESET, or has been provided initial configuration of more than one TCI states for the CORESET by tci-StatesPDCCH-ToAddList and tci-StatesPDCCH-ToReleaseList but has not received a MAC CE activation command for one of the TCI states as described in [11, TS 38.321], the UE assumes that the DM-RS antenna port associated with PDCCH receptions is quasi co-located with the SS/PBCH block the UE identified during the initial access procedure; - if a UE has been provided a configuration of more than one TCI states by tci-StatesPDCCH-ToAddList and tci-StatesPDCCH-ToReleaseList for the CORESET as part of Reconfiguration with sync procedure as described in [12, TS 38.331] but has not received a MAC CE activation command for one of the TCI states as described in [11, TS 38.321], the UE assumes that the DM-RS antenna port associated with PDCCH receptions is quasi co-located with the SS/PBCH block or the CSI-RS resource the UE identified during the random access procedure initiated by the Reconfiguration with sync procedure as described in [12, TS 38.331]. For a CORESET with index 0, the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET is quasi co-located with - the one or more DL RS configured by a TCI state, where the TCI state is indicated by a MAC CE activation command for the CORESET, if any, or - a SS/PBCH block the UE identified during a most recent random access procedure not initiated by a PDCCH order that triggers a contention-free random access procedure, if no MAC CE activation command indicating a TCI state for the CORESET is received after the most recent random access procedure. For a CORESET other than a CORESET with index 0, if a UE is provided a single TCI state for a CORESET, or if the UE receives a MAC CE activation command for one of the provided TCI states for a CORESET, the UE assumes that the DM-RS antenna port associated with PDCCH receptions in the CORESET is quasi co-located with the one or more DL RS configured by the TCI state. For a CORESET with index 0, the UE expects that QCL-TypeD of a CSI-RS in a TCI state indicated by a MAC CE activation command for the CORESET is provided by a SS/PBCH block - if the UE receives a MAC CE activation command for one of the TCI states, the UE applies the activation command in the first slot that is after slot where is the slot where the UE would transmit a PUCCH with HARQ-ACK information for the PDSCH providing the activation command and is the SCS configuration for the PUCCH. The active BWP is defined as the active BWP in the slot when the activation command is applied. For each DL BWP configured to a UE in a serving cell, the UE is provided by higher layers with search space sets where, for each search space set from the search space sets, the UE is provided the following by SearchSpace: - a search space set index , , by searchSpaceId - an association between the search space set and a CORESET by controlResourceSetId - a PDCCH monitoring periodicity of slots and a PDCCH monitoring offset of slots, by monitoringSlotPeriodicityAndOffset - a PDCCH monitoring pattern within a slot, indicating first symbol(s) of the CORESET within a slot for PDCCH monitoring, by monitoringSymbolsWithinSlot - a duration of slots indicating a number of slots that the search space set exists by duration - a number of PDCCH candidates per CCE aggregation level by aggregationLevel1, aggregationLevel2, aggregationLevel4, aggregationLevel8, and aggregationLevel16, for CCE aggregation level 1, CCE aggregation level 2, CCE aggregation level 4, CCE aggregation level 8, and CCE aggregation level 16, respectively - an indication that search space set is either a CSS set or a USS set by searchSpaceType - if search space set is a CSS set - an indication by dci-Format0-0-AndFormat1-0 to monitor PDCCH candidates for DCI format 0_0 and DCI format 1_0 - an indication by dci-Format2-0 to monitor one or two PDCCH candidates for DCI format 2_0 and a corresponding CCE aggregation level - an indication by dci-Format2-1 to monitor PDCCH candidates for DCI format 2_1 - an indication by dci-Format2-2 to monitor PDCCH candidates for DCI format 2_2 - an indication by dci-Format2-3 to monitor PDCCH candidates for DCI format 2_3 - an indication by dci-Format2-4 to monitor PDCCH candidates for DCI format 2_4 - an indication by dci-Format2-6 to monitor PDCCH candidates for DCI format 2_6 - if search space set is a USS set, an indication by dci-Formats to monitor PDCCH candidates either for DCI format 0_0 and DCI format 1_0, or for DCI format 0_1 and DCI format 1_1, or an indication by dci-Formats-Rel16 to monitor PDCCH candidates for DCI format 0_0 and DCI format 1_0, or for DCI format 0_1 and DCI format 1_1, or for DCI format 0_2 and DCI format 1_2, or, if a UE indicates a corresponding capability, for DCI format 0_1, DCI format 1_1, DCI format 0_2, and DCI format 1_2, or for DCI format 3_0, or for DCI format 3_1, or for DCI format 3_0 and DCI format 3_1 - a bitmap by freqMonitorLocation-r16, if provided, to indicate one or more RB sets for the search space set s, where the MSB k in the bitmap corresponds to RB set k-1 in the DL BWP. For RB set k indicated in the bitmap, the first PRB of the frequency domain monitoring location confined within the RB set is given by RBk,DLstart,μNRB, set kstart+NRBoffset, where RB k,DLstart,μNRB,set kstart is the index of first PRB of the RB set k, and NRBoffset is provided by rb-offset or NRBoffset=0 if rb-offset is not provided. The frequency domain resource allocation pattern for each monitoring location is determined based on the first NRBG, set 0size bits in frequencyDomainResources provided by the associated CORESET configuration. <unchanged text omitted>

From Spreadtrum [12]

-------------------------------------- Text Proposal for 38.213, Section 10.1 ----------------------------------------- *** Unchanged text omitted *** For each CORESET in a DL BWP of a serving cell, a respective frequencyDomainResources provides a bitmap. - if a CORESET is not associated with any search space set configured with freqMonitorLocation-r16, the bits of the bitmap have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP bandwidth of NRBBWP PRBs with starting common RB position NBWPstart, where the first common RB of the first group of 6 PRBs has common RB index 6⋅NBWPstart/6 if rb-offset is not provided, or the first common RB of the first group of 6 PRBs has common RB index NBWPstart+NRBoffset where NRBoffset is provided by rb-offset. - if a CORESET is associated with at least one search space set configured with freqMonitorLocation-r16, the first NRBG,set0size bits of the bitmap have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP bandwidth of NRBBWP PRBs with starting common RB position NBWPstart, where the first common RB of the first group of 6 PRBs has common RB index NBWPstart+NRBoffset. NRBG,set0size=(NRB,set0size-NRBoffset)/6, NRB,set0size is a number of available PRBs in the RB set 0 for the DL BWP, and NRBoffset is provided by rb-offset or NRBoffset=0 if rb-offset is not provided. - If a CORESET is associated with at least one search space set configured with freqMonitorLocation-r16, and the search space set configured with freqMonitorLocation-r16 indicates that CORESET is not in RB-set 0, the UE ignores the configuration of freqMonitorLocations-r16 in RB-set 0 and follows the configuration of frequencyDomainResources in RB-set 0. *** Unchanged text omitted *** ----------------------------------------------------- End Text Proposal ------------------------------------------------------

Issue C1

From Nokia [11],

### 5.1.4 PDSCH resource mapping When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 0, the UE shall assume that no SS/PBCH block is transmitted in REs used by the UE for a reception of the PDSCH. When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 1, RA-RNTI, MsgB-RNTI, P-RNTI or TC-RNTI, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst, and if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted. A UE expects a configuration provided by ssb-PositionsInBurst in ServingCellConfigCommon to be same as a configuration provided by ssb-PositionsInBurst in SIB1. When receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCHs with SPS, the REs corresponding to the configured or dynamically indicated resources in Clauses 5.1.4.1, 5.1.4.2 are not available for PDSCH. Furthermore, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources, the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted. A UE is not expected to handle the case where PDSCH DM-RS REs are overlapping, even partially, with any RE(s) not available for PDSCH, unless PDSCH DM-RS REs collide with resource of RateMatchPattern configured to be fully overlapping with an intra-cell GB defined in sub-clause 7.

From Apple [13],

>>> Text Proposal TP1 for 38.214, Section 7>>> 7 UE procedures for transmitting and receiving on a carrier with intra-cell guard bands *** Unchanged text omitted *** For a carrier with intra-carrier guard bands, the UE does not expect to receive a BWP configuration by BWP-Downlink or BWP-Uplink partially overlapping with a RB-set. RB-sets within BWP form a set SRB-sets of cardinality NRB-setBWP. For a carrier with intra-carrier guard bands, the UE assumes that any PRG that are fully/partially overlapped with an intra-carrier guard band is not used for downlink resource allocation type 0 if the availability of corresponding RB-set of the intra-carrier guard band is not provided to UE by DCI format 2_0 and precoding granularity is determined as one of the values among {2,4}. For a carrier with intra-carrier guard bands, the UE assumes that any PRB that are fully/partially overlapped with an intra-carrier guard band is not used for downlink resource allocation type 0 if the availability of corresponding RB-set of the intra-carrier guard band is not provided to UE by DCI format 2_0 and precoding granularity is determined as “wideband”. For a carrier with intra-carrier guard bands, the UE assumes that any PRG that are fully/partially overlapped with an intra-carrier guard band is not used for downlink resource allocation type 0 if precoding granularity is determined as one of the values among {2,4} and the availability of corresponding RB-set is provided to UE by Available RB set Indicator field in a DCI format 2_0 which indicates at least one of two corresponding RB-sets of the intra-carrier guard band is not available for PDSCH . For a carrier with intra-carrier guard bands, the UE assumes that any PRB that are fully/partially overlapped with an intra-carrier guard band is not used for downlink resource allocation type 0 if precoding granularity is determined as “wideband” and the availability of corresponding RB-set is provided to UE by Available RB set Indicator field in a DCI format 2_0 which indicates at least one of two corresponding RB-sets of the intra-carrier guard band is not available for PDSCH . >>> End Text Proposal >>>

Issue C2

From OPPO [4],

----------------------------------------TP2: Start of 38.214 section 5.1.6.1.1 -------------------------------------- 5.1.6.1.1 CSI-RS for tracking <Unchanged text is omitted> Each CSI-RS resource, defined in Subclause 7.4.1.5.3 of [4, TS 38.211], is configured by the higher layer parameter NZP-CSI-RS-Resource with the following restrictions: - the time-domain locations of the two CSI-RS resources in a slot, or of the four CSI-RS resources in two consecutive slots (which are the same across two consecutive slots), as defined by higher layer parameter CSI-RS-resourceMapping, is given by one of - , , or for frequency range 1 and frequency range 2, - , , , , , or for frequency range 2. - a single port CSI-RS resource with density given by Table 7.4.1.5.3-1 from [4, TS 38.211] and higher layer parameter density configured by CSI-RS-ResourceMapping. - the bandwidth of the CSI-RS resource, as given by the higher layer parameter freqBand configured by CSI-RS-ResourceMapping, is the minimum of 52 and NBWP,isize resource blocks, or is equal to NBWP,isize resource blocks. For operation with shared spectrum channel access, freqBand configured by CSI-RS-ResourceMapping, is the minimum of 48 and NBWP,isize resource blocks within a RB set, or is equal to NBWP,isize resource blocks. <Unchanged text is omitted> ----------------------------------------End of 38.214 section 5.1.6.1.1 --------------------------------------

Issue C3

From Huawei [1],

## TP#2: TS38.214v16.0.0 ##### 6.1.2.2.1 Uplink resource allocation type 0 In uplink resource allocation of type 0, the resource block assignment information includes a bitmap indicating the Resource Block Groups (RBGs) that are allocated to the scheduled UE where a RBG is a set of consecutive virtual resource blocks defined by higher layer parameter rbg-Size configured in pusch-Config and the size of the bandwidth part as defined in Table 6.1.2.2.1-1. For operation with shared spectrum channel access mechansim, The UE shall determine the resource allocation in frequency domain as an intersection of the resource blocks of the indicated RBGs and the union of RB sets overlapping with the indicated RBGs and intra-cell guard bands defined in Clause 7 between the adjacent RB sets overlapping with the indicated RBGs, if any. *** Unchanged text is omitted *** ##### 6.1.2.2.3 Uplink resource allocation type 2 In uplink resource allocation of type 2, the resource block assignment information defined in [5, TS 38.212] indicates to a UE a set of up to M interlace indices, and a set of up to N RB sets, where M and interlace indexing are defined in Clause 4.4.4.6 in [4, TS 38.211]. The UE shall determine the resource allocation in frequency domain as an intersection of the resource blocks of the indicated interlaces and the union of indicated set of RB sets and intra-cell guard bands defined in Clause 7 between the indicated RB sets, if any. *** Unchanged text is omitted ***

Appendix B: Previous agreements

Agreement: (RAN1#92bis)

- At least for band where absence of Wi-Fi cannot be guaranteed (e.g. by regulation), LBT can be performed in units of 20 MHz.

  - FFS: details on how to perform LBT for as single carrier with bandwidth greater than 20 MHz, i.e., integer multiples of 20 MHz.

Agreement: (RAN1#94bis)

- NR-U should support that a serving cell can be configured with bandwidth larger than 20 MHz.

  - For DL operation, the following options for BWP-based operation within a carrier with bandwidth larger than 20 MHz can be considered.

    - Option 1a: Multiple BWPs configured, multiple BWPs activated, transmission of PDSCH on one or more BWPs

    - Option 1b: Multiple BWPs configured, multiple BWPs activated, transmission of PDSCH on single BWP

    - Option 2: Multiple BWPs can be configured, single BWP activated, gNB transmits PDSCH on a single BWP if CCA is successful at gNB for the whole BWP

    - Option 3: Multiple BWPs can be configured, single BWP activated, gNB transmits PDSCH on parts or whole of single BWP where CCA is successful at gNB

  - Note: CCA is declared to be successful or not in multiples of 20 MHz.

  - FFS for UL operation including some or all of above options can be applied

- Note: Capture the following in TR only after further discussion for down-selecting from the options in RAN1#95.

Agreement: (RAN1#95)

- For wideband operation for both DL and UL,

  - Bandwidth larger than 20 MHz can be supported with multiple serving cells.

  - NR-U should support that a serving cell can be configured with bandwidth larger than 20 MHz.

- For DL operation, the following options for BWP-based operation within a carrier with bandwidth larger than 20 MHz can be considered.

  - Option 1a: Multiple BWPs configured, multiple BWPs activated, transmission of PDSCH on one or more BWPs

  - Option 1b: Multiple BWPs configured, multiple BWPs activated, transmission of PDSCH on single BWP

  - Option 2: Multiple BWPs can be configured, single BWP activated, gNB transmits PDSCH on a single BWP if CCA is successful at gNB for the whole BWP

  - Option 3: Multiple BWPs can be configured, single BWP activated, gNB transmits PDSCH on parts or whole of single BWP where CCA is successful at gNB

- For UL operation, the following options for BWP-based operation within a carrier with bandwidth larger than 20 MHz can be considered.

  - Option 1a: Multiple BWPs configured, multiple BWPs activated, transmission of PUSCH on one or more BWPs

  - Option 1b: Multiple BWPs configured, multiple BWPs activated, transmission of PUSCH on single BWP

  - Option 2: Multiple BWPs can be configured, single BWP activated, UE transmits PUSCH on a single BWP if CCA is successful at UE for the whole BWP

  - Option 3: Multiple BWPs can be configured, single BWP activated, UE transmits PUSCH on parts or whole of single BWP where CCA is successful at UE

- It is noted that CCA is declared to be successful or not in multiples of 20 MHz.

- Detailed design and potential selection from the above options can be further discussed when specifications are developed considering protocol and RF aspects.

Agreement: (RAN1#AH1901)

- For wideband operation in DL with a single serving cell operation within a carrier with bandwidth larger than 20 MHz

  - Multiple BWPs can be configured, single BWP activated, gNB may transmit PDSCH on parts or whole of single active BWP where CCA is successful at gNB (i.e., option 2 and 3 from previous agreement)

    - FFS: Restrictions on supportable gaps and combinations of gaps between discontiguous blocks where

      - each block spans contiguous (one or) multiple successful LBT sub-bands

      - each gap spans one or multiple contiguous unsuccessful LBT sub-bands

    - FFS: Transmission bandwidth adaptation delay, potentially different delay for e.g., different number of supported gaps, different transmission bandwidths and different positions of the LBT sub-bands where transmissions occur

    - FFS: Limit on the occupied LBT sub-bands due to regulation and coexistence considerations (not intended to imply that regulation and coexistence considerations will not be addressed)

    - FFS: Whether/how to indicate gNB’s transmitted LBT sub-bands

    - FFS: Enhancements to PDCCH/PDSCH configuration/transmission for the parts of BWP where gNB does not transmit due to CCA failure

- Send LS to RAN4 to inform above decision with the description that RAN1 requires RAN4’s feedback on the first three FFS parts in addition to what was requested in earlier LSs.

Agreement: (RAN1#AH1901)

Operation with multiple active BWPs for a carrier on unlicensed bands is not supported for DL or UL at least in Rel-16 NR-U WI.

- Inform RAN2 of this decision

Agreement: (RAN4#90bis)

- It is feasible to operate single carrier wideband operation when when LBT is successful in all LBT sub-bands

  - FFS whether guardbands are needed in between LBT sub-bands or not

- Mode 2 (Single wideband carrier when LBT is successful in a subset of the LBT sub-bands which are contiguous) is feasible at least if PRBs within the guardband of two contiguous LBT sub-bands are not scheduled by gNB.

  - FFS filter adaptation time if PRBs within the guardband of two contiguous LBT sub-bands are scheduled by gNB.

  - is feasible at least for WiFi-like requirements for in-carrier leakage (e.g. 20dbr).

  - FFS what regional regulatory requirements apply in LBT sub-bands where LBT fails.

    - RAN4 will investigate the feasibility whether regional regulatory requirements are met or not for in-carrier leakage.

- Mode 3 (Single wideband carrier when LBT is successful in a subset of the LBT sub-bands which are non-contiguous)

  - is feasible at least if PRBs within the guardband of two contiguous LBT sub-bands are not scheduled by gNB.

  - is feasible at least for WiFi-like requirements for in-carrier leakage (e.g. 20dbr).

  - FFS what regional regulatory requirements apply in LBT sub-bands where LBT fails.

    - RAN4 will investigate the feasibility whether regional regulatory requirements are met or not for in-carrier leakage.

  - FFS what level of in-carrier leakage and blocking requirements can be met at the BS and UE

  - FFS how to specify this in RAN4

  - FFS filter adaptation time if PRBs within the guardband of two contiguous LBT sub-bands are scheduled by gNB.

Agreement: (RAN1#96bis)

For UL transmissions in a serving cell with carrier bandwidth greater than LBT bandwidth, for the case where UE performs CCA before UL transmission, support at least Alt. 1 among the following alternatives

- Alt. 1: UE transmits the PUSCH only if CCA is successful at UE in all LBT bandwidths of the scheduled PUSCH.

- Alt. 2: UE transmits the PUSCH in all or a subset of LBT bandwidths of the scheduled PUSCH for which CCA is successful at the UE.

  - Decision on whether this alternative is supported will depend on feedback from RAN4

  - FFS on restrictions to the subset of LBT bandwidths, e.g., only contiguous LBT bandwidths allowed, based on feedback from RAN4

- Necessity of guard bands within the scheduled PUSCH should be determined by RAN4

- FFS: Whether this applies also to configured grant PUSCH

- FFS: Whether this applies also to PUCCH

Agreement: (RAN1#96bis)

- Support a mechanism for a UE to detect gNB is transmitting across

- Multiple carriers

- Multiple LBT bandwidths in a carrier.

- The following mechanisms are to be considered:

- Option 1: Explicit indication via PDCCH

  - FFS: The type of PDCCH (e.g., group common PDCCH or UE-specific PDCCH)

  - FFS: Signaling details of the indication

- Option 2: Explicit indication via selection of a PDCCH DM-RS sequence from a set of PDCCH DM-RS sequences

  - FFS: Details of the indication

- Option 3: Via UE implementation, i.e., implicit method based on NR-based signal such as DM-RS and/or corresponding PDCCH detection

  - FFS: Which signals/channels or combination of signals/channels could be used by the UE

- Note: Above options are not mutually exclusive

Agreement: (RAN1#97)

When GC-PDCCH is configured, explicit indication via GC-PDCCH is supported as a mechanism to inform the UE that one or more carriers and/or LBT bandwidths are not available or available for DL reception, at least for slot(s) that are not at the beginning of DL transmission burst.

- FFS: Signalling details of the indication, including e.g., the time domain validity of the indication

- FFS: Whether and how to support the mechanism at the beginning of DL transmission burst

- FFS: Whether and how to handle the case when GC-PDCCH is not configured or not received by the UE

Conclusion: (RAN1#97)

A UE can receive a PDSCH scheduled within an LBT bandwidth or over multiple LBT bandwidths as per Rel-15 and current agreements in Rel-16.

Conclusion: (RAN1#98)

The following are unchanged from Rel-15 for PDCCH.

- The maximum number of monitored PDCCH candidates per slot and per serving cell.

- The maximum number of non-overlapped CCEs per slot and per serving cell.

- CCE-to-REG mapping rule and hashing function.

Agreement: (RAN1#98)

For CORESET configuration in a serving cell with carrier bandwidth greater than LBT bandwidth,

- For the case where a CORESET is confined within a LBT bandwidth, the search space set configuration associated with the CORESET can have multiple monitoring locations in the frequency domain (per LBT bandwidth)

  - Send an LS to RAN2 informing them of this agreement and providing clarifications on the above if necessary

- Note: For scenarios in which gNB transmits PDCCH/PDSCH on a single BWP if CCA is successful at gNB for the whole BWP, CORESET(s) need not all be confined within an LBT bandwidth, and no specification impact is foreseen

Agreement: (RAN1#98bis)

For a search space set configuration associated with multiple monitoring locations in the frequency domain (as per the previous agreement defining such a search space set associated with a CORESET confined within an LBT bandwidth):

- PRBs allocated by frequencyDomainResources in the CORESET configuration are confined within one of LBT bandwidths within the BWP corresponding to the CORESET.

- Within the search space set configuration associated with the CORESET, each of the one or more monitoring locations in the frequency domain corresponds to (and is confined within) an LBT bandwidth and has a frequency domain resource allocation pattern that is replicated from the pattern configured in the CORESET.

- CORESET parameters other than frequency domain resource allocation pattern are identical for each of the one or more monitoring locations in the frequency domain.

- Include this and the prior agreement on this issue in an LS to RAN2

Agreement: (RAN1#98bis)

The intra-carrier guard bands on a carrier can be semi-statically adjusted with an RB level granularity. The RAN4 minimum guard band requirements are used as the guard bands when no semi-static adjustment is applied.

- The guard bands adjustments do not affect the already agreed restrictions on PUCCH resource allocation.

- FFS: Whether and how to handle the case where the intra-carrier guard bands are part of a resource allocation

Agreement: (RAN1#98bis)

- For UL transmissions in a serving cell with carrier bandwidth greater than the LBT bandwidth, for the case where UE performs CCA before UL transmission, UE transmits on the UL only if CCA is successful at UE in all LBT bandwidths that overlap with the resource allocation for the UL transmission

- The UE is not expected to receive resource allocations in discontiguous LBT bandwidths within a wideband carrier

  - This does not preclude such resource allocation in discontiguous LBT bandwidths being supported by specifications managed by RAN1 in Rel-16.

Agreement: (RAN1#99)

UE determines the number of RB sets (corresponding to LBT bandwidths) and the available PRBs in each RB set, both for DL and UL, based on,

- If configured, the RRC parameters intraCellGuardBandDL-r16 and intraCellGuardBandUL-r16 (if UL is configured) configure the lists of intra-carrier guard-bands per cell, e.g.,

  - If intraCellGuardBandDL-r16 or intraCellGuardBandUL-r16 are given as {GB1low, GB1high}, …, {GBN-1low, GBN-1high} where GB is given by CRB index, the UE determines

    - The number of RB sets for a cell: N

    - The available PRBs in each RB set: [RBstart, GB1low-1] for RB set #1, [GB1high+1, GB2low-1] for RB set #2,…, [GBN-1high+1, RBend] for RB set#N, where RBstart and RBend corresponds to starting and ending RB index of cell, respectively.

    - Note: {GB1low, GB1high}, …, {GBN-1low, GBN-1high} may be provided separately for DL and UL

- If intraCellGuardBandDL-r16 is not configured, then {GB1low, GB1high}, …, {GBN-1low, GBN-1high} is derived from the RAN4 specifications

  - Note: This supersedes a previous agreement

- If intraCellGuardBandUL-r16 is not configured, then {GB1low, GB1high}, …, {GBN-1low, GBN-1high} is derived from the RAN4 specifications

  - Note: This supersedes a previous agreement

- Note: This addresses the FFS in sections 5.1.2.2 and 6.1.2.2 in 38.214.

Agreement: (RAN1#99)

For the frequency domain resource allocation that is provided with frequencyDomainResources in CORESET configuration,

- Introduce a new RRC parameter rb-Offset (with the value range of 0,1,…,5) in ControlResoureSet IE.

  - If rb-Offset is not configured, rb-Offset is 0

- The bits of the 45-bit bitmap frequencyDomainResources have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the BWP with the starting PRB position as {the first PRB index in the BWP + rb-Offset} for a CORESET.

- FFS: For multi-cluster CORESET configuration, rb-Offset also applies to the RB offset between the starting PRB index of the first 6 PRB group and the first PRB index in each RB set. Full 6 PRB groups are counted till the end of the RB set. The bits in frequencyDomainResources sequentially maps to the 6 RB groups in all RB sets in the BWP.

- Note: Cluster above implies a group of resource blocks that are not contiguous in frequency

Conclusion: (RAN1#99)

For a legacy CORESET configuration, the UE can expect to process PDCCH as per Rel-15 behaviour

Agreement: (RAN1#99)

For a search space set configuration with multiple monitoring locations in the frequency domain,

- Within the SearchSpace IE, the agreed RRC parameter freqMonitorLocations-r16 provides a bitmap (where the first bit in the bitmap corresponds to the first RB set in the BWP, and the second bit corresponds to the second RB set, and so on). For a RB set indicated in the bitmap, the first PRB of the frequency domain monitoring location confined within the RB set is aligned with {the first PRB of the RB set + rb-Offset provided by the associated CORESET configuration}.

- The frequency domain resource allocation pattern for each monitoring location is determined based on the first A bits in frequencyDomainResources provided by the associated CORESET configuration, where A = floor({the number of available PRBs in the first RB set (accounting for rb-Offset) for the BWP}/6).

Agreement: (RAN1#99)

- The RRC parameters intraCellGuardBandDL-r16 and intraCellGuardBandUL-r16 include a mechanism to indicate that no intra-carrier guard-bands are configured

  - Note: This configuration may be used for the case where transmission only occurs in a BWP if LBT is successful in all RB sets within the BWP

- For a carrier with intra-carrier guard bands, the UE does not expect that the dedicated BWP is configured to include parts of a RB set.

Agreement: (RAN1#99)

If a UE is configured with a CSI-RS spanning over multiple LBT bandwidths,

- The UE assumes that the CSI-RS is not transmitted if the UE is monitoring DCI format 2_0 carrying an LBT BW indication and detects the DCI format 2_0 indicating any of corresponding LBT bandwidths is not available for DL reception.

Agreement: (RAN1#99)

For CSI-RS for tracking in unlicensed spectrum,

- Text proposal for section 5.1.6.1.1 in TS 38.214:

  - The bandwidth of the CSI-RS resource, as given by the higher layer parameter freqBand configured by CSI-RS-ResourceMapping, is the minimum of 48 and NBWP,i size resource blocks, or is equal to NBWP,i size resource blocks.

Agreement: (RAN1#100-e)

- If CORESET p is not configured with rb-offset, and is not associated with any search space set configured withfreqMonitorLocation-r16,

  - The bits of the 45-bit bitmap frequencyDomainResources of the CORESET p have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP with the starting common RB position NBWPstart, where the first common RB of the first group of 6 consecutive RBs has common RB index 6∙NBWPstart6, i.e., same as in Rel-15.

- If CORESET p is not configured with rb-offset, and is associated with at least one search space set configured with freqMonitorLocation-r16,

  - The bits of the first A bits of the 45-bit bitmap frequencyDomainResources of the CORESET p have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP with the starting common RB position NBWPstart , where the first common RB of the first group of 6 consecutive RBs has common RB index NBWPstart+rb-Offset, where the UE assumes the default value rb-Offset = 0.

- If CORESET p is configured with rb-offset, and is not associated with any search space set configured with freqMonitorLocation-r16,

  - The bits of the 45-bit bitmap frequencyDomainResources of the CORESET p have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP with the starting common RB position NBWPstart , where the first common RB of the first group of 6 consecutive RBs has common RB index NBWPstart+rb-Offset.

- If CORESET p is configured with rb-offset, and is associated with at least one search space set configured with freqMonitorLocation-r16,

  - The bits of the first A bits of the 45-bit bitmap frequencyDomainResources of the CORESET p have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP with the starting common RB position NBWPstart , where the first common RB of the first group of 6 consecutive RBs has common RB index NBWPstart+rb-Offset.

- Note: A bits in above bullets is defined as floor({the number of available PRBs in the first RB set (accounting for rb-Offset) for the BWP}/6), as per previous agreement.

- TS 38.213 editor to implement this agreement