3GPP TSG RAN WG1 #100bis																		R1- 200xxxx

e-Meeting 2, April 20th – 30th, 2020

Source:	Moderator (CATT)

Title:	FL Summary of Remaining issues on NR Positioning Measurements

Agenda Item:	7.2.8.3

Document for:	Discussion and Decision

# Introduction

This document provides a summary of the remaining issues and proposals for AI 7.2.8.3 related to UE and gNB measurements for NR Positioning based on the contributions submitted under this AI [1-7]. According to the guidance of RAN1 Chairman, the intention of this summary is to identiy a list of critical issues/proposals for further email discussion in RAN1#100bis. To facilitate the discussion, the issues/proposals from the submitted contributions and the FL views are presented in Section 2. The summary of the issues/proposals is provided in Section 3, where the feature lead has also provided his assessment on the critical issues/proposals. Interested companies are welcome to present their views on the priority of these issues.

Issues/proposals for UE/gNB measurements

## UL RTOA reference time

Submitted Proposals

- (Huawei) Proposal 1:

  - RAN1 continues to discuss the topic of gNB measurement and reaches agreements in RAN1#100b at least on the following issues and sends an LS to RAN3 to notify RAN3 regarding the reached agreements:

    - UL RTOA definition

    - Search window configuration for gNB to receive SRS.

- (Huawei) Proposal 2:

  - Introduce the following new parameter to the higher layer parameter list.

| NR_pos-Core | NR UL Measurement Report Configuration |  |  | FFS in RAN3 WG | SFN Initialization time | SFN initialization time | New |  | The nominal beginning time of SFN 0 for SRS | Same as SFN initialization time in LPPa BIT STRING (64) |  |  | NRPPa 38.455 |  | Time in seconds relative to 00:00:00 on 1 January 1900 (calculated as continuous time without leap seconds and traceable to a common time reference) where binary encoding of the integer part is in the first 32 bits and binary encoding of the fraction part in the last 32 bits. The fraction part is expressed with a granularity of 1 /2**32 second. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- (Huawei) Proposal 3:

  - The RTOA reference time is defined as T0+tSRS, where

    - T0  is the nominal beginning time of SFN 0 provided by LMF.

    - tSRS is the nominal time offset of the beginning of the subframe that contains the target SRS  relative to the nominal beginning time of SFN 0.

- Proposal 4: Adopt the following TP to TS 38.215.

===================== Unchanged parts omitted ====================== 2 References ===================== Unchanged parts omitted ====================== [xx] 3GPP TS 38.455: "NG-RAN; NR Positioning Protocol A (NRPPa)" ===================== Unchanged parts omitted ====================== 5.2.2 UL Relative Time of Arrival (TUL-RTOA) Definition; The UL Relative Time of Arrival (TUL-RTOA) is the beginning of subframe i containing SRS received in positioning node j, relative to the UL RTOA reference time. The UL RTOA reference time is defined as T0+tSRS, where - T0 is the nominal beginning time of SFN 0 provided by [yy] [xx, TS 38.455] - tSRS is the nominal time offset of the beginning of the subframe that contains the target SRS relative to the nominal beginning time of SFN0. Multiple SRS resources for positioning can be used to determine the beginning of one subframe containing SRS received at a positioning node. The reference point for TUL-RTOA shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Rx antenna (i.e. the centre location of the radiating region of the Rx antenna), - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector.. ===================== Unchanged parts omitted ======================

FL Comments

This issue of the definition of the UL RTOA reference time was discussed in RAN1#100-e[8]. At that time, the majority view was to let RAN3 to handle this issue. Given that RAN3 has not provided the definition so far, we suggest RAN1 to take a look at this issue again to see if there is a need or RAN1 to provide inputs to RAN3. Intereted companies are also welcome to express their view on whether it is critical for RAN1 to provide the definition in this meeting.

Comments from interested companies

| Company |  | Comments |
| --- | --- | --- |

## Search window for configuration gNB to receive SRS

Submitted Proposals

- (Huawei) Proposal 1:

  - RAN1 continues to discuss the topic of gNB measurement and reaches agreements in RAN1#100b at least on the following issues and sends an LS to RAN3 to notify RAN3 regarding the reached agreements:

    - UL RTOA definition

    - Search window configuration for gNB to receive SRS.

- (Huawei) Proposal 5:

  - Introduce the following new parameters to the higher layer parameter list.

| NR_pos-Core | NR UL Measurement Report Configuration |  |  | FFS in RAN3 WG | Expected Propagation Delay | Expected Propagation Delay | New |  | For providing an indication of when the SRS is expected to arrive in time at the gNB relative to the UL-RTOA reference time. | FFS in RAN4 WG |  |  | NRPPa 38.455 |  | Similar to expected propagation delay in SLmAP. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_pos-Core | NR UL Measurement Report Configuration |  |  | FFS in RAN3 WG | Delay Uncertainty | Delay Uncertainty | New |  | For providing an indication of when the SRS is expected to arrive in time at the gNB with uncertainty (search window). | FFS in RAN4 WG |  |  | NRPPa 38.455 |  | Similar to delay uncertainty in SLmAP. |

FL Comments

This issue was not brought up before. In LTE ULTDOA, the location server does not provide the search window configuration for the reception of SRS. Intereted companies are welcome to express their views on whether it is critical to provide the search window for gNB  for SRS reception.

Comments from interested companies

| Company |  | Comments |
| --- | --- | --- |

## NR-TimingMeasQuality for RSTD

Submitted Proposals

- (Huawei) Proposal 6:

  - RAN1 to clarify which interpretation should be used for the timing measurement quality for RSTD.

    - Interpretation 1: σk2+σ12

    - Interpretation 2: σk

    - Where σ1 is the TOA quality of the reference and σk is the TOA quality of the TRP k.

- (Huawei) Proposal 7:

  - RAN1 to clarify whether the reference TRP can have NR-TimingMeasQuality

FL Comments

Suggest having a discussion in this meeting for the clarification on the timing measurement quality for RSTD.

Comments from interested companies

| Company |  | Comments |
| --- | --- | --- |

## Inter-frequency UE Rx – Tx time difference

Submitted Proposals

- (Huawei) Proposal 8:

    - Limit UE Rx – Tx time difference only to PRS and SRS in the same band.

- (Huawei) Proposal 9:

  - Adopt the following TP to TS 38.215

- ===================== Unchanged parts omitted ======================

| Definition | The UE Rx – Tx time difference is defined as TUE-RX – TUE-TX Where: TUE-RX is the UE received timing of downlink subframe #i from a positioning node, defined by the first detected path in time. TUE-TX is the UE transmit timing of uplink subframe #j that is closest in time to the subframe #i received from the positioning node. TUE-RX and TUE-TX shall be measured on the same band. Multiple DL PRS resources can be used to determine the start of one subframe of the first arrival path of the positioning node. For frequency range 1, the reference point for TUE-RX measurement shall be the Rx antenna connector of the UE and the reference point for TUE-TX measurement shall be the Tx antenna connector of the UE. For frequency range 2, the reference point for TUE-RX measurement shall be the Rx antenna of the UE and the reference point for TUE-TX measurement shall be the Tx antenna of the UE. |
| --- | --- |
| Applicable for | RRC_CONNECTED intra-frequency RRC_CONNECTED inter-frequency |

- ===================== Unchanged parts omitted ======================

FL Comments

Our understanding is that UE Rx – Tx time difference is not limited to the scenarios that PRS and SRS are in the same band.

Comments from interested companies

| Company |  | Comments |
| --- | --- | --- |

## Reference point for gNB RSRP measurements

Submitted Proposals

- (Huawei) Proposal 10:

  - Send a reply LS to RAN4 to ask RAN4 whether the reference point for the UL SRS RSRP at gNB, currently captured in TS 38.215, is correct or not.

FL Comments

This seems not critical at this moment. If RAN4 finds any issue on the reference point for the UL SRS RSRP, or any other issues on the measurement definition in TS 38.215, we may assume RAN4 would send LS to RAN1 for the clarification.

.

Comments from interested companies

| Company |  | Comments |
| --- | --- | --- |

## Cell ID of DL RSTD measurements

Submitted Proposals

- (ZTE) Proposal 1

  - The terminology ‘cell’ should have a clear indication to be associated with the [ID] provided by DL-PRS-RstdReferenceInfo. Adopt the following text changes,

FL Comments

May need to discuss whether there is a need to make further clarification on the definition of ‘cell’ in the requirement. Our understanding is that a cell is identied by nr-PhysCellId together with nr-CellGlobalId etc.

Comments from interested companies

| Company |  | Comments |
| --- | --- | --- |

## TP for PRS reception procedure

Submitted Proposals

- (OPPO): Proposal 1:

  - TP for TS 38.214

#### 5.1.6.5 PRS reception procedure *** Unchanged text is omitted *** For the DL RSTD, DL PRS-RSRP, and UE Rx-Tx time difference measurements the UE can report an associated higher layer parameter Timestamp. The Timestamp can include the SFN and the slot number for a subcarrier spacing. These values correspond to the reference which is provided by DL-PRS-RSTDReferenceInfo. The UE is expected to measure the DL PRS resource outside the active DL BWP or with a numerology different from the numerology of the active DL BWP if the measurement is made during a configured measurement gap. When not configured with a measurement gap, the UE is only required to measure DL PRS within the active DL BWP and with the same numerology as the active DL BWP. When the UE is expected to measure the DL PRS resource outside the active DL BWP or with a numerology different from the numerology of the active DL BWP or on any symbols indicated as UL symbol by the serving cell, it may request a measurement gap in higher layer parameter [XYZ]. The UE assumes that the DL PRS from the serving cell is not mapped to any symbol that contains SS/PBCH block from the serving cell. If the time frequency location of the SS/PBCH block transmissions from non-serving cells are provided to the UE then the UE also assumes that the DL PRS from a non-serving cell is not mapped to any symbol that contains the SS/PBCH block of the same non-serving cell. The UE may be configured to report, subject to UE capability, up to 4 DL RSTD measurements per pair of cells with each measurement between a different pair of DL PRS resources or DL PRS resource sets within the DL PRS configured for those cells. The up to 4 measurements being performed on the same pair of cells and all DL RSTD measurements in the same report use a single reference timing. The UE may be configured to measure and report up to 8 DL PRS RSRP measurements on different DL PRS resources from the same cell. When the UE reports DL PRS RSRP measurements from one DL PRS resource set, the UE may indicate which DL PRS RSRP measurements have been performed using the same spatial domain filter for reception.When UE reports one DL PRS RSRP measurement, the UE may indicate whether the DL PRS RSRP measurement has been performed using multiple different spatial domain receive filters for receiving the corresponding PRS resource. If the UE is configured with DL-PRS-QCL-Info and the QCL relation is between two DL PRS resources, then the UE assumes those DL PRS resources are from the same cell. If DL-PRS-QCL-Info is configured to the UE with 'QCL-Type-D' with a source DL-PRS-Resource then the DL-PRS-ResourceSetId and the DL-PRS-ResrouceId of the source DL-PRS-Resource are expected to be indicated to the UE. *** Unchanged text is omitted ***

FL Comments

The proposal is related to UE measurement procedure. Suggest the issue to be discussed in AI 7.2.8.4 together with other similar proposals to AI 7.2.8.4.

Comments from interested companies

| Company |  | Comments |
| --- | --- | --- |

## ‘Positioning node’ in TS 38.215

Submitted Proposals

- (Futurewei, R1-2002048)  Proposal 1:

  - The terminology ‘Positioning Node’ in TS 38.215 is replaced by ‘Transmission Point (TP)’ or ‘Reception Point (RP)’, where applicable, as defined in TS38.305.

FL Comments

This issue was discussed in RAN1#100-e. It seems it is simple and reasonable to make the change since ‘TP’ and ‘RP’ are defined in TS 38.305, but ‘Positioning Node’ is undefined.

Comments from interested companies

| Company |  | Comments |
| --- | --- | --- |

## FFSs on Maximum numbers of DL PRS resources and UE capability

Submitted Proposals

- (Samsung) Proposal 1

      - The following values for X1 to X7 should be supported.:

| Description | Maximum numbers for DL PRS resources | Values that can be signaled as part of UE Capability |
| --- | --- | --- |
| Max number of frequency layers (X1) | X1=4 | Values = {1,4} No other values |
| Max number of TRPs per frequency layer (X2) | X2=64 |  |
| Max number of PRS resource sets per TRP (X3) per frequency layer | X3=2 | Values = {1,2} |
| Max number of Resources per PRS resource set (X4) | X4=64 | Values = {64, 32, 16, 8 ,4, 2,1} |
| Max number of DL PRS Resources per UE (X5) | NA | Values = {16, 8, 4, 2 ,1} |
| Max number of TRPs for all frequency layers (X6) per UE | 256 | Values = {1,…,256} |
| Max number of Resources per frequency layer (X7) | NA | Values = {128, 64, 32, 16, 8 ,4, 2, 1} |

- (Samsung) Proposal 2

      - There is no need to support FR1/FR2 differentiation for the values of X1 to X7

FL Comments

The FFSs related to UE capability will be further discussed in AI 7.2.11.8.  It seems no need to further discuss this issue under this AI.

Comments from interested companies

| Company |  | Comments |
| --- | --- | --- |

## UE capability on DL PRS RSRP

Following agreements were made on Rel-16 UE/gNB positioning measurements in RAN1#99, where it is FFS on whether N is a UE capability

Agreement: The prior agreement is updated as given below: UE can be configured to measure and report up to N=8 (> 1) DL PRS RSRP measurements on different DL PRS resources from the same TRP - N=[3] FFS: N is a UE capability

Submitted Proposals

- (Samsung) Proposal 3

      - The number of reported DL PRS RSRP measurements N is not a UE capability

- (CATT) Proposal 2

        - There is no need to set N as an UE capability, since the UE will at least try to measure  all DL PRS from one TRP, and can report N=8 DL PRS RSRP measurements if the UE detects 8 or more DL PRS..

FL Comments

The issues related to UE capability will be covered by AI 7.2.11.8. It seems no need to further discuss this issue under this AI.

Comments from interested companiese

| Company |  | Comments |
| --- | --- | --- |

## UE/gNB RX-TX time difference measurements for NR E-CID

Submitted Proposals

- (Ericsson, ) Proposal 1:

      - Support reuse of Rel-15 SRS resource set for gNB Rx-Tx and UE Rx-Tx measurements for positioning in NR.

      - Send an LS to RAN4 regarding UE Rx-Tx requirement

Note: There is no impact to specifications managed by RAN1

FL Comments

This proposal was discussed in RAN1#100-e[8]. It was not considered as critical at that time by some companies. Intereted companies are welcome to express their view on whether it is critical for RAN1 to further discuss this issue in RAN1#100bis.

Comments from interested companies

| Company |  | Comments |
| --- | --- | --- |

# Summary on the Priority of the Issues/Proposals

Table 1 summarizes the views of interested companies on the priority of the Issues/Proposals for the discussion in RAN1#100 e-meeting:

Table 1 Summary on the Priority of the Issues/Proposals for RAN1#100 e-meeting discussion

| Issues/Proposals | Companies |  |  |  |
| --- | --- | --- | --- | --- |
|  | High Priority | Low Priority | No Need | Other comments |
| - UL RTOA reference time | FL |  |  |  |
| - Search window for gNB to receive SRS |  | FL |  |  |
| - NR-TimingMeasQuality for RSTD | FL |  |  |  |
| - Inter-frequency UE Rx – Tx time difference | FL |  |  |  |
| - Reference point for gNB RSRP measurements |  |  | FL |  |
| - Cell ID of DL RSTD measurements |  | FL |  |  |
| - TPs for PRS reception procedure in TS 38.214 |  |  | FL |  |
| - ‘Positioning node’ in TS 38.215 |  | FL |  |  |
| - FFSs on Maximum numbers of DL PRS resources and UE capability |  |  | FL |  |
| - UE capability on DL PRS RSRP |  |  | FL |  |
| - UE/gNB RX-TX time difference measurements for NR E-CID |  | FL |  |  |

Notes:

- High priority: Critical issues/Proposals need to be discussed and resolved in this AI in this meeting.

- Low priority: Issues/Proposals may be discussed in this meeting with low priority.

- No Need: Issues/Proposals may not necessarily be discussed in this AI.

- Other comments: Additional comments. For example, if there is a strong view on whether the issues/proposals should be included in, or excluded from this meeting.

References

- R1-2001560	Maintenance of NR positioning measurements	Huawei, HiSilicon

- R1-2001602	Maintenance of UE and gNB measurements for NR positioning	ZTE

- R1-2001733	Remaining Issues on Measurements for NR Positioning	OPPO

- R1-2002048	Remaining details on Measurements	Futurewei

- R1-2002097	Remaining issues on NR Positioning Measurements	CATT

- R1-2002146	UE and gNB measurements for NR Positioning	Samsung

- R1-2002622	Maintenance of rel16 UE and gNB measurements for NR Positioning	Ericsson

- R1- 2001156 	FL Summary #2 of Remaining issues on NR Positioning Measurements CATT