3GPP TSG RAN WG1 #101		R1-2003366

e-Meeting, May 25th – June 5th, 2020

Source:	vivo

Title:	Remaining issues on 2-step RACH procedure

Agenda Item:	7.2.1.2

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss the reaming issues on the related procedure of 2-step RACH, including additional PUSCH validation rule in shared spectrum and Tx beam for PDCCH/PDSCH for CF 2-step RACH.

- Discussion

  - PUSCH occasion validation rule

In RAN1#99, the following agreement was made on PRACH validation due to that any transmission including PRACH is not allowed in FBE idle period [1]:

Agreement:

A PRACH resource is considered invalid if:

- it overlaps with the IDLE period of a fixed frame period when FBE operation is indicated

Similarly, for 2-step RACH, a PUSCH occasion for 2-step RACH is considered invalid if it overlaps with the IDLE period of a fixed frame period when FBE operation is indicated. Therefore, following text proposal in Section 8.1A in 38.213 is suggested.

Proposal 1: Adopt the following text proposal #1 in Section 8.1A in 38.213.

---------------------------------Text proposal #1 starts for TS 38.213, Section 8.1A -------------------------------- =====omitted text ====== A PUSCH occasion is valid if it does not overlap in time and frequency with any PRACH occasion associated with either a Type-1 random access procedure or a Type-2 random access procedure. Additionally, for unpaired spectrum and for SS/PBCH blocks with indexes provided by ssb-PositionsInBurst in SIB1 or by ServingCellConfigCommon - if a UE is not provided tdd-UL-DL-ConfigurationCommon, a PUSCH occasion is valid if the PUSCH occasion - does not precede a SS/PBCH block in the PUSCH slot, and - starts at least Ngap symbols after a last SS/PBCH block symbol, where Ngap is provided in Table 8.1-2 - if a UE is provided tdd-UL-DL-ConfigurationCommon, a PUSCH occasion is valid if the PUSCH occasion - is within UL symbols, or - does not precede a SS/PBCH block in the PUSCH slot, and - starts at least Ngap symbols after a last downlink symbol and at least Ngap symbols after a last SS/PBCH block symbol, where Ngap is provided in Table 8.1-2 - if ChannelAccessType-r16 = semistatic is provided, does not overlap with a set of consecutive symbols before the start of a next channel occupancy time where there shall not be any transmissions, as described in [15, TS 37.213] =====omitted text ====== ------------------------------- Text proposal #1 ends for TS 38.213, Section 8.1A -----------------------------------

  - Tx beam for PDCCH/PDSCH for 2-step RACH

In RAN2#109bis-e meeting, it was agreed that From RAN2 perspective, if the UE supports 2-step RA, it shall support 2-step CFRA for the SSB case (i.e. same as 4-step RACH). RAN2 will continue discussing how to support 2-step CFRA with limited RAN1 impact.

Per RAN2 agreement, text proposal #1 for section 8.2A of TS 38.213 to support CF 2-step RACH is adopted.

Proposal 2: RAN1 adopts the following text proposal #2 for section 8.2A of TS 38.213:

---------------------------------Text proposal #2 starts for TS 38.213, Section 8.2A -------------------------------- =====omitted text ====== If the UE detects a DCI format 1_0 with CRC scrambled by the corresponding MsgB-RNTI and receives a transport block in a corresponding PDSCH, the UE may assume same DM-RS antenna port quasi co-location properties, as described in [6, TS 38.214], as for a SS/PBCH block the UE used for PRACH association, as described in Clause 8.1, regardless of whether or not the UE is provided TCI-State for the CORESET where the UE receives the PDCCH with the DCI format 1_0. =====omitted text ====== ------------------------------- Text proposal #2 ends for TS 38.213, Section 8.2A -----------------------------------

- Conclusion

In this contribution, we provide our views on the remaining issues on 2-step RACH procedure. The proposals are summarized below.

Proposal 1: Adopt the following text proposal #1 in Section 8.1A in 38.213.

---------------------------------Text proposal #1 starts for TS 38.213, Section 8.1A -------------------------------- =====omitted text ====== A PUSCH occasion is valid if it does not overlap in time and frequency with any PRACH occasion associated with either a Type-1 random access procedure or a Type-2 random access procedure. Additionally, for unpaired spectrum and for SS/PBCH blocks with indexes provided by ssb-PositionsInBurst in SIB1 or by ServingCellConfigCommon - if a UE is not provided tdd-UL-DL-ConfigurationCommon, a PUSCH occasion is valid if the PUSCH occasion - does not precede a SS/PBCH block in the PUSCH slot, and - starts at least Ngap symbols after a last SS/PBCH block symbol, where Ngap is provided in Table 8.1-2 - if a UE is provided tdd-UL-DL-ConfigurationCommon, a PUSCH occasion is valid if the PUSCH occasion - is within UL symbols, or - does not precede a SS/PBCH block in the PUSCH slot, and - starts at least Ngap symbols after a last downlink symbol and at least Ngap symbols after a last SS/PBCH block symbol, where Ngap is provided in Table 8.1-2 - if ChannelAccessType-r16 = semistatic is provided, does not overlap with a set of consecutive symbols before the start of a next channel occupancy time where there shall not be any transmissions, as described in [15, TS 37.213] =====omitted text ====== ------------------------------- Text proposal #1 ends for TS 38.213, Section 8.1A -----------------------------------

Proposal 2: RAN1 adopts the following text proposal #2 for section 8.2A of TS 38.213:

---------------------------------Text proposal #2 starts for TS 38.213, Section 8.2A -------------------------------- =====omitted text ====== If the UE detects a DCI format 1_0 with CRC scrambled by the corresponding MsgB-RNTI and receives a transport block in a corresponding PDSCH, the UE may assume same DM-RS antenna port quasi co-location properties, as described in [6, TS 38.214], as for a SS/PBCH block the UE used for PRACH association, as described in Clause 8.1, regardless of whether or not the UE is provided TCI-State for the CORESET where the UE receives the PDCCH with the DCI format 1_0. =====omitted text ====== ------------------------------- Text proposal #2 ends for TS 38.213, Section 8.2A -----------------------------------

- References

- Chairman’s Notes, RAN1 #99.