3GPP TSG RAN WG1 #101		R1-2003367

e-Meeting, May 25th – June 5th, 2020

Source:	vivo

Title:	Remaining issues on initial access signals and channels

Agenda Item:	7.2.2.1.1

Document for:	Discussion and Decision

- Introduction

This contribution discusses remaining issues on initial access signals and channels for NR-U.

- Discussion

LBT Gap between RACH occasions

In NR Rel-15, only back to back RACH occasions in time is configured. In NR-U, a UE has to pass LBT before transmitting RACH preamble. The UE may get blocked by a RACH transmission of another UE in the previous RACH occasion. To reduce the PRACH delay caused by the blockage issue, LBT gap between RACH occasions is preferred. The following alternative can be considered:

LBT gap can be realized by mapping the ROs non-contiguously in time without truncating the PRACH transmission, sparser RO mapping pattern can be defined additionally. In NR Rel-15, the RACH occasions in a RACH slot/frame and periodicity of the RACH frame are indicated by the higher layer parameter prach-ConfigurationIndex, 255 configurations can be indicated, no extra entry is left for additional patterns. To avoid significant specification work, the sparser RO mapping pattern can be designed based on the existing configuration by muting some of the RACH instances. In other words, some resources in time domain are defined as invalid. As shown in Figure 1, only the odd/even numbered RACH instances configured in a RACH slot are valid, LBT gap with at least 2 symbols can be left to perform LBT before PRACH transmission in each valid RACH instance. Both CAT2 and CAT4 LBT can be performed during this time interval.

Figure 1	Sparser ROs in PRACH slot based on existing configuration

Proposal 1: To reduce the PRACH delay caused by the blockage issue, LBT gap between RACH occasions should be supported, i.e. only even or odd numbered time domain RACH occasions in a RACH slot are used based on existing PRACH configurations.

Proposal 2: RAN1 adopts TP1 for section 8.1 of TS 38.213.

----------------------------------------TP1: Start TP for Section 8.1 of TS 38.213 -------------------------------------

For unpaired spectrum,

-	if a UE is not provided tdd-UL-DL-ConfigurationCommon, a PRACH occasion in a PRACH slot is valid if it does not precede a SS/PBCH block in the PRACH slot and starts at least  symbols after a last SS/PBCH block reception symbol, where  is provided in Table 8.1-2.

-	the index of the SS/PBCH block is provided by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon

-	If a UE is provided 	 tdd-UL-DL-ConfigurationCommon, a PRACH occasion in a PRACH slot is valid if

-	it is within UL symbols, or

-	it does not precede a SS/PBCH block in the PRACH slot and starts at least  symbols after a last downlink symbol and at least  symbols after a last SS/PBCH block symbol, where  is provided in Table 8.1-2, and if ChannelAccessType-r16 = semistatic is provided, does not overlap with a set of consecutive symbols before the start of a next channel occupancy time where there shall not be any transmissions, as described in [15, TS 37.213]

-	the index of the SS/PBCH block is provided by ssb-PositionsInBurst in SIB1 or in
ServingCellConfigCommon.

-	if operating in shared spectrum access, only odd numbered RACH occasions within one slot in time domain based on higher layer configuration for PRACH transmission [4, TS 38.211] are valid.

----------------------------------------TP1: End TP for Section 8.1 of TS 38.213 -------------------------------------

CSI-RS transmission with SSB

In NR Rel-15, the CSI-RS framework offers enough flexibility to achieve various types of resource configuration. Within one CSI-RS resource configuration for a serving cell, it could include one resource set comprising of multiple CSI-RS resources. The CSI-RS transmission as a part of DRS together with SSB could also adopt such methods instead of introducing a new separate CSI-RS configuration association with SSB as that in LAA. Multiple CSI-RS resources could be configured with different slot offsets to achieve that they locate in different slots in which the SSBs have the same QCL assumption. One example is shown below: CSI-RS resource 1 and 2 with the same TCI state configuration (i.e. QCL with SSB index #0) locate in the same slot with candidate SSB#0 and #4 by appropriate configuration of respective slot offset. Note that candidate SSB #0 and #4 have the same QCL assuming the value of Q in MIB is 4. In this way, it achieves the same result of independent configuration as in LAA.

Figure 2	CSI-RS configuration with SSB

Proposal 3: NR Rel-15 CSI-RS frame work offers enough flexibility for configuration of CSI-RS as a part of DRS and there is no need to introduce a separate configuration for this purpose.

Another discussion point would be how to determine which CSI-RS needs to be measured from UE side for NRU since only one CSI-RS is transmitted depending on LBT result in this case. In LAA, if the UE is configured with CSI-RS for RRM using DRS, the UE assumes the presence of CSI-RS for RRM in first detected subframe containing PSS/SSS/CRS within DMTC. Similarly, the CSI-RS presence could be validated by the SSB transmission located in the same slot in NRU. If only one SSB is transmitted, only the CSI-RS located in the same slot with this SSB is validated. The same behavior is performed by UE as in LAA.

Proposal 4: If the configured CSI-RS is located in the same slot as the QCLed SSB, UE will assume the CSI-RS is transmitted when the SSB is detected.

- Conclusion

In this contribution, we focus on the remaining issues of initial access signals and channels, and have the following proposals:

Proposal 1: To reduce the PRACH delay caused by the blockage issue, LBT gap between RACH occasions should be supported, i.e.

Proposal 2: RAN1 adopts TP1 for section 8.1 of TS 38.213.

Proposal 3: NR Rel-15 CSI-RS frame work offers enough flexibility for configuration of CSI-RS as a part of DRS and there is no need to introduce a separate configuration for this purpose.

Proposal 4: If the configured CSI-RS is located in the same slot as the QCLed SSB, UE will assume the CSI-RS is transmitted when the SSB is detected.

- References

- RAN1 chairman notes, RAN1 #99, Reno, USA, November 18th – 22nd, 2019.