3GPP TSG RAN WG1 #101	R1-2003396

e-Meeting, May 25th – June 5th, 2020

Source:	vivo

Title:	Discussion on remaining issues on MUCSI

Agenda Item:	7.2.6.1

Document for:	Discussion and Decision

Introductions

In this contribution, we discuss some of the remaining open issue on Rel-16 MUCSI enhancement.

Remaining issues and TP for MU-CSI

In RAN 1 #100 meeting [1], it was agreed only wideband reporting is supported for BWP size less than 24 PRB. And the following was agreed [2]

- If csi-ReportingBand is configured, the number of subbands is determined based on the subband size configured by the higher-level parameter subbandSize and the total number of PRBs in the bandwidth part according to Table 5.2.1.4-2. Otherwise, the number of subbands equals one and the subband size equals the total number of PRBs in the bandwidth part.

Referring to the definition of R = 2 [3], it controls the total number of precoding matrices N3 as a function of the number of subbands in csi-ReportingBand, which implies two precoding matrices per CQI subband. Therefore, only support R = 1 for BWP size less than 24 PRBs.

Proposal 1: UE is not expected to be configured with R =2 for BWP size less than 24 PRBs.

Proposal 2: agree following TP for 38.214.

-------- Start of proposed TP for 38.214 (section 5.2.2.2.5)-------

5.2.2.2.5  Enhanced Type II Codebook

------- Unchanged text is omitted -------

For 4 antenna ports {3000, 3001, …, 3003}, 8 antenna ports {3000, 3001, …, 3007}, 12 antenna ports {3000, 3001, …, 3011}, 16 antenna ports {3000, 3001, …, 3015}, 24 antenna ports {3000, 3001, …, 3023}, and 32 antenna ports {3000, 3001, …, 3031}, and UE configured with higher layer parameter codebookType set to ' typeII-r16'

- The values of N1 and N2 are configured with the higher layer parameter n1-n2-codebookSubsetRestriction-r16. The supported configurations of N1,N2 for a given number of CSI-RS ports and the corresponding values of O1,O2 are given in Table 5.2.2.2.1-2. The number of CSI-RS ports, PCSI-RS, is 2N1N2.

- The values of L, β and pυ are determined by the higher layer parameter paramCombination-r16, where the mapping is given in Table 5.2.2.2.5-1.

- The UE is not expected to be configured with paramCombination-r16 equal to

- 3, 4, 5, 6, 7, or 8 when PCSI-RS=4,

- 7 or 8 when PCSI-RS<32

- 7 or 8 when higher layer parameter typeII-RI-Restriction-r16 is configured with ri=1 for any i>1.

- 7 or 8 when R=2.

- If csi-ReportingBand is configured, the number of subbands is determined based on the subband size configured by the higher-level parameter subbandSize and the total number of PRBs in the bandwidth part according to Table 5.2.1.4-2. Otherwise, the number of subbands equals one and the subband size equals the total number of PRBs in the bandwidth part. UE does not expect to be configured with R =2 for BWP size less than 24 PRBs.

- -------- end of the TP-------

Conclusion

In this contribution we discussed two different aspects of Rel-16 MU CSI enhancement. Based on our analysis, we have following proposal and TP:

Proposal 1: UE is not expected to be configured with R =2 for BWP size less than 24 PRBs.

Proposal 2: agree following TP for 38.214.

-------- Start of proposed TP for 38.214 (section 5.2.2.2.5)-------

5.2.2.2.5 Enhanced Type II Codebook

------- Unchanged text is omitted -------

For 4 antenna ports {3000, 3001, …, 3003}, 8 antenna ports {3000, 3001, …, 3007}, 12 antenna ports {3000, 3001, …, 3011}, 16 antenna ports {3000, 3001, …, 3015}, 24 antenna ports {3000, 3001, …, 3023}, and 32 antenna ports {3000, 3001, …, 3031}, and UE configured with higher layer parameter codebookType set to ' typeII-r16'

- The values of N1 and N2 are configured with the higher layer parameter n1-n2-codebookSubsetRestriction-r16. The supported configurations of N1,N2 for a given number of CSI-RS ports and the corresponding values of O1,O2 are given in Table 5.2.2.2.1-2. The number of CSI-RS ports, PCSI-RS, is 2N1N2.

- The values of L, β and pυ are determined by the higher layer parameter paramCombination-r16, where the mapping is given in Table 5.2.2.2.5-1.

- The UE is not expected to be configured with paramCombination-r16 equal to

- 3, 4, 5, 6, 7, or 8 when PCSI-RS=4,

- 7 or 8 when PCSI-RS<32

- 7 or 8 when higher layer parameter typeII-RI-Restriction-r16 is configured with ri=1 for any i>1.

- 7 or 8 when R=2.

- If csi-ReportingBand is configured, the number of subbands is determined based on the subband size configured by the higher-level parameter subbandSize and the total number of PRBs in the bandwidth part according to Table 5.2.1.4-2. Otherwise, the number of subbands equals one and the subband size equals the total number of PRBs in the bandwidth part. UE does not expect to be configured with R =2 for BWP size less than 24 PRBs.

- -------- end of the TP-------

References

- Chairman notes, 3GPP TSG RAN WG1 Meeting #100

- R1-2001215 Feature lead summary for [100e-NR-eMIMO-MUCSI-01]

- 3GPP TS 38.214 V16.1.0 2020-03