3GPP TSG RAN WG1 Meeting #101-e                                         R1-2003466

e-Meeting, May 25th – June 5th, 2020

Source: 	ZTE

Title: 	Maintenance of additional SRS symbols

Agenda item:	6.2.3.1.1

Document for:	Discussion/Decision

Introduction

In Rel-16 LTE, additional SRS symbols are introduced for DL MIMO efficiency enhancement. In this meeting, we provide our views on some issues of the latest endorsed specifications.

Discussion

In Rel-16 LTE, virtual cell ID is introduced for additional SRS symbol(s) or both additional SRS symbol(s) and legacy SRS symbol. In latest TS 36.331 [3], high layer parameters srs-VirtualCellID and srs-VirtualCellID-AllSRS are introduced, which are not aligned with the name of nSRS-Identity and nSRS-Identity-Legacy in TS 36.211. In addition, the value of srs-VirtualCellID-AllSRS is TRUE or FALSE.

Proposal 1: nSRS-Identity and nSRS-Identity-Legacy in TS 36.211 should be replaced with srs-VirtualCellID and srs-VirtualCellID-AllSRS.

In TS 36.213 [2], trigger type 2 SRS is introduced for Rel-16 LTE, but it is not clarified that trigger type 2 SRS corresponds to additional SRS. In addition, it should be clarified that trigger type 0 SRS and trigger type 1 SRS correspond to periodic SRS and aperiodic SRS of basic SRS respectively.

Proposal 2: In order to avoid misalignment between TS 36.211 and TS 36.213, it should be clarified that trigger type 0 and type1 SRS correspond to periodic SRS and aperiodic SRS of basic SRS respectively, trigger type 2 SRS corresponds to additional SRS.

In RAN 1 #100bis-e, there was a discussion about whether trigger type 2 SRS can be applied to the scenario of eMTC and LAA. Considering that there is no use case for eMTC and LAA UEs to transmit SRS on additional SRS symbols, and support of type 2 SRS will impact the current specification/implementation of eMTC and LAA, we prefer not to support trigger type 2 SRS for eMTC and LAA UEs.

Proposal 3: Trigger type 2 SRS is not supported for eMTC and LAA UEs.

Based on above proposals, we provide TP1 and TP2 for the section 5.5.1.5 of TS 36.211 [1] and section 8.2 of TS 36.213 [2] respectively.

  - TP1 for TS 36.211 [1]:

- 5.5.1.5 Determining virtual cell identity for sequence generation The definition of depends on the type of transmission. Transmissions associated with PUSCH: - if no value for is configured by higher layers or if the PUSCH transmission corresponds to a Random Access Response Grant or a retransmission of the same transport block as part of the contention based random access procedure, - otherwise. Transmissions associated with SPUCCH/PUCCH: - if no value for is configured by higher layers, - otherwise. Basic sounding reference signals: - nIDRS=nIDSRS if the value of higher-layer parameter srs-VirtualCellID-AllSRS is TRUE and srs-VirtualCellID is configured, where nIDSRS equals the higher-layer parameter srs-VirtualCellID - nIDRS=NIDcell otherwise. Additional sounding reference signals: - nIDRS=NIDcell if no value for nIDSRS is configured by the higher-layer parameter srs-VirtualCellID - nIDRS=nIDSRS otherwise.

  - TP2 for TS 36.213 [2]:

- 8.2 UE sounding procedure If the UE is configured with a PUCCH-SCell, the UE shall apply the procedures described in this clause for both primary PUCCH group and secondary PUCCH group unless stated otherwise - When the procedures are applied for the primary PUCCH group, the terms 'secondary cell', 'secondary cells', 'serving cell', and 'serving cells' in this clause refer to secondary cell, secondary cells, serving cell or serving cells belonging to the primary PUCCH group respectively unless stated otherwise. - When the procedures are applied for secondary PUCCH group, the terms 'secondary cell', 'secondary cells', 'serving cell' and 'serving cells' in this clause refer to secondary cell, secondary cells (not including the PUCCH-SCell), serving cell, serving cells belonging to the secondary PUCCH group respectively unless stated otherwise. The term 'primary cell' in this clause refers to the PUCCH-SCell of the secondary PUCCH group. A UE shall transmit Sounding Reference Symbol (SRS) on per serving cell SRS resources based on three trigger types: - trigger type 0: higher layer signalling - trigger type 1: DCI formats 0/0A/0B/4/4A/4B/1A/6-0A/6-1A for FDD, TDD, and frame structure type 3 and DCI formats 2B/2C/2D/3B for TDD, and frame structure type 3, and DCI format 7-0A/7-0B/7-1E/7-1F/7-1G for TDD if the UE is configured by higher layers for SRS triggering via DCI format 7-0A and has indicated the capability srs-DCI7-Triggering-FS2-r15/ srs-DCI7-Triggering-FS2-r16 and the UE is configured for SRS triggering with srs-DCI7-TriggeringConfig-r15/ srs-DCI7-Triggering-FS2-r16. - trigger type 2: DCI formats 0/4/1A for FDD and TDD, and DCI formats 2B/2C/2D/3B for TDD, and DCI format 7-0A/7-0B/7-1E/7-1F/7-1G for TDD if the UE is configured by higher layers for SRS triggering via DCI format 7-0A and has indicated the capability srs-DCI7-Triggering-FS2-r16 and the UE is configured for SRS triggering with srs-DCI7-TriggeringConfig-r16. Trigger type 0 and type 1 SRS correspond to periodic SRS and aperiodic SRS of basic SRS respectively in Subclause 5.5.3 of [1]. Trigger type 2 SRS corresponds to additional SRS in Subclause 5.5.3 of [1]. A UE is not expected to be configured with SRS trigger type 0 on a LAA SCell. In case both trigger type 0 and trigger type 1 SRS transmissions would occur in the same subframe in the same serving cell, the UE shall only transmit the trigger type 1 SRS transmission. This prioritization rule shall be applied before other prioritization rules defined in this subclause. In case both trigger type 1 and trigger type 2 SRS transmissions would occur in the same subframe, the UE shall transmit both the trigger type1 and type 2 SRS transmissions. In case both trigger type 0 and trigger type 2 SRS transmissions would occur in the same subframe, the UE shall transmit both the trigger type 0 and type 2 SRS transmissions. <Unchanged parts are omitted>

Conclusion

In this contribution, we provide our proposals and corresponding TPs for LTE additional SRS. The details of the TPs can be found in section 2.

Proposal 1: nSRS-Identity and nSRS-Identity-Legacy in TS 36.211 should be replaced with srs-VirtualCellID and srs-VirtualCellID-AllSRS.

Proposal 2: In order to avoid misalignment between TS 36.211 and TS 36.213, it should be clarified that trigger type 0/1 SRS correspond to periodic SRS and aperiodic SRS of basic SRS respectively, trigger type 2 SRS corresponds to additional SRS.

Proposal 3: Trigger type 2 SRS is not supported for eMTC and LAA UEs.

Reference

- 36.211 g10, E-UTRA Physical channels and modulations (Release 16)

- 36.213 g10, E-UTRA Physical layer procedures (Release 16)

- 36.331 g10, Radio Resource Control (RRC) (Release 16)