3GPP TSG RAN WG1 #101-e                                                                           R1-2003482

e-Meeting, May 25th – June 5th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | xxxx | rev | - | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on SRS for 38.214 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | - |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-05-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - The current description may cause misunderstanding that simultaneous SRS and PUCCH/PUSCH/PRACH transmissions can never be allowed in inter-band CA band combination. However, whether to support simultaneous SRS and PUCCH/PUSCH/PRACH transmissions is up to UE capability ‘parallelTxSRS-PUCCH-PUSCH’ and ‘parallelTxPRACH-SRS-PUCCH-PUSCH’ based on 38.306. - The terminology of ‘inter-band CA band-band combination’ in 38.214 and ‘inter-band CA band combination’ in 38.306 should be aligned. 38.306: parallelTxSRS-PUCCH-PUSCH Indicates whether the UE supports parallel transmission of SRS and PUCCH/ PUSCH across CCs in an inter-band CA band combination.. parallelTxPRACH-SRS-PUCCH-PUSCH Indicates whether the UE supports parallel transmission of PRACH and SRS/PUCCH/PUSCH across CCs in an inter-band CA band combination.. |
| Summary of change: |  | 1.Clarify that SRS and PUCCH/PUSCH transmissions in the same symbol are not expected only if UE does not support the capability in case of inter-band CA band combination. 2. Clarify that SRS and PRACH transmissions in the same symbol are not expected only if UE does not support the capability in case of inter-band CA band combination. 3. Replace ‘inter-band CA band-band combination’ by ‘inter-band CA band combination’ in order to align the terminologies in 38.214 and 38.306. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It may be understood that simultaneous SRS and PUCCH/PUSCH/PRACH transmissions can never be allowed in case of inter-band CA band combination. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: This CR makes 38.214 clearer and aligned with 38.306. There is no isolated impact on either gNB or UE side. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version for this CR. |  |  |  |  |  |  |  |  |

6.2.1	UE sounding procedure

< Unchanged parts are omitted >

In case of intra-band carrier aggregation or in inter-band CA band combination if simultaneous SRS and PUCCH/PUSCH transmissions are not supported by UE, the UE is not expected to be configured with SRS from a carrier and PUSCH/UL DM-RS/UL PT-RS/PUCCH formats from a different carrier in the same symbol.

In case of intra-band carrier aggregation or in inter-band CA band combination if simultaneous SRS and PRACH transmissions are not supported by UE, the UE shall not transmit simultaneously SRS resource(s) from a carrier and PRACH from a different carrier.

< Unchanged parts are omitted >