3GPP TSG RAN WG1 Meeting #101-e	R1-2003532

E-meeting, 25 May –5 June, 2020

Agenda Item:	7.2.6.3

Source:	Huawei, HiSilicon

Title:	Remaining issues on multi-beam enhancements in R16

Document for:	Discussion and Decision

Introduction

The outcome of email discussions on Rel-16 multi-beam enhancements in RAN1#100b-e meeting was summarized in [1]. In this paper, we share our views on several remaining issues. The rest of this paper is structured as follows. Section 2 is on latency/overhead reduction, Section 3 is on L1-SINR and Section 4 is on SCell BFR.

Remaining issues on latency/overhead reduction for BM

Enabling default PL RS in FR1

Reason for changes

In current spec, with default PL RS enabled, UE determines PL RS based on the RS resource with 'QCL-TypeD' in the TCI state or the QCL assumption of a CORESET with the lowest index in the active DL BWP. In addition, for dedicated SRS, if CORESETs are not provided in the active DL BWP, UE determines PL RS based on the RS resource with 'QCL-TypeD' in lowest active TCI state for PDSCH. However, as shown in the agreements listed below, TCI state in FR1 does not contain any RS resource with 'QCL-TypeD'. Therefore, this feature of default PL RS update cannot be used in FR1 and RRC reconfigurations may still be required in FR1 to update the PL RS for PUSCH scheduled with DCI format 0_0, dedicated PUCCH, and dedicated SRS.

Agreement @ RAN1#91 QCL linkage for below 6GHz before RRC; signalling. SSB DMRS for PDCCH w.r.t Doppler shift, Doppler spread, average delay, delay spread, spatial RX parameters (spatial RX parameters are used only for above 6GHz). QCL linkage for below 6GHz after RRC; signalling. TRS DMRS: Doppler shift, Doppler spread, average delay, delay spread; QCL type: A. CSI-RS DMRS: Doppler shift, Doppler spread, average delay, delay spread; QCL type: A.

Observation 1: Existing design of default PL RS is ‘FR2-only’. RRC reconfigurations may still be required to update the PL RS for PUSCH scheduled with DCI format 0_0, dedicated PUCCH, and dedicated SRS in FR1.

From overhead/latency reduction perspective, it is not preferred to do RRC reconfigurations. Such ‘FR2-only’ restriction does not bring any technical advantages but preventing FR1 system to benefit from Rel-16 enhancements. For the feature of default PL RS in particular, it seems that it can be supported in FR1 already via relaxing some unnecessary restrictions, for example, via removing the restriction on ‘QCL-TypeD’ RS, and allow other QCL-Type reference RS to be used as the default PL RS for FR1.

Summary of changes

Allow QCL reference RS other than TypeD RS to be used as default PL RS derived from TCI state.

Consequences if not approved

Default PL RS cannot be supported in FR1.

To summarize above discussions, we have a proposal as follows.

Proposal 1: Allow QCL reference RS other than TypeD RS to be used as default PL RS derived from TCI state.

An indicative TP is provided as below. Please note that we use SRS power control in TS 38.213 Section 7.3.1 as an example. Similar updates are needed for PUSCH power control in TS 38.213 Section 7.1.1 and for PUCCH power control in Section 7.2.1.

< Start of text proposal on TS 38.213 v16.1.0 Section 7.3.1> < Unchanged parts are omitted > - If the UE - is not provided pathlossReferenceRS or SRS-PathlossReferenceRS, - is not provided spatialRelationInfo, and - is provided enableDefaultBeamPlForSRS the UE determines a RS resource index providing a RS resource, if there are two RS indexes in the TCI state or the QCL assumption, the UE selects the one corresponding to 'QCL-TypeD', in - the TCI state or the QCL assumption of a CORESET with the lowest index, if CORESETs are provided in the active DL BWP - the active PDSCH TCI state with lowest ID [6, TS 38.214], if CORESETs are not provided in the active DL BWP < Unchanged parts are omitted > < End of text proposal on TS 38.213 v16.1.0 Section 7.3.1>

Remaining issues on L1-SINR measurement/reporting

NZP+ZP IMR based interference measurement

Reason for changes

NZP+ZP based interference measurement for L1-SINR was captured in the spec but under brackets. In previous meetings, companies had concerns on how to measure from the single ZP IMR if associated CMRs are configured with different QCL-TypeD assumptions. To address this concern, we propose to add a restriction that all CMRs should be QCLed w.r.t ‘QCL-TypeD’ if both NZP IMRs and ZP IMR are configured for L1-SINR.

Summary of changes

Add a restriction that all CMRs should be QCLed w.r.t ‘QCL-TypeD’ if both NZP IMRs and ZP IMR are configured, and remove brackets on NZP+ZP IMR based interference measurement for L1-SINR

Consequences if not approved

NZP+ZP IMR based interference measurement for L1-SINR is not supported.

Proposal 2: If NZP + ZP IMRs are configured for L1-SINR report, support CMRs are QCLed with respect to ‘QCL-TypeD’.

An indicative TP is provided as below.

< Start of text proposal 38.214 v16.1.0 Section 5.2.1.2> < Unchanged parts are omitted > - When three Resource Settings are configured, the first one Resource Setting (given by higher layer parameterresourcesForChannelMeasurement) is for channel measurement on SSB or NZP CSI-RS. The second one (given by either higher layer parameter csi-IM-ResourcesForInterference) is for interference measurement performed on CSI-IM, where each NZP CSI-RS resource set for channel measurement is associated with one CSI-IM resource for interference measurement. The Third one (given by higher layer parameter nzp-CSI-RS-ResourcesForInterference) is for interference measurement performed on 1 port NZP CSI-RS with density 3 REs/RB. - UE expects that the NZP CSI-RS resources in each NZP CSI-RS resource set for channel measurement are QCLed with respect to ‘QCL-TypeD’. < Unchanged parts are omitted > < End of text proposal 38.214 v16.1.0 Section 5.2.1.2>

NZP CMR without QCL indication

Reason for changes

In current spec it is allowed that a periodic NZP CSI-CS resource as CMR of L1-SINR measurement can be configured without QCL indication. In that case, the UE assumption/expectation is unclear. Therefore, it is necessary to specify UE assumption/expectation for CMR with NZP CSI-RS without QCL indication.

Summary of changes

If a periodic NZP CSI-RS resource without QCL indication is configured as CMR for LI-SINR, a UE may assume all instances of the periodic NZP-CSI-RS resource are transmitted with the same downlink spatial domain transmission filter.

Consequences if not approved

Unclear UE assumption/expectation when a periodic NZP CSI-CS resource is configured as CMR for L1-SINR measurement but without QCL indication.

Proposal 3: If no QCL assumption is provided for a periodic NZP CSI-RS resource as CMR for L1-SINR measurement, the UE may assume all the instances of this CSI-RS are transmitted with the same downlink spatial domain transmission filter.

An indicative TP is provided as below.

< Start of text proposal on TS 38.214 v16.1.0 Section 5.2.1.2> #### < Unchanged parts are omitted > For L1-SINR measurement: - When one Resource Setting is configured, the Resource Setting (given by higher layer parameter resourcesForChannelMeasurement) is for channel and interference measurement for L1-SINR computation. UE may assume that same 1 port NZP CSI-RS resource(s) with density 3 REs/RB is used for both channel and interference measurements. - When two Resource Settings are configured, the first one Resource Setting (given by higher layer parameter resourcesForChannelMeasurement) is for channel measurement on SSB or 1 port NZP CSI-RS with density 3 REs/RB and the second one (given by either higher layer parameter csi-IM-ResourcesForInterference or higher layer parameter nzp-CSI-RS-ResourcesForInterference) is for interference measurement performed on CSI-IM or on 1 port NZP CSI-RS with density 3 REs/RB, where each SSB or NZP CSI-RS resource for channel measurement is associated with one CSI-IM resource or one NZP CSI-RS resource for interference measurement by the ordering of the SSB or NZP CSI-RS resource for channel measurement and CSI-IM resource or NZP CSI-RS resource for interference measurement in the corresponding resource sets. The number of SSB(s) or CSI-RS resources for channel measurement equals to the number of CSI-IM resources or the number of NZP CSI-RS resource for interference measurement. - UE may apply 'QCL-TypeD' assumption of the SSB or 'QCL-TypeD' configured to the NZP CSI-RS resource for channel measurement to measure the associated CSI- IM resource or associated NZP CSI-RS resource for interference measurement configured for one CSI reporting - UE may expect that the NZP CSI-RS resource set for channel measurement and the NZP-CSI-RS resource set for interference measurement, if any, are configured with the higher layer parameter repetition. - [When three Resource Settings are configured, the first one Resource Setting (given by higher layer parameterresourcesForChannelMeasurement) is for channel measurement on SSB or NZP CSI-RS. The second one (given by either higher layer parameter csi-IM-ResourcesForInterference) is for interference measurement performed on CSI-IM, where each NZP CSI-RS resource set for channel measurement is associated with one CSI-IM resource for interference measurement. The Third one (given by higher layer parameter nzp-CSI-RS-ResourcesForInterference) is for interference measurement performed on 1 port NZP CSI-RS with density 3 REs/RB.] #### If a periodic NZP CSI-RS resource for channel measurement for L1-SINR computation is configured without the high layer parameter of qcl-InfoperiodicCSI-RS, UE may assume all the instances of the periodic NZP CSI-RS resource are transmitted with the same downlink spatial domain transmission filter. < Unchanged parts are omitted > < End of text proposal on TS 38.214 v16.1.0 Section 5.2.1.2>

Resource multiplexing with NZP IMR

Reason for changes

In Rel-15, for a CSI-RS resource associated with a NZP-CSI-RS-ResourceSet with higher layer parameter repetition set to 'on', the UE shall not expect to be configured with CSI-RS over the symbols during which the UE is also configured to monitor the CORESET. For L1-SINR measurement in Rel-16, each NZP CMR can be associated with one NZP IMR and UE may apply 'QCL-TypeD' assumption of NZP CMR to measure the associated NZP IMR. Following the resource multiplexing rule in Rel-15, it is natural to have that when NZP CMRs are configured with repetition set to 'on', the associated NZP IMR cannot be configured over the symbols during which the UE is also configured to monitor the CORESET.

Summary of changes

For L1-SINR measurement, if the CMR is associated with a NZP-CSI-RS-ResourceSet with higher layer parameter repetition set to 'on', both the CMR and the associated IMR cannot be configured over the symbols during which the UE is also configured to monitor the CORESET.

Consequences if not approved

Possible collisions between NZP IMRs and the CORESETs, and unclear UE behavior.

Proposal 4: If NZP IMRs are configured for L1-SINR report, and associated NZP CMR set is with higher layer parameter repetition set to 'on', the NZP IMRs cannot be configured over the symbols during which the UE is also configured to monitor the CORESET.

An indicative TP is provided as below.

< Start of text proposal on TS 38.214 v16.1.0 Section 5.1.6.1> < Unchanged parts are omitted > For a CSI-RS resource for channel measurement associated with a NZP-CSI-RS-ResourceSet with the higher layer parameter repetition set to 'on', the UE shall not expect to be configured with CSI-RS over the symbols during which the UE is also configured to monitor the CORESET. If the CSI-RS resource for channel measurement is associated with a NZP CSI-RS resource for interference measurement, UE shall not expect the NZP CSI-RS resource for interference measurement to be configured over the symbols during which the UE is also configured to monitor the CORESET. While for other NZP-CSI-RS-ResourceSet configurations, if the UE is configured with a CSI-RS resource and a search space set associated with a CORESET in the same OFDM symbol(s), the UE may assume that the CSI-RS and a PDCCH DM-RS transmitted in all the search space sets associated with CORESET are quasi co-located with 'QCL-TypeD', if 'QCL-TypeD' is applicable. This also applies to the case when CSI-RS and the CORESET are in different intra-band component carriers, if 'QCL-TypeD' is applicable. Furthermore, the UE shall not expect to be configured with the CSI-RS in PRBs that overlap those of the CORESET in the OFDM symbols occupied by the search space set(s). < Unchanged parts are omitted > < End of text proposal on TS 38.214 v16.1.0 Section 5.1.6.1>

Remaining issues on beam failure recovery for SCell

New beam identification from CSI-RS for BM

Reason for changes

It has been agreed in RAN1#96b that new beam identification for SCell can be based on SSB and CSI-RS for beam management, with which CSI-RS for other purposes are not included. However the current spec allows for CSI-RS for other purposes, which is not aligned with the agreement.

Agreement (RAN1#96b) Downlink RS for new beam identification can be based on SSB and CSI-RS for BM

Summary of changes

Capture the agreement that new beam identification for SCell can be based on SSB and CSI-RS for BM only.

Consequences if not approved

Missing agreement and extra UE complexity.

Proposal 5: Clarify that new beam identification for SCell can be based on SSB and CSI-RS for beam management only.

An indicative TP is provided as below.

< Start of text proposal on TS 38.213 v16.1.0 Section 6> < Unchanged parts are omitted > A UE can be provided, for each BWP of a serving cell, a set of periodic CSI-RS resource configuration indexes by failureDetectionResources or beamFailureDetectionResourceList and a set of periodic CSI-RS resource configuration indexes associated with NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, and/or SS/PBCH block indexes by candidateBeamRSList or candidateBeamRSListExt-r16 for radio link quality measurements on the BWP of the serving cell. If the UE is not provided by failureDetectionResources or beamFailureDetectionResourceList for a BWP of the serving cell, the UE determines the set to include periodic CSI-RS resource configuration indexes with same values as the RS indexes in the RS sets indicated by TCI-State for respective CORESETs that the UE uses for monitoring PDCCH and, if there are two RS indexes in a TCI state, the set includes RS indexes with QCL-TypeD configuration for the corresponding TCI states. The UE expects the set to include up to two RS indexes. The UE expects single port RS in the set . < Unchanged parts are omitted > < End of text proposal on TS 38.213 v16.1.0 Section 6>

Summary of proposals

The observations and proposals of this paper are summarized as follows.

Overhead/latency reduction

Observation 1: Existing design of default PL RS is ‘FR2-only’. RRC reconfigurations may still be required to update the PL RS for PUSCH scheduled with DCI format 0_0, dedicated PUCCH, and dedicated SRS in FR1.

Proposal 1: Allow QCL reference RS other than TypeD RS to be used as default PL RS derived from TCI state.

L1-SINR

Proposal 2: If NZP + ZP IMRs are configured for L1-SINR report, support CMRs are QCLed with respect to ‘QCL-TypeD’.

Proposal 3: If no QCL assumption is provided for a periodic NZP CSI-RS resource as CMR for L1-SINR measurement, the UE may assume all the instances of this CSI-RS are transmitted with the same downlink spatial domain transmission filter.

Proposal 4: If NZP IMRs are configured for L1-SINR report, and associated NZP CMR set is with higher layer parameter repetition set to 'on', the NZP IMRs cannot be configured over the symbols during which the UE is also configured to monitor the CORESET.

SCell BFR

Proposal 5: Clarify that new beam identification for SCell can be based on SSB and CSI-RS for beam management only.

- References

3GPP, “RAN1 Chairman’s Notes”, RAN WG1 Meeting #100b-e, e-Meeting, April 20th – 30th, 2020