3GPP TSG RAN WG1 Meeting #101		                     	   	R1-2003633

e-Meeting, May 25th – June 5th, 2020

Source:	CATT

Title:	Remaining issues on UL SRS for NR Positioning

Agenda item:	7.2.8.2

Document for:	Discussion and Decision

Introduction

Most of the issues for Rel-16 NR Positioning Support WI were resolved in RAN1#98bis [1] and RAN1#99 [2]. In RAN1#100bis-e meeting, the following agreements were achieved on several remaining issues on UL SRS for positioning (SRS-Pos) [9]. In this contribution, we discuss some remaining issues on SRS-Pos, including simultaneous SRS-Pos transmission in a single symbol, intra-band collision between SRS-Pos and SRS-MIMO, collision handling between aperiodic SRS-Pos and PUSCH and symbol-specific cyclic shifts for SRS-Pos.

Agreement: # Adopt the following text proposal for Clause 6.2.1.4 of TS 38.214: # ===================== Unchanged parts omitted ====================== For single carrier operations, the UE does not expect to be configured on overlapping symbols with more than one SRS resources configured by the higher layer parameter SRS-PosResource with resourceType of the SRS resources as ‘periodic’. For single carrier operations, the UE does not expect to be triggered to transmit SRS on overlapping symbols with more than one SRS resources configured by the higher layer parameter SRS-Pos-Resource with resourceType of the SRS resources as ‘semi-persistent’ or ‘aperiodic’. ===================== Unchanged parts omitted ====================== Agreement: # Introduce a new UE capability for the number of SRS resources for positioning on a symbol for intra-band CA - FFS: Capability for simultaneous SRS transmission across bands for inter-band CA - Continue discussion on capability for intra-band/inter-band CA, including potential TP to 38.214 to reflect the new capability. Agreement: Change all occurrences of “[SRS-for-positioning]” to the higher layer parameter agreed by RAN2 WG called “SRS-PosResource-r16” in TS 38.211 Section 6.4.1.4 # Conclusion: # For release 16, type3 PHR based on SRS for positioning is not supported. Agreement: - A new UE capability is introduced for the number of simultaneous transmissions of SRS resources for positioning for inter-band CA, where the SRS resources are on different CCs. - The text in TP#A below is added to 38.214 TP#A ===================== Unchanged parts omitted ====================== For intra-band and inter-band CA operations, a UE can simultaneously transmit more than one SRS resources configured by SRS-PosResource on different CCs, subject to UE’s capability provided by [XX] and [YY] respectively. ===================== Unchanged parts omitted ======================. Agreement: Adopt the text proposal TP#B in Section 2.3.1 for TS 38.213 Agreement: Adopt the text proposal TP#D in Section 2.4.1 for TS 38.213 # LS on UL RTOA reference time approved in R1-2003054 # Agreement: # The following TP (TP#2 in the email discussion) was agreed: Start of Text Proposal #1 to the TS 38.214 -------- ------------------------------------------------------- 6.2.1 UE sounding procedure <omitted text> For operations in the same carrier, the UE is not expected to be configured on overlapping symbols with a SRS resource configured by the higher layer parameter srs-PosResource-r16 and a SRS resource configured by the higher layer parameter SRS-Resource with resourceType of both SRS resources as ‘periodic’. For operations in the same carrier, the UE is not expected to be triggered to transmit SRS on overlapping symbols with a SRS resource configured by the higher layer parameter srs-PosResource-r16 and a SRS resource configured by the higher layer parameter SRS-Resource with resourceType of both SRS resources as ‘semi-persistent’ or ‘aperiodic’. <omitted text> End of Text Proposal #1 to the TS 38.214------------------------------------------------------------------.

Discussion

Simultaneous SRS-Pos transmission in a single symbol

In RAN1#100bis-e meeting, the following agreements were achieved on simultaneous SRS-Pos transmission in a single symbol.

Agreement: # Adopt the following text proposal for Clause 6.2.1.4 of TS 38.214: # ===================== Unchanged parts omitted ====================== For single carrier operations, the UE does not expect to be configured on overlapping symbols with more than one SRS resources configured by the higher layer parameter SRS-PosResource with resourceType of the SRS resources as ‘periodic’. For single carrier operations, the UE does not expect to be triggered to transmit SRS on overlapping symbols with more than one SRS resources configured by the higher layer parameter SRS-Pos-Resource with resourceType of the SRS resources as ‘semi-persistent’ or ‘aperiodic’. ===================== Unchanged parts omitted ======================

We suggest the above agreed TP to be changed to the following proposed TP-A, in order to align the description to the common wording in the specifications, and avoid ambiguity of single carrier operations by using the same carrier instead of single carrier.

Proposal 1: Adopt the following text proposal (TP-A) for simultaneous SRS-Pos transmission in a single symbol in 38.214:

# Proposed TP-A # -----------------------------------------------Start of Text Proposal for 38.214---------------------------------------------- # 6.2.1.4 UE sounding procedure for positioning purposes # ----------------------------------------------------- Unchanged part omitted ------------------------------------------------ For operations in the same carrier, the UE is not expected to be configured on overlapping symbols with more than one SRS resources configured by the higher layer parameter srs-PosResource-r16 with resourceType of the SRS resources as ‘periodic’. For operations in the same carrier, the UE is not expected to be triggered to transmit SRS on overlapping symbols with more than one SRS resources configured by the higher layer parameter srs-PosResource-r16 with resourceType of the SRS resources as ‘semi-persistent’ or ‘aperiodic’. ----------------------------------------------------- Unchanged part omitted ------------------------------------------------ --------------------------------------------------------End of Text Proposal --------------------------------------------------

Intra-band collision between SRS-Pos and SRS-MIMO

In RAN1#100bis-e meeting, the following agreements were achieved on Intra-band collision between SRS-Pos and SRS-MIMO.

Agreement: - Introduce a new UE capability for the number of SRS resources for positioning on a symbol for intra-band CA - A new UE capability is introduced for the number of simultaneous transmissions of SRS resources for positioning for inter-band CA, where the SRS resources are on different CCs. - The text in TP#A below is added to 38.214 TP#A ===================== Unchanged parts omitted ====================== For intra-band and inter-band CA operations, a UE can simultaneously transmit more than one SRS resources configured by SRS-PosResource on different CCs, subject to UE’s capability provided by [XX] and [YY] respectively. ===================== Unchanged parts omitted ======================. Agreement: The following TP (TP#2 in the email discussion) was agreed: Start of Text Proposal #1 to the TS 38.214 --------------------------------------------------------------- 6.2.1 UE sounding procedure <omitted text> For operations in the same carrier, the UE is not expected to be configured on overlapping symbols with a SRS resource configured by the higher layer parameter srs-PosResource-r16 and a SRS resource configured by the higher layer parameter SRS-Resource with resourceType of both SRS resources as ‘periodic’. For operations in the same carrier, the UE is not expected to be triggered to transmit SRS on overlapping symbols with a SRS resource configured by the higher layer parameter srs-PosResource-r16 and a SRS resource configured by the higher layer parameter SRS-Resource with resourceType of both SRS resources as ‘semi-persistent’ or ‘aperiodic’. <omitted text> End of Text Proposal #1 to the TS 38.214 --------------------------------------------------------------------.

Although it had been agreed that introducing a new UE capability for the number of SRS resources for positioning on a symbol for intra-band CA, which means multiple SRS-Pos may be transmitted on a symbol in different CCs subjected to UE’s capability, we see some issues for supporting SRS-Pos and SRS-MIMO transmitted on the same symbol in different CCs. For intra-band CA case, suppose SRS-Pos and SRS-MIMO are transmitted in different CCs, intra-band collision for inter-feature “SRS-Pos + SRS-MIMO” on overlapping symbols is different with intra-feature “SRS-Pos + SRS-Pos”, as SRS-MIMO may take up large proportion of Tx power on overlapping symbols, therefore will affect the transmission power of SRS-Pos, or vice versa. However, “SRS-Pos + SRS-Pos” can easily avoid such issue by intra-feature coordination.

Therefore, we suggest extending the above TP from the same carrier case to intra-band CA case with the following proposed TP-B.

Proposal 2: Adopt the following text proposal (TP-B) for intra-band collision between SRS-Pos and SRS-MIMO in 38.214:

# Proposed TP-B # -----------------------------------------------Start of Text Proposal for 38.214---------------------------------------------- # 6.2.1 UE sounding procedure # ----------------------------------------------------- Unchanged part omitted ------------------------------------------------ For operations in the same carrier or intra-band CA case(when a SRS resource configured by the higher layer parameter srs-PosResource-r16 and a SRS resource configured by the higher layer parameter SRS-Resource are in different component carriers), the UE is not expected to be configured on overlapping symbols with a SRS resource configured by the higher layer parameter srs-PosResource-r16 and a SRS resource configured by the higher layer parameter SRS-Resource with resourceType of both SRS resources as ‘periodic’. For operations in the same carrier or intra-band CA case(when a SRS resource configured by the higher layer parameter srs-PosResource-r16 and a SRS resource configured by the higher layer parameter SRS-Resource are in different component carriers), the UE is not expected to be triggered to transmit SRS on overlapping symbols with a SRS resource configured by the higher layer parameter srs-PosResource-r16 and a SRS resource configured by the higher layer parameter SRS-Resource with resourceType of both SRS resources as ‘semi-persistent’ or ‘aperiodic’. ----------------------------------------------------- Unchanged part omitted ------------------------------------------------ --------------------------------------------------------End of Text Proposal --------------------------------------------------

Collision handling between aperiodic SRS-Pos and PUSCH

In this section, the issue on collision handing between aperiodic SRS-Pos and PUSCH and related solution will be discussed. Before discussing the issue on collision handling between aperiodic SRS-Pos and PUSCH, we would like to explain the necessity of solving the issue.

In RAN2#109bis-e meeting, RAN2 have not decided whether support aperiodic SRS-Pos in Rel-16. In this meeting, RAN2 agreed to send an LS to RAN3, indicating that the RAN2 signalling can currently configure aperiodic SRS-Pos, but RAN2 need to know if it the specification effort is feasible to complete from RAN3 perspective.

Although RAN2 has not yet decided whether to support aperiodic SRS-Pos, however, since RAN1 has agreed that aperiodic SRS-Pos needs to be supported in RAN1 scope, the issue on collision between aperiodic SRS-Pos and PUSCH still needs to be solved to finish the specifications supporting aperiodic SRS-Pos in RAN1’s scope.

In RAN1#99 meeting [2], the following rules for collision handling between SRS-Pos and PUCCH/PUSCH were agreed.

Agreement: The SRS collision rules for Rel-15 are reused for Rel-16 for SRS for positioning. Agreement: If the SRS for positioning collides with PUSCH, the SRS is dropped in the symbols where the collision occurs. - Note: The phase continuity between transmitted symbols on either side of a transmission gap may not be maintained.

In the email discussion [100e-NR-Pos-ULRS-02] in RAN1#100-e, RAN1 reached the agreements about collision handling between SRS and PUSCH as follows [8],

Agreement: The text proposal in Section 3.1 of R1-2001287 for TS38.214 is agreed. ************* start TP for 38.214 *************** 6.2.1 UE sounding procedure ===================== Unchanged part omitted ======================== If a UE is not configured with [intraUEPrioritization] and PUSCH and SRS configured by SRS-Resource are transmitted in the same slot on a serving cell, the UE may only be configured to transmit the SRS after the transmission of the PUSCH and the corresponding DM-RS. If a UE is configured with [intraUEPrioritization] and a PUSCH transmission would overlap in time with an SRS transmission on a serving cell, the UE does not transmit the SRS in the overlapping symbol(s). ==================== Unchanged part omitted ======================= ************* end TP for 38.214 ***************

In Rel-15, SRS occupies at most 4 symbols in one slot, and it will be configured to be transmitted after the transmission of the PUSCH, so Rel-15 SRS will not collide with a PUSCH transmission. In Rel-16, SRS-Pos can be configured to occupy at most 12 symbols in one slot to expand its coverage and facilitate reception of neighboring gNBs. Due to limited uplink resource, a PUSCH transmission may be scheduled in overlapped symbols in one slot with an SRS-Pos transmission on a serving cell. In this case, the collision between SRS-Pos and PUSCH will occur. According to the agreement above, SRS-Pos is dropped in the overlapped symbols where the collision occurs. Moreover, in TS 38.214 [10], there is the description on the collision handling between PUCCH and SRS as follows,

- For PUCCH and SRS on the same carrier, a UE shall not transmit SRS when semi-persistent and periodic SRS are configured in the same symbol(s) with PUCCH carrying only CSI report(s), or only L1-RSRP report(s), or only L1-SINR report(s).

- A UE shall not transmit SRS when semi-persistent or periodic SRS is configured or aperiodic SRS is triggered to be transmitted in the same symbol(s) with PUCCH carrying HARQ-ACK, link recovery request and/or SR.

- PUCCH shall not be transmitted when aperiodic SRS is triggered to be transmitted to overlap in the same symbol with PUCCH carrying semi-persistent/periodic CSI report(s) or semi-persistent/periodic L1-RSRP report(s) only, or only L1-SINR report(s).

The current collision rules between SRS-Pos and PUCCH/PUSCH is shown in Table 1:

Table 1: Collision rules between SRS-Pos and PUCCH/PUSCH in current spec

| PUCCH/PUSCH SRS-Pos | PUCCH carrying semi-persistent/periodic CSI report(s) or semi-persistent/periodic L1-RSRP report(s) only, or only L1-SINR report(s) | PUCCH carrying HARQ-ACK, link recovery request and/or SR | PUSCH |
| --- | --- | --- | --- |
| Semi-persistent and Periodic SRS-Pos | Drop SRS-Pos | Drop SRS-Pos | Drop SRS-Pos |
| Aperiodic SRS-Pos | Drop PUCCH | Drop SRS-Pos | Drop SRS-Pos |

From the above collision rules marked as BLUE color in Table 1, it can be observed that PUCCH and PUSCH have different behavior when aperiodic SRS-Pos collides with PUSCH and PUCCH carrying semi-persistent/periodic CSI report(s) or semi-persistent/periodic L1-RSRP report(s) only, or only L1-SINR report(s). As the aperiodic SRS has a higher transmission priority than semi-persistent/periodic CSI report(s) or semi-persistent/periodic L1-RSRP report(s) only, or only L1-SINR report(s), and considering that aperiodic SRS-Pos will be received by multiple TRPs involved in positioning, aperiodic SRS-Pos should has a higher transmission priority than PUSCH, and the aperiodic SRS should transmitted in the overlapped symbols when it collides with PUSCH. Therefore, we suggest the following rule of collision handling between SRS-Pos and PUSCH:

- PUSCH shall not be transmitted when aperiodic SRS configured by the higher layer parameter [SRS-for-positioning] is triggered to be transmitted to overlap in the same symbol with PUSCH.

The proposed collision rules between SRS-Pos and PUCCH/PUSCH is shown in Table 2, the changed collision rule is marked as RED color:

Table 2: Proposed collision rules between SRS-Pos and PUCCH/PUSCH

| PUCCH/PUSCH SRS-Pos | PUCCH carrying semi-persistent/periodic CSI report(s) or semi-persistent/periodic L1-RSRP report(s) only, or only L1-SINR report(s) | PUCCH carrying HARQ-ACK, link recovery request and/or SR | PUSCH |
| --- | --- | --- | --- |
| Semi-persistent and Periodic SRS-Pos | Drop SRS-Pos | Drop SRS-Pos | Drop SRS-Pos |
| Aperiodic SRS-Pos | Drop PUCCH | Drop SRS-Pos | Drop PUSCH |

Proposal 3: Aperiodic SRS-Pos should have a higher transmission priority than PUSCH, and PUSCH should be dropped in the overlapped symbols when colliding with aperiodic SRS-Pos.

Proposal 4: Adopt the following text proposal (TP-C) for collision handling between SRS-Pos and PUSCH in 38.214:

# Proposed TP-C -------------------------------------------------Start of Text Proposal for 38.211-------------------------------------------- 6.2.1 UE sounding procedure ----------------------------------------------------- Unchanged part omitted ------------------------------------------------- For PUCCH and SRS on the same carrier, a UE shall not transmit SRS when semi-persistent and periodic SRS are configured in the same symbol(s) with PUCCH carrying only CSI report(s), or only L1-RSRP report(s), or only L1-SINR report(s). A UE shall not transmit SRS when semi-persistent or periodic SRS is configured or aperiodic SRS is triggered to be transmitted in the same symbol(s) with PUCCH carrying HARQ-ACK, link recovery request (as defined in clause 9.2.4 of [6, 38.213]) and/or SR. In the case that SRS is not transmitted due to overlap with PUCCH, only the SRS symbol(s) that overlap with PUCCH symbol(s) are dropped. PUCCH shall not be transmitted when aperiodic SRS is triggered to be transmitted to overlap in the same symbol with PUCCH carrying semi-persistent/periodic CSI report(s) or semi-persistent/periodic L1-RSRP report(s) only, or only L1-SINR report(s). For PUSCH and SRS on the same carrier, PUSCH shall not be transmitted when aperiodic SRS configured by the higher layer parameter srs-PosResource-r16 is triggered to be transmitted to overlap in the same symbol with PUSCH. ----------------------------------------------------- Unchanged part omitted -------------------------------------------- --- --------------------------------------------------------End of Text Proposal -------------------------------------------------

Symbol-specific cyclic shifts for SRS-Pos

For Rel-16 positioning, staggered patterns were agreed for the transmission of the UL SRS for positioning (SRS-Pos) as shown in the following agreement [2]:

Agreement: (RAN1#99)

For each pair of comb size and number of symbols of SRS, there is one single RE pattern. The patterns in the table below are selected

| Number of symbols / Comb size | 1 | 2 | 4 | 8 | 12 |
| --- | --- | --- | --- | --- | --- |
| 2 | {0} | {0, 1} | {0, 1, 0, 1} | N/A | N/A |
| 4 | N/A | {0, 2} | {0,2,1,3}, | {0,2,1,3, 0,2,1,3} | {0,2,1,3, 0,2,1,3, 0,2,1,3} |
| 8 | N/A | N/A | {0,4,2,6} | {0,4,2,6,1,5,3,7} | {0,4,2,6,1,5,3,7,0,4,2,6} |

With the introduction of the staggered patterns for SRS-Pos, when the formula of the SRS cyclic shifts is re-used for the SRS-Pos, there are a number of potential issues. For example, there are phase discontinuities when a staggered SRS-Pos pattern is de-staggered (combined) for the SRS-Pos detection at the receiver. Also, when multiple UEs are multiplexed to the same REs for the transmission SRS-Pos based on the assignment of different cyclic shifts, the phase increments for different UEs on the REs are different. This may prevent the gNB from detecting the SRS-Pos signals from multiple UEs with the same correlation process. To overcome these issues, introducing different formulas for cyclic shifts for SRS-Pos was discussed in previous meetings without reaching a consensus [3].

In Rel-16, the maximum number of SRS resources per set for positioning is defined to be 16, and the maximum number of supported SRS resource sets for positioning can be up to 16 resource sets per BWP for a UE. In addition, for supporting the need to allocating many UEs simultaneously, the number of bits for the sequence ID for SRS-Pos is increased to 16. Furthermore, Rel-16 supports the configuration of a SRS resource with NsymbSRS∈1,2,4,8,12 symbols located in anywhere is a UL slot and comb-size {2, 4, 8}. Therefore, the detection of SRS-Pos signals is expected to be a much more difficult than the detection of the SRS signals, especially for the SRS-Pos from remote neighboring TRPs.

According to TS 38.211, SRS/SRS-Pos sequence in a OFDM symbol l  will be generated according to

anKTC+k0(pi),l=βSRSejαnru,vnn=0,1,…,Msc,bRS-1l=0,1,...,NsymbSRS-10otherwise

α=2πnSRScsnSRScs,max

k0(pi)=nshiftNscRB+kTC(pi)+koffsetl mod KTC

Where KTC∈{2,4,8}  is the come-size, Msc,bSRS  is the SRS-Pos sequence length. The cyclic shift applies to each element of the base sequence ru,vn is related to the element index n and the configured cyclic shift index nSRScs∈{0, 1, …., nSRScs, max-1} , nSRScs, max={8, 12,6} for KTC∈{2,4,8} respectively. For SRS, koffsetl=0, and for SRS-Pos the quantity koffsetl is given the SRS-Pos staggering pattern, as shown in Table 3.

Table 3: The offset koffsetl  for SRS-Pos

| KTC | koffset0, …, koffsetNsymbSRS-1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | NsymbSRS=1 | NsymbSRS=2 | NsymbSRS=4 | NsymbSRS=8 | NsymbSRS=12 |
| 2 | 0 | 0,1 | 0,1,0,1 | - | - |
| 4 | - | 0, 2 | 0, 2, 1, 3 | 0, 2, 1, 3, 0, 2, 1, 3 | 0, 2, 1, 3, 0, 2, 1, 3, 0, 2, 1, 3 |
| 8 | - | - | 0, 4, 2, 6 | 0, 4, 2, 6, 1, 5, 3, 7 | 0, 4, 2, 6, 1, 5, 3, 7, 0, 4, 2, 6 |

For SRS, if we use comb-size 4 as an example, the maximum number of cyclic shifts M=nSRScs, max is 12. If we assume one UE i is configured for SRS transmission with cyclic shift indexes ni∈{0, . 1, …, 11} and base sequence run, we have the mapping of SRS sequences to the REs as shown in Table 4, where  αi=j2πni12

Table 4: Mapping of SRS sequences (UE i, Comb-size 4, Comb-size 4)

| … | … | … | … | … | … |
| --- | --- | --- | --- | --- | --- |
| 4m+3 |  |  |  |  |  |
| 4m+2 |  |  |  |  |  |
| 4m+1 |  |  |  |  |  |
| 4m | ejmairum | ejmairum | ejmairum | ejmairum | 4ejmairum |
| … |  |  |  |  |  |
| … | … | … | … | … | … |
| 11 |  |  |  |  |  |
| 10 |  |  |  |  |  |
| 9 |  |  |  |  |  |
| 8 | ej2airu2 | ej2airu2 | ej2airu2 | ej2airu2 | 4ej2airu2 |
| 7 |  |  |  |  |  |
| 6 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 4 | ejairu1 | ejairu1 | ejairu1 | ejairu1 | 4ejairu1 |
| 3 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 1 |  |  |  |  |  |
| 0 | ru0 | ru0 | ru0 | ru0 | 4ru0 |
| Subcarrier ↑ Symbols → | 0 | 1 | 2 | 3 | Combined |

As shown in Table 4, SRS sequence has the same phase incensement of αi=j2πni12 between the SRS REs for UE i.

For SRS-Pos, however, due to the staggered pattern with koffsetl={0, 2, 1, 3}, the mapping of SRS-Pos sequences is shown in Table 5. It shows that different phase offset incensements between the SRS-Pos REs for UE i.

Table 5: Mapping of SRS-Pos sequences (UE i, Comb-size 4, SRS-Pos pattern (0, 2, 1, 3))

| … | … | … | … | … | … |
| --- | --- | --- | --- | --- | --- |
| 4m+3 |  |  |  | ejmairum | ejmairum |
| 4m+2 |  | ejmairum |  |  | ejmairum |
| 4m+1 |  |  | ejmairum |  | ejmairum |
| 4m | ejmairum |  |  |  | ejmairum |
| … |  |  |  |  |  |
| … | … | … | … | … | … |
| 11 |  |  |  | ej2airu2 | ej2airu2 |
| 10 |  | ej2airu2 |  |  | ej2airu2 |
| 9 |  |  | ej2airu2 |  | ej2airu2 |
| 8 | ej2airu2 |  |  |  | ej2airu2 |
| 7 |  |  |  | ejairu2 | ejairu1 |
| 6 |  | ejairu1 |  |  | ejairu1 |
| 5 |  |  | ejairu1 |  | ejairu1 |
| 4 | ejairu1 |  |  |  | ejairu1 |
| 3 |  |  |  | ru0 | ru0 |
| 2 |  | ru0 |  |  | ru0 |
| 1 |  |  | ru0 |  | ru0 |
| 0 | ru0 |  |  |  | ru0 |
| Subcarrier ↑ Symbols → | 0 | 1 | 2 | 3 | Combined |

Now let us assume there are total N UEs that are assigned to the same REs, but with different cyclic shift indexes ni∈{0, . 1, …, N-1}. Then, under the same assumption of comb-size 4, with the staggered pattern with koffsetl={0, 2, 1, 3}, the mappings of SRS-Pos sequences are shown in Table 6. It shows different phase offset incensements between the SRS-Pos subcarriers. It is known that the mapping makes it difficult to remove the cyclic shifts for the detection for the SRS-Pos, and causes aliasing problem of correlation peaks in SRS-Pos detection as discussed in previous meetings ([3-7]).

Table 6: Mapping of SRS-Pos sequences (N UEs, Comb-size 4, SRS-Pos pattern (0, 2, 1, 3), the ∑=i=0N-1

| … | … | … | … | … | … |
| --- | --- | --- | --- | --- | --- |
| 4m+3 |  |  |  | ∑ejmairum | rum∑ejmai |
| 4m+2 |  | ∑ejmairum |  |  | rum∑ejmai |
| 4m+1 |  |  | ∑ejmairum |  | rum∑ejmai |
| 4m | ∑ejmairum |  |  |  | rum∑ejmai |
| … |  |  |  |  |  |
| … | … | … | … | … | … |
| 11 |  |  |  | ∑ej2airu2 | ru2∑ej2ai |
| 10 |  | ∑ej2airu2 |  |  | ru2∑ej2ai |
| 9 |  |  | ∑ej2airu2 |  | ru2∑ej2ai |
| 8 | ∑ej2airu2 |  |  |  | ru2∑ej2ai |
| 7 |  |  |  | ∑ ejairu1 | ru1∑ejai |
| 6 |  | ∑ ejairu1 |  |  | ru1∑ejai |
| 5 |  |  | ∑ ejairu1 |  | ru1∑ejai |
| 4 | ∑ ejairu1 |  |  |  | ru1∑ejai |
| 3 |  |  |  | Nru0 | Nru0 |
| 2 |  | Nru0 |  |  | Nru0 |
| 1 |  |  | Nru0 |  | Nru0 |
| 0 | Nru0 |  |  |  | Nru0 |
| Subcarrier ↑ Symbols → | 0 | 1 | 2 | 3 | Combined |

A number of solutions were proposed to resolve above problem with the similar idea to introducing an adjustment of the cyclic shifts as a function of the staggered pattern, such that the phase offset incensement between the SRS-Pos subcarriers to be the same ([3-7]), which can be summarized into the following two options in [3]:

Option 1 : ru,v(αi,δ,l')n= ejαKTCn+ fl´maxKTC/NsymbSRS,1ru,vn if configured ru,vα,δn=ejαnru,vn, otherwise Option 2: ru,vαi,δ,l'n=ru,vα,δn ejfl´KTCα' α'=α+2πa α' configured via nSRSeff∈{0,…,KTCnmax-1} and a=nSRSeffnSRScs,max nSRSeff is configurable (range for cyclicshift is extended) Note: the maximum value of cyclic shift nSRScs,max is not changed

In this paper, we propose a similar, but a simpler solution to the cyclic shits of the SRS-Pos as follows:

anKTC+k0(pi),l=βSRSejαnru,vnejγl'n=0,1,…,Msc,bRS-1l=0,1,...,NsymbSRS-10otherwise

where

γl'=αimodk0pi,KTCKTc

The k0pi is the frequency-domain starting position of symbol l'.

With above change, we have the same phase increment for the SRS-Pos symbols, and the aliasing problem due to the uneven phase increment should be resolved as shown in Table 7.

Table 7: Mapping of SRS-Pos sequences (Comb-size 4, SRS-Pos pattern (0, 2, 1, 3)

| … | … | … | … | … | … |
| --- | --- | --- | --- | --- | --- |
| 4m+3 |  |  |  | ej3/4αiejmairim | ej3/4αiejmairim |
| 4m+2 |  | ej2/4αi ejmairim |  |  | ej2/4αiejmairim |
| 4m+1 |  |  | ej1/4αiejmairim |  | ej1/4αiejmairim |
| 4m | ejmairim |  |  |  | ejmairim |
| … |  |  |  |  |  |
| … | … | … | … | … | … |
| 11 |  |  |  | ej3/4αiej2airi2 | ej3/4αiej2airi2 |
| 10 |  | ej2/4αi ej2airi2 |  |  | ej2/4αiej2airi2 |
| 9 |  |  | ej1/4αiej2airi2 |  | ej1/4αiej2airi2 |
| 8 | ej2airi2 |  |  |  | ej2airi2 |
| 7 |  |  |  | ej3/4αiejairi1 | ej3/4αiejairi1 |
| 6 |  | ej2/4αi ejairi1 |  |  | ej2/4αiejairi1 |
| 5 |  |  | ej1/4αiejairi1 |  | ej1/4αiejairi1 |
| 4 | ejairi1 |  |  |  | ejairi1 |
| 3 |  |  |  | ej3/4αiri0 | ej3/4αiri0 |
| 2 |  | ej2/4αi ejairi0 |  |  | ej2/4αiri0 |
| 1 |  |  | ej1/4αiri0 |  | ej1/4αiri0 |
| 0 | ri0 |  |  |  | ri0 |
| Subcarrier ↑ Symbols → | 0 | 1 | 2 | 3 | Combined |

| … | … | … | … | … | … |
| --- | --- | --- | --- | --- | --- |
| 4m+3 |  |  |  | ∑ej(m+3/4)airum | rum∑ej(m+3/4)ai |
| 4m+2 |  | ∑ej(m+2/4)airum |  |  | rum∑ej(m+2/4)ai |
| 4m+1 |  |  | ∑ej(m+1/4)airum |  | rum∑ej(m+1/4)ai |
| 4m | ∑ejmairum |  |  |  | rum∑ejmai |
| … |  |  |  |  |  |
| … | … | … | … | … | … |
| 11 |  |  |  | ∑ej(2+3/4)airu2 | ru2∑ej(2+3/4)ai |
| 10 |  | ∑ej(2+2/4)airu2 |  |  | ru2∑ej(2+2/4)ai |
| 9 |  |  | ∑ej(2+1/4)airu2 |  | ru2∑ej(2+1/4)ai |
| 8 | ∑ej2airu2 |  |  |  | ru2∑ej2ai |
| 7 |  |  |  | ∑ej(1+3/4)airu1 | ru1∑ej(1+3/4)ai |
| 6 |  | ∑ej(1+2/4)airu1 |  |  | ru1∑ej(1+2/4)ai |
| 5 |  |  | ∑ej(1+1/4)airu1 |  | ru1ej(1+1/4)ai |
| 4 | ∑ ejairu1 |  |  |  | ru1∑ejai |
| 3 |  |  |  | ∑ej(3/4)airu0 | ru0∑ej(3/4)ai |
| 2 |  | ∑ej(2/4)airu0 |  |  | ru0∑ej(2/4)ai |
| 1 |  |  | ∑ej(1/4)airu0 |  | ru0∑ej(1/4)ai |
| 0 | Nru0 |  |  |  | Nru0 |
| Subcarrier ↑ Symbols → | 0 | 1 | 2 | 3 | Combined |

Proposal 5: Symbol-specific cyclic shifts for SRS-Pos should be supported in order to keep phase continuities when a staggered SRS-Pos pattern is de-staggered for the SRS-Pos detection at the receiver.

Proposal 6: Adopt the following text proposal (TP-D) for the generation of cyclic shits for the SRS-Pos in 38.214:

# Proposed TP-D -----------------------------------------------Start of Text Proposal for 38.211------------------------------------------- - 6.4.1.4.3 Mapping to physical resources When SRS is transmitted on a given SRS resource, the sequence rpi(n,l') for each OFDM symbol and for each of the antenna ports of the SRS resource shall be multiplied with the amplitude scaling factor in order to conform to the transmit power specified in [5, 38.213] and mapped in sequence starting with to resource elements in a slot for each of the antenna ports according to aKTCk'+k0(pi),l'+l0(pi)=1NapβSRSrpik',l'ejγl'k'=0,1,…,Msc,bRS-1l'=0,1,...,NsymbSRS-10otherwise γl'=αimodk0pi,KTCKTc The k0(pi) is the frequency-domain starting position of symbol l'. ----------------------------------------------------- Unchanged part omitted ------------------------------------------ --- -------------------------------------------------------End of Text Proposal -------------------------------------------------

Conclusions

In this contribution, we discuss several remaining issues on UL SRS for NR positioning. Based on the discussion, we propose:

Proposal 1: Adopt the following text proposal (TP-A) for simultaneous SRS-Pos transmission in a single symbol in 38.214:

# Proposed TP-A # -----------------------------------------------Start of Text Proposal for 38.214---------------------------------------------- # 6.2.1.4 UE sounding procedure for positioning purposes # ----------------------------------------------------- Unchanged part omitted ------------------------------------------------ For operations in the same carrier, the UE is not expected to be configured on overlapping symbols with more than one SRS resources configured by the higher layer parameter SRS-PosResource-r16 with resourceType of the SRS resources as ‘periodic’. For operations in the same carrier, the UE is not expected to be triggered to transmit SRS on overlapping symbols with more than one SRS resources configured by the higher layer parameter SRS-PosResource-r16 with resourceType of the SRS resources as ‘semi-persistent’ or ‘aperiodic’. ----------------------------------------------------- Unchanged part omitted ------------------------------------------------ --------------------------------------------------------End of Text Proposal --------------------------------------------------

Proposal 2: Adopt the following text proposal (TP-B) for intra-band collision between SRS-Pos and SRS-MIMO in 38.214:

# Proposed TP-B # -----------------------------------------------Start of Text Proposal for 38.214---------------------------------------------- # 6.2.1 UE sounding procedure # ----------------------------------------------------- Unchanged part omitted ------------------------------------------------ For operations in the same carrier or intra-band CA case(when a SRS resource configured by the higher layer parameter srs-PosResource-r16 and a SRS resource configured by the higher layer parameter SRS-Resource are in different component carriers), the UE is not expected to be configured on overlapping symbols with a SRS resource configured by the higher layer parameter srs-PosResource-r16 and a SRS resource configured by the higher layer parameter SRS-Resource with resourceType of both SRS resources as ‘periodic’. For operations in the same carrier or intra-band CA case(when a SRS resource configured by the higher layer parameter srs-PosResource-r16 and a SRS resource configured by the higher layer parameter SRS-Resource are in different component carriers), the UE is not expected to be triggered to transmit SRS on overlapping symbols with a SRS resource configured by the higher layer parameter srs-PosResource-r16 and a SRS resource configured by the higher layer parameter SRS-Resource with resourceType of both SRS resources as ‘semi-persistent’ or ‘aperiodic’. ----------------------------------------------------- Unchanged part omitted ------------------------------------------------ --------------------------------------------------------End of Text Proposal --------------------------------------------------

Proposal 3: Aperiodic SRS-Pos should have a higher transmission priority than PUSCH, and PUSCH should be dropped in the overlapped symbols when colliding with aperiodic SRS-Pos.

Proposal 4: Adopt the following text proposal (TP-C) for collision handling between SRS-Pos and PUSCH in 38.214:

# Proposed TP-C -------------------------------------------------Start of Text Proposal for 38.211-------------------------------------------- 6.2.1 UE sounding procedure ----------------------------------------------------- Unchanged part omitted ------------------------------------------------- For PUCCH and SRS on the same carrier, a UE shall not transmit SRS when semi-persistent and periodic SRS are configured in the same symbol(s) with PUCCH carrying only CSI report(s), or only L1-RSRP report(s), or only L1-SINR report(s). A UE shall not transmit SRS when semi-persistent or periodic SRS is configured or aperiodic SRS is triggered to be transmitted in the same symbol(s) with PUCCH carrying HARQ-ACK, link recovery request (as defined in clause 9.2.4 of [6, 38.213]) and/or SR. In the case that SRS is not transmitted due to overlap with PUCCH, only the SRS symbol(s) that overlap with PUCCH symbol(s) are dropped. PUCCH shall not be transmitted when aperiodic SRS is triggered to be transmitted to overlap in the same symbol with PUCCH carrying semi-persistent/periodic CSI report(s) or semi-persistent/periodic L1-RSRP report(s) only, or only L1-SINR report(s). For PUSCH and SRS on the same carrier, PUSCH shall not be transmitted when aperiodic SRS configured by the higher layer parameter [SRS-for-positioning] is triggered to be transmitted to overlap in the same symbol with PUSCH. ----------------------------------------------------- Unchanged part omitted -------------------------------------------- --- --------------------------------------------------------End of Text Proposal -------------------------------------------------

Proposal 5: Symbol-specific cyclic shifts for SRS-Pos should be supported in order to keep phase continuities when a staggered SRS-Pos pattern is de-staggered for the SRS-Pos detection at the receiver.

Proposal 6: Adopt the following text proposal (TP-D) for the generation of cyclic shits for the SRS-Pos in 38.214:

# Proposed TP-D -----------------------------------------------Start of Text Proposal for 38.211------------------------------------------- - 6.4.1.4.3 Mapping to physical resources When SRS is transmitted on a given SRS resource, the sequence rpi(n,l') for each OFDM symbol and for each of the antenna ports of the SRS resource shall be multiplied with the amplitude scaling factor in order to conform to the transmit power specified in [5, 38.213] and mapped in sequence starting with to resource elements in a slot for each of the antenna ports according to aKTCk'+k0(pi),l'+l0(pi)=1NapβSRSrpik',l'ejγl'k'=0,1,…,Msc,bRS-1l'=0,1,...,NsymbSRS-10otherwise γl'=αimodk0pi,KTCKTc The k0(pi) is the frequency-domain starting position of symbol l'. ----------------------------------------------------- Unchanged part omitted ------------------------------------------ --- -------------------------------------------------------End of Text Proposal -------------------------------------------------

References

- 3GPP RAN1 Chairman’s Notes of TSG RAN WG1 Meeting #98-bis

- 3GPP RAN1 Chairman’s Notes of TSG RAN WG1 Meeting #99

- R1-1913507, “Session notes #2 on offline discussion 7.2.10.2 UL Reference Signals for NR Positioning”,  Ericsson

- R1-1912113, “Uplink RS design for NR positioning”, MediaTek Inc.

- R1-1913101, “Remaining details on SRS enhancements for positioning”, Fraunhofer IIS, Fraunhofer HHI

- R1-1913136, “UL Reference Signals for NR Positioning”, Ericsson

- R1-1911897, “Remaining issues on SRS for NR positioning”, Huawei, HiSilicon

- RAN1 List of email approvals (after the preparation phase), 3GPP TSG RAN WG1 Meeting #100-e.

- 3GPP RAN1 Chairman’s Notes of TSG RAN WG1 e-Meeting #100bis-e

- 3GPP TS 38.214, “NR; Physical layer procedures for data”，v16.1.0，March 2020.