3GPP TSG RAN WG1 #101                                                         R1-2003657

e-Meeting, May 25th – June 5th, 2020

Agenda item:	7.2.2.2.2

Source: 	MediaTek Inc.

Title: 	Remaining issues on initial access procedure for NR-U operation

Document for:	Discussion and decision

# Introduction

In this contribution, we discuss two remaining issues about initial access procedure and mobility measurements. The first one is about an agreement missing from TS38.213. The second one is about the gap between msgA PRACH and msgA PUSCH.

# Number of transmitted SSBs in a DRS window

Agreement: (RAN1 #94bis 1810) For SSB transmissions as part of DRS: - It is considered beneficial to expand the maximum number of candidate SSB positions within DRS transmission window to [Y], for e.g., Y = [64] - FFS: How to derive frame timing from detected SS/PBCH block - Transmitted SSBs do not overlap - FFS: Shift granularity between candidate SSBs positions/candidate groups of SSBs - Maximum number of transmitted SSBs is [X] within DRS transmission window. X <= 8 - FFS: Duration of DRS transmission window - FFS: Duration of the transmitted DRS within the window, including SSBs and other multiplexed signals/channels - FFS: relationship between transmitted SSB index and QCL assumption at UE - FFS: If and how to support beam repetition for soft combining of SSBs within the same DRS transmission

At RAN1 #94bis meeting, the above agreements were made. However, the agreement highlighted in yellow is not captured into specification. We hence provide the following text proposal.

Proposal 1: Adopt the text proposal in Section 2.1 to capture a previous agreement on the maximum number of transmitted SS/PBCH blocks in a discovery burst transmission window.

## Text Proposal 1

============== Start of Text Proposal for Clause 4.1 of TS 38.213 ==============

*** Unchanged text is omitted ***

The candidate SS/PBCH blocks in a half frame are indexed in an ascending order in time from 0 to Lmax-1, where Lmax is determined according to SS/PBCH block patterns for Cases A through E. Lmax is a maximum number of SS/PBCH block indexes in a cell, and

-	for operation without shared spectrum channel access, Lmax=Lmax

-	for operation with shared spectrum channel access, Lmax=8 for Lmax=10 and 15 kHz SCS of SS/PBCH blocks and for Lmax=20 and 30 kHz SCS of SS/PBCH blocks. The maximum number of transmitted SS/PBCH blocks within a discovery burst transmission window is 8.

For Lmax=4, a UE determines the 2 LSB bits of a candidate SS/PBCH block index per half frame from a one-to-one mapping with an index of the DM-RS sequence transmitted in the PBCH as described in [4, TS 38.211].

*** Unchanged text is omitted ***

============== End of Text Proposal for Clause 4.1 of TS 38.213 ==============

# Gap between MsgA PRACH and MsgA PUSCH

In Rel-16, msgA PRACH and msgA PUSCH are only allowed to be transmitted in different slots. Looking at the R15 PRACH configuration for TDD in FR1 (Table 6.3.3.2-3 in 38.211), there would be always gaps between msgA PRACH and msgA PUSH for formats A1/A2/A3. While for formats B1, C0, A1/B1, A2/B2, A3/B3, there would be always a gap in between the associated RACH occasion and PUSCH occasion expect for the last RACH occasion in a slot. Now, it only leaves formats B4 and C2 on the table which in total have 46 out of the 256 preambles. If not all the UEs in the cell can support no gap in between its msgA PRACH and msgA PUSCH transmissions, it means there should be gaps in the cell-specifically configured PRACH resources and PUSCH resources. The “no-gap msgA” is only applicable to CFRA.

Observation 1:  The cases that UE can take advantage from supporting no gap in between msgA PRACH and msgA PUSCH are not many.

Proposal 2: Apply the same gap defined in the R15 to msgA PRACH for NR-U.

Proposal 3: Adopt the text proposal in Section 3.1 to TS 38.213.

## Text Proposal 2

============== Start of Text Proposal for Clause 8.1A of TS 38.213 ==============

*** Unchanged text omitted ***

For a Type-2 random access procedure, a UE transmits a PUSCH, when applicable, after transmitting a PRACH. The UE encodes a transport block provided for the PUSCH transmission using redundancy version number 0. The PUSCH transmission is after the PRACH transmission by at least N symbols where N=2 for μ=0 or μ=1, N=4 for μ=2 or μ=3, and μ is the SCS configuration for the active UL BWP.

*** Unchanged text omitted ***

============== Start of Text Proposal for Clause 8.1A of TS 38.213 ==============

# Conclusion

In this contribution, we make the following proposals.

Proposal 1: Adopt the text proposal in Section 2.1 to capture a previous agreement on the maximum number of transmitted SS/PBCH blocks in a discovery burst transmission window.

Proposal 2: Apply the same gap defined in the R15 to msgA PRACH for NR-U.

Proposal 3: Adopt the text proposal in Section 3.1 to TS 38.213.