3GPP TSG-RAN WG1 Meeting #101	Tdoc R1-2003707

eMeeting, May 25th – June 5th, 2020

Agenda Item:	7.2.6.5

Source:	Ericsson

Title:	Remaining issues on low PAPR RS

Document for:	Discussion and Decision

# 1	Introduction

This document contains a remaining issue on low PAPR RS after Ran1 100-b e-meeting.

# 2 Remaining issue

In R1-2002297, the following correction was suggested. In our view, this correction is useful for correctness. Hence, we propose to adopt the following TP for 38.211.

===========================================================================

6.4.1.1.1.2	Sequence generation when transform precoding is enabled

<unchanged text omitted>

The sequence group u=fgh+nIDRS mod 30, where nIDRS is given by

-	nIDRS=nIDPUSCH if nIDPUSCH is configured by the higher-layer parameter nPUSCH-Identity in the DMRS-UplinkConfig IE and,

- the higher-layer parameter dmrsUplinkTransformPrecoding-r16 is not configured or the higher-layer parameter dmrsUplinkTransformPrecoding-r16 is configured and π/2-BPSK modulation is not used for PUSCH, and

- the PUSCH is neither scheduled by RAR UL grant nor scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI according to clause 8.3 in [5, TS 38.213];

-	nIDRS=NIDnSCID if the higher-layer parameter dmrs-UplinkTransformPrecoding-r16 is configured, π/2-BPSK modulation is used for PUSCH, the PUSCH transmission is not a msg3 transmission, and the transmission is not scheduled using DCI format 0_0 in a common search space;

-	nIDRS=NIDcell otherwise

<unchanged text omitted>

===========================================================================