3GPP TSG RAN WG1 #101-e	R1- 2003719

Agenda item:		8.2.1

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Additional scenarios for evaluation of NR positioning

Document for: 	Discussion and Decision

# Introduction

In Rel-16 native NR positioning support was standardized. At RAN#86 a new SI was approved on enhancements in Rel-17 to positioning [1]. This contribution discusses our initial views on the additional scenarios for evaluation. Our companion contributions discuss our views on evaluations and potential enhancements in [6], [7].

# Discussion

In the email discussion [2], study scopes and scenarios on IIoT Positioning Enhancements have been discussed. We continue to add views on the IIoT SI.

IIOT use cases and performance requirement

When Rel-16 SI started, RAN1 has defined the target performance requirements. The performance targets for RAT dependent solutions for commercial use cases in Rel-16 are defined in TR 38.855 [3]

- Horizontal positioning error < 3m for 80% of UEs in indoor deployment scenarios

- Vertical positioning error < 3m for 80% of UEs in indoor deployment scenarios

- Horizontal positioning error < 10m for 80% of UEs in outdoor deployments scenarios

- Vertical positioning error < 3m for 80% of UEs in outdoor deployment scenarios

- End to end latency < 1s

For Rel-16 IIOT positioning requirement SI, new IIOT scenarios are given in TR 22.804 [4], the scenarios have different performance requirements in terms of horizontal and vertical positioning accuracy, latency, and power consumption etc. RAN1 needs to define the target positioning requirements for Rel-17 as RAN1 did in Rel-16. Also a goal of RAN1 requirements study is to know exemplary performance under selected scenarios, the scenario selection is important to demonstrate good performances as intended for a practical usecase. In the email discussion, RAN1 has discussed as the box below.

RAN#100 email discussion [2] Proposal X1 : Define target positioning requirements in Rel-17 with one of the following options: - Option 1: Select one IIOT scenario (or multiple IIOT scenarios) from Table 8.1.7 in TR 22.804 as the target IIOT scenario(s), and then define the target positioning requirements in Rel-17 based on the positioning requirements defined in TR 22.804 for the selected IIOT scenario(s); - Option 2: Define the target positioning requirements in Rel-17 with the consideration of the positioning performance defined in Table 8.1.7 in TR 22.804 (e.g., using the exemplary performance targets in the SID), but the target positioning requirements may not necessarily be associated with particular IIOT scenario(s). Proposal X2 : For assessing scalability of positioning solutions, latency of positioning procedure should be studied as a function of number of devices to be positioned. Average power consumption of UE should also be studied as a function of number of UEs to be positioned. Such assessment would give a clear picture of scalability of a positioning solution while also providing metrics for latency and power consumption.

We don’t see benefits from selecting scenario from previous studies of other groups such as TR22.804. RAN1 study may refer to the scenarios from TR22.804, the application requirements are regarded in actual implementation. In the meantime, RAN1 studies exemplary performance based on standard positioning methods under given scenarios, the meaning of performance requirement are different depending on study purposes. Whether RAN1 studies can meet the agreed requirements for InF sub-scenario(s) with Rel-16 solutions will be the clear first step of this SI in our view.

Proposal 1: We prefer option-1 defining the exemplary positioning requirements in Rel-17 by selecting one IIOT scenario from Table 8.1.7 in TR 22.804.

- The target accuracy performance must be indicated with CDF values in a statistic manner for horizontal and vertical positioning methods

- Power consumption and latency requirements must be minimum performance requirements.

The commercial use cases and requirements are applicable to a limited geographic area. RAN1 performance requirements are exemplary requirements as it was in TR 38.855. It is performance target in IIoT scenario, it would be available at the end of SI simulation works from companies.

Also, multiple aspects of performances can be studies as listed like power consumption, latency, scalability or service UE capacity. However, the scope seems to be narrow down and more specific. Proposal X2 in the box addresses ‘, latency of positioning procedure as a function of number of devices’ or ‘Average power consumption of UE as a function of number of UEs’. Overall, we agree to intention, possibly the latency can appear depending on the service UE loads. However, our view on the performance study scopes are different.

Proposal 2: Latency of positioning procedure should be studied. A goal of latency study is to identify latency bottle neck in the positioning service process, and improve the bottle neck issue.

- UE<>gNB measurement and report latency requirement

- gNBs<>LMF request and report latency requirement (may include RAN3 for the study)

We also discuss our views on latency evaluation in our companion contribution [6].

IIOT InF scenarios selection

For the evaluation of positioning performance with the target IIOT scenarios, there is a need to define the details of the evaluation parameters, including the channel parameters for the IIOT scenarios based on TR 38.901[2], where the following 5 Indoor Factory (InF) scenarios are defined:

- InF-SL	Indoor Factory with Sparse clutter and Low base station height (both Tx and Rx are below the average height of the clutter)

- InF-DL	Indoor Factory with Dense clutter and Low base station height (both Tx and Rx are below the average height of the clutter)

- InF-SH	Indoor Factory with Sparse clutter and High base station height (Tx or Rx elevated above the clutter)

- InF-DH	Indoor Factory with Dense clutter and High base station height (Tx or Rx elevated above the clutter)

- InF-HH	Indoor Factory with High Tx and High Rx (both elevated above the clutter)

RAN#100 email discussion [2] - Proposal X3: Reuse the common parameters defined in Table 6.1.1-1 in TR 38.855 (including Table 6.1.1-2 for UE radiation pattern in FR2) for IIOT scenarios. - Proposal X4: Select one InF scenario (or more target InF scenarios) from the InF scenarios scenarios defined in TR 38.901 for supporting positioning evolution in Rel-17.

It is agreeable to proposal X3 to reuse the common parameters defined in 38.901 for the evaluation of IIoT scenarios. However, the parameter setting are still broad, further down scoping is required.

Proposal 3: Reuse the common parameters used in Rel-16 SI performance study (in Table 6.1.1-1 in TR 38.855). Further down scoping of BW, carrier frequency must be considered.

Observation 1: InF-DH is regarded as the most feasible study scenario in terms of usecase, but its LOS probability with the current setting causes performance concern.

Proposal 4: In order to make reasonable LOS assumption for InF-DH, adjust cluster density or cluster size factors.

Proposal 5 : Select one scenario with relatively high LOS probability for targeted performance demonstration.

Option -1: select InF-SH and InF-DH scenarios

Option -2: select InF-DH scenario only with adjusting cluster density or cluster size
          ( current setting cluster density r=0.6, hc=6m, d_cluster=2m in Table 5 [2] )

Selection on the options must be done with consideration of practical usecase as well as satisfying the target performance requirements reasonably, and the two scenarios of InF-SH and InF-DH make sense for study purpose (i.e., our preference is option 1 above). In LOS analysis in InF scenarios, approximate distance to gNB is 10m as appeared in Table 5 in [2], and the LOS probability drops to 10%, which especially makes the AoA performance questionable in the study.

Figure : LOS probability of InF-DH and SL (upper), 
LOS probability depending on cluster density and cluster size

RAN#100 email discussion [2] - Proposal X : The scenario parameters common to InF scenario(s) can be developed with the consideration of the parameters for InF scenarios defined in Table 7.8-7 in TR 38.901, Table 7.2-4 of 38.901 (see Appendix 7.2)and the parameters for indoor office scenarios in Table 6.1.1-3 in TR 38.855, e.g., as shown in Table 5.

DL PRS and UL SRS Configurations

The main interest of Rel-17 study is high accuracy performance of IIoT positioning. Therefore, it would be reasonable to prioritize configuration of full resources of PRS, SRS (i.e. comb-2, n12) over other configurations. The goal should be to see what is the best accuracy we can achieve.

The study should proceed based on the parameters you would capture in Section 4 to proceed with the various evaluations in a IIoT scenario. Performance target is achieved with the best performance achievable with full resource allocation.

Proposal 6: Performance target is achieved with the best performance achievable with resource allocation, accordingly the DL PRS and UL SRS configuration selections must be done with the consideration of the best performance.

Other Scenarios

The SID also lists “general commercial uses cases” as a target and mentions that scenarios uses in TR 38.855 should be reused when applicable. The three scenarios considered in Rel-16 were Urban Macro (UMa), Urban Mirco (UMi), and Indoor Office. As the indoor office scenario is somewhat related to InF models we propose to not evaluate that scenario for Rel-17. UMa seems to be both quite challenging for positioning due to high ISD and is mostly related to regulatory use cases in our understanding which have been addressed in Rel-16. We think that it would be most relavant to consider the UMi scenario to try to address the “general commercial use cases” and we can also consider to adapt the assumptions used in Rel-16 (e.g., ISD, additional nodes, indoor/outdoor UE split, etc).

Observation 2: Simulating Rel-16/17 techniques for all InF subscenarios and for the three scenarios from Rel-16 may require very high effort.

Proposal 7: In addition to evaluating IIoT scenarios RAN1 should at most evaluate UMi. Note: RAN1 to consider if changes to the UMi assumptions from TR 38.855 are needed.

# Conclusion

In this contribution we make the following proposals and observations:

Proposal 1: We prefer option-1 defining the exemplary positioning requirements in Rel-17 by selecting one IIOT scenario from Table 8.1.7 in TR 22.804.

- The target accuracy performance must be indicated with CDF values in a statistic manner for horizontal and vertical positioning methods

- Power consumption and latency requirements must be minimum performance requirements.

Proposal 2: Latency of positioning procedure should be studied. A goal of latency study is to identify latency bottle neck in the positioning service process, and improve the bottle neck issue.

- UE<>gNB measurement and report latency requirement

- gNBs<>LMF request and report latency requirement (may include RAN3 for the study)

Proposal 3: Reuse the common parameters used in Rel-16 SI performance study (in Table 6.1.1-1 in TR 38.855). Further down scoping of BW, carrier frequency must be considered.

Observation 1: InF-DH is regarded as the most feasible study scenario in terms of usecase, but its LOS probability with the current setting causes performance concern.

Proposal 4: In order to make reasonable LOS assumption for InF-DH, adjust cluster density or cluster size factors.

Proposal 5: Select one scenario with relatively high LOS probability for targeted performance demonstration.

Option -1: select InF-SH and InF-DH scenarios and check if the performance requirements are satisfied.

Option -2: select InF-DH scenario only with adjusting cluster density or cluster size
          ( current setting cluster density r=0.6, hc=6m, d_cluster=2m in Table 5 [2] )

Proposal 6: Performance target is achieved with the best performance achievable with resource allocation, accordingly the DL PRS and UL SRS configuration selections must be done with the consideration of the best performance.

Observation 2: Simulating Rel-16/17 techniques for all InF subscenarios and for the three scenarios from Rel-16 may require very high effort.

Proposal 7: In addition to evaluating IIoT scenarios RAN1 should at most evaluate UMi. Note: RAN1 to consider if changes to the UMi assumptions from TR 38.855 are needed.

# References

[1] RP-193237, New SID on NR Positioning Enhancements, Qualcomm.

[2] R1-200xxxx, email discussion Summary of discussion on IIOT Scenarios for NR Positioning Enhancements, CATT, RAN1#100, Athens, Greece, Feb 2020

[3]	TR 38.855, “Study on NR positioning support”, V16.0.0 (2019-03)

[4] TR 22.804, “Study on Communication for Automation in Vertical Domains”, V16.2.0

[5] TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz, V16.1.0

[6] R1-2003720

[7] R1-2003721