3GPP TSG RAN WG1 Meeting #101-E						R1-2003741

e-Meeting, May 25th – June 5th, 2020

Source:	Intel Corporation

Title:	Corrections for DL SPS and intra-UE prioritization involving CG PUSCH

Agenda item:	7.2.5.7

Document for:	Discussion and Decision

# Introduction

In this contribution, we present our views on some remaining details on enhanced DL SPS and on intra-UE prioritization between two CG PUSCH occasions or between a CG and DG PUSCH for Rel-16 IIoT.

# SPS Release DCI and SPS PDSCH occasion in same slot

With the support of DL SPS periodicities as frequent as every slot, it becomes imperative that the SPS release DCI would need to be transmitted to the UE in a slot with a SPS PDSCH occasion for the same DL SPS configuration. According to the agreed HARQ-ACK CB construction procedure, a location in the Type-1 HARQ-ACK codebook for HARQ-ACK information corresponding to a single SPS PDSCH release is same as for a corresponding SPS PDSCH reception. A location in the Type-1 HARQ-ACK codebook for HARQ-ACK information corresponding to multiple SPS PDSCH releases by a single DCI format is same as for a corresponding SPS PDSCH reception with the lowest SPS configuration index among the multiple SPS PDSCH releases.

For UEs supporting reception of a maximum of a single unicast PDSCH per slot, the above leads to a collision of the HARQ-ACK corresponding to the release DCI and the SPS PDSCH occasion in the slot. To address this, two alternatives have been discussed so far for such UEs:

- Alt. 1: In a slot with SPS PDSCH occasion in which the UE detects an SPS release DCI, only the HARQ-ACK for the SPS release DCI is generated and the UE is not expected to receive and report HARQ-ACK feedback for the SPS PDSCH occasion.

  - Further constraints on relative timings between the release DCI and the SPS PDSCH occasion in the slot are necessary to ensure that the processing timelines are not adversely impacted.

- Alt. 2: The UE does not expect to receive SPS PDSCH release and unicast PDSCH in a same slot associated with the same PUCCH slot.

For Alt. 1, to ensure that the UE does not run into processing timeline issues towards prioritizing the HARQ-ACK feedback for the release DCI over the SPS PDSCH, it is necessary to define the latest time such a release DCI may be received by the UE with respect to the SPS PDSCH occasion. For instance, if the PDSCH occasion occurs in the first few symbols of the slot, while the release DCI is detected at the end of the slot, it may not be possible for the UE to cancel the SPS PDSCH processing and corresponding HARQ-ACK generation in lieu of the HARQ-ACK feedback for the SPS release DCI. Therefore, under the assumption that it would take considerably less time for detection of the release DCI, as long as the release DCI ends no later than the last symbol of the SPS PDSCH occasion, the UE should be able to prioritize HARQ-ACK generation for the release DCI over the SPS PDSCH processing even if it might have started processing the PDSCH first (i.e., if the SPS PDSCH occasions starts before the release DCI monitoring occasion).

Alt. 2 solves the core issue at hand by enforcing that the gNB indicates a different value of the K1-offset to move the HARQ-ACK transmission for the SPS release DCI to a different slot. However, this also implies that now the UE has to process both release DCI and the SPS PDSCH in a slot, and valid HARQ-ACK feedback is expected for both. This is a departure from current behavior in the direction of increased UE complexity, and considering that Alt. 1 can address the issue without incurring the additional demands on UE implementation, Alt. 1 is preferred.

During the RAN1 #100b-E meeting, the following exhaustive list of cases were identified:

- Case 1: In a slot, if SPS release DCI is received before the end of the SPS PDSCH for the same SPS configuration

  - Case 1-1: A UE is not required to receive the SPS PDSCH if HARQ-ACK for the SPS release and the SPS reception would map to different PUCCHs

    - Expected consequence: separate HARQ-ACK bits but NACK for the SPS PDSCH?

  - Case 1-2: A UE is not required to receive the SPS PDSCH if HARQ-ACK for the SPS release and the SPS reception would map to the same PUCCH

    - Expected consequence: only 1 bit for SPS release

- Case 2: In a slot, if SPS release DCI is received after the end of the SPS PDSCH for the same SPS configuration

  - Case 2-1: SPS PDSCH is received if HARQ-ACK for the SPS release and the SPS reception would map to different PUCCHs

    - Expected consequence: Separate HARQ-ACK bits

  - Case 2-2: A UE is not required to receive the SPS PDSCH if HARQ-ACK for the SPS release and the SPS reception would map to the same PUCCH

    - Expected consequence: only 1 bit for SPS release

From the above discussion, we observe that Cases 1-1 and 2-1 are assuming a new UE behavior whereby the HARQ-ACK for the SPS PDSCH and that for the SPS release DCI that are scheduled in the same slot are mapped to different PUCCHs. As mentioned above, it results in additional UE behavior while hardly offering any material benefits/flexibility in terms of real use case. Thus, in our view, Cases 1-1 and 2-1 should NOT be supported, and only Cases 1-2 and 2-2 should be supported.

Further, for cases with multiple DL SPS configurations configured (and activated), and joint release DCI, the above can be generalized to consider the handling of the release DCI that includes at least the SPS configuration with the lowest DL SPS configuration index among the SPS configuration(s) configured to be received in the slot.

Also, for cross-carrier scheduling, both the SPS release DCI and SPS PDSCH occasion should be interpreted to correspond to the same scheduled cell, and thus, the same approach as for same-cell scheduling should apply.

Proposal 1

- For a UE not indicating a capability to receive more than one unicast PDSCH per slot, if a UE is configured to receive at least one SPS PDSCH in a slot on a serving cell, for same- or cross-carrier scheduling cases,

  - if a PDCCH with SPS PDSCH release including an SPS configuration with the lowest configuration index among the SPS configuration(s) configured to be received in the slot is received in the slot, only HARQ-ACK corresponding to the PDCCH with SPS PDSCH release is generated.

    - The UE is not expected to receive the PDCCH with SPS PDSCH release such that the PDCCH ends after the last symbol of the SPS PDSCH of an SPS configuration with the lowest configuration index among the SPS configuration(s) configured to be received in the slot.

  - The UE is not expected to receive a PDCCH with SPS PDSCH release which does not include an SPS configuration with the lowest configuration index among the SPS configuration(s) configured to be received in the slot.

Moreover, to address UEs capable of receiving multiple unicast PDSCHs in a slot, the above proposal can be generalized such that the consideration is made between an SPS PDSCH occasion and an SPS release DCI corresponding to the same configuration in the same slot.

Thus, with such generalization, Proposal 1 could be adapted to address UEs capable of receiving more than one unicast PDSCH per slot as follows.

Proposal 2

- For a UE indicating a capability to receive more than one unicast PDSCH per slot, if a UE is configured to receive at least one SPS PDSCH in a slot on a serving cell, for same- or cross-carrier scheduling cases,

  - if a PDCCH with SPS PDSCH release, that includes an SPS configuration for which an SPS PDSCH is to be received in the slot after resolution of any overlapping SPS occasions, is received in the slot, only HARQ-ACK corresponding to the PDCCH with SPS PDSCH release is generated.

    - The UE is not expected to receive the PDCCH with SPS PDSCH release for an SPS configuration such that the PDCCH ends after the last symbol of the SPS PDSCH of the same SPS configuration.

  - The UE is not expected to receive a PDCCH with SPS PDSCH release which does not include an SPS configuration with configuration index that is not among the SPS configuration(s) to be received in the slot after resolution of any overlapping SPS occasions.

# Intra-UE prioritization between CG-CG PUSCH and CG-DG PUSCH

In RAN1 99 [1], following agreements were made regarding cancelation time for dropping a low priority UL transmission and preparation time of high priority UL transmission after receiving the PDCCH of high priority UL transmission.

Agreement When a high-priority UL transmission overlaps with a low-priority UL transmission in a slot, - The UE is expected to cancel the low-priority UL transmission starting from Tproc,2 +d1 after the end of PDCCH scheduling the high-priority transmission, where - Tproc,2 is correponding to UE processing time capability for the carrier. - Value d1 is the time duration corresponding to 0,1,2 symbols reported by UE capability - Note: d_2,1=0 is for cancellation - The minimum processing time of the high priority channel is extended by d2 symbols - Value d2 is the time duration corresponding to 0,1,2 symbols reported by UE capability The overlapping condition is per repetition of the uplink transmission

First, for cases involving overlaps between two CG PUSCH occasions with different priorities, there is no issue with timelines as the uplink grants corresponding to both CG PUSCH occasions are available to the UE semi-statically.

Observation 1

- There is no timeline issue for the handling of intra-UE prioritization for CG-CG PUSCH overlaps with different priorities.

However, the above agreement does not address the possibility wherein the UL transmission with larger priority index may start before the earliest point in time when the UL transmission with smaller priority index may be canceled. Certainly, such a case should be avoided, and the following is proposed.

Proposal 3

- When a high-priority UL transmission overlaps with a low-priority UL transmission in a slot,

  - The UE does not expect to be scheduled with a high-priority UL transmission that starts before the earliest symbol in which the low-priority UL transmission may be canceled.

In this regard, and as a specific case of intra-UE prioritization, regarding the resource conflict of a PUSCH corresponding to a configured grant and a PUSCH scheduled by a PDCCH, the following is captured in Section 6.1 of TS 38.214.

If [a UE reports the capability of intra-UE prioritization], and if a PUSCH corresponding to a configured grant and a PUSCH scheduled by a PDCCH on a serving cell are partially or fully overlapping in time, - If the PUSCH corresponding to the configured grant has priority in configuredGrantConfig set to 1 (i.e., high priority), and the PUSCH scheduled by the PDCCH is indicated as low priority by having the [priority indicator] field in the scheduling DCI set to 0 or by not having the [priority indicator] field present in the scheduling DCI, the UE is expected to transmit the PUSCH corresponding to the configured grant, and cancel the PUSCH transmission scheduled by the PDCCH at latest starting at the first symbol of the PUSCH corresponding to the configured grant. - Otherwise, the UE shall cancel the PUSCH transmission corresponding to the configured grant at latest starting M symbols after the end of the last symbol of the PDCCH carrying the DCI scheduling the PUSCH, and transmit the PUSCH scheduled by the PDCCH, where - M = Tproc,2 +d1, where Tproc,2 is given by clause 6.4 for the corresponding PUSCH timing capability assuming d2,1 = 0 and d1 is determined by the reported UE capability [XXXXX], - In this case, the UE is not expected to be scheduled for the PUSCH by the PDCCH where the PUSCH starts earlier than N symbols after the end of the last symbol of the PDCCH, where - N = Tproc,2 + d2, where Tproc,2 is the PUSCH preparation time of the PUSCH scheduled by the PDCCH using the associated PUSCH timing capability according to clause 6.4 and d2 is determined by the reported UE capability [YYYYY]. - In case of PUSCH repetitions, the overlapping handling is performed for each PUSCH repetition separately. - The UE is not expected to be scheduled for another PUSCH by a PDCCH where this PUSCH starts no earlier than the end of the prioritized transmitted PUSCH and before the end of the time domain allocation of the cancelled PUSCH.

The specification does not capture the scenario if there can be any symbol(s) of the PUSCH  corresponding to the configured grant within M symbols after the last symbol of PDCCH carrying the DCI scheduling the PUSCH that may overlap with any symbol of the PUSCH scheduled by the DCI. In our view, specification should capture that the UE does not expect this case to occur. Hence, we add the following text proposal.

Proposal 4

- Capture the below text proposal regarding collision handling of DG and CG PUSCH and update section 6.1 in TS 38.214.

-------------------------- Text proposal starts for TS 38.214, Section 6.1 ----------------------------------------

If [a UE reports the capability of intra-UE prioritization], and if a PUSCH corresponding to a configured grant and a PUSCH scheduled by a PDCCH on a serving cell are partially or fully overlapping in time,

-	If the PUSCH corresponding to the configured grant has priority in configuredGrantConfig set to 1 (i.e., high priority), and the PUSCH scheduled by the PDCCH is indicated as low priority by having the [priority indicator] field in the scheduling DCI set to 0 or by not having the [priority indicator] field present in the scheduling DCI, the UE is expected to transmit the PUSCH corresponding to the configured grant, and cancel the PUSCH transmission scheduled by the PDCCH at latest starting at the first symbol of the PUSCH corresponding to the configured grant.

Otherwise, the UE shall cancel the PUSCH transmission corresponding to the configured grant at latest starting M symbols after the end of the last symbol of the PDCCH carrying the DCI scheduling the PUSCH, and transmit the PUSCH scheduled by the PDCCH, where

-	M = Tproc,2 +d1, where Tproc,2 is given by clause 6.4 for the corresponding PUSCH timing capability assuming d2,1 = 0 and d1 is determined by the reported UE capability [XXXXX],

-	In this case, the UE is not expected to be scheduled for the PUSCH by the PDCCH where the PUSCH starts earlier than max (M, N) symbols after the end of the last symbol of the PDCCH, where

-	N = Tproc,2 + d2, where Tproc,2 is the PUSCH preparation time of the PUSCH scheduled by the PDCCH using the associated PUSCH timing capability according to clause 6.4 and d2 is determined by the reported UE capability [YYYYY].

-	In case of PUSCH repetitions, the overlapping handling is performed for each PUSCH repetition separately.

-	The UE is not expected to be scheduled for another PUSCH by a PDCCH where this PUSCH starts no earlier than the end of the prioritized transmitted PUSCH and before the end of the time domain allocation of the cancelled PUSCH.]

---------------------------- Text proposal ends for TS 38.214, Section 6. 1----------------------------------------

With the above correction, it should be sufficient to facilitate intra-UE prioritization between a CG PUSCH and DG PUSCH based on agreements so far in RAN1.

However, considering that PUSCH vs. PUSCH prioritization is not supported for the general case, to further aid UE implementation a possible additional constraint that may be considered for the CG-DG case to ensure that the LP PUSCH can be canceled in entirety, i.e., the UE does not have to perform any partial cancelation of one PUSCH and replace with another on the overlapping resources. Such a constraint would enable the UE to cancel the LP PUSCH at the “uplink grant”-level, essentially making it similar to the handling of CG-DG prioritization in Rel-15 (wherein DG PUSCH is always prioritized). Thus, the uplink grant for the HP PUSCH should be available at the UE at least Tproc,2 before the first symbol of the LP PUSCH.

For the CG-DG case, if CG PUSCH is of higher priority, the above condition is automatically satisfied, and thus, existing timelines are sufficient.

On the other hand, for CG-DG case, if the DG PUSCH is of higher priority, the above condition effectively makes the prioritization handling exactly same as Rel-15 CG-DG prioritization rule with the same constraints/timelines and outcome. However, it should be noted that even in this case, although the prioritization outcome is the same between the cases when: (A) DG is HP and CG is LP, and (B) both DG and CG are of same priority, it does not mean that the intra-UE prioritization feature becomes redundant in this case, since assigning certain CG PUSCH configuration or a DG PUSCH with priority index 1 can be helpful to address any other collisions (e.g., for other CG PUSCH occasions or collisions with a third channel, etc.). On the other hand, the effectiveness of the feature would certainly be compromised with such additional constraints.

Proposal 5

- RAN1 to confirm that intra-UE prioritization for the following cases are supported in Rel-16

  - Time domain overlaps between two CG PUSCH occasions with different priorities

  - Time domain overlaps between a CG PUSCH occasion and a DG PUSCH with different priorities.

# Conclusions

In this contribution, we discussed some remaining issues for enhanced DL SPS for Rel-16 IIoT, and the following proposals are made:

Proposal 1

- For a UE not indicating a capability to receive more than one unicast PDSCH per slot, if a UE is configured to receive at least one SPS PDSCH in a slot on a serving cell, for same- or cross-carrier scheduling cases,

  - if a PDCCH with SPS PDSCH release including an SPS configuration with the lowest configuration index among the SPS configuration(s) configured to be received in the slot is received in the slot, only HARQ-ACK corresponding to the PDCCH with SPS PDSCH release is generated.

    - The UE is not expected to receive the PDCCH with SPS PDSCH release such that the PDCCH ends after the last symbol of the SPS PDSCH of an SPS configuration with the lowest configuration index among the SPS configuration(s) configured to be received in the slot.

  - The UE is not expected to receive a PDCCH with SPS PDSCH release which does not include an SPS configuration with the lowest configuration index among the SPS configuration(s) configured to be received in the slot.

Proposal 2

- For a UE indicating a capability to receive more than one unicast PDSCH per slot, if a UE is configured to receive at least one SPS PDSCH in a slot on a serving cell, for same- or cross-carrier scheduling cases,

  - if a PDCCH with SPS PDSCH release, that includes an SPS configuration for which an SPS PDSCH is to be received in the slot after resolution of any overlapping SPS occasions, is received in the slot, only HARQ-ACK corresponding to the PDCCH with SPS PDSCH release is generated.

    - The UE is not expected to receive the PDCCH with SPS PDSCH release for an SPS configuration such that the PDCCH ends after the last symbol of the SPS PDSCH of the same SPS configuration.

  - The UE is not expected to receive a PDCCH with SPS PDSCH release which does not include an SPS configuration with configuration index that is not among the SPS configuration(s) to be received in the slot after resolution of any overlapping SPS occasions.

Observation 1

- There is no timeline issue for the handling of intra-UE prioritization for CG-CG PUSCH overlaps with different priorities.

Proposal 3

- When a high-priority UL transmission overlaps with a low-priority UL transmission in a slot,

  - The UE does not expect to be scheduled with a high-priority UL transmission that starts before the earliest symbol in which the low-priority UL transmission may be canceled.

Proposal 4

- Capture the below text proposal regarding collision handling of DG and CG PUSCH and update section 6.1 in TS 38.214.

-------------------------- Text proposal starts for TS 38.214, Section 6.1 ----------------------------------------

If [a UE reports the capability of intra-UE prioritization], and if a PUSCH corresponding to a configured grant and a PUSCH scheduled by a PDCCH on a serving cell are partially or fully overlapping in time,

-	If the PUSCH corresponding to the configured grant has priority in configuredGrantConfig set to 1 (i.e., high priority), and the PUSCH scheduled by the PDCCH is indicated as low priority by having the [priority indicator] field in the scheduling DCI set to 0 or by not having the [priority indicator] field present in the scheduling DCI, the UE is expected to transmit the PUSCH corresponding to the configured grant, and cancel the PUSCH transmission scheduled by the PDCCH at latest starting at the first symbol of the PUSCH corresponding to the configured grant.

Otherwise, the UE shall cancel the PUSCH transmission corresponding to the configured grant at latest starting M symbols after the end of the last symbol of the PDCCH carrying the DCI scheduling the PUSCH, and transmit the PUSCH scheduled by the PDCCH, where

-	M = Tproc,2 +d1, where Tproc,2 is given by clause 6.4 for the corresponding PUSCH timing capability assuming d2,1 = 0 and d1 is determined by the reported UE capability [XXXXX],

-	In this case, the UE is not expected to be scheduled for the PUSCH by the PDCCH where the PUSCH starts earlier than max (M, N) symbols after the end of the last symbol of the PDCCH, where

-	N = Tproc,2 + d2, where Tproc,2 is the PUSCH preparation time of the PUSCH scheduled by the PDCCH using the associated PUSCH timing capability according to clause 6.4 and d2 is determined by the reported UE capability [YYYYY].

-	In case of PUSCH repetitions, the overlapping handling is performed for each PUSCH repetition separately.

-	The UE is not expected to be scheduled for another PUSCH by a PDCCH where this PUSCH starts no earlier than the end of the prioritized transmitted PUSCH and before the end of the time domain allocation of the cancelled PUSCH.]

---------------------------- Text proposal ends for TS 38.214, Section 6. 1----------------------------------------

Proposal 5

- RAN1 to confirm that intra-UE prioritization for the following cases are supported in Rel-16

  - Time domain overlaps between two CG PUSCH occasions with different priorities

  - Time domain overlaps between a CG PUSCH occasion and a DG PUSCH with different priorities.