3GPP TSG RAN WG1 Meeting #101                     R1- 2003800

e-Meeting, May 25th – June 5th, 2020

Source: 	ZTE Corporation

Title: 	Discussion on LTE Rel-16 UE Features for NB-IoT

Agenda item:	6.2.5.2

Document for:	Discussion/Decision

Introduction

After RAN1#100b-e meeting, it has been declared all the Rel-16 RAN1-led WIs as finished from RAN1 perspective. To facilitate the UE features discussion, two email discussions (i.e., 100e-b-NR-UEFeature and 100e-b-LTE-UEFeature) on NR and LTE Rel-16 UE features were initiated to collect companies’ input.

In this contribution, remailing detail about LTE Rel-16 UE features for NB-IoT are presented.

Discussions

It is noted that many of the same features exist in both LTE_eMTC5 and NB_IOTenh3 WI, therefore it is suggested to align the naming and the corresponding description of these FGs in the feature file.

| 1-1 | Group WUS without group resource alternation | 1. Group WUS without group resource alternation |
| --- | --- | --- |
| 1-2 | Group WUS with group resource alternation | 1. Group WUS with group resource alternation |

| 2-1 | UE-group wake-up signal (Group WUS) with a wake-up time before the first associated PO (without group resource alternation) | - UE-group wake-up signal (Group WUS) with a wake-up time before the first associated PO (without group resource alternation) |
| --- | --- | --- |
| 2-2 | UE-group WUS with a wake-up time before the first associated PO (with group resource alternation) | - UE-group WUS with a wake-up time before the first associated PO (with group resource alternation) |

- Either add UE-group wake-up signal in 1-1/2 or remove this term and use Group WUS in 2-1/2.

| 1-3 | PUR for full-PRB in CEmodeA |
| --- | --- |

| 2-3 | Transmission in preconfigured UL resources (PUR) (with potential UE-specific cyclic shift for DMRS) |
| --- | --- |

- Align the description for PUR. Either change the name of FG 1-3 to 'Transmission in preconfigured UL resources (PUR) for full-PRB in CEmodeA' , or change the name of FG 2-3 to ' PUR (with potential UE-specific cyclic shift for DMRS )

| 1-7 | PUR serving cell RSRP TA validation |
| --- | --- |

| 2-4 | PUR with serving cell RSRP for TA validation |
| --- | --- |

- For 1-7 change to 'PUR with serving cell RSRP for TA validation'

| 1-10 | Multi-TB unicast for DL in CEmodeA |
| --- | --- |

| 2-6 | Multi-TB scheduling for unicast in DL with a single DCI(Interleaved transmission) |
| --- | --- |

- Change the name 'Multi-TB' to 'Multi-TB scheduling' in LTE_eMTC5

| 1-23 | Subframe level resource reservation for DL in CEmodeA |
| --- | --- |

| 2-12 | DL resource reservation with subframe-level granularity of NB-IoT non-anchor carriers. |
| --- | --- |

- Align the description for resource reservation.

| 1-29 | DL quality report in Msg3 in Idle |
| --- | --- |
| 1-30 | DL quality report in Connected |

| 2-14 | Quality report in Msg3 for non-anchor access |
| --- | --- |
| 2-15 | Quality report in connected mode |

- Align the description for quality report.

Proposal 1: Align the naming and the description of corresponding FGs for LTE_eMTC5 and NB_IOTenh3

Conclusion

In this contribution, proposals on LTE Rel-16 UE features are introduced.

Proposal 1: Align the naming and the description of corresponding FGs for LTE_eMTC5 and NB_IOTenh3

Reference

Chairman note RAN1#99.

Chairman’s note RAN1#94bis.

Chairman’s note RAN1#AH1901.