3GPP TSG RAN WG1 #100bis	R1-2003887

e-Meeting, May 25th – June 5th, 2020

Agenda item:	7.2.8.2

Source: 	Samsung

Title: 	UL reference signals for NR Positioning

Document for:	Discussion and Decision

# Introduction

A new work item on “NR positioning support” was approved in RAN#83 and the following agreements were made in RAN1#100b-e [1].

Agreement:

Adopt the following text proposal for Clause 6.2.1.4 of TS 38.214:

--------------- Begin of TP -------------------

<Text is omitted>

For single carrier operations, the UE does not expect to be configured on overlapping symbols with more than one SRS resources configured by the higher layer parameter SRS-PosResource with resourceType of the SRS resources as ‘periodic’.

For single carrier operations, the UE does not expect to be triggered to transmit SRS on overlapping symbols with more than one SRS resources configured by the higher layer parameter SRS-Pos-Resource with resourceType of the SRS resources as ‘semi-persistent’ or ‘aperiodic’.

--------------- End of TP -------------------

Agreement:

Introduce a new UE capability for the number of SRS resources for positioning on a symbol for intra-band CA

- FFS: Capability for simultaneous SRS transmission across bands for inter-band CA

- Continue discussion on capability for intra-band/inter-band CA, including potential TP to 38.214 to reflect the new capability.

Agreement:

Change all occurrences of “[SRS-for-positioning]” to the higher layer parameter agreed by RAN2 WG called “SRS-PosResource-r16” in TS 38.211 Section 6.4.1.4

Conclusion:

For release 16, type3 PHR based on SRS for positioning is not supported.

Agreement:

- Introduce a new parameter to the higher layer parameter list according to the table below.

- The RTOA reference time is defined as T0+tSRS, where

  - T0 is the nominal beginning time of SFN 0 provided by LMF.

  - tSRS = (10nf + nsf) x 10-3tSRS=10nf+nsf×10-3, where nf and nsfnsf are the system frame number and the subframe number of the SRS, respectively

- Send an LS to RAN3 reflecting the consensus.

Table 1 additional parameter to the higher layer parameter list

| NR_pos-Core | NR UL Measurement Report Configuration |  |  | FFS in RAN3 WG | SFN Initialization time | SFN initialization time | New |  | The nominal beginning time of SFN 0 for SRS | Same as SFN initialization time in LPPa BIT STRING (64) |  |  | NRPPa 38.455 |  | Time in seconds relative to 00:00:00 on 1 January 1900 (calculated as continuous time without leap seconds and traceable to a common time reference) where binary encoding of the integer part is in the first 32 bits and binary encoding of the fraction part in the last 32 bits. The fraction part is expressed with a granularity of 1 /2**32 second. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Decision: As per email decision posted on Apr.30th,

Agreement:

- A new UE capability is introduced for the number of simultaneous transmissions of SRS resources for positioning for inter-band CA, where the SRS resources are on different CCs.

- The text in TP#A below is added to 38.214

TP#A ===================== Unchanged parts omitted ====================== For intra-band and inter-band CA operations, a UE can simultaneously transmit more than one SRS resources configured by SRS-PosResource on different CCs, subject to UE’s capability provided by [XX] and [YY] respectively. ===================== Unchanged parts omitted ======================

Agreement:

Adopt the text proposal TP#B in Section 2.3.1 of R1-2003136 for TS 38.213

Agreement:

Adopt the text proposal TP#D in Section 2.4.1 of R1-2003136 for TS 38.215

R1-2003054	LS on UL RTOA reference time	RAN1, Ericsson

Decision: As per email decision posted on May 1st, the LS is approved.

Agreements:

The following TP (TP#2 in the email discussion) was agreed:

Start of Text Proposal to TS 38.214 -------------------------------------------------------------- 6.2.1 UE sounding procedure <omitted text> <omitted text> End of Text Proposal to TS 38.214---------------------------------------------------------------

In this contribution, we discuss the remaining issues for NR UL positioning.

# NR UL PRS design

It has been agreed that the maximum number of supported SRS resource sets for positioning is a UE capability, with configurability of up to 16 resource sets per BWP and the values for the UE capability include {1 ,16}. Since the number of resources within one resource set can be adjusted, the current values should be enough to support beam sweeping in a variety of cases. Therefore, there is no need to support additional values.

Proposa1 1:  The values for the maximum number of supported SRS resource sets for positioning include {1, 16} only.

In the last meeting, a new UE capability for the number of SRS resources for positioning on a symbol for intra-band CA is introduced and it is FFS for inter-band CA. Multiple positioning schemes can be combined to improve the positioning accuracy, e.g., angle based scheme and timing based scheme. For angle based positioning scheme, it might require FR2 band to form a narrow beam. For timing based positioning scheme, FR1 band can be used to guarantee the coverage. In this case, inter-band CA is needed and the capability of UE for the number of SRS resources for positioning on a symbol should be extended to the intra-band CA case.

Proposa1 2:  The capability of UE for the number of SRS resources for positioning on a symbol should be extended to the intra-band CA case.

# Conclusions

In this contribution, we discussed the UL reference signals design for NR positioning. Our proposals are summarized below.

Proposa1 1:  The values for the maximum number of supported SRS resource sets for positioning include {1, 16} only.

Proposa1 2:  The capability of UE for the number of SRS resources for positioning on a symbol should be extended to the intra-band CA case.

# References

- RAN1#100b-e Chairman’s Notes.