3GPP TSG RAN WG1 #101-e	R1-2003914

# e-Meeting, May 25th – June 5th, 2020

Agenda Item:		8.4.1.1

Source:				Samsung

Title:					Scenarios and simulation assumptions for coverage enhancement in FR1

Document for:	Discussion and decision

Introduction

In RAN#86 meeting, NR coverage enhancement SI was approved and the objective of coverage enhancement SI is listed below [1]:

The objective of this study item is to study potential coverage enhancement solutions for specific scenarios for both FR1 and FR2. The detailed objectives are as follows. - The target scenarios and services include - Urban (outdoor gNB serving indoor UEs) scenario, and rural scenario (including extreme long distance rural scenario) for FR1 - Indoor scenario (indoor gNB serving indoor UEs), and urban/suburban scenario (including outdoor gNB serving outdoor UEs and outdoor gNB serving indoor UEs) for FR2. - TDD and FDD for FR1. - VoIP and eMBB service for FR1. - eMBB service as first priority and VoIP as second priority for FR2. - LPWA services and scenarios are not included. - Identify baseline coverage performance for both DL and UL for the above scenarios and services based on link-level simulation - UL channels (including PUSCH and PUCCH) are prioritized for FR1. - Both DL and UL channels for FR2. - Identify the performance target for coverage enhancement, and study the potential solutions for coverage enhancements for the above scenarios and services - The target channels include at least PUSCH/PUCCH - Study enhanced solutions, e.g., time domain/frequency domain/DM-RS enhancement (including DM-RS-less transmissions) - Study the additional enhanced solutions for FR2 if any - Evaluate the performance of the potential solutions based on link level simulation.

As listed in the objective, link-level simulations are to be used for the identification of baseline coverage performance for both DL and UL for the various scenarios and services. This contribution mainly focuses on the simulation assumptions and preliminary results for FR1.

Evaluation methodology and simulation assumptions

Evaluation methodology

In Rel-11 LTE coverage enhancement [2], Maximum Coupling Loss (MCL) was chosen as the metric to evaluate coverage of a radio access technology. The coupling loss is defined as the total long-term channel loss over the link between the Tx antenna ports and the Rx antenna ports. The maximum coupling loss (MCL) is the limit value of the coupling loss at which the service can be delivered, and then MCL can be calculated as the difference between the maximum Tx power and RX sensitivity. Therefore, the coverage of the service can be expressed by MCL. However, MCL used in [2] does not include the antenna gains, shadowing, and penetration loss which depend on the scenarios and channel.

In Rel-15 IMT2020 self-evaluation [3], available path loss in link budget is used in terms of coverage performance for a radio technology. Available path loss includes the loss in the signal path due to shadow fading and penetration loss depending on the scenarios and channel. In order to take into account the different scenarios, the link budget methodology employed in IMT-2020 self-evaluation for 3GPP submission is preferred. The detailed evaluation procedure for available path loss and corresponding maximum range is provided in the Appendix in Table A-1. The calculated maximum range would provide evidence to identify the coverage bottlenecks of transmission channels in both UL and DL.

Proposal 1. The available path loss should be considered in the link budget calculation for coverage performance in FR1 to take into account the different scenarios and channel.

Simulation assumptions for FR1

The available path losses in link budget template with several scenarios, such as urban, rural and indoor, and TDD/FDD configuration were calculated for data and control channel in IMT2020 self-evaluation [3]. Hence, most simulation parameters used in IMT 2020 self-evaluation can be used in the coverage enhancement study. Since the target scenarios and services considered in this SI are different, however, transmission bit rate and occupied channel bandwidth should be changed. In detail, general simulation parameters are summarized below in Table 1 for the four scenarios, urban scenario for 4GHz, rural scenario for 4GHz and 2GHz, and rural scenario with long distance, respectively.

Table 1. General simulation parameters for FR1

| Scenario | FR1 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Urban | Rural for 4GHz | Rural for 2GHz | Rural with long distance |
| System configuration | 4 GHz TDD | 4 GHz TDD | 2GHz FDD | 700MHz FDD |
| Physical channel | DL: PDCCH, PDSCH; UL: PUCCH, PUSCH |  |  |  |
| SCS | 30 kHz | 30 kHz | 15kHz | 15kHz |
| Channel state | NLOS O-to-I | NLOS O-to-I, NLOS |  | LOS |
| Channel model | NLOS: TDL-C | NLOS: TDL-C |  | LOS: TDL-D |
|  | Delay spread = 300ns |  |  |  |
| UE speed | NLOS O-to-I: 3km/h | NLOS: 120km/h NLOS O-to-I: 3km/h |  | LOS: 120km/h |
| # of gNB Ant. | 192 | 64 | 64 | 64 |
| # of TXRUs for gNB | 2 | 2 | 2 | 2 |
| # of UE Ant. | 4(DL) / 2(UL) | 2(DL) / 1(UL) | 2(DL) / 1(UL) | 2(DL) / 1(UL) |
| # of TxRUs for UE | 2 | 2(DL) / 1(UL) | 2(DL) / 1(UL) | 2(DL) / 1(UL) |

In detail, channel specific parameters are summarized below in Table 2 for the four physical channel, such as PDSCH, PDCCH, PUSCH, and PUCCH, respectively.

Table 2. Channel specific parameters for FR1

|  | DL control channel | DL data channel | UL control channel | UL data channel |
| --- | --- | --- | --- | --- |
| Transmission bit rate (bit/s) | DCI format 1-0/0-0; DCI size = 68 bit; QPSK, aggregation level = 16 CCE | eMBB: 10/1 (Urban/Rural) Mbps VoIP: 12.2kbps (304bits: 244 + 60 (header for RoHC compress)) | PUCCH Format 1 w/ 14 OFDM symbols, UCI 2 bit | eMBB: 1 Mbps /100kbps (Urban/Rural) VoIP: 12.2kbps (304bits: 244 + 60 (header for RoHC compress)) |
| Occupied channel BW | FDD: 10 MHz TDD: 20 MHz | FDD: 10 MHz TDD: 20 MHz | 1 PRB | 4 PRB/30 PRB (for high data rate) |
| Performance target | 1% BLER | eMBB: 10% iBLER VoIP: 2% rBLER | 1% BLER | eMBB: 10% iBLER VoIP: 2% rBLER |
| Overhead assumption for DMRS | DMRS: - For 3km/h or 30km/h: Type I, one DMRS symbol, no multiplexing with data. - For 120km/h: Type I, 2 DMRS symbol (one front- loaded and one additional), no multiplexing data. |  |  |  |

Proposal 2. Consider IMT-2020 link budget simulation assumptions including Table 1 and Table 2 as the starting point for coverage enhancement evaluation in FR1.

Preliminary results

The following link budget results using link budget template in Table A-1 have been performed based on the parameter values in Table 1 and 2. In addition, the required SNR in the link budget template is calculated based on the link-level simulation at performance target for each channel in Table 2. The baseline performance in dB and coverage distance in meter are defined as available path-loss (29) and maximum range (30) in link budget template in Table A-1, respectively. ISD in meter is evaluated based on the coverage distance in meter. The target performance uses the expected ISD for each scenario (e.g., 500m for urban, 1732m for rural, and 30km for rural with long distance) to compute the target path loss from the path-loss model in [Section 7.4.1, 4] and corresponding performance gap represented as: Gap = target path loss – available path loss in baseline performance.

Preliminary results for urban scenario

In this section, baseline performance is provided for urban scenario with 4GHz carrier frequency and TDD system. For urban scenario, the target ISD is 500m and thus the target performance for ISD=500m is calculated based on the path loss model in [Section 7.4.1, 4] as 121.75 dB in terms of path loss.

Table 3. Preliminary results for urban scenario at FR1

| Carrier | Service and data rate | Frame structure | Pathloss model | # of PRBs | Required SNR (dB) | Baseline performance (dB) | Coverage distance/ISD (m) | Target path loss (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | ISD=500m | Gap |
| 4GHz | 1Mbps for eMBB | DDDSUDDSUU | NLOS(O-to-I) | 30 | -1.05 | 109.35 | 139/240.8 | 121.75 | 12.4 |
|  |  | DDDSU |  |  | 0.4 | 107.9 | 127.7/221.1 |  | 13.85 |
|  | VoIP | - | NLOS(O-to-I) | 4 | 1 | 116.05 | 206.3/357.4 |  | 5.7 |
|  | VoIP for 0.02 target residual BLER | DDDSU | NLOS(O-to-I) | 4 | -2.4 | 119.5 | 252.1/436.7 |  | 2.3 |
|  | PUCCH format 1 with 14 symbols for UCI 2bits | - | NLOS(O-to-I) | 1 | -6.23 | 122.65 | 304.3/527.1 |  | -0.9 |

As observed in Table 3, there are 12.4 dB and 13.85 dB gap from target ISD=500m for PUSCH with 1Mbps eMBB service in case of TDD configuration with DDDSUDDSUU and DDDSU, respectively. For VoIP service, there are 5.7 dB and 2.3dB gap for 0.1 target initial BLER and 0.02 target residual BLER, respectively. Based on the results, it is observed that for urban scenario, PUSCH enhancement in both eMBB and VoIP services is needed with ISD=500m. In case of PUCCH format 1 with 14 symbols for UCI 2bits, there is a -0.9dB gap and hence PUCCH enhancement is not needed with ISD=500m for urban scenario.

Observation 1. For urban scenario, PUSCH enhancement in both eMBB and VoIP services is needed with ISD=500m.

Observation 2. For urban scenario, PUCCH enhancement is not needed with ISD=500m.

Preliminary results for rural scenario at 4GHz

In this section, baseline performance is provided for rural scenario with 4GHz carrier frequency and TDD system. For rural scenario, the target ISD is 1732m and thus the target performance for ISD=1732m is calculated based on the path loss model in [Section 7.4.1, 4] as 131.57 dB in terms of path loss.

Table 4. Preliminary results for rural scenario at 4GHz

| Carrier | Service and data rate | Frame structure | Pathloss model | # of PRBs | Required SNR (dB) | Baseline performance(dB) | Coverage distance/ISD (m) | Target path loss (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | ISD =1732m | Gap |
| 4GHz | 100kbps for eMBB | DDDSUDDSUU | NLOS(O-to-I) | 4 | -1.1 | 126.48 | 738.2/1278.8 | 131.57 | 5.09 |
|  |  |  | NLOS(O-to-O) |  | -1.0 | 128.40 | 827.7/1433.8 |  | 3.17 |
|  |  | DDDSU | NLOS(O-to-I) |  | 0.37 | 125.01 | 676.3/1171.5 |  | 6.56 |
|  |  |  | NLOS(O-to-O) |  | 0.37 | 127.03 | 762.8/1321.4 |  | 4.54 |
|  | VoIP | - | NLOS(O-to-I) | 4 | 1.2 | 124.18 | 643.6/1115 |  | 7.39 |
|  |  |  | NLOS(O-to-O) |  | 1.2 | 126.2 | 726/1257.6 |  | 5.37 |
|  | VoIP for 0.02 target residual BLER | DDDSU | NLOS(O-to-I) |  | -2.4 | 127.8 | 797.7/1381.8 |  | 3.79 |
|  |  |  | NLOS(O-to-O) |  | -6.3 | 133.7 | 1135.1/1966.5 |  | -2.13 |
|  | PUCCH format 1 with 14 symbols for UCI 2bits |  | NLOS(O-to-I) | 1 | -6.23 | 130.74 | 951.3/1648 |  | 0.91 |
|  |  |  | NLOS(O-to-O) |  | -5.93 | 131.94 | 1021.8/1770.1 |  | -0.44 |

As observed in Table 4, there are 5.09 dB and 3.17 dB gap from target ISD=1732m for PUSCH with 100kbps eMBB service in case of TDD configuration with DDDSUDDSUU on NLOS O-to-I and NLOS O-to-O channel, respectively. Also, there are 6.56 dB and 4.54 dB gap from target ISD=1732m for PUSCH with 100kbps eMBB service in case of TDD configuration with DDDSU on NLOS O-to-I and NLOS O-to-O channel, respectively. For VoIP service at 0.1 target initial BLER, there are 7.39 dB and 5.37 dB gap on NLOS O-to-I and NLOS O-to-O channel, respectively. For VoIP service at 0.02 target residual BLER, there are 3.79 dB and -2.13 dB gap on NLOS O-to-I and NLOS O-to-O channel, respectively. Based on the results, it is observed that for rural scenario at 4GHz, PUSCH enhancement in both eMBB and VoIP services is needed with ISD=1732m except for the case of VoIP service at target residual BLER with NLOS O-to-O channel. In case of PUCCH format 1 with 14 symbols for UCI 2bits, there are 0.91dB and -0.44 dB gap on NLOS O-to-I and NLOS O-to-O channel, respectively, and hence PUCCH enhancement is needed with ISD=1732m for rural scenario on NLOS O-to-I but PUCCH enhancement is not needed with ISD=1732m for rural scenario on NLOS O-to-O.

Observation 3. For rural scenario at 4GHz, PUSCH enhancement in both eMBB and VoIP services is needed with ISD=1732m.

Observation 4. For rural scenario at 4GHz, PUCCH enhancement with NLOS O2I channel is needed but that with NLOS O2O channel is not needed with ISD=1732m.

Preliminary results for rural scenario at 2GHz

In this section, baseline performance is provided for rural scenario with 2GHz carrier frequency and FDD system. For rural scenario, the target ISD is 1732m and thus the target performance for ISD=1732m is calculated based on the path loss model in [Section 7.4.1, 4] as 125.55 dB in terms of path loss.

Table 5. Preliminary results for rural scenario at 2GHz

| Carrier | Service and data rate | Frame structure | Pathloss model | # of PRBs | Required SNR (dB) | Baseline Performance (dB) | Coverage distance/ISD (m) | Target Performance (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | ISD=1732m | Gap |
| 2GHz | 100Kbps for eMBB | FDD | NLOS(O-to-I) | 4 | -1.95 | 130.34 | 1330.2/2304.4 | 125.55 | -4.79 |
|  |  |  | NLOS(O-to-O) | 4 | -2.3 | 132.71 | 1532.1/2654 |  | -7.16 |
|  | VoIP | - | NLOS(O-to-I) | 4 | 1.1 | 127.29 | 1109.1/1921.3 |  | -1.74 |
|  |  |  | NLOS(O-to-O) | 4 | 1.1 | 129.31 | 1251.0/2167.2 |  | -3.76 |
|  | VoIP for 0.02 target residual BLER | FDD | NLOS(O-to-I) | 4 | -5.1 | 133.49 | 1605/2780 |  | -7.94 |
|  |  |  | NLOS(O-to-O) | 4 | -8.95 | 139.36 | 2277.2/3944.9 |  | -13.81 |
|  | PUCCH format 1 with 14 symbols for UCI 2bits | - | NLOS(O-to-I) | 1 | -6.1 | 133.62 | 1617.0/2801.2 |  | -8.07 |
|  |  |  | NLOS(O-to-O) |  | -6.1 | 135.12 | 1768.2/3063.2 |  | -9.57 |

As observed in Table 5, there are -4.79 dB and -7.16 dB gap from target ISD=1732m for PUSCH with 100kbps eMBB service in case of FDD system on NLOS O-to-I and NLOS O-to-O channel, respectively. For VoIP service at 0.1 target initial BLER, there are -1.74 dB and -3.76 dB gap on NLOS O-to-I and NLOS O-to-O channel, respectively. For VoIP service at 0.02 target residual BLER, there are -7.94 dB and -13.81 dB gap on NLOS O-to-I and NLOS O-to-O channel, respectively. Based on the results, it is observed that for rural scenario at 2GHz, there is no need of PUSCH enhancement in both eMBB and VoIP services with ISD=1732m. In case of PUCCH format 1 with 14 symbols for UCI 2bits, there are -8.07 dB and -9.57 dB gap on NLOS O-to-I and NLOS O-to-O channel, respectively, and hence PUCCH enhancement is not needed with ISD=1732m for rural scenario at 2GHz.

Observation 5. For rural scenario at 2GHz, no PUSCH and PUCCH enhancement is needed with ISD=1732m.

Preliminary results for rural scenario with long distance

In this section, baseline performance is provided for rural scenario with long distance and FDD system. For rural scenario with long distance, the target ISD is 30km and thus the target performance for ISD=30km is calculated based on the path loss model in [Section 7.4.1, 4] as 142.91 dB in terms of path loss.

Table 6. Preliminary results for rural scenario with long distance

| Carrier | Service and data rate | Frame structure | Pathloss model | # of PRBs | Required SNR (dB) | Baseline performance(dB) | Coverage distance/ISD (km) | Target Performance (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | ISD=30km | Gap |
| 0.7GHz | 100kbps for eMBB | FDD | LOS(O-to-O) | 4 | -7.55 | 139.78 | 14.5/25.1 | 142.91 | 3.13 |
|  | VoIP | - |  | 4 | -3.5 | 135.73 | 11.5/19.8 |  | 7.18 |
|  | VoIP for 0.02 target residual BLER | FDD |  | 4 | -8.7 | 140.93 | 15.5/26.8 |  | 1.98 |
|  | PUCCH format 1 with 14 symbols for UCI 2bits | - |  | 1 | -10.3 | 144.71 | 19.2/33.3 |  | -1.8 |

As observed in Table 6, there is a 3.13 dB gap from target ISD=30km for PUSCH with 100kbps eMBB service in case of FDD system. For VoIP service, there are 7.18 dB and 1.98 dB gap for 0.1 target initial BLER and 0.02 target residual BLER, respectively. Based on the results, it is observed that for rural scenario with long distance, PUSCH enhancement in both eMBB and VoIP services is needed with ISD=30km. In case of PUCCH format 1 with 14 symbols for UCI 2bits, there is a -1.8 dB gap and hence PUCCH enhancement is not needed with ISD=30km for rural scenario with long distance.

Observation 6. For rural scenario with long distance, PUSCH enhancement in both eMBB and VoIP services is needed with ISD=30km.

Observation 7. For urban scenario with long distance, PUCCH enhancement is not needed with ISD=30km.

Conclusion

This contribution discusses the evaluation methodology and simulation assumption for FR1 to identify the baseline coverage performance. In addition, we provide the preliminary results for target scenarios. Following observations and proposals are made:

Proposal 1. The available path loss should be considered in the link budget calculation for coverage performance in FR1 to take into account the different scenarios and channel in FR1.

Proposal 2. Consider IMT-2020 link budget simulation assumptions including Table 1 and Table 2 as the starting point for coverage enhancement evaluation in FR1.

Observation 1. For urban scenario, PUSCH enhancement in both eMBB and VoIP services is needed with ISD=500m.

Observation 2. For urban scenario, PUCCH enhancement is not needed with ISD=500m.

Observation 3. For rural scenario at 4GHz, PUSCH enhancement in both eMBB and VoIP services is needed with ISD=1732m.

Observation 4. For rural scenario at 4GHz, PUCCH enhancement with NLOS O2I channel is needed but that with NLOS O2O channel is not needed with ISD=1732m.

Observation 5. For rural scenario at 2GHz, no PUSCH and PUCCH enhancement is needed with ISD=1732m.

Observation 6. For rural scenario with long distance, PUSCH enhancement in both eMBB and VoIP services is needed with ISD=30km.

Observation 7. For urban scenario with long distance, PUCCH enhancement is not needed with ISD=30km.

- References

- RP-193240, New SID on NR coverage enhancement, RAN#86, Dec. 2019.

- TR 36.824, Evolved Universal Terrestrial Radio Access (E-UTRA); LTE coverage enhancements (Release 11).

- TR 37.910, 	Study on self-evaluation towards IMT-2020 submission (Release 16).

- TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz (Release 16).

- Appendix

Table A-1. Link budget template in [3]

| Parameter | Values |
| --- | --- |
| Scenario |  |
| Frame structure |  |
| Carrier frequency (Hz) |  |
| BS antenna heights (m) |  |
| UT antenna heights (m) |  |
| Cell area reliability for control channel |  |
| Cell area reliability for data channel |  |
| Transmission bit rate for control channel (bit/s) |  |
| Transmission bit rate for data channel (bit/s) |  |
| Target packet error rate for the required SNR in item (19a) for control channel |  |
| Target packet error rate for the required SNR in item (19b) for data channel |  |
| Spectral efficiency (bit/s/Hz) |  |
| Pathloss model (select from LoS or NLoS) |  |
| UE speed (km/h) |  |
| Feeder loss (dB) |  |
| Transmitter |  |
| (1) Number of transmit antennas. (The number shall be within the indicated range in § 8.4 of Report ITU-R M.2412-0) |  |
| (2) Maximal transmit power per antenna (dBm) |  |
| (3) Total transmit power = function of (1) and (2) (dBm) (The value shall not exceed the indicated value in § 8.4 of Report ITU-R M.2412-0) |  |
| (4) Transmitter antenna gain (dBi) |  |
| (5) Transmitter array gain (depends on transmitter array configurations and technologies such as adaptive beam forming, CDD (cyclic delay diversity), etc.) (dB) |  |
| (6) Control channel power boosting gain (dB) |  |
| (7) Data channel power loss due to pilot/control boosting (dB) |  |
| (8) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for downlink) |  |
| (9a) Control channel EIRP = (3) + (4) + (5) + (6) – (8) dBm |  |
| (9b) Data channel EIRP = (3) + (4) + (5) – (7) – (8) dBm |  |
| Receiver |  |
| (10) Number of receive antennas (The number shall be within the indicated range in § 8.4 of Report ITU-R M.2412-0) |  |
| (11) Receiver antenna gain (dBi) |  |
| (11bis) Receiver array gain (depends on transmitter array configurations and technologies such as adaptive beam forming, etc.) (dB) |  |
| (12) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for uplink) |  |
| (13) Receiver noise figure (dB) |  |
| (14) Thermal noise density (dBm/Hz) |  |
| (15a) Receiver interference density for control channel (dBm/Hz) |  |
| (15b) Receiver interference density for data channel (dBm/Hz) |  |
| (16a) Total noise plus interference density for control channel = 10 log (10^(((13) + (14))/10) + 10^((15a)/10)) dBm/Hz |  |
| (16b) Total noise plus interference density for data channel = 10 log (10^(((13) + (14))/10) + 10^((15b)/10)) dBm/Hz |  |
| (17a) Occupied channel bandwidth for control channel (for meeting the requirements of the traffic type) (Hz) |  |
| (17b) Occupied channel bandwidth for data channel (for meeting the requirements of the traffic type) (Hz) |  |
| (18a) Effective noise power for control channel = (16a) + 10 log((17a)) dBm |  |
| (18b) Effective noise power for data channel = (16b) + 10 log((17b)) dBm |  |
| (19a) Required SNR for the control channel (dB) |  |
| (19b) Required SNR for the data channel (dB) |  |
| (20) Receiver implementation margin (dB) |  |
| (21a) H-ARQ gain for control channel (dB) |  |
| (21b) H-ARQ gain for data channel (dB) |  |
| (22a) Receiver sensitivity for control channel = (18a) ++ (19a) + (20) – (21a) dBm |  |
| (22b) Receiver sensitivity for data channel = (18b) ++ (19b) + (20) – (21b) dBm |  |
| (23a) Hardware link budget for control channel = (9a) + (11) + (11bis) − (22a) dB |  |
| (23b) Hardware link budget for data channel = (9b) + (11) + (11bis) − (22b) dB |  |
| Calculation of available pathloss |  |
| (24) Lognormal shadow fading std deviation (dB) |  |
| (25a) Shadow fading margin for control channel (function of the cell area reliability and (24)) (dB) |  |
| (25b) Shadow fading margin for data channel (function of the cell area reliability and (24)) (dB) |  |
| (26) BS selection/macro-diversity gain (dB) |  |
| (27) Penetration margin (dB) |  |
| (28) Other gains (dB) (if any please specify) |  |
| (29a) Available path loss for control channel = (23a) – (25a) + (26) – (27) + (28) – (12) dB |  |
| (29b) Available path loss for data channel = (23b) – (25b) + (26) – (27) + (28) – (12) dB |  |
| Range/coverage efficiency calculation |  |
| (30a) Maximum range for control channel (based on (29a) and according to the system configuration section of the link budget) (m) |  |
| (30b) Maximum range for data channel (based on (29b) and according to the system configuration section of the link budget) (m) |  |