3GPP TSG RAN WG1 #101			R1-2003991

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	7.2.4.2.2

Source:	 Spreadtrum Communications

Title:	 Remaining issues in NR sidelink mode 2 resource allocation

Document for:	 Discussion and decision

- Introduction

Based on the latest 3GPP progress [1-6], in this contribution, we discuss some remaining issues of resource allocation mode 2 for NR V2X communication and put forward our considerations and possible solutions for them.

- Discussion

  - Re-evaluation

In RAN1 #100e, the following agreements were achieved for re-evaluation.

Agreements: - For re-evaluation of a pre-selected resource contained in a slot ‘k’ to be first time signaled in a slot ‘m’, where k ≥ m, - Step 1 of the resource (re-)selection procedure is performed at least at the moment ‘m-T3’, and if the pre-selected resource is not in the identified candidate resource set, Step 2 is triggered for reselection of the resource - Re-evaluations before the moment ‘m-T3’ or after ‘m-T3’ but before ‘m’ are not precluded and are up to UE implementation - FFS whether to mandate a UE to perform Step 1 checking every slot before ‘m-T3’ - FFS whether evaluation of Step 2 has to ensure any introduced timing restrictions between pre-selected and re-selected resources when re-evaluation is triggered, and whether it is allowed to change the pre-selected but not reserved resources which are still in the candidate resource set in order to ensure the timing restrictions. - FFS whether for the case of enabled periodic reservation, already reserved resources in upcoming periods can be re-evaluated.

In our view, there is no need to mandate a UE to perform Step 1 checking every slot before ‘m-T3’ because the UE complexity will increase, although it is possible to reselect an earlier resource if performing Step 1 at the moment before ‘m-T3’, which may reduce transmission latency. Furthermore, since UE can obtain more sufficient sensing results at the moment ‘m-T3’, it is enough to perform Step 1 at the moment ‘m-T3’. Therefore, the existing agreements are sufficient and Step 1 checking every slot is up to UE implementation.

Proposal 1: Mandating a UE to perform Step 1 checking every slot before ‘m-T3’ is not supported.

  - Pre-emption

In RAN1 #100b-e, the following agreements were achieved for the RRC parameter for pre-emption configuration.

Agreements: Finalize the RRC parameter for pre-emption activation per resource pool by - Disabled - Enabled. Default is without a priority level (i.e., pre-emption is applicable to all levels). - Can optionally configuring a priority level p_preemption {1…8} (the value range is a working assumption), and (as a working assumption regarding “<”) if prioRX < p_preemption, and prioTX > prioRX, then pre-emption can be triggered - Note: In the inequalities it is assumed that the lowest priority value corresponds to the highest priority/importance traffic - prioRX is the priority associated with the resource indicated in SCI, as per 8.1.4 in 38.214 - prioTX is L1 priority within a UE associated with the reserved resources, as per 8.1.4 in 38.214

According to the intention of the agreements, pre-emption can be enabled/disabled per resource pool. When pre-emption is enabled in a resource pool, default case is without a priority level in which pre-emption is enabled for all priority levels. Furthermore, high layers can optionally configure a priority level p_preemption, causing that pre-emption is enabled for some priority levels and disabled for the others. However, if captured by “p_preemption {1…8}” and “prioRX < p_preemption”, it will be unclear, that is, the following confusing issues exist.

- Issue 1: when pre-emption is enabled in a resource pool with configuring a priority level p_preemption=1, pre-emption cannot be enabled for any priority level, i.e., pre-emption is actually disabled in the resource pool, which is inconsistent with the enabling configuration for the resource pool.

- Issue 2: when pre-emption is enabled in a resource pool with configuring a priority level p_preemption=8, pre-emption can be enabled for all priority levels except level 8, which is essentially the same function as the default case.

In order to avoid confusion stated above, there are two alternatives.

- Alt 1: “p_preemption {2…7}” and “prioRX < p_preemption”

- Alt 2: “p_preemption {1…6}” and “prioRX≤p_preemption”

Proposal 2: To make the RRC configuration clearer, the following two alternatives can be used:

- Alt 1: “p_preemption {2…7}” and “prioRX < p_preemption”

- Alt 2: “p_preemption {1…6}” and “prioRX≤p_preemption”

Agreements: - Once pre-emption re-selection condition is met at the UE, re-selection is performed for all resources which satisfy the pre-emption re-selection condition - A UE ensures the HARQ RTT related minimum time gap Z agreed in RAN1#100-e, between re-selected and non-preempted resources during the re-selection triggered by pre-emption - FFS cases when timing restriction could not be met - FFS whether/how to extend it to periodic reservations Agreements: - It is up to UE implementation to reselect any pre-selected but not reserved resource which is still in the identified resource set after Step 1 in order to ensure the timing restrictions during reselection triggered by re-evaluation and/or pre-emption - The timing restrictions at least include the HARQ RTT related minimum gap Z agreed in RAN1#100e - FFS how to handle the case that there is no resources satisfying the timing restrictions in the identified resource set after Step 1

In RAN1 #100b-e, the above agreements were achieved.

As for the cases when HARQ RTT could not be met in resource reselection for pre-emption and/or re-evaluation, UE can reselect resources without HARQ RTT restriction. In such cases, mixed retransmission of HARQ retransmission and blind retransmission should be supported by indicating RX UE to send or not send HARQ feedback in SCI.

Proposal 3: When HARQ RTT could not be met in resource reselection for pre-emption, mixed retransmission is supported.

Regarding to the FFS “whether/how to extend it to periodic reservations”, there are two aspects to be discussed.

The first is whether the periodically reserved resource can be a pre-empted resource. If yes, as shown in Figure 1, the initial resource in a period has been already periodically reserved and satisfies the pre-emption reselection condition, resource reselection of the resource is triggered. Furthermore, the reselection is performed for all resources which satisfy the pre-emption reselection condition in the period.

The second is whether the reselected resources for pre-emption will be periodically reserved. According to the above agreements, re-selection for pre-emption is performed for all resources which satisfy the pre-emption reselection condition and any pre-selected but not reserved resource which is still in the identified resource set after Step 1 by UE implementation, i.e., pre-emption related resources in this period are reselected. However, if the reselected resources are periodically reserved, it means that the pre-emption unrelated resources in the subsequent periods are also reselected. Furthermore, it may cause resource collision since other UEs can only sense the previously periodically reserved resources but not the re-reserved resources before the transmission of the reselected resources. Therefore, we think the reselected resources for pre-emption cannot be periodically reserved.

Figure 1. Resource reselection for pre-emption in periodic reservations

Proposal 4: If the initial resource in a period has been already periodically reserved and satisfies the pre-emption reselection condition, resource reselection of the resource is triggered.

Proposal 5: The reselected resources for pre-emption cannot be periodically reserved.

  - Backward indication

In RAN1 #100e, the following agreements were achieved.

Agreements: - Down-select in the next meeting one of the following options - Option 1: There is no separate field in the first stage SCI indicating a resource index for the purpose of backward indication, i.e., backward indication is not supported - Option 2: When periodic reservations are enabled in a resource pool, a separate field of 1 bit in the first stage SCI indicates a resource index for the purpose of backward indication - Option 3: When periodic reservations are enabled in a resource pool, a separate field of ceil(log2(Nmax)) bit in the first stage SCI indicates a resource index for the purpose of backward indication

In our view, backward indication should not be supported in NR V2X.

First, the motivation of backward indication is not clear. In LTE V2X, backward indication can restore reservation information about the missed first transmission. However, in NR V2X, thanks to reserving up to 3 resources by 1 transmission, for certain missed transmission, it has already supported to restore it from previous transmission if exists. The benefit provided by backward indication is quite limited. Then, in NR V2X, it supports to (pre-)configure up to 32 (re-)transmissions, which is different from the maximum 2 (re-)transmissions in LTE V2X. If backward indication is supported in NR V2X, it will sacrifice the opportunity to reserve future transmission(s), which is not friendly to sensing mechanism.

Proposal 6: Support Option 1: There is no separate field in the first stage SCI indicating a resource index for the purpose of backward indication, i.e., backward indication is not supported.

  - Periods for exclusion if a slot is not monitored in a sensing window

In the email discussion of RAN1 #100b e-meeting, the following proposal has been discussed.

Proposal: - For exclusion of slots in the selection window which correspond to slots not monitored in the sensing window, a UE applies a separately (pre-)configured sub-set (including empty and full set possibilities) of periodicities and applies the period used for transmission, if any.

Regarding to periods for exclusion if a slot is not monitored in a sensing window, we support reusing the mechanism in LTE V2X at least in R16, i.e. applying all the periods configured for a resource pool. We don’t support applying a separately (pre-)configured subset of periodicities, since a subset cannot fully cover all the periods actually used, and thus collision will be caused. Furthermore, we don’t expect to perform worse than LTE V2X.

Proposal 7: For exclusion of slots in the selection window which correspond to slots not monitored in the sensing window, LTE V2X mechanism is reused at least in R16.

  - Processing time of Tproc,0, Tproc,1 and T3

In RAN1 #98b, the following agreements were achieved for sensing window and resource selection window.

Agreements: - For a given time instance n when resource (re-)selection and re-evaluation procedure is triggered - The resource selection window starts at time instance (n + T1), T1 ≥ 0 and ends at time instance (n + T2) - The start of selection window T1 is up to UE implementation subject to T1 ≤ Tproc,1  T2 is up to UE implementation with the following details as a working assumption:  T2 ≥ T2min  If T2min > Remaining PDB, then T2min is modified to be equal to Remaining PDB  FFS other details of T2min including whether the minimum window duration T2min - T1 is a function of priority  UE selection of T2 shall fulfil the latency requirement, i.e. T2 ≤ Remaining PDB - A sensing window is defined by time interval [n–T0, n–Tproc,0) - T0 is (pre-)configured, T0 > Tproc,0 FFS further details - FFS, if Tproc,0 and Tproc,1 are defined separately or as a sum - FFS relation of T3, Tproc,0, Tproc,1 - Time instances n, T0, T1, T2, T2min are measured in slots, FFS Tproc,0 and Tproc,1

According to the agreements, some details of the Tproc,0, Tproc,1 and T3 need to be further discussed.

For the issue whether Tproc,0 and Tproc,1 are defined separately or as a sum, sensing and resource selection window for both resource (re-)selection and re-evaluation/pre-emption procedure should be taken into account. For these procedures, Tproc,0 is the required time for processing the sensing results. As for Tproc,1, it is the required time for resource selection processing, and also for data encoding processing time for resource (re-)selection or for dropping transmission on previous reserved resource for re-evaluation/pre-emption. Considering the usage of Tproc,0 and Tproc,1, Tproc,0 and Tproc,1 should be defined separately. Furthermore, since resource time granularity and other time instances are measured in slots, Tproc,0 and Tproc,1 should be also measured in slots for consistency.

Regarding T3, based on the above analysis, we propose that T3=Tproc,1. The re-evaluation should be triggered no later than ‘m-T3’, i.e. the required processing time needs to be ensured.

Proposal 8: For sensing and resource selection window:

- Tproc,0 and Tproc,1 are defined separately.

- Tproc,0 and Tproc,1 are measured in slots.

- T3=Tproc,1.

  - Handling of reserved but unused resources

In RAN1 #97, the following agreements were achieved.

Agreements: - NR V2X Mode-2 supports resource reservation for feedback-based PSSCH retransmissions by signaling associated with a prior transmission of the same TB - FFS impact on subsequent sensing and resource selection procedures - At least from the transmitter perspective of this TB, usage of HARQ feedback for release of unused resource(s) is supported - No additional signaling is defined for the purpose of release of unused resources by the transmitting UE - FFS the behavior of the receiver UE(s) of this TB and other UEs

According to the agreements, Tx UE can use the unused resource for another TB transmission without additional signaling. In this case, Rx UE is able to distinguish between a retransmission for the same TB and a new transmission for another TB by NDI, HARQ ID, source ID and destination ID. While in other cases, i.e., associated Rx UE(s) or other UEs using the unused resource, it causes some problems.

Firstly, if the associated Rx UE is supported to use the unused resource, when Tx UE misses to detect ACK sent by Rx UE, both Rx UE and Tx UE will use the same resource for transmission and thus resource collision will be caused. To avoid this, the associated Rx UE(s) should not use the unused resource. Secondly, if other UEs are supported to use the unused resource, other UEs need to detect all potential HARQ feedback information on PSFCH. However, it was agreed that no additional sensing for other channels. Therefore, usage of the unused resource by the associated RX UE(s) and other UEs is not supported.

Proposal 9: Usage of the unused resource by the associated RX UE(s) and other UEs is not supported.

In NR V2X, even for blind retransmission, each single transmission is accompanied with an individual SCI, which is different from the mechanism in Uu link. Therefore, there is no need to indicate the total number of (re)transmissions of a TB in SCI, instead, Rx UE can identify multiple transmissions for the same TB and perform HARQ combining based on HARQ ID, NDI, source ID, and destination ID.

Proposal 10: The total number of (re)transmissions of a TB is not indicated in SCI.

  - Converting the physical interval to logical slots

According to TS 38.214[8], “the resource reservation interval, Prsvp_TX, if provided, is converted from units of ms to units of logical slots, resulting in Prsvp_TX'”, the mapping rule for converting the physical interval to logical slots need to be discussed in NR V2X. In LTE V2X, a parameter Pstep is defined to convert the resource reservation interval to logical subframes. The conversion formula is Prsvp_TX'=Pstep×Prsvp_TX/100, where the value of Pstep for each TDD configuration is shown in the Table 1 and the value of Pstep/100 is equal to the proportion of UL subframes in one frame since only UL subframes are supported for SL transmission in shared carrier.

Table 1. Determination of Pstep in LTE V2X

|  | Pstep |
| --- | --- |
| TDD with UL/DL configuration 0 | 60 |
| TDD with UL/DL configuration 1 | 40 |
| TDD with UL/DL configuration 2 | 20 |
| TDD with UL/DL configuration 3 | 30 |
| TDD with UL/DL configuration 4 | 20 |
| TDD with UL/DL configuration 5 | 10 |
| TDD with UL/DL configuration 6 | 50 |
| Otherwise | 100 |

Similarly, the conversion formula in NR V2X can be derived. Since one or two DL-UL pattern(s) on a reference SCS is/are configured for cell-specific semi-static slot configuration in NR Uu, consider the configured pattern(s) as a period P in ms. NR V2X supports SL transmission in cell-specific UL resources that meet the SL symbol configuration. According to SL symbol configuration，assume that the number of slots that can be used for SL transmission in a period is N. Therefore, the conversion formula in NR V2X can be

Prsvp_TX'slot=NslotPms×2μ×Prsvp_TXms×2μ=NP×Prsvp_TX

Where the value of N/(P×2μ) is equal to the proportion of slots that can be used for SL transmission in period P. μ is the numerology of BWP.

Proposal 11: Support the formula to convert the physical interval to logical slots in NR V2X:

Prsvp_TX'=NP×Prsvp_TX

where P is the period of configured DL-UL pattern(s) in NR Uu , N is the total number of slots that can be used for SL transmission in period P, and Prsvp_TX denotes the reservation interval.

  - UE procedure for determining the subset of resources to be reported to higher layers

According to Section 8.1.4 of TS 38.214 [8], resource identification procedure step 6 is reused from LTE V2X, which is only applicable when Prsvp_TX is provided. However, in NR V2X, it is also necessary to consider the case where Prsvp_TX is not provided. If Prsvp_TX is not provided, only the resource collision avoidance in resource selection window needs to be considered.

Proposal 12: Resource identification procedure should consider the case where Prsvp_TX is not provided.

The following TP for TS 38.214 [8] is provided:

----------------------------------------------------Begin text proposal for 38.214----------------------------------------------------

8.1.4	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

<Unchanged parts omitted>

6)	The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions:

<Unchanged parts omitted>

c)	the SCI format received in slot tmSLor the same SCI format which, if and only if the "Resource reservation period" field is present in the received SCI format 0-1, is assumed to be received in slot(s) tm+q×Prsvp_RX'SL determines according to clause [TBD] in [6, TS 38.213] the set of resource blocks and slots which overlaps with Rx,y if Prsvp_TX is not provided or Rx,y+j×Prsvp_TX' if Prsvp_TX is provided, for q=1, 2, …, Q and j=0, 1, …, Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots,  Q=TscalPrsvp_RX  if Prsvp_RX< Tscal and  n'-m≤Prsvp_RX', where tn'SL = n if slot n belongs to the set t0SL,t1SL,...,tTmaxSL, otherwise slot tn'SL is the first slot after slot n belonging to the set t0SL,t1SL,...,tTmaxSL; otherwise Q=1. Tscal is FFS.

<Unchanged parts omitted>

-----------------------------------------------------End text proposal for 38.214-----------------------------------------------------

Since the number of candidate single-slot resources remaining in the set SA may be greater than 0.2⋅Mtotal, 0.2⋅Mtotal candidates in the set SA should be selected and reported to higher layers. Then how to select is up to UE implementation, for example, it can be selected randomly or based on RSRP measurement.

Proposal 13: 0.2⋅Mtotal candidates should be selected and reported to higher layers.

The following TP for TS 38.214 [8] is provided:

----------------------------------------------------Begin text proposal for 38.214----------------------------------------------------

8.1.4	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

<Unchanged parts omitted>

4)	The set SA is initialized to the set of all the candidate single-slot resources. The set SB is initialized to an empty set.

<Unchanged parts omitted>

7)	If the number of candidate single-slot resources remaining in the set SA is smaller than 0.2⋅Mtotal, then Th(pi) is increased by 3 dB for each priority value Th(pi) and the procedure continues with step 4. Otherwise, the UE selects the candidate single-slot resource Rx,y from the set SA to SB. This step is repeated until the number of candidate single-slot resources in the set SB becomes greater than or equal to 0.2⋅Mtotal.

The UE shall report set SBA to higher layers.

<Unchanged parts omitted>

-----------------------------------------------------End text proposal for 38.214-----------------------------------------------------

- Conclusions

In this contribution, we discussed the resource allocation mode 2 for NR V2X sidelink transmission and following conclusions are proposed:

Proposal 1: Mandating a UE to perform Step 1 checking every slot before ‘m-T3’ is not supported.

Proposal 2: To make the RRC configuration clearer, the following two alternatives can be used:

- Alt 1: “p_preemption {2…7}” and “prioRX < p_preemption”

- Alt 2: “p_preemption {1…6}” and “prioRX≤p_preemption”

Proposal 3: When HARQ RTT could not be met in resource reselection for pre-emption, mixed retransmission is supported.

Proposal 4: If the initial resource in a period has been already periodically reserved and satisfies the pre-emption reselection condition, resource reselection of the resource is triggered.

Proposal 5: The reselected resources for pre-emption cannot be periodically reserved.

Proposal 6: Support Option 1: There is no separate field in the first stage SCI indicating a resource index for the purpose of backward indication, i.e., backward indication is not supported.

Proposal 7: For exclusion of slots in the selection window which correspond to slots not monitored in the sensing window, LTE V2X mechanism is reused at least in R16.

Proposal 8: For sensing and resource selection window:

- Tproc,0 and Tproc,1 are defined separately.

- Tproc,0 and Tproc,1 are measured in slots.

- T3=Tproc,1.

Proposal 9: Usage of the unused resource by the associated RX UE(s) and other UEs is not supported.

Proposal 10: The total number of (re)transmissions of a TB is not indicated in SCI.

Proposal 11: Support the formula to convert the physical interval to logical slots in NR V2X:

Prsvp_TX'=NP×Prsvp_TX

where P is the period of configured DL-UL pattern(s) in NR Uu , N is the total number of slots that can be used for SL transmission in period P, and Prsvp_TX denotes the reservation interval.

Proposal 12: Resource identification procedure should consider the case where Prsvp_TX is not provided.

The following TP for TS 38.214 [8] is provided:

----------------------------------------------------Begin text proposal for 38.214----------------------------------------------------

8.1.4	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

<Unchanged parts omitted>

6)	The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions:

<Unchanged parts omitted>

c)	the SCI format received in slot tmSLor the same SCI format which, if and only if the "Resource reservation period" field is present in the received SCI format 0-1, is assumed to be received in slot(s) tm+q×Prsvp_RX'SL determines according to clause [TBD] in [6, TS 38.213] the set of resource blocks and slots which overlaps with Rx,y if Prsvp_TX is not provided or Rx,y+j×Prsvp_TX' if Prsvp_TX is provided, for q=1, 2, …, Q and j=0, 1, …, Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots,  Q=TscalPrsvp_RX  if Prsvp_RX< Tscal and  n'-m≤Prsvp_RX', where tn'SL = n if slot n belongs to the set t0SL,t1SL,...,tTmaxSL, otherwise slot tn'SL is the first slot after slot n belonging to the set t0SL,t1SL,...,tTmaxSL; otherwise Q=1. Tscal is FFS.

<Unchanged parts omitted>

-----------------------------------------------------End text proposal for 38.214-----------------------------------------------------

Proposal 13: 0.2⋅Mtotal candidates should be selected and reported to higher layers.

The following TP for TS 38.214 [8] is provided:

----------------------------------------------------Begin text proposal for 38.214----------------------------------------------------

8.1.4	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

<Unchanged parts omitted>

4)	The set SA is initialized to the set of all the candidate single-slot resources. The set SB is initialized to an empty set.

<Unchanged parts omitted>

7)	If the number of candidate single-slot resources remaining in the set SA is smaller than 0.2⋅Mtotal, then Th(pi) is increased by 3 dB for each priority value Th(pi) and the procedure continues with step 4. Otherwise, the UE selects the candidate single-slot resource Rx,y from the set SA to SB. This step is repeated until the number of candidate single-slot resources in the set SB becomes greater than or equal to 0.2⋅Mtotal.

The UE shall report set SBA to higher layers.

<Unchanged parts omitted>

-----------------------------------------------------End text proposal for 38.214-----------------------------------------------------

- References

- “RAN1 Chairman's Notes”, RAN1#97, Reno, USA, May 13th – 17th, 2019.

- “RAN1 Chairman's Notes”, RAN1#98, Reno, USA, August 26th – 30th, 2019.

- “RAN1 Chairman's Notes”, RAN1#98bis, Chongqing, China, October 14th – 20th, 2019.

- “RAN1 Chairman's Notes”, RAN1#99, Reno, USA, November 18th – 22nd, 2019.

- “RAN1 Chairman's Notes”, RAN1#100e, e-Meeting, February 24th – March 6th, 2020.

- “RAN1 Chairman's Notes”, RAN1#100bis, e-Meeting, April 20th –30th, 2020.