3GPP TSG RAN WG1 Meeting #101-e	                              						R1-2004145

E-meeting, 25 May – 5 June, 2020

Agenda Item:	7.2.11.11

Source:	Huawei, HiSilicon

Title:	Rel-16 UE features for CLI/RIM

Document for:	Discussion and Decision

# Introduction

In this contribution, we will provide some views towards the CLI/RIM related part.

# Discussion

- FG 17-1 & FG 17-2

It is FFS regarding licensed and unlicensed spectrum differentiation. However, in the past discussion of CLI, there is no discussion regarding the unlicensed spectrum. Furthermore, unlicensed spectrum has different transmission mechanism, thus, the CLI feature cannot be assumed as applicable to unlicensed spectrum. In our view, CLI on unlicensed spectrum is not supported in Rel-16.

Proposal 1: CLI on unlicensed spectrum is not supported in Rel-16.

Secondly, since 17-3 and 17-4 are to be introduced, instead of a note in 17-3/4, the content of R1-2001523 should be captured in both 17-1 and 17-2 as part of component 1, making them more self-contained, e.g. add “UE shall prioritize the DL receiving in case DL signal/channel and CLI RSSI resources are FDMed” in component 1 in 17-1 and add  “UE shall prioritize the DL receiving in case DL signal/channel and SRS-RSRP resources are FDMed” in 17-2. And remove the corresponding part from FG17-3 and FG17-4.

Proposal 2:

- FG 17-1: add “UE shall prioritize the DL receiving in case DL signal/channel and CLI RSSI resources are FDMed” in component 1 in both 17-1.

- FG 17-2: add “UE shall prioritize the DL receiving in case DL signal/channel and SRS-RSRP resources are FDMed” in component 1 in both 17-2.

- FG 17-3/4: remove the corresponding notes from FG17-3 and FG17-4.

# Conclusions

In this contribution, we discuss the remaining issues for Rel-16 UE feature groups of CLI/RIM, and have the following proposals, along with a table with corresponding changes in Appendix.

Proposal 1: CLI on unlicensed spectrum is not supported in Rel-16.

Proposal 2:

- FG 17-1: add “UE shall prioritize the DL receiving in case DL signal/channel and CLI RSSI resources are FDMed” in component 1 in both 17-1.

- FG 17-2: add “UE shall prioritize the DL receiving in case DL signal/channel and SRS-RSRP resources are FDMed” in component 1 in both 17-2 and

- FG 17-3/4: remove the corresponding notes from FG17-3 and FG17-4.

# References

R1-2003203, “Summary on email discussion [100b-e-NR-UEFeatures-Remaining] NR_CLI_RIM”, 3GPP, RAN1#100b-e

# Appendix

| Index | Feature group | Components | Prerequisite feature groups | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 17-1 | CLI-RSSI measurement | 1. Support CLI-RSSI measurement. The max number of resources across all CCs configured to measure RSSI simultaneously shall not exceed 64. UE shall prioritize the DL receiving in case DL signal/channel and CLI RSSI resources are FDMed 2. Maximum number of measurement resources configured for CLI-RSSI measurement |  | Per UE] | No (TDD only) | Yes | Candidate values for component 2 are {8, 16, 32, 64}. Only licensed spectrum is supported | Optional with capability signaling |
| 17-2 | SRS-RSRP measurement | 1. Support SRS-RSRP measurement. The max number of SRS resources across all CCs configured to measure SRS-RSRP simultaneously shall not exceed 32. UE shall prioritize the DL receiving in case DL signal/channel and SRS-RSRP resources are FDMed 2. Maximum number of measurement resources configured for SRS-RSRP measurement 3. Maximum number of measurement resources configured for SRS-RSRP measurement within a slot |  | Per UE | No (TDD only) | Yes | Candidate values for component 2 are {4, 8, 16, 32}. Candidate values for component 3 are {2, 4, 8}. Only licensed spectrum is supported | Optional with capability signaling |
| 17-3 | Simultaneous reception of DL signals/channels and CLI-RSSI measurement resource | Support simultaneous reception of DL signals/channels and CLI-RSSI measurement resource | 17-1 | Per UE | No (TDD only) | Yees |  | Optional with capability signaling |
| 17-4 | Simultaneous reception of DL signals/channels and SRS-RSRP measurement resource | Support simultaneous reception of DL signals/channels and SRS-RSRP measurement resource | 17-2 | Per UE | No (TDD only) | Yes |  | Optional with capability signaling |