3GPP TSG-RAN WG1 Meeting #101-e	Tdoc R1-2004203

e-Meeting, May 25th– June 5th, 2020

Agenda Item:	7.2.11.9

Source:	Ericsson

Title:	UE features for NR mobility enhancements

Document for:	Decision

# 1	Introduction

In this contribution, we discuss the open issues on the UE features for NR mobility enhancements.

# 2	Discussion

In [1], the outcome of the email discussion for the UE features for Release-16 are captured. Here we provide input to the discussion on the UE features related to mobility enhancements.

The currently proposed features for mobility enhancements are listed below. Our proposed changes are introduced directly in Table 1, and commented below.

Table 1: Proposed updates for UE features related to mobility enhancements

| Index | Feature group | Components | Prerequisite feature groups |  | Need for the gNB to know |  | V2X | Consequence if the feature is not supported by the UE | Type (the ‘type’ | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 21-1a | Intra-frequency DAPS HO | Support of intra-frequency DAPS-HO 1) Support of simultaneous DL reception of PDCCH and PDSCH from source and target cell in DAPS-HO 2) Support of PDCCH blind decoding capability in the first MCG and second MCG. |  | DAPS (Note: RAN2 feature) |  | Yes | N/A | The network cannot configure UE with DAPS HO | Per Band | No | N/A | N/A |  | [Optional with capability signalling] |
| 21-1b | Inter-frequency DAPS HO | Support of inter-frequency DAPS-HO 1) Support of simultaneous DL reception of PDCCH and PDSCH from source and target cell in DAPS-HO 2) Support of PDCCH blind decoding capability in the first MCG and second MCG. |  | DAPS (Note: RAN2 feature) |  | Yes | N/A | The network cannot configure UE with DAPS HO | Per BC | No | N/A | N/A |  | [Optional with capability signalling] |
| 21-2 | Basic UE power sharing for DAPS HO | Support of semi-static power sharing mode1 |  | DAPS (Note: RAN2 feature) |  | Yes | N/A | FFS: The UE is only able to drop the transmission to the source. | Per BC | No | N/A | N/A |  | Optional with capability signalling |
| 21-2a | Semi-static UL power sharing mode 2 for DAPS HO | Support of semi-static power sharing mode 2 |  | 21-2 |  | Yes | N/A |  | Per BC | No | N/A | N/A |  | Optional with capability signalling |
| 21-2b | Dynamic UL power sharing for DAPS HO | Support of dynamic power sharing |  | 21-2 |  | Yes | N/A |  | Per BC | No | N/A | N/A |  | Optional with capability signaling |
| [21-2d] | [UL transmission cancellation] | [Indicates support of cancelling UL transmission to the source cell] |  | DAPS (Note: RAN2 feature) |  |  |  |  |  |  |  |  |  |  |

Comments:

FG 21-2d:

This feature is not needed, RAN1 has already agreed that the UE should be able to drop in case there is an overlap.

For UE features related to mobility enhancements, agree on the updates captured in Table 1.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	For UE features related to mobility enhancements, agree on the updates captured in Table 1.

# References

R1-2003073, RAN1 UE features list for Rel-16 NR after RAN1#100bis-E, AT&T, NTT DOCOMO INC., RAN1 #100bis-e, April 2020