3GPP TSG RAN WG1 Meeting #101-e		R1-2004269

e-Meeting, May 25th – June 5th, 2020

Agenda item:       8.5

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Discussion on Rel-17 NR FeMIMO

Document for:     Discussion and Decision

# 1	Introduction

In the new Rel-17 FeMIMO WID [1], the objectives were identified for further enhancing CSI measurement and reporting in NR MIMO:

- Evaluate and, if needed, specify CSI reporting for DL multi-TRP and/or multi-panel transmission to enable more dynamic channel/interference hypotheses for NCJT, targeting both FR1 and FR2

- Evaluate and, if needed, specify Type II port selection codebook enhancement (based on Rel.15/16 Type II port selection) where information related to angle(s) and delay(s) are estimated at the gNB based on SRS by utilizing DL/UL reciprocity of angle and delay, and the remaining DL CSI is reported by the UE, mainly targeting FDD FR1 to achieve better trade-off among UE complexity, performance and reporting overhead.

In this document we present our initial considerations on the EVM assumptions for the partial FDD reciprocity evaluation.

# 2	EVM assumptions for partial reciprocity in FDD

The SLS assumptions for Rel-16 MU-CSI enhancement (copied in the Appendix) can be a starting point with additional updates/modifications summarised below in the following sections:

- System model for FDD reciprocity

- SRS configuration

- Beamformed CSI-RS configuration

- Performance evaluation metrics and baseline

## 2.1	System model for FDD reciprocity

In this section, we discuss aspects of the channel model and measuring the channel state information.  Since the work item objective targets partial reciprocity in an FDD system, the carrier frequency used in the evaluation should reflect currently defined NR bands as found in Table 5.2-1 of TS 38.104 [3].  Two alternatives are:

Alt. 1:  2 GHz

Alt. 2:  4 GHz

Alt. 2 was used in the evaluation assumptions for Rel-16 MU-CSI enhancements.  However, the FDD bands defined in [3] are more often close to 2 GHz.  In addition, the duplexing distance found in [3] ranges from 30 MHz to several hundred MHz, but most often lies near 100 MHz where the UL frequency is smaller than the DL frequency.  Therefore, we propose to use Alt. 1 as the carrier frequency with a duplexing distance of 100 MHz.

Proposal 1:  Adopt 2 GHz as the carrier frequency with a duplexing distance of 100 MHz in the evaluation methodology.

The base channel model most naturally comes from TR 38.901 using the Dense Urban scenario with the UMa channel model.  While this choice makes sense for one transmission direction, there are three existing alternatives for the partially reciprocal channel in the reverse direction:

Alt. 1:  The reciprocity model of Section 5.3 of TR 36.897 [4]

Alt. 2:  The correlation model for multi-frequency simulations in Section 7.6.5 of TR 38.901 [4]

Alt. 3:  The alternative correlation model in Section 7.6.5.1 of TR 38.901 [4]

The model of Alt. 1 has been designed specifically for reciprocity, is consistent, and is more specific about parameter generation than the models in TR 38.901.  The model of Alt. 2 is similar to the model of Alt. 1 but is less clear.  For example, the model indicates that the cluster delays are the same for all frequency bands, yet also indicates a method for the delay spread to vary with frequency.  This inconsistency would need to be addressed for this model.  In addition, Alt. 2 appears to be designed for cases such as multiple DL frequencies (e.g., carrier aggregation) where frequency separation may be greater than in the FDD UL/DL case.  The channel of Alt. 3 is an alternative channel generation method which differs significantly from the approach used for the base models in TR 38.901.  This model should be avoided so that comparisons against the reference system (without partial reciprocity) can easily be made.  Therefore, we propose to adopt Alt. 1.

Proposal 2:  Adopt the reciprocity channel model of Section 5.3 of TR 36.897.

When simulating estimates of the UL channel based on SRS signals, it is important to realistically model the channel estimation error.  Two alternatives for modelling the error include:

Alt. 1:  Using a realistic channel estimator.

Alt. 2:  Using the SRS error model in Table A.1-2 of TR 36.897.

Alt. 1 has the advantage of directly modeling the channel estimation error, while Alt. 2 involves determining several parameters and disclosing the method used to determine those parameters.  Therefore, we prefer Alt. 1.

Proposal 3:  Use a realistic channel estimator to model the estimation error for SRS signals.

In order to obtain CSI on the DL, companies may find it desirable to test schemes were more than 32 beamformed CSI-RS ports are used.  This may require a longer CSI-RS periodicity where ports are time-multiplexed.  Companies should indicate the period and multiplexing approach for the CSI-RS signals.

Proposal 4:  Companies should indicate the period and multiplexing approach for CSI-RS signals for DL channel measurement.

Table 1 summarises the system model assumptions discussed above. SLS assumptions from Rel-16 MU-CSI enhancement are taken as reference with modifications specific to the new WID.

Table 1. Proposed system model assumptions. The SLS assumptions for Rel-16 CSI WI are takes as reference, with modifications marked in bold.

| Parameter |  |  | Value |
| --- | --- | --- | --- |
| Duplex, Waveform |  |  | FDD, OFDM |
| Multiple access |  |  | OFDMA |
| Scenario |  |  | Dense Urban (Macro only) is a baseline. |
| Frequency Range |  |  | FR1 only |
| DL carrier frequency |  |  | Alt1. 2GHz Alt2. 4GHz |
| UL carrier frequency |  |  | DL carrier - duplexing gap |
| Duplexing gap |  |  | 100MHz |
| Inter-BS distance |  |  | 200m |
| Channel model |  |  | Alt1. According to the TR 38.901 with reciprocity model of Section 5.3, TR 36.897 Alt2. According to the TR 38.901 with reciprocity model of Section 7.6.5, TR 36.901 Alt3. According to the TR 38.901 with alternative model of Section 7.6.5.1, TR 36.901 |
| Antenna setup and port layouts at gNB |  |  | Baseline options. Companies to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations for reciprocity-based beamformed CSI-RS ports are not precluded. |
| Antenna setup and port layouts at UE |  |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for rank ≤ 4 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for rank ≤ 2 Other configurations are not precluded. |
| BS Tx power |  |  | 41 dBm |
| BS antenna height |  |  | 25m |
| UE antenna height & gain |  |  | Follow TR36.873 |
| UE receiver noise figure |  |  | 9dB |
| BS receiver noise figure |  |  | 5dB |
| Modulation |  |  | Up to 256QAM |
| Coding on PDSCH |  |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot |  | 14 OFDM symbol slot |
|  | SCS |  | 15kHz |
| Number of RBs |  |  | 52 for 15 kHz SCS |
| Simulation bandwidth |  |  | 10 MHz for 15kHz as a baseline, and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. |
| Frame structure |  |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  |  | SU/MU-MIMO with rank adaptation is a baseline for rank ≤ 2. For low RU, SU-MIMO or SU/MU-MIMO with rank adaptation are assumed for rank ≤ 4. For medium/high RU, SU/MU-MIMO with rank adaptation is assumed for rank ≤ 4. |
| MIMO layers |  |  | For all evaluation, companies to provide the assumption on the maximum MU layers (e.g. 8 or 12) |
| CSI feedback |  |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback): 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms Companies to indicate if longer CSI feedback periodicity is used |
| Overhead |  | CSI feedback | UCI bits |
|  |  | SRS | Companies to indicate overhead relative to a baseline SRS configuration - (BW, SRS period, comb, number of OFDM symbols, number of users) = (...) |
| Traffic model |  |  | FTP model 1 with packet size 0.5 Mbytes Other FTP model is not precluded. |
| Traffic load (Resource utilization) |  |  | - 50/70 % for rank ≤ 2 - 20/50 % for rank ≤ 4 Companies are encouraged to report the MU-MIMO utilization. |
| UE distribution |  |  | - 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  |  | Realistic |
| DL Channel estimation |  |  | Realistic |
| UL Channel estimation |  |  | Alt 1. Realistic Alt 2. SRS error model in Table A.1-2 of TR 36.897 |

## 2.2	SRS configuration

An important set of assumptions needed for evaluating partial FFD channel reciprocity are those relative to the SRS configuration. These assumptions impact the quality of the UL channel measurements performed by the gNB on the SRS and therefore the performance of a CSI scheme relying on those measurements. The SRS configurations also impact the overhead calculation, which needs to take account of the SRS resources, particularly when a large number of UEs are scheduled for SRS transmission.

Generally, simulation results should indicate the SRS configuration and period and how UE antennas are sounded. For example, if antenna switching is used, sounding multiple antennas but one at a time every 20ms so the SRS period is 10ms.

Table 2 summarises the main SRS configuration parameters and proposed values. For the purpose of controlling the SRS overhead, we could set a baseline SRS configuration and measure the overhead in number of occupied REs/s/Hz, relative to the baseline. One example of baseline configuration for SRS could be (BW, SRS period, comb, number of OFDM symbols, number of users) = (10MHz, 10ms, 2, 1,4).

Proposal 5:  Agree a set of parameters and values for the SRS configuration as indicated in Table 2.

Proposal 6:  Agree on a realistic modelling of SRS transmit power, based on UL power control.

Proposal 7:  For SRS overhead calculation, agree on a baseline SRS configuration and measure the overhead in number of occupied REs/s/Hz, relative to the baseline. One example of baseline configuration for SRS could be (BW, SRS period, comb, number of OFDM symbols, number of users) = (10MHz, 10ms, 2, 1,4).

With respect to what is estimated at the gNB based on the SRS measurements, companies should indicate what is estimated from SRS, such as angles and delays, the UL channel, the covariance matrix of the channel, etc., at least until we gain a better understanding of what schemes are available.

Table 2. Proposed assumptions for SRS configuration and use at the gNB.

| Parameter |  | Value |
| --- | --- | --- |
| SRS configuration | BW | Same as CSI-RS (10MHz). Smaller BW is not precluded |
|  | Time/frequency structure | Companies to indicate which options are used between: - Comb: 2,4 - Number of OFDM symbols: 1,2,4 |
|  | Ports | Companies to indicate which option(s) are used - 2 for rank ≤ 2 - 2 or 4 for rank ≤ 4 |
|  | Periodicity | Companies to indicate how UE antennas are sounded with SRS period not shorter than 10ms |
|  | Tx power | Alt1. Realistic, based on UL power control (max power 23dBm) Alt2. Fixed power for all UEs |
|  | Time multiplexing for large number of UEs | Companies to indicate if time multiplexing is used |
| What is estimated from SRS/how it is used |  | Companies to indicate what is estimated from SRS (angles, and delays, the channel, the covariance matrix of the channel, etc.) until we further understand the direction the WI will take |

## 2.3	Beamformed CSI-RS configuration

In order to benchmark performance of a reciprocity-aided Type II PS codebook against Rel-16 eType II PS, it would be helpful to agree a baseline configuration for the beamformed CSI-RS ports, which can be used as reference for Rel-16 eType II PS. One possible choice for the baseline beamformed CSI-RS is a DFT-based grid of beams.

Proposal 8: Agree on a baseline configuration for beamformed CSI-RS based on a DFT-grid of beams. Others beamforming solutions are not precluded but should be indicated in the simulation results.

Table 3. Proposed SLS assumption on beamformed CSI-RS configuration.

| Parameter | Value |
| --- | --- |
| Beamformed CSI-RS configuration | Baseline could be DFT-based grid of beams. Companies to indicate configuration details if different beamforming is used |

## 2.4	Performance evaluation metrics and baseline

Table 4 summarises our proposal for evaluation metric and baseline.

Proposal 9: Use UPT vs overhead as main evaluation metric. Both CSI feedback and SRS overhead can be indicated. SRS overhead may be indicated relative to a baseline SRS configuration.

Proposal 10: Agree on Rel-16 eType II PS codebook using DFT grid-of-beams as baseline for performance/CSI feedback overhead trade-off.

Rel-16 eType2 regular codebook can also be used as reference for performance and UE complexity

- Note: eType2 Port selection codebook performance depends on beam ordering, so order is important in the comparison and could either be agreed on beforehand or reported each company.

Table 4. Proposed assumptions for performance evaluation metric and baseline.

| Parameter | Value |
| --- | --- |
| Evaluation metric | UPT vs overhead as main evaluation metric. Both CSI feedback and SRS overhead can be indicated. SRS overhead may be indicated relative to a baseline SRS configuration |
| Baseline for performance evaluation | Rel-16 eType II PS codebook using DFT grid-of-beams is the baseline for performance/CSI feedback overhead tradeoff - Note: eType II Port selection codebook performance depends on beam ordering, so order is important in the comparison and could either be agreed on beforehand or reported by each company. Rel-16 eType II regular codebook can also be used as reference for performance and UE complexity |

Proposal 11: For the initial evaluation phase we propose to:

- Generate baseline performance results (using agreed baseline configuration)

- Generate results showing possible gains:

  - UL eigen-beams with ideal SRS and CSI-RS

  - Also show more results with more realistic approach to gauge the likely actual gains

# 3	Conclusion

In this contribution, we present our initial considerations on the EVM assumptions for the partial FDD reciprocity evaluation, which are summarised in the following proposals:

Proposal 1:  Adopt 2 GHz as the carrier frequency with a duplexing distance of 100 MHz in the evaluation methodology.

Proposal 2:  Adopt the reciprocity channel model of Section 5.3 of TR 36.897.

Proposal 3:  Use a realistic channel estimator to model the estimation error for SRS signals.

Proposal 4:  Companies should indicate the period and multiplexing approach for CSI-RS signals for DL channel measurement.

Proposal 5:  Agree a set of parameters and values for the SRS configuration as indicated in Table 2.

Proposal 6:  Agree on a realistic modelling of SRS transmit power, based on UL power control.

Proposal 7:  For SRS overhead calculation, agree on a baseline SRS configuration and measure the overhead in number of occupied REs/s/Hz, relative to the baseline. One example of baseline configuration for SRS could be (BW, SRS period, comb, number of OFDM symbols, number of users) = (10MHz, 10ms, 2, 1,4).

Proposal 8: Agree on a baseline configuration for beamformed CSI-RS based on a DFT-grid of beams. Others beamforming solutions are not precluded but should be indicated in the simulation results.

Proposal 9: Use UPT vs overhead as main evaluation metric. Both CSI feedback and SRS overhead can be indicated. SRS overhead may be indicated relative to a baseline SRS configuration.

Proposal 10: Agree on Rel-16 eType II PS codebook using DFT grid-of-beams as baseline for performance/CSI feedback overhead trade-off.

Rel-16 eType2 regular codebook can also be used as reference for performance and UE complexity

- Note: eType2 Port selection codebook performance depends on beam ordering, so order is important in the comparison and could either be agreed on beforehand or reported each company.

Proposal 11: For the initial evaluation phase we propose to:

- Generate baseline performance results (using agreed baseline configuration)

- Generate results showing possible gains:

  - UL eigen-beams with ideal SRS and CSI-RS

  - Also show more results with more realistic approach to gauge the likely actual gains

# References

- Samsung, RP-193133, “New WID: Further enhancements on MIMO for NR,” RAN#86, Dec 2019

- TS 38.104, “Base Station (BS) radio transmission and reception,” V16.2.0, December 2019.

- TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz,” V16.0.0, October 2019.

- TR 36.897, “Study on elevation beamforming / Full-Dimension (FD) Multiple Input Multiple Output (MIMO) for LTE,” V13.0.0, June 2015.

# Appendix

In Rel-16 MU-CSI enhancement the EVM assumptions were taken from Table A.2.1-1 of TS38.802 or NR phase 2 EVM, with modifications as follows

Table 2-1. SLS assumptions for CSI enhancement

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) is a baseline. Other scenarios (e.g. UMi@4GHz 2GHz, Urban Macro) are not precluded. |
| Frequency Range |  | FR1 only, 4GHz. |
| Inter-BS distance |  | 200m |
| Channel model |  | According to the TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded. |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for rank > 2 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Type II overhead reduction Other configuration is not precluded. |
| BS Tx power |  | 41 dBm |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Number of RBs |  | 52 for 15 kHz SCS |
| Simulation bandwidth |  | 10 MHz for 15kHz as a baseline, and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU/MU-MIMO with rank adaptation is a baseline for overhead reduction. For low RU, SU-MIMO or SU/MU-MIMO with rank adaptation are assumed for higher rank extension. For medium/high RU, SU/MU-MIMO with rank adaptation is assumed for higher rank extension. |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers (e.g. 8 or 12) |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | Companies shall provide the downlink overhead assumption |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes Other FTP model is not precluded. |
| Traffic load (Resource utilization) |  | - 50/70 % for CSI overhead reduction - 20/50 % for high rank extension Companies are encouraged to report the MU-MIMO utilization. |
| UE distribution |  | - 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic |
| Evaluation Metric |  | Throughput and CSI feedback overhead as baseline metrics. Additional metrics, e.g., ratio between throughput and CSI feedback overhead, can be used. Maximum overhead (payload size for CSI feedback)for each rank at one feedback instance is the baseline metric for CSI feedback overhead, and companies can provide other metrics. |
| Baseline for performance evaluation |  | Rel-15 Type II Codebook is the baseline for performance and overhead evaluation for overhead reduction. (Type I Codebook can be considered at least for performance evaluation) - Companies are encouraged to compare the proposed overhead reduction scheme with Rel-15 overhead reduction scheme, Rel-15 Type I Codebook is the baseline for performance and overhead evaluation for higher rank codebook. |