3GPP TSG RAN WG1 #101		                                                                               R1-2004304

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	8.4.1.1

Source:	InterDigital Inc.

Title:	Simulation assumptions and throughput performance for FR1 UL

Document for:	Discussion

# Introduction

RAN approved an SI on NR coverage enhancements in RAN#86 [1]. The objective of the study item is to study potential coverage enhancement solutions for specific scenarios for both FR1 and FR2. Use cases include eMBB and VoIP. In RAN#101, simulation assumptions for evaluation of techniques are to be discussed. Based on the use cases, simulation parameters for UL transmission are proposed and initial results for FR1 are presented in this contribution.

# Simulation assumptions for FR1 UL

Coverage expansion finds use cases in both eMBB and VoIP. Considerations for simulation parameters for UL transmission are presented in this section.

Waveform

Due to its low PAPR, DFTsOFDM (i.e. use of transform precoding) should be considered as one of the potential waveforms to increase coverage.  However, given that OFDM can achieve both flexible data multiplexing and multi-layer transmission, coverage expansion should also be considered for OFDM.

Depending on application, suitable waveform should be chosen. For example, as mentioned previously, DFTsOFDM, combined with power efficient modulation schemes such as QPSK and pi/2 BPSK, can be implemented when significant coverage enhancement is needed for power-limited devices. Use case can be VoIP or applications requiring remote sensor which are trying to transmit video image (eMBB) from the cell edge.

OFDM can be useful in scenarios where multiplexing of UL data is needed. For example, sensors placed in corners of factories may need to be multiplexed flexibly; OFDM can be useful in such scenario. Transmission diversity can also be applied easily to OFDM while transmission diversity for DFTsOFDM has to be applied carefully to preserve low PAPR.

Coverage expansion for OFDM will provide flexibility in deployment, i.e., extending maximum coverage area for factories. It is important to note that as both waveforms are normative, potential techniques for coverage expansion should be applicable to both waveforms.

Proposal 1: Include both OFDM and DFTsOFDM as baseline waveforms for evaluation for both VoIP and eMBB

Baseline DMRS density

In addition, in FR1, in long range communication, reflection from multipath will become prominent. Thus, the optimum number of DMRS symbols should be determined from evaluation, as explained in our contribution [2]. As severe multipath from long-distance communication in FR1 is expected to generate frequency selectivity, MMSE channel estimation spanning multiple DMRS symbols should be used as baseline approach. Performance evaluation should consider all possible DMRS arrangement, and determine the optimum number of DMRS symbols, which can be used as the benchmark for evaluation of techniques.

Proposal 2: Number and locations of DMRS symbols in a slot should be decided as benchmark

Channel and antenna configurations for FR1

In TR 38.901 [3], channel models for a variety of scenarios are defined. A channel model with appropriate delay spread value should be chosen to simulate realistic long range communication. Furthermore, designs should be based on the worst case scenario, targeting a large value of DS. In this contribution, we recommend TDL-C with DS values listed as examples as “long delay profile” in TR 38.901.

Following one of the configurations assumed in LTE scenarios in TR 36.824 [4], at least 1TX 2RX configuration should be included in the evaluation.

Proposal 3: DS=616ns and DS=153 with TDL-C are recommended as Urban and Rural scenario, respectively

Proposal 4: For FR1, include 1TX2RX scenario in simulation assumption

Amplifier model

For realistic computation of MCL, nonlinearity should be incorporated in evaluation. Inclusion of nonlinearity will also provide insights into difference in performance between OFDM and DFTsOFDM as OFDM requires larger backoff than OFDM.

Proposal 5: Nonlinear amplification model should be included for MCL

Slot hopping domain

To benefit from time diversity, inter-slot hopping should be included in evaluation. We have shown results for inter-slot frequency hopping in the next section to demonstrate performance improvement that can be obtained by capturing frequency and time diversity.

Number of repetitions:

As explained in our contribution [2], temporal diversity gain can be achieved by repetition of transmission, i.e., slot bundling, of transport blocks. As one of the baseline techniques, repetition should be included.

Proposal 6: Repetition should be the baseline technique for coverage enhancement.

Evaluation of MCL for VoIP and eMBB

As described in TR 36.824, coverage of service is defined by maximum coupling loss (MCL). Based on evaluation results, SINR to meet a specific performance requirement is determined, and used to compute MCL.

For VoIP, the residual error is defined as an occasion in which a transport block could not be recovered before the number of repetition exceeds the limit. The error rate is defined for VoIP due to its necessity to meet the latency requirement. In addition, a latency requirement is needed for practical VoIP service. As described in [2], following evaluation assumptions in TR 36.824, MCL should be evaluated for 2% residual BLER within 50 ms delay budget.

For eMBB, as there are no latency requirements, SINR required to meet the target throughput should be used. Thus, the following proposals are made.

Proposal 7: For VoIP, MCL is evaluated for 2% residual BLER within 50 ms delay budget

Proposal 8: For eMBB, MCL is evaluated for the target throughput

Finally, following the parameters in TR 36.824 [4] and TR 38.802 [6], maximum UE transmit power of 23 dBm and gNB noise figure of 5 dB can be assumed for calculation of MCL.

Proposal 9: For calculation of MCL for FR1, UE transmit power of 23dBm and gNB noise figure of 5dB are assumed

# Initial evaluation results

Table of simulation parameters

The proposed table of simulation parameters is summarized in Table 1. Details of each DMRS configuration are shown in Table 2. In this contribution, we focused on performance evaluation for DFTsOFDM and inter-slot frequency hopping combined with repetition. The number of PRBs has been computed to meet the target data rate for eMBB and VoIP. In addition, in evaluation, the number of DMRS symbols ranging from 1 to 4 DMRS symbols has been evaluated. For VoIP both number of PRBs and MCS number are adjusted such that approximately 320 bits are contained in one slot. The number of bits for VoIP is derived from the summation of MAC header bits, RLC header bits, PDCP bits and CRC bits.

When repetition is not used, 12 HARQ processes are assumed, while 3 HARQ processes are assumed when a TB is transmitted 4 times in the repetition scheme. To assess impact of number of DMRS symbols on the performance, ideal power amplifier is assumed. The frequency hopping pattern specified in TS 38.214 [4] is assumed in the evaluation.

In addition, for QPSK and pi/2 BPSK, for maximum transmission efficiency, modulation and coding pair that yields the highest spectral efficiency form TS. 38.214 [5] was chosen in the evaluation. In the evaluation, 14 PUSCH symbols are assumed and DMRS mapping type B are assumed.

BLER and Throughput results for eMBB

In this subsection, BLER and throughput performances for eMBB are shown. All simulations are generated with FDD. In Figure 1 and 2, BLER and throughput performance for eMBB QPSK in the urban scenario is shown. From the figure, scheduling with repetition and inter-slot frequency hopping brings nearly 10dB and 5dB gain in BLER and throughput performance. It is noticeable that increasing the number of DMRS symbols improves BLER while the throughput improvement becomes limited as the number of DMRS symbols increases. Increasing the number of DMRS symbols improves quality of channel estimates, thanks to averaging of channel estimates over time. However, including too many DMRS symbols in a slot limits the throughput performance, as shown in Figure 2. From Figure 2, it is clear that the target data rate can be achieved at around 0dB with repetition for all DMRS configurations.

In Figure 3 and 4, QPSK performance for the rural scenario is shown. From Figure 3 and 4, it is clear that both throughput and BLER performance can be improved with repetitions and frequency hopping. It is also clear that one DMRS symbol in a slot is not enough to obtain satisfactory channel estimation performance. Increasing the number of DMRS symbols beyond 2 degrades throughput performance. The target data rate can be reached at SINR=-4dB when the number of DMRS symbols is set at 2 or 3.

Based on the performance shown in Figure 1 through 4, the following observations are made.

Observation 1: Repetition, combined with Inter-slot hopping yields performance improvement by approximately 5dB

Observation 2: For QPSK eMBB in both rural and urban scenarios, 2 and 3 DMRS symbols per slot yields satisfactory performance in both BLER and throughput

In practice, the number of DMRS symbols should be minimized to allow flexible multiplexing between RS and data; minimum number of DMRS should be used.  Based on the observations above, we make the following proposal

Proposal 10: At least for eMBB QPSK scenario, as the baseline, the number of DMRS symbols in a 14-symbol slot should be 2

BLER and rBLER performance for VoIP

Both BLER and rBLER performances for VoIP are shown in Figure 5 through 8. It is clear that the 1-symbol DMRS configuration suffers from performance loss due to poor quality of channel estimates while increasing the number of symbols beyond 2 DMRS symbols yield performance gain. The following observation is made.

Observation 3: For QPSK VoIP in both rural and urban scenarios, increasing the number of DMRS symbols improves both BLER and rBLER performance

In VoIP, since the size of the payload is fixed, a recommendation here is to set the number of DMRS symbols to the maximum number allowed in 3GPP, and use it as benchmark.

Proposal 11: At least for VoIP QPSK scenario, as the baseline, the number of DMRS symbols in a 14-symbol slot should be 4

Figure 1 : BLER performance for eMBB QPSK Urban, FDD

Figure 2 : Throughput performance  for eMBB Urban, QPSK, FDD

Figure 3 : BLER performance for eMBB QPSK Rural, FDD

Figure 4 : Throughput performance for eMBB QPSK Rural, FDD

Figure 5 BLER performance for QPSK in Urban scenario, FDD

Figure 6 rBLER performance for QPSK in Urban scenario, FDD

Figure 7  BLER performance for QPSK in Rural scenario, FDD

Figure 8 rBLER performance for QPSK in Rural scenario, FDD

# Conclusion

Simulation parameters for FR1 are shown in this contribution. In this contribution, for eMBB, BLER and throughput performance for FDD in FR1 are shown. For VoIP, BLER and rBLER performances for FDD in FR1 are shown. In all scenarios, QPSK is assumed. Based on the evaluation results and analysis presented in this contribution, we make the following observations and proposals.

Proposal 1: Include both OFDM and DFTsOFDM as baseline waveforms for evaluation for both VoIP and eMBB

Proposal 2: Number and locations of DMRS symbols in a slot should be decided as benchmark

Proposal 3: DS=616ns and DS=153 with TDL-C are recommended as Urban and Rural scenario, respectively

Proposal 4: For FR1, include 1TX2RX scenario in simulation assumption

Proposal 5: Nonlinear amplification model should be included for MCL

Proposal 6: Repetition should be baseline technique for coverage enhancement.

Proposal 7: For VoIP, MCL is evaluated for 2% residual BLER within 50 ms delay budget

Proposal 8: For eMBB, MCL is evaluated for the target throughput

Proposal 9: For calculation of MCL for FR1, UE transmit power of 23dBm and gNB noise figure of 5dB are assumed

Proposal 10: At least for eMBB QPSK scenario, as the baseline, the number of DMRS symbols in a 14-symbol slot should be 2

Proposal 11: At least for VoIP QPSK scenario, as the baseline, the number of DMRS symbols in a 14-symbol slot should be 4

Observation 1: Repetition, combined with Inter-slot hopping yields performance improvement by approximately 5dB

Observation 2: For QPSK eMBB in both rural and urban scenarios, 2 and 3 DMRS symbols per slot yields satisfactory performance in both BLER and throughput

Observation 3: For QPSK VoIP in both rural and urban scenarios, increasing the number of DMRS symbols improves both BLER and rBLER performance

# References

RP-193240, “New SID on NR coverage enhancements”, China Telecom.

R1-2004273, “Potential techniques for coverage enhancements,” InterDigital. RAN1#101e, May, 2020.

TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz,” ver. 16.0.1, Dec. 2019.

TR 36.824, “LTE coverage enhancements,” ver. 11.0.0, June, 2016.

TS 38.214, “Physical layer procedures for data,” ver. 16.0.1, Dec. 2019.

TR 38.802, “Study on New Radio Access Technology,” ver. 14.2.0, Sept. 2017.

TS 38.211, “Physical channels and modulation,” ver. 16.1.0, March, 2020.

Table 1 Simulation Parameters for FR1

| Simulation Assumptions (to maximize MCL) | eMBB Values |  | VoIP Values |
| --- | --- | --- | --- |
| Frequency Region | FR1 |  |  |
| Duplexing mode | FDD, TDD |  |  |
| Carrier Frequency (GHz) | 3.5, 5 |  |  |
| Subcarrier Spacing (kHz) | 15 |  |  |
| Tx Bandwidth Configuration (MHz) | 20 |  |  |
| Waveform | W CP DFT-s-OFDM |  |  |
| CP type | Normal |  |  |
| PHY channel | PUSCH |  |  |
| Allocation (# of PRB) | - {5, 6, 6, 7} for MCS index 9 to achieve 1 Mbps - {21, 23, 25, 28} for MCS index 1 to achieve 1 Mbps - {1, 1, 1, 1} for MCS index 9 to achieve 100 kbps - {3, 3, 3, 3} for MCS index 1 to achieve 100 kbps Note : For repetition “off”, above allocation is used. For repetition “on”, number of RBs is scaled up by the number of repetitions per bundle (4) to achieve the target bit error rate |  | - {2, 2, 2, 3} for MCS index {7, 8, 9, 9} Note : TB size is 320 bits |
| Allocation (# of OFDM symbols) | 14 |  |  |
| DM-RS Type | Configuration Type 1 (according to Table 6.4.1.1.3-1 in TS 38.211 [7]) |  |  |
| DM-RS (# of OFDM symbols) | Type B, {1, 2, 3, 4} (Positions of DMRS symbols in the time domain are shown in Table 2) |  |  |
| MCS index, table | - Table 6.1.4.1-1 in TS 38.214 |  |  |
| HARQ sequence | 0,2,3,1 |  |  |
| Repetition/HARQ scheme | Repetition OFF: - 12 HARQ processes Repetition ON: - 4 repetitions per bundle - 3 HARQ processes Max number of HARQ retransmissions: Unlimited | Repetition ON: - 4 repetitions per bundle - 3 HARQ processes Maximum number of HARQ retransmissions: 5 bundles Note: there is no “Repetition OFF” case for VoIP |  |
| Hopping | Inter-slot |  |  |
| Antenna Configuration | 1x2 (i.e., 1 TX at UE and 2 RX at BS) |  |  |
| PA impairment | ideal |  |  |
| Channel model (TDL type, DS) | TDL-C DS=616ns (Urban Macro O2I long delay profile from TR 38.901), largest DS chosen as the worst case scenario TDL-C DS=153ns (RMa & RMa O2I long delay profile from TR 38.901), largest DS chosen as the worst case scenario |  |  |
| TX pattern per frame (for TDD) | For TDD, referring to one of the patterns from LTE, use the pattern XXUUXXXUUX (10 slots=10ms) where U is a uplink slot and X is downlink/flexible; 2 out of 5 slots are reserved for uplink transmission. For inter-slot frequency hopping pattern, refer to 6.3 in TS 38.214. Below is a section in TS 38.214 that describes frequency hopping for inter-slot. [TS 38.214, 6.3] In case of inter-slot frequency hopping, the starting RB during slot is given by: , where is the current slot number within a radio frame, where a multi-slot PUSCH transmission can take place, is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Subclause 6.1.2.2.2) and is the frequency offset in RBs between the two frequency hops. |  |  |
| Speed | 3 km/hr |  |  |
| Receiver: CHEST | Realistic |  |  |
| Data rate | 1 Mbps for urban 100 kbps for rural |  | N/A |
| Residual BLER | N/A |  | 2% |

Table 2 DMRS placement for PUSCH (type B) Table 6.4.1.1.3-3 from TS 38.211

| 1 symbol DMRS configuration (# symbols used for PUSCH = 13) | 2 symbol DMRS configuration (# symbols used for PUSCH = 12) |
| --- | --- |
| 3 symbol DMRS configuration (# symbols used for PUSCH = 11) | 4 symbol DMRS configuration (# symbols used for PUSCH = 10) |