3GPP TSG-RAN WG1 Meeting #101	R1-2004339

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	7.2.5.2

Source:	ITRI

Title:	Remaining issue on the HARQ-ACK/PUSCH priority

Document for:	Discussion and Decision

1	Introduction

During RAN1#99 meeting, we have following agreement:

Agreements:

When both DCI format 0_1/1_1 and DCI format 0_2/1_2 are configured to be monitored per BWP, a DCI format (from the formats 0_1/1_1/0_2/1_2) can be used to schedule PDSCH with different HARQ-ACK priorities or PUSCH with different priorities.

- This feature is UE optional

However, it is possible that both DCI format 0_1/1_1 and DCI format 0_2/1_2 are configured to be monitored per BWP but the UE is not capable of supporting dynamic switching of PUSCH/HARQ-ACK priority via both DCI format 0_1/1_1 and 0_2/1_2. In this contribution we provide our view on how to specify the corresponding UE behavior.

2	Discussion

When a UE is not capable of supporting dynamic switching of PUSCH/HARQ-ACK priority via both DCI format 0_1/1_1 and 0_2/1_2, according to the discussion during RAN1#100bis-e meeting, we have two options to define the UE behavior:

- (Option 1) Fix the priority of PUSCH/HARQ-ACK by corresponding DCI format. For example, a PUSCH/HARQ-ACK would be treated as low priority if it is triggered by DCI format 0_1/1_1; otherwise, the PUSCH/HARQ-ACK would be treated as high priority if it is triggered by DCI format 0_2/1_2.

- (Option 2) The PUSCH/HARQ-ACK triggered by DCI format 0_1/1_1 should always treated as low priority, and follow the indicated priority in the scheduling DCI for the case of DCI format 0_2/1_2.

Consider the UE capability discussed above, our understanding is that the UE cannot dynamically determine the priority of PUSCH/HARQ-ACK according to the priority field in DCI. In this case, the UE may not be configured with priority indicator in a DCI format. Even if the priority field is configured in a DCI format, the UE should ignore this field. Therefore, it is reasonable that the priority could be determined according to a pre-determined rule. Moreover, since the DCI format 0_2/1_2 is intended for URLLC, it’s reasonable to assume it corresponds to high priority in case UE is not capable of scheduling different priorities by a DCI format while both DCI format 0_2/1_2 and DCI format 0_1/1_1 are configured to be monitored for a BWP. Therefore, we suggest to adopt option 1 to handle the priority issue.

Proposal:

- If a UE is not capable of supporting dynamic switching of HARQ-ACK/PUSCH priority via both DCI format 0_1/1_1 and 0_2/1_2, and the UE is configured with DCI format 0_1/1_1 and 0_2/1_2:

- The UE is expected to assume fixed priority by DCI format (i.e., low priority for DCI format 0_1/1_1, high priority for DCI format 0_2/1_2)

References

RAN WG1#99 chairman’s note.

RAN WG1#100bis-e chairman’s note.

R1-2002784, “Summary of email thread [100b-e-NR-L1enh_URLLC-UCI_Enh-02]”, OPPO.