3GPP TSG RAN WG1 #101			 R1-2004407

e-Meeting, May 25th – June 5th, 2020

Source:	NTT DOCOMO, INC.

Title:	Discussion on UE features for UE power saving

Agenda Item:	7.2.11.7

Document for: 	Discussion and Decision

- Introduction

At the RAN1#100bis-e meeting, UE features for UE power saving were discussed and RAN1 made agreements [1]. Also, a baseline of UE feature list was agreed in the email discussion after the meeting [2]. In this contribution, we discuss the remaining issues on UE features for UE power saving.

- UE features for UE power saving

- FG 19-1 DRX Adaptation

In email discussion after RAN1#100bis-e meeting, the interpretation issue was raised if the type is not “per UE without FR1/FR2 differentiation”. On the assumption that the UE which supports 19-1 in FR1 but not in FR2 is configured FR1 as PCell and FR2 as SCell, our views on the interpretation are following since DRX is per MAC entity but the function on monitoring DCI can be different between FR1 and FR2:

- UE supports to monitor DCI format 2_6 and all rerated functions as long as PCell is FR1.

- If UE does not start drx-onDurationTimer by DCI format 2_6, the UE does not monitor PDCCH in both PCell and SCell.

Proposal 1: On the assumption that the UE which supports 19-1 in FR1 but not in FR2 is configured FR1 as PCell and FR2 as SCell, the interpretation is:

- UE supports to monitor DCI format 2_6 and all rerated functions as long as PCell is FR1.

- If UE does not start drx-onDurationTimer by DCI format 2_6, the UE does not monitor PDCCH in both PCell and SCell.

- FG 19-3 Maximum MIMO Layer Adaptation

The type of FG 19-3 Maximum MIMO Layer Adaptation was discussed during the last meeting. The UE just reports whether or not to support the function of maximum MIMO layer adaptation, although the maximum number of MIMO layers is different per BC. Thus, the type of this feature should be per UE. Regarding FR differentiation, we are fine to have FR1/FR2 differentiation.

Proposal 2: For FG 19-3 “Maximum MIMO Layer Adaptation”, the type should be per UE with FR1/FR2 differentiation.

- Conclusion

In this contribution, the remaining issues on UE features for UE power saving were discussed. Based on the discussion, the following proposals were made:

Proposal 1: On the assumption that the UE which supports 19-1 in FR1 but not in FR2 is configured FR1 as PCell and FR2 as SCell, the interpretation is:

- UE supports to monitor DCI format 2_6 and all rerated functions as long as PCell is FR1.

- If UE does not start drx-onDurationTimer by DCI format 2_6, the UE does not monitor PDCCH in both PCell and SCell.

Proposal 2: For FG 19-3 “Maximum MIMO Layer Adaptation”, the type should be per UE with FR1/FR2 differentiation.

References

- 3GPP RAN1#100bis-e, Chairman’s note, April 2020.

- R1-2003195, “Summary of Email Approval [100e-b-NR-UEFeatures-Remaining] — UE Power Saving Aspects”, Moderator (AT&T), April 2020.