3GPP TSG RAN WG1 Meeting #101-e	                                                                          R1-2004441

May 25th – June 5th, 2020

Agenda item:    7.2.2.1.2

Source:             Qualcomm Incorporated

Title:                  TP for DL signals and channels for NR-U

Document for:  Discussion and Decision

# Introduction

This paper summarizes the text proposals for CRs for NR-U DL signals and channels agenda item.

# CSI-RS

In RAN1 #100bis-e, the P/SP-CSI-RS validation in time domain has been discussed extensively but more discussion is needed to converge. In our view, validation is necessary for UE to properly process the P/SP-CSI-RS. The validation needs to be CRC protected. The source of validation naturally can be COT-SI (either COT duration or SFI-index, if provided). Other DCI, such as DL grant granting the symbols carry the CSI-RS is overlapping with a PDSCH, can be used as a source of validation, but this validation is very opportunistic in nature and is not a consistent solution. For FBE, as discussed in Section 4 below, we believe COT duration or SFI-index may not need to be explicitly configured, and the UE can assume the gNB acquired the COT if detected DL transmission in earlier part of an FFP.

For AP-CSI-RS, if we enforce the AP-CSI-RS is always triggered in the same COT as the triggering DCI, the validation is automatic.

The periodic CSI-RS for tracking (TRS) consists of a TRS burst of 1 or 2 slots that occurs periodically. The CSI-RS symbols in one TRS burst is processed together. Thus, it makes sense that they are transmitted only when the all the CSI-RS symbols in one TRS burst falls in the DL portion of the same COT.

Proposal : For periodic and semi-persistent CSI-RS reception, UE is expected to process periodic and semi-persistent CSI-RS only if

- For LBE case, the UE is able to confirm through COT-SI (either COT duration or SFI-index) that the CSI-RS is inside a gNB COT.

- For FBE case, the UE can receive a DL transmission in the earlier part of the FFP where the CSI-RS is configured.

- When the CSI-RS is a TRS, UE is expected to process the TRS only if it can confirm that all CSI-RS symbols in one TRS burst are inside the same gNB COT.

Based on the above proposal, we can add the following text to 38.214, section 5.1.6.1 and 5.1.6.1.1

============TP for 38.214 Section 5.1.6.1 ====================================

-------Unchanged part omitted----------------------------

5.1.6.1 CSI-RS reception procedure

-------Unchanged part omitted----------------------------

For operation with shared spectrum channel access, the UE is not expected to receive CSI-RS if it is not able to confirm through DCI 2_0 that the CSI-RS symbol is inside a gNB channel occupancy duration.

-------Unchanged part omitted----------------------------

============TP for 38.214 Section 5.1.6.1.1 ====================================

5.1.6.1.1 CSI-RS for tracking

For operation with shared spectrum channel access, the UE is not expected to receive the NZP CSI-RS resources in a NZP CSI-RS resource set configured with trs-Info as true, if it is not able to confirm through DCI 2_0 that all the NZP CSI-RS resources in the NZP CSI-RS resource set is inside a same gNB channel occupancy duration.

-------Unchanged part omitted----------------------------

=======================================================================

In NR, an aperiodic CSI-RS is triggered by a DCI. The offset from the trigger to the CSI-RS resource is RRC configured. In NRU, to avoid LBT related issues and simplify the CSI-RS reception at the UE, it is beneficial to transmit the DCI trigger and the aperiodic CSI-RS in the same COT.

Proposal : Aperiodic CSI-RS and its triggering DCI are transmitted in the same COT.

CSI-RS for RLM can be supported in NR-U. However, due to LBT subband based access in the DL, the number of RB sets used for each COT can be different, and in general, the CSI-RS transmit power can vary. For RLM purpose, the UE can only assume a fixed power for CSI-RS if detected. Therefore, even though CSI-RS for CSI acquisition purpose may support variable power for different COT, the CSI-RS for RLM should be transmitted with fixed power.

Proposal : UE expects CSI-RS for RLM to have fixed transmit power over all COTs.

# DCI 2_0

When a UE is configured with DCI 2_0 monitoring for a shared spectrum cell with multiple LBT bandwidth, if the availableRB-setPerCell-r16 is configured, UE can figure out which RB set is available, which may impact CSI-RS reception and RRC configured UL transmission validation. However, it is not yet defined if the DCI 2_0 monitoring is configured, but availableRB-setPerCell-r16 is not configured for a cell with multiple LBT bandwidth. There is no way for a UE to blindly detect which LBT bandwidth is available. Therefore, we propose for this case, the UE will consider all RB sets transmitted and included in COT.

Proposal : For a cell with multiple LBT bandwidth but availableRB-setPerCell-r16 not configured, the UE will consider all RB sets in the COT when DCI 2_0 is detected.

===========TP for 38.213 11.1.1 ==================

11.1.1	UE procedure for determining slot format

*** Unchanged text is omitted ***

For each serving cell in the set of serving cells, the UE can be provided:

-	an identity of the serving cell by servingCellId

-	a location of a SFI-index field in DCI format 2_0 by positionInDCI

-	a set of slot format combinations by slotFormatCombinations, where each slot format combination in the set of slot format combinations includes

-	one or more slot formats indicated by a respective slotFormats for the slot format combination, and

-	a mapping for the slot format combination provided by slotFormats to a corresponding SFI-index field value in DCI format 2_0 provided by slotFormatCombinationId

-	for unpaired spectrum operation, a reference SCS configuration  by subcarrierSpacing and, when a supplementary UL carrier is configured for the serving cell, a reference SCS configuration  by subcarrierSpacing2 for the supplementary UL carrier

-	for paired spectrum operation, a reference SCS configuration  for a DL BWP by subcarrierSpacing and a reference SCS configuration  for an UL BWP by subcarrierSpacing2

-	a location of an available RB set indicator field in DCI format 2_0 that is

-  	one bit, if intraCellGuardBandDL-r16 for the serving cell indicates no intra-cell guard-bands are configured, where a value of '1' indicates that the serving cell is available for receptions, a value of '0' indicates that the serving cell is not available for receptions, by availableRB-SetPerCell-r16, and the serving cell remains available or unavailable for reception until the end of the indicated channel occupancy duration

-  	a bitmap having a one-to-one mapping with the RB sets [6, TS 38.214] of the serving cell, if intraCellGuardBandDL-r16 for the serving cell indicates intra-cell guard-bands are configured, where the bitmap includes NRB,set,DL bits and NRB,set,DL is the number of RB sets in the serving cell, a value of '1' indicates that an RB set is available for receptions, a value of '0' indicates that an RB set is not available for receptions, by availableRB-SetPerCell-r16, and an RB set remains available or unavailable for receptions until the end of the indicated channel occupancy duration

-	If availableRB-SetPerCell-r16 is not provided, all RB sets in the active BWP are considered available for receptions.

-	a location of a channel occupancy duration field in DCI format 2_0, by CO-DurationPerCell-r16, that indicates a remaining channel occupancy duration for the serving cell starting from a first symbol of a slot where the UE detects the DCI format 2_0 by providing a value from CO-DurationList-r16. The channel occupancy duration field includes maxlog2COdurationListSize,1 bits, where COdurationListSize is the number of values provided by CO-DurationList-r16. If CO-DurationPerCell-r16 is not provided, the remaining channel occupancy duration for the serving cell is a number of slots, starting from the slot where the UE detects the DCI format 2_0, that the SFI-index field value provides corresponding slot formats

-	a location of a search space set group switching field in DCI format 2_0, by SearchSpaceSwitchTrigger-r16, that indicates a group from two groups of search space sets for PDCCH monitoring for scheduling on the serving cell as described in Clause 10.4.

*** Unchanged text is omitted ***

=================================================

During email discussion of RAN1 #100bis-e, if DCI 2_0 can be configured without SFI has been discussed without conclusion. In our view, RAN2 spec already clearly captured that the DCI 2_0 can be configured without SFI, even in Rel.15. Additionally, for NR-U operation, COT duration and available RB set field is more important for normal operation, while SFI is not always needed. Additionally, search space set group switching is an independent functionality. All these should not force SFI to be configured.

Proposal . DCI 2_0 can be configured without SFI field being configured to support COT duration, available RB set indicator and search space set group switching functionality.

==================38.213 11.1.1=================

---------unchanged text omitted------------------

- a location of a channel occupancy duration field in DCI format 2_0, by CO-DurationPerCell-r16, that indicates a remaining channel occupancy duration for the serving cell starting from a first symbol of a slot where the UE detects the DCI format 2_0 by providing a value from CO-DurationList-r16. The channel occupancy duration field includes maxlog2COdurationListSize,1 bits, where COdurationListSize is the number of values provided by CO-DurationList-r16. If CO-DurationPerCell-r16 is not provided, and SFI-index field is provided, the remaining channel occupancy duration for the serving cell is a number of slots, starting from the slot where the UE detects the DCI format 2_0, that the SFI-index field value provides corresponding slot formats

---------unchanged text omitted------------------

==============================================================

# COT duration for semi-static channel access

During the COT duration configuration discussion, the focus is for dynamic channel access system. However, for semi-static channel access (FBE), how to handle COT duration needs some additional discussion.

For initial access UE, there is no COT duration configuration yet. Therefore, effectively the UE will treat the entire fixed frame period (other than the idle region) as in COT if a DL signal/channel is detected in the earlier part of the fixed frame period. For connected mode UE, after the COT duration can be configured, we need clarifications on

- Do we need the COT duration configuration at all?

- If we allow COT duration configuration, what are the restriction on the length of COT duration

Given we already have a clear definition for COT duration for semi-static channel access, as is already used by initial access UEs, we don’t believe there is a need to mandate the COT duration configuration for semi-static channel access. Additionally, by regulation, the initiating device cannot acquire the COT in the next fixed frame period, it does not make sense if an indicated COT duration is beyond the current fixed frame period or extend into the idle period. We have the following proposals

Proposal . When COT duration is not explicitly configured for a semi-static channel access system, the UE assumes the COT ends at the beginning of the idle period in the same fixed frame period.

Proposal . If COT duration is explicitly configured, UE does not expect the COT duration to indicate the COT ends later than the beginning of the idle period in the same fixed frame period that the COT duration information is detected.

On the other hand, it is not clear why the gNB wants to indicate a COT duration ends earlier than the current fixed frame period, we can go one step further to say for semi-static channel access, the COT duration is not configured and the UE always interpret the COT duration from the fixed frame period structure.

Proposal . COT duration is not explicitly configured for a semi-static channel access system. The UE assumes the COT ends at the beginning of the idle period in the same fixed frame period.

==============TP for Proposal 6 and proposal 7=======38.213 11.1.1=================

---------unchanged text omitted------------------

- a location of a channel occupancy duration field in DCI format 2_0, by CO-DurationPerCell-r16, that indicates a remaining channel occupancy duration for the serving cell starting from a first symbol of a slot where the UE detects the DCI format 2_0 by providing a value from CO-DurationList-r16. The channel occupancy duration field includes maxlog2COdurationListSize,1 bits, where COdurationListSize is the number of values provided by CO-DurationList-r16. If CO-DurationPerCell-r16 is not provided, the remaining channel occupancy duration for the serving cell is a number of slots, starting from the slot where the UE detects the DCI format 2_0, that the SFI-index field value provides corresponding slot formats

  - If a gNB provides UE(s) with higher layer parameters ChannelAccessMode-r16 ='semistatic', and CO-DurationPerCell-r16 is not configured, the UE assumes the COT duration end at Tz=max0.05Tx , 100us before the start of the next periodic channel occupancy starting position [4.3 of 37.213].

  - If a gNB provides UE(s) with higher layer parameters ChannelAccessMode-r16 ='semistatic', and CO-DurationPerCell-r16 is configured, the UE does not expect the COT duration to end later than Tz=max0.05Tx , 100us before the start of the next periodic channel occupancy starting position [4.3 of 37.213].

---------unchanged text omitted------------------

==============================================================

==============TP for Proposal 8=======38.213 11.1.1=================

---------unchanged text omitted------------------

- a location of a channel occupancy duration field in DCI format 2_0, by CO-DurationPerCell-r16, that indicates a remaining channel occupancy duration for the serving cell starting from a first symbol of a slot where the UE detects the DCI format 2_0 by providing a value from CO-DurationList-r16. The channel occupancy duration field includes maxlog2COdurationListSize,1 bits, where COdurationListSize is the number of values provided by CO-DurationList-r16. If CO-DurationPerCell-r16 is not provided, the remaining channel occupancy duration for the serving cell is a number of slots, starting from the slot where the UE detects the DCI format 2_0, that the SFI-index field value provides corresponding slot formats. If a gNB provides UE(s) with higher layer parameters ChannelAccessMode-r16 ='semistatic', the UE does not expect CO-DurationPerCell-r16 to be configured, and the COT duration is Tz=max0.05Tx , 100us before the start of the next periodic channel occupancy starting position [4.3 of 37.213].

---------unchanged text omitted------------------

==============================================================

# Summary

In this section, we summarize the new proposals for DL signals and channels for NR-U in SUB-7GHZ BAND.

Proposal 1: For periodic and semi-persistent CSI-RS reception, UE is expected to process periodic and semi-persistent CSI-RS only if

- For LBE case, the UE is able to confirm through COT-SI (either COT duration or SFI-index) that the CSI-RS is inside a gNB COT.

- For FBE case, the UE can receive a DL transmission in the earlier part of the FFP where the CSI-RS is configured.

- When the CSI-RS is a TRS, UE is expected to process the TRS only if it can confirm that all CSI-RS symbols in one TRS burst are inside the same gNB COT.

Proposal 2: Aperiodic CSI-RS and its triggering DCI are transmitted in the same COT.

Proposal 3: UE expects CSI-RS for RLM to have fixed transmit power over all COTs.

Proposal 4: For a cell with multiple LBT bandwidth but availableRB-setPerCell-r16 not configured, the UE will consider all RB sets in the COT when DCI 2_0 is detected.

Proposal 5. DCI 2_0 can be configured without SFI field being configured to support COT duration, available RB set indicator and search space set group switching functionality.

Proposal 6. When COT duration is not explicitly configured for a semi-static channel access system, the UE assumes the COT ends at the beginning of the idle period in the same fixed frame period.

Proposal 7. If COT duration is explicitly configured, UE does not expect the COT duration to indicate the COT ends later than the beginning of the idle period in the same fixed frame period that the COT duration information is detected.

Proposal 8. COT duration is not explicitly configured for a semi-static channel access system. The UE assumes the COT ends at the beginning of the idle period in the same fixed frame period.