3GPP TSG RAN WG1 #101		     R1-2004560

e-Meeting, May 25th – June 5th, 2020, 2020

Source: 	Nokia, Nokia Shanghai Bell

Title:	On UE features for NR Unlicensed

Agenda: 	7.2.11.2

Document for:	Discussion and Decision

# Introduction

The first baseline for Rel-16 UE features has been created in RAN1#100bis-e [1] and updated in [2]. In this contribution we present our further views regarding the UE features for NR Unlicensed.

# Discussion

In RAN1#100bis-e the following agreements have been made on the structure of NR-U UE features:

Agreements:

- Define new basic FGs with components that have tightly related functionality to replace current basic FGs

  - In “mandatory/optional” column for the possible basic FGs, it should be clarified that the FG may be a part of basic operation for a particular scenario

    - If the FG is decided as a basic FG, the note will be updated to clarify that the FG is “optional with capability signaling and is required to be supported for the scenario”

  - Note: each basic FG will have capability bit

- [Working assumption] Take either one of following alternatives

Alt.1:

- Define a table to capture the basic FGs required for a certain NR-U deployment scenario in specification

  - Note: the table does not have impact on capability signaling

  - Note: the grouping of FGs in the table does not have impact on “prerequisite FGs” column in features list

Alt.2:

- Capture an association between the basic FGs required to be supported and a certain NR-U deployment scenario in the UE features list

Agreements:

- Define new basic FGs with components that have tightly related functionality to replace current basic FGs

  - In “mandatory/optional” column for the possible basic FGs, it should be clarified that the FG may be a part of basic operation for a particular scenario

    - If the FG is decided as a basic FG, the note will be updated to clarify that the FG is “optional with capability signaling and is required to be supported for the scenario”

  - Note: each basic FG will have capability bit

- [Working assumption] Take either one of following alternatives

Alt.1:

- Define a table to capture the basic FGs required for a certain NR-U deployment scenario in specification

  - Note: the table does not have impact on capability signaling

  - Note: the grouping of FGs in the table does not have impact on “prerequisite FGs” column in features list

Alt.2:

- Capture an association between the basic FGs required to be supported and a certain NR-U deployment scenario in the UE features list

Agreements:

- Replace FG10-1/1a/2/2a/2b by following FGs

  - UL channel access for dynamic channel access mode

  - UL channel access for semi-static channel access mode

  - SSB-based RRM [for dynamic channel access mode]

  - SSB-based RRM [for semi-static channel access mode]

  - MIB reading

  - SSB-based RLM [for dynamic channel access mode]

  - SSB-based RLM [for semi-static channel access mode]

  - SIB1 reception

Agreements:

- Replace FG10-1/1a/2/2a/2b by following FGs

  - UL channel access for dynamic channel access mode

  - UL channel access for semi-static channel access mode

  - SSB-based RRM [for dynamic channel access mode]

  - SSB-based RRM [for semi-static channel access mode]

  - MIB reading

  - SSB-based RLM [for dynamic channel access mode]

  - SSB-based RLM [for semi-static channel access mode]

  - SIB1 reception

The targeted scenarios listed in the WID [3] are as follows:

- Scenario A: Carrier aggregation between licensed band NR (PCell) and NR-U (SCell).

  - NR-U SCell may have both DL and UL, or DL-only.

  - In this scenario, NR PCell is connected to 5G-CN.

- Scenario B: Dual connectivity between licensed band LTE (PCell) and NR-U (PSCell)

  - In this scenario, LTE PCell connected to EPC as higher priority than PCell connected to 5G-CN.

- Scenario C: Stand-alone NR-U

  - In this scenario, NR-U is connected to 5G-CN.

- Scenario D: A stand-alone NR cell in unlicensed band and UL in licensed band (single cell architecture).

  - In this scenario, NR-U is connected to 5G-CN.

- Scenario E: Dual connectivity between licensed band NR and NR-U.

  - In this scenario, PCell is connected to 5G-CN.

Table 1 and 2 below provides our proposed mapping for the basic FGs to the NR-U deployment scenarios for dynamic and semi-static channel access modes, respectively. In the tables, ‘O’ indicates the FG can be optional for the corresponding scenario, while ‘M’ indicates the FG should be mandatory for the corresponding scenario.

Table 1: Proposed mapping for the basic FGs to the NR-U deployment scenarios assuming dynamic channel access mode

| Index | FG | Scen. A (DL-only) | Scen. A (UL+DL) | Scen. B | Scen. C | Scen. D | Scen. E | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10-1 | UL channel access for dynamic channel access mode | N/A | M | M | M | N/A | M | Implies support of transmission of relevant UL channels/signals as well |
| 10-2 | SSB-based RRM [for dynamic channel access mode] | M | M | M | M | M | M |  |
| 10-2b | MIB reading on unlicensed cell | N/A | N/A | M | M | M | M |  |
| 10-2c | SSB-based RLM [for dynamic channel access mode] | N/A | N/A | M | M | M | M |  |
| 10-2e | SIB1 reception on unlicensed cell | N/A | N/A | M | M | M | M |  |
| 10-2f | Support monitoring of extended RAR window | N/A | N/A | M | M | M | M |  |
| 10-30 | Support channel occupancy duration indicator field in DCI 2_0 | [O/M] | [O/M] | [O/M] | [O/M] | [O/M] | [O/M] | Depends on whether SFI is mandatory or optional in DCI 2_0 |

Table 2: Proposed mapping for the basic FGs to the NR-U deployment scenarios assuming semi-static channel access mode

| Index | FG | Scen. A | Scen. A (UL+DL) | Scen. B | Scen. C | Scen. D | Scen. E | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10-1a | UL channel access for semi-static channel access mode | M (DL+UL) | M | M | M | N/A | M | Not applicable to Scenario A with DL-only operation. |
| 10-2a | SSB-based RRM [for semi-static channel access mode] | M | M | M | M | M | M |  |
| 10-2b | MIB reading on unlicensed cell | N/A | N/A | M | M | M | M |  |
| 10-2d | SSB-based RLM [for semi-static channel access mode] | N/A | N/A | M | M | M | M |  |
| 10-2e | SIB1 reception on unlicensed cell | N/A | N/A | M | M | M | M |  |
| 10-2f | Support monitoring of extended RAR window | N/A | N/A | M | M | M | M |  |
| 10-30 | Support channel occupancy duration indicator field in DCI 2_0 | [O/M] | [O/M] | [O/M] | [O/M] | [O/M] | [O/M] | This depends on whether “end of FFP is treated as end of COT in the specification” |

Other aspects can be considered when mapping basic feature groups to scenarios. One such aspect is wideband operation modes 2/3, where such a mapping is proposed in Table 3 below. Given the simpler mapping in this case, it might be straightforward to handle the relationships by mean of pre-requisites in the FG table as well, depending on the final decisions regarding the FGs themselves and applicability to deployment scenarios.

Table 3: Basic FGs for wideband operation mode 2/3

| Index | FG | Comment |
| --- | --- | --- |
| 10-20 | Support search space set configuration with freqMonitorLocation-r16 | Applies for scenarios A, B, C, D, E |
| 10-29 | Support available RB set indicator field in DCI 2_0 | Applies for scenarios A, B, C, D, E |
| [10-19a] | [Support DL reception in a carrier with intra-cell guard-bands] | If FG is confirmed |
| [10-19b] | [Support UL transmission with subset of RB sets passing LBT] | If FG is confirmed |

On the detailed FGs, we have the following comments:

- 10-2/2a/2c/2d: it is OK to keep the differentiation depending on dynamic/static access modes.

- 10-2f: RAN2 has agreed already that 40ms is mandatory.

- 10-8: We are OK to remove brackets from [9,10].

- 10-19a: This FG is not needed in our view, and in any case this should not be based on guard-bands being allowed or not, but whether UE is capable of inband blocking or not

- 10-19b: This FG is OK in principle, but it should be clear how many RB sets are supported.

- 10-31: This is still open in NR-U maintenance, wait for conclusion.

# Conclusion

In this contribution we presented our proposals on UE features for NR Unlicensed.

# References

[1] R1-2003073, “RAN1 UE features list for Rel-16 NR after RAN1#100bis-E,” AT&T, NTT DOCOMO, Apr. 2020

[2] R1-2003198, “Summary on email discussion [100b-e-NR-UEFeatures-Remaining] NR-unlicensed,” NTT DOCOMO, May 2020

[3] RP-192926, “Revised WID on NR-based Access to Unlicensed Spectrum,” Qualcomm, Dec. 2019

# Annex

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type ( 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10. NR-unlicensed | 10-1 | UL channel access for dynamic channel access mode | 1. Type 1 channel access 2. Type 2A channel access 3. Type 2B channel access 4. Type 2C channel access 5. 20MHz LBT bandwidth 6. CP extension up to 1 symbol for PUSCH/PUCCH transmission |  | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-1a | UL channel access for semi-static channel access mode | 1. Type 2C channel access 2. Single sensing slot of 9us channel access 3. 20MHz LBT bandwidth | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-2 | SSB-based RRM [for dynamic channel access mode] | 1. SSB-based RRM with Q [for dynamic channel access mode] | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A | Q indicates the value of RAN1 parameter NSSBQCL | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-2a | SSB-based RRM [for semi-static channel access mode] | 1. SSB-based RRM with Q [for semi-static channel access mode] | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A | Q indicates the value of RAN1 parameter NSSBQCL | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-2b | MIB reading on unlicensed cell | 1. MIB reading on unlicensed cell | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-2c | SSB-based RLM [for dynamic channel access mode] | 1. SSB-based RLM with Q [for dynamic channel access mode] | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A | Q indicates the value of RAN1 parameter NSSBQCL | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-2d | SSB-based RLM [for semi-static channel access mode] | 1. SSB-based RLM with Q [for semi-static channel access mode] | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A | Q indicates the value of RAN1 parameter NSSBQCL | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-2e | SIB1 reception on unlicensed cell | 1. SIB1 reception on unlicensed cell | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-2f | Support monitoring of extended RAR window | 1. Support of RAR extension from 10ms to [40ms] by decoding of the 2-bit SFN indication in DCI 1_0 | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-7 | UL channel access for 10 MHz SCell | - 10 MHz LBT bandwidth | one of {10-1, 10-1a} | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-10 | RSSI and channel occupancy measurement and reporting | - RSSI measurement - Channel occupancy reporting | TBD | Yes | N/A |  | FFS: Per band or Per UE | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-11 | SRS starting position at any OFDM symbol in a slot | - Support transmitting SRS starting in all symbols (0,…,13) of a slot | TBD | Yes | N/A |  | FFS: Per band or Per UE | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-20 | Support search space set configuration with freqMonitorLocation-r16 | 1. Support search space set configuration with freqMonitorLocations-r16 | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-20a | Support coreset configuration with rb-Offset | 1. Support coreset configuration with rb-Offset | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-23 | CGI reading on unlicensed cell [based on off-sync raster SSB] for ANR functionality | 1. Support acquisition of relevant information from a neighbouring NR unlicensed cell in an unlicensed carrier by reading the RMSI of the neighbouring unlicensed cell and reporting the acquired information to the network | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A | Support reading RMSI from an unlicensed cell [with an off-sync raster SSB] for ANR | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-25 | Enable configured UL transmissions when DCI 2_0 is configured but not detected | 1. Support configuration of enableConfiguredUL-r16 and enable transmission of higher-layer configured UL *SRS, PUCCH, CG-PUSCH etc) when DCI 2_0 is configured but not detected | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-27 | Wideband PRACH | - Enhanced PRACH design for NR-U by adopting a single long ZC sequence, with ZC sequence = 1151 for 15kHz and ZC sequence = 571 for 30kHz | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-29 | Support available RB set indicator field in DCI 2_0 | - Support monitoring DCI 2_0 to read availableRB-Sets-r16 | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-30 | Support channel occupancy duration indicator field in DCI 2_0 | - Support monitoring DCI 2_0 to read COT duration | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-8 | Type B PDSCH length {3, 5, 6, 8, [9, 10,] 11, 12, 13} without DMRS shift due to CRS collision | - Type B PDSCH length {3, 5, 6, 8, [9, 10,] 11, 12, 13} without DMRS shift due to CRS collision | 5-6a | Yes | N/A |  | FFS: Per band or Per UE | N/A | N/A | N/A | Note length 9/10 with DMRS shift due to CRS collision are already covered by 14-3 | Optional with capability signalling |
| 10. NR-unlicensed | 10-9 | Search space set group switching with explicit DCI 2_0 bit field trigger or with implicit PDCCH decoding with DCI 2_0 monitoring | 1. Two groups of search space sets 2. Monitor DCI 2_0 with a search space set switching field 3. Support switching the search space set group with PDCCH decoding in group 1 4. Support a timer to switch back to original search space set group 5. Monitor DCI 2_0 for channel occupancy time and use the end of channel occupancy time to switch back to the original search space set group | TBD | Yes | N/A |  | FFS: Per UE or per band or per BC | N/A | N/A | N/A | Being configured with two groups of search spaces, and switch between them. Some search space sets can be configured in both groups. | Optional with capability signalling |
| 10. NR-unlicensed | 10-9b | Search space set group switching with implicit PDCCH decoding without DCI 2_0 monitoring | 1. Two groups of search space sets 2. Support switching the search space set group with PDCCH decoding in group 1 3. Support a timer to switch back to original search space set group | TBD | Yes | N/A |  | FFS: Per UE or per band | N/A | N/A | N/A | Being configured with two groups of search spaces, and switch between them. Some search space sets can be configured in both groups. | Optional with capability signalling |
| 10. NR-unlicensed | 10-9c | Joint search space group switching across multiple cells | - Configured with a group of cells and switch search space set group jointly over these cells | one of {10-9, 10-9b} | Yes | N/A |  | FFS: Per UE or per band or per BC | N/A | N/A | N/A | Without this capability, the UE will switch search space set groups for different cells independently | Optional with capability signalling |
| 10. NR-unlicensed | 10-9d | Support Search space set group switching capability 2 | - Search space set group switching Capability-2: P=10/12/22 symbols for µ = 0/1/2 SCS | one of {10-9, 10-9b} | Yes | N/A |  | FFS: Per UE or per band | N/A | N/A | N/A | Without this capability, the UE supports search space set group switching capability-1: P=25/25/25 symbols for µ=0/1/2 | Optional with capability signalling |
| 10. NR-unlicensed | 10-14 | Non-numerical PDSCH to HARQ-ACK timing | - Support configuration of a value for dl-DataToUL-ACK indicating an inapplicable time to report HARQ ACK | TBD | Yes | N/A |  | FFS: Per UE or per band | N/A | N/A | N/A | If non-numerical K1 value is supported | Optional with capability signalling |
| 10. NR-unlicensed | 10-15 | Enhanced dynamic HARQ codebook | 1. Support of bit fields signalling PDSCH HARQ group index and NFI in DCI 1_1 (configuration of nfi-TotalDAI-Included) 2. Support of bit field in DCI 0_1 for other group total DAI if configured. (configuration of ul-TotalDAI-Included) 3. Support the retransmission of HARQ ACK (pdsch-HARQ-ACK-Codebook = enhancedDynamic-r16) | TBD | Yes | N/A |  | FFS: Per UE or per band | N/A | N/A | N/A | Enhanced dynamic HARQ codebook supporting grouping of HARQ ACK and triggering the retransmission of HARQ ACK in each groups | Optional with capability signalling |
| 10. NR-unlicensed | 10-16 | One-shot HARQ ACK feedback | - Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_1 scheduling a PDSCH - Support feedback of type 3 HARQ-ACK codebook , triggered by a DCI 1_1 without scheduling a PDSCH using a reserved FDRA value | TBD | Yes | N/A |  | FFS: Per band or Per UE | N/A | N/A | N/A | Upon triggering, UE reports A/N for all HARQ processes and all CCs in a PUCCH group. | Optional with capability signalling |
| 10. NR-unlicensed | 10-17 | Multi-PUSCH UL grant | 1. Support of scheduling up to 8 PUSCH with a single DCI 0_1 | TBD | Yes | N/A |  | FFS: Per band or Per UE | N/A | N/A | N/A |  | Optional with capability signalling |
| 10. NR-unlicensed | [10-19a] | [Support DL reception in a carrier with intra-cell guard-bands] | 1. [When DL BWP has multiple RB sets, support using the available RB set bitmap in DCI 2_0 to validate the periodic CSI-RS transmission if the CSI-RS is over multiple RB-sets] | TBD | Yes | N/A |  | Per band | N/A | N/A |  | Without this capability, UE will assume all RB sets in the DL BWP are all transmitted or none of them are transmitted | Optional with capability signalling |
| 10. NR-unlicensed | [10-19b] | [Support UL transmission with subset of RB sets passing LBT] | 1. [When UL BWP has multiple RB sets, support transmission of UL signal or channels when LBT passes for only the RB sets the UL signals or channels are located] | TBD | Yes | N/A |  | Per band | N/A | N/A |  | Without this capability, UE will transmit UL when all RB sets in the UL BWP pass LBT | Optional with capability signalling |
| 10. NR-unlicensed | 10-26 | CSI-RS based RLM for NR-U | [CSI-RS based RLM for NR-U] | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 10. NR-unlicensed | 10-26a | CSI-RS based RRM for NR-U | [CSI-RS based RRM for NR-U] | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 10. NR-unlicensed | [10-31] | [Support of CSI-RS measurements for CSI reporting and tracking without COT duration from DCI 2_0] | [· Perform CSI measurements for reporting and tracking using CSI-RS resources that are not within a COT duration indicated by DCI 2_0 · Note: This includes the cases when DCI 2_0 is not configured and when DCI 2_0 is configured but COT duration is not provided by either CO duration field or SFI.] | TBD | Yes | N/A |  | [Per band] | N/A | N/A |  |  | Optional with capability signaling |
| 10. NR-unlicensed | 10-3 | PRB interlace mapping for PUSCH | 1. PRB interlace frequency domain resource allocation for PUSCH | TBD One of {10-1, 10-1a} | Yes | N/A |  | Per band | N/A | N/A | N/A | Support of PRB interlace PUSCH | Optional with capability signalling |
| 10. NR-unlicensed | 10-3a | PRB interlace mapping for PUCCH | - PRB interlace frequency domain resource allocation for PUCCH format 0 and format 1 - PRB interlace frequency domain resource allocation for PUCCH format 2 - PRB interlace frequency domain resource allocation for PUCCH format 3 | TBD One of {10-1, 10-1a} | Yes | N/A |  | Per band | N/A | N/A | N/A | Support of PRB interlace PUCCH format 0/1 | Optional with capability signalling |
| 10. NR-unlicensed | 10-12 | OCC for PRB interlace mapping for PF2 and PF3 | 1. OCC2 2. OCC4 | 10-3a | Yes | N/A |  | Per band | N/A | N/A | N/A | UE OCC capability for EPF2/EFP3 | Optional with capability signalling |
| 10. NR-unlicensed | 10-13a | Extended CP range of more than one symbol for CG-PUSCH | - UE supports generating a CP extension of length longer than 1 symbol for Configured Grant PUSCH transmission | TBD One of {5-19, 5-20} | Yes | N/A |  | Per band | N/A | N/A | N/A | How long a UE can generate the CP extension beyond 1 symbol for CG-PUSCH | Optional with capability signalling |
| 10. NR-unlicensed | 10-18 | Configured grant with retransmission in CG resources | 1. Support retransmission in CG resources 2. Support configured grant retransmission timer 3. Support DFI monitoring 4. Support CG-UCI in CG-PUSCH | TBD One of {5-19, 5-20} | Yes | N/A |  | Per band | N/A | N/A | N/A | Support configured grant with retransmission in configured grant resource | Optional with capability signalling |
| 10. NR-unlicensed | 10-21a | Support using ED threshold given by gNB for UL to DL COT sharing | 1. Use ULtoDL-CO-SharingED-Threshold-r16 for cat 4 LBT for scheduled UL to share COT with gNB for DL 2. Use ULtoDL-CO-SharingED-Threshold-r16 for cat 4 LBT for CG-PUSCH to share COT with gNB for DL 3. Indicate in CG-UCI the COT sharing information | TBD 10-1 | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 10. NR-unlicensed | 10-24 | CG-UCI multiplexing with HARQ ACK | 1. Support multiplexing CG-UCI with HARQ ACK | 10-18 | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 10. NR-unlicensed | 10-28 | Configured grant with Rel-16 enhanced resource configuration | 1. Support configuration of resources with cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16, | TBD One of {5-19, 5-20} | Yes | N/A |  | FFS: Per UE or per band | N/A | N/A | N/A |  | Optional with capability signalling |