3GPP TSG RAN WG1 #101		     R1-2004565

e-Meeting, May 25th – June 5th, 2020, 2020

Source: 	Nokia, Nokia Shanghai Bell

Title:	On UE features for UE Power Savings

Agenda: 	7.2.11.7

Document for:	Discussion and Decision

# Introduction

The first baseline for Rel-16 UE features has been created in RAN1#100bis-e [1] and updated in [2]. In this contribution we present our further views regarding the UE features for UE power savings.

# Discussion

The key aspects to be resolved regarding UE features for UE power savings are the FG types and the potential need for licensed/unlicensed differentiation. Most of the UE power saving features defined in Rel-16 impact baseband operation only, and hence they should be “per UE” without a need for further xDD/FRx differentiation. Even FG 19-3 “Maximum MIMO Layer Adaptation” in practice it is only an indication of the UE’s capability of benefiting from adapting the maximum number of MIMO layers, and the UE will in any case support less MIMO layers than the maximum possible as part of normal operation in any band.

The UE power saving features do not impact the channel access mechanisms in any way, and they do not depend on dynamic relationship with the channel occupancy time, or other specific features to operate in unlicensed spectrum. The amount of power savings obtained in licensed and unlicensed operation may change as the UE might need to perform other tasks in unlicensed band, but those aspects are unrelated to the ability to support the FGs themselves.

Proposal 1: UE power saving FGs are “per UE” without xDD/FRx differentiation.

Proposal 2: Do not introduce licensed/unlicensed differentiation for UE power saving FGs.

# Conclusion

In this contribution we presented our proposals on UE features for UE power savings.

# References

[1] R1-2003073, “RAN1 UE features list for Rel-16 NR after RAN1#100bis-E,” AT&T, NTT DOCOMO, Apr. 2020

[2] R1-2003195, “Summary of Email Approval [100e-b-NR-UEFeatures-Remaining] — UE Power Saving Aspects,” Moderator (AT&T), May 2020

# Annex

| Features | Index | Feature group | Components | Prerequisite feature groups |  | Need for the gNB to know if the feature is supported |  | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 19.UE Power Saving | 19-1 | DRX Adaptation | - Configured PS_offset for the detection of DCI format 2_6 with CRC scrambling by PS-RNTI and minimum time gap before DRX ON duration - Indication of UE whether or not to start drx_OnDuration timer at the DRX ON by detection of DCI format 2_6 - Configured UE wakeup or not when DCI format 2_6 is not detected at all monitoring occasions outside Active time - Configured P-CSI / L1-SINR report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON - Configured L1-RSRP report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON | N/A |  | Yes |  | N/A |  | Per UE | No | Yes | N/A | The minimum time gap between the end of the slot of last DCI format 3_0 2_6 monitoring occasion and the start of the DRX ON is a UE capability based on subcarrier spacing. - The reporting is per SCS in units of slots of the respective SCS - The reported value for a SCS is taken from two possible values per SCS - The largest value of minimum time gap in UE capability is no more than the number of slots equal to 3 ms UE is not required to monitor PDCCH for detection of DCI format 2_6 during the minimum time gap FFS: whether or how to handle licensed/unlicensed differentiation | Optional with capability signalling |
|  | 19-2 | Cross Slot Scheduling | (1) Dynamic indication of applicable minimum scheduling restriction by DCI format 0_1 and 1_1 minimumSchedulingOffset K0 configuration for PDSCH and aperiodic CSI-RS triggering offset (2) minimumSchedulingOffset K2 configuration for PUSCH |  | FFS |  | Yes | N/A | Dynamic adaptation of the minimum value of K0min/K2min for cross-slot scheduling is not supported | Per UE | No | No | N/A | FFS: whether or how to handle licensed/unlicensed differentiation | Optional with capability signalling |
|  | 19-3 | Maximum MIMO Layer Adaptation | - 1. Support of maximum number of MIMO layer configuration per DL BWP |  | See Note |  | Yes | N/A |  | [Per UE ] | No | [Yes] | N/A | This capability is indicated only if UE supports the network configuration of maxMIMO-Layers according to maxLayersMIMO-Indication FFS: whether or how to handle licensed/unlicensed differentiation | Optional with capability signalling |
|  | 19-4a | UE assistance information | Support of reporting preferred minimum K0/K2 via UE assistance information - 15kHz/30kHz SCS: {1, 2, 4, 6} slots - 60kHz/120kHz SCS: {2, 4, 8, 12} slots |  | 19-2 |  | Yes | N/A |  | Per UE | No | No | N/A | The minimum applicable value of K0 (K2) for an active DL (UL) BWP for the carrier where PDSCH(PUSCH) is transmitted FFS: whether or how to handle licensed/unlicensed differentiation | Optional with capability signalling |