3GPP TSG RAN WG1 Meeting #101-e                                                                                               	R1-2004278(Revision of R1-2003394)

e-Meeting, May 25th – June 5th, 2020

Source:	Moderator (vivo)

Title:	Summary#2 of UL inter UE Tx prioritization

Agenda Item:	7.2.5.5

Document for:	Discussion and Decision

- Introduction

The document provides a summary for discussion based on the contribution submitted to agenda item 7.2.5.5-UL inter UE Tx prioritization/multiplexing.

Email discussion list for RAN1#101-e

As the outcome of email discussion [101-e-Prep-NR-eURLLC-interUE], the email discussion threads for RAN1#101-e are agreed as following

[101-e-NR-L1enh-URLLC-InterUE-01] Issue#1 in R1-2003394 by 5/29 and corresponding TP (if any) by 6/5 – Xueming (vivo) - Including the processing order of UL cancellation by TDD configuration/SFI and other UL multiplexing/cancellation. [101-e-NR-L1enh-URLLC-InterUE-02] Issue#2, Issue #4, Issue #6 in R1-2003394 and a possible conclusion by 5/29 and corresponding TP (if any) by 6/5 – Xueming (vivo) - Note: issue#4 includes the following - Whether and how to clarify that data-data collision procedure is not affected by UL CI (limited to CG-DG collision case with Rel-15 timeline and potential Rel-16 timeline, if agreed in AI7.2.5.7) - Text proposal to clarify that for PUSCH repetition type B, the cancellation is applied on each actual repetition - Possible Conclusion - The 1st editor’s note in section 11.2A of endorsed 38.213CR (R1-2003176) is removed [101-e-NR-L1enh-URLLC-InterUE-03] Issue#7, Issue #8, Issue #10 in R1-2003394 by 5/29 and corresponding TP (if any) by 6/5 – Xueming (vivo)

During the discussion in [101-e-Prep-NR-eURLLC-interUE], issue#1 was selected for email discussion due to common interest and during the discussion point#5 and #6 were added to issue#1.

Among the other issues, issue#2,#4,#6,#7,#8,#10 were selected due to higher interest and issue#3,#5,#9 were dropped. A conclusion was reached for issue#11 in [101-e-Prep-NR-eURLLC-interUE] but will be formally approved during RAN#101-e in [101-e-NR-L1enh-URLLC-InterUE-02].

| Company | Issue 2 | Issue 3 | Issue 4 | Issue 5 | Issue 6 | Issue 7 | Issue 8 | Issue 9 | Issue 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Nokia, NSB | High would be good to support | Low no need for optimization | High good to capture and should not be controversial | Low no need identified and already getting one RRC parameter was hard, so we prefer to not open this again | High Somehow aligned also with intra-UE cancelation behavior | High simple change, should be relatively easy to get some consensus | High to complete the discussion on this issue finally… | Low No need as identified by FL | High Agree with FL assessment, should be easily agreeable |
| Sony | High. Should be straight forward to support. | High This seems easy enough to support based on ZTE’s TP. | High | Low Involves RRC configuration. Also is this for each CG or all CGs. | Low Doesn’t the UE cancels the entire PUSCH and so it should by definition cancel before the first indicated symbol. | High | High | Low | High Easy enough to fix and provide clarification. |
| Samsung | Low No need to support | Low Non-essential correction | High Trivial to conclude | Low No need, no use case. Non-essential, no need for new RRC | High Will be good to clarify | High Trivial to conclude | Low Non-essential correction. Questionable/ unclear benefit. | Low No need. | High Trivial to conclude |
| ZTE | Low No need to support | High Improve the efficiency of UL CI with simple standardization effort | High | Low No essential and no need for a new RRC parameter | Low Current description is reasonable, i.e., it is better for gNB to know which is the valid part of canceled transmission | High Essential correction | High Essential correction | Low | High Essential correction |
| DOCOMO | High Should be straight forward to support | Low Optimization | High Not controversial | Low No need to discuss | High | High | High | Low No need to discuss | High Essential and easily agreeable |
| Panasonic | High Good to conclude whether to support it or not. | Low It is optimization issue. | High Agree to the proposed TP. | Low It would not be essential for introducing new RRC. | High It seems essential clarification. | High Agree to the proposed TP. |  | Low Agree with FL suggestion. | High Agree to the proposed TP. |
| Spreadtrum | High We need a conclusion for this issue. | Low Optimization | High | Low Optimization | High Good to clarify. | High | High It seems essential clarification. | Low No need to discuss. | High It is an essential correction. |
| Qualcomm | Low. | High and easy to fix | No need to discuss as the timeline for CG-DG collision handling is clear from Rel. 15. No change is needed. There is no agreement on changing the timeline. | Low. No need for optimization. | High. | Low. No need for any change. gNB can solve the issue by properly configuring the first set. | High. | Low. Agree with the FL assessment. | High |
| HW/HiSi | Low No need for UL CI in FR2. | High Important for support of UL CI in TDD. Otherwise eMBB performance could suffer significantly. | Medium. Should be included but is straight forward. | Low Agree with Nokia on the reason. | Low This should not be left to implementation and is different from the intra-UE case. How about when the UE is configured with CBGs? The gNB might want the UE to transmit earlier CBGs? | Medium Should be included. A simple change and should be straight forward. | High This has been discussed before and it should be a rather quick fix, | Low Support the same view as the FL | High Agree with FL |
| vivo | High It is not preferable to restrict the frequency band for the feature | Low no need for optimization | High It should be clarified that intra-UE handling of data-data collision is not affected by UL CI | Low No need | High Will be good to clarify | High Good to clarify | High | Low No need | High |
| CATT | Low | Low. It is an optimization and we believe it can be handled by proper configuration | High. It’s better to clearly capture it to make the spec complete. | Low. No need | High. Better to clarify. | High. To fixed the hole in the current spec. | High. It should be an easy one and should be fixed to complete the UL CI feature. | Low | High |
| LG | Low There seems no practical use case | Low Optimization | High Need to be conclude. | Low No need to handle same priority differently. | High Agree with Nokiae same priority differently. fixed to complete | High Simple and necessary correction . | High Fine to discuss. Current TP are to use FrequencyInfoUL within ServingCellConfigCommon, or FrequencyInfoUL-SIB within ServingCellConfigCommon-SIB. For SIB only, there may be no issue. If we consider ServingCellConfigCommon, it would be necessary to clarify the case of cross-carrier indication of UL CI. | Low If TO which UE able to start is cancelled, UE cannot start. we are fine not to change anything. | High |
| Apple | Low No need to support in FR2 | Low No need for optimization | Medium It seems to make sense to include PUSCH in the sentence, but it should not have anything to do with PUSCH-PUSCH collision. That is, the TP is not directly connected to the agreement in our view. In addition, should we clarify a repetition is an actual repetition for PUSCH repetition Type B? | Low No need (especially this requires more RRC parameters) | High Should clarify, similar to the discussion in UCI AI | High Should fix it. | High | Low Agree with FL’s comments | High Should fix it. |
| Motorola Mobility, Lenovo | Low: No need to support in FR2 | Low: optimization | Good to discuss: In our view, may be better to simplify the spec text in section 11.2A and remove “, as determined in Clauses 9 and 9.2.5 or in Clause 6.1 in [6, TS 38.214],”. The respective UE behavior for determining PUSCH repetitions is/should be provided in respective clauses. | Low: no need to add new RRC parameters | High We agree with the proposal. Good to discuss if any spec change is needed. | High This is for the case that only one of the DCI formats 0-1 and 0-2 has SRI field configured. | High | Good to clarify/have a common understanding | High: should be added |
| Intel | Low (use case is questionable) | Low (non-essential/optimization) | High (should be straightforward to conclude) | Low (non-essential, need new RRC parameter) | Low (For UL CI, in contrast to intra-UE cases, cancelation time is rather tight. Hence, the probability that UE could know where to cancel sufficiently ahead of time would be quite low in practice. So, use case for the relaxation seems somewhat questionable.) | High (seems simple to conclude) | High (essential correction to avoid ambiguity) | Low (agree with FL comment) | High (seems easy to fix) |
| OPPO | High Straightforward to support | Low No need for optimization | High Should be captured based agreement | High 1) RRC parameter details are discussing in RAN2 , which needs more information. 2) For CG-PUSCH, inter-UE interference still exists if priority based cancellation is configured. | Low No need for optimization | High Simple and essential correction | High Essential correction | Low | High |
| WILUS | Low No solid use case in FR2 | Low Optimization | High Straightforward to capture agreements | Low Need to add new RRC parameters | High Good to clarify it | High Need to fix it | High Need to fix it | Low Agree with FL’s comments | High Need to fix it |
| Ericsson | High, It impacts UE feature discussion | Low | High Seems like an easy fix | Low We dont think we need to have this | High Agree that this is an issue | Low | Low Non essential | Low | High |
| Summary | High: 8 Low: 10 | High: 4 Low: 14 | High: 15 Low/Medium: 3 | High: 1 Low: 17 | High: 13 Low: 5 | High: 15 Low/Medium: 3 | High: 15 Low: 2 | High: 1 Low: 17 | High: 18 Low: 0 |

Discussions

- Issue 1: Handling of UL grant no earlier than UL CI, and whether another UL transmission can be scheduled on the cancelled symbols [1][2][4][5][6][7][8][9][10][11][12][13][14][15][16][17][18][19][20]

There are following discussion points

Discussion point #1: UL CI is applicable to UL grant sent earlier than the UL CI

There seems to be a general consensus on this aspect, a slight difference is whether UL CI is applicable to an overlapping UL grant

- Alt 1: A DCI format 2_4 is only applicable to an uplink grant scheduling PUSCH/SRS if the ending symbol of the PDCCH carrying the UL grant is earlier than the first symbol of the PDCCH carrying DCI format 2_4. (i.e. UL CI is not applicable to an overlapping UL grant) [2][4][5][6][7][9][12][15][16][17]

- Alt 2: A DCI format 2_4 is only applicable to an uplink grant scheduling PUSCH/SRS if the ending symbol of the PDCCH carrying the UL grant is earlier than the last symbol of the PDCCH carrying DCI format 2_4. (i.e. UL CI is applicable to an overlapping UL grant) [11]

For the applicability of UL CI to the UL grant sent no earlier than the UL CI, following cases are to be discussed:

Case 1:

Discussion point #2: If UE does not cancel a DG-PUSCH based on the detected UL CI, whether another DG-PUSCH can be scheduled on the resource indicated by UL CI if the 2nd UL grant is received no earlier than the UL CI (i.e. Case 1)

  - Yes: [1][2][5][7][10][11][12][15][16][17]

  - No: [4] [20]

- FL Proposal:

- To consider the following alternatives

  - Alt1: If the UE does not cancel a transmission in resources indicated by DCI format 2_4, the UE can receive an UL grant scheduling a transmission in any resources and transmit accordingly, if the ending symbol the PDCCH carrying UL grant is no earlier than the first symbol of the PDCCH carrying DCI format 2_4.

  - Alt 2: If the UE does not cancel a transmission in resources indicated by DCI format 2_4, the UE can receive an UL grant scheduling a transmission in any resources and transmit accordingly, if the ending symbol the PDCCH carrying UL grant is no earlier than the last symbol of the PDCCH carrying DCI format 2_4.

- To discuss if the above only applies to UE behaviour #2 or both behaviour #1 and #2

  - Note: UE behaviour#1 – UL CI only applicable to low priority transmissions, UE behaviour#2- UL CI applicable irrespective of transmission priority

Case 2:

Case 3:

Discussion point #3: If UE has to cancel a DG-PUSCH1 based on the detected UL CI, whether another DG-PUSCH can be scheduled on cancelled symbols of DG-PUSCH1, if the 2nd UL grant is received no earlier than the UL CI? The cancelled symbol may (case 3) or may not (case 2) within the resource indicated by the UL CI

  - Yes under some condition

    - Condition 2 [7]: The offset between the end of PDCCH carrying UL grant and the start of its scheduling transmission is no less than Tproc,2 +d1, where Tproc,2 is determined by UE processing time capability for the carrier, and d1 is the time duration corresponding to 0,1,2 symbols reported by UE capability.

    - Condition 3 [19]: If DG-PUSCH2 is of high priority

    - Condition 4 [11]: if the gap between the 2nd UL grant and the DG-PUSCH2 is at least maxTproc,2,T'proc,2+Tproc,2-Tdelta and Tdelta is the gap between the ending symbol of the UL CI and the ending symbol of the 2nd UL grant

      - Similar to condition #1

    - Condition 6 [9][10][19]: if DG-PUSCH2 does not include resource indicated by the UL CI (i.e. case 2)

  - No: [1][2][4] [5][6][8] [11][12][13][14][15][16][17][18][20]

- FL proposal:

- If UE has to cancel a DG-PUSCH1 based on the detected UL CI, another DG-PUSCH2 can NOT be scheduled on cancelled symbols of DG-PUSCH1, [if the 2nd UL grant is received no earlier than the UL CI]

  - The cancelled symbols of DG-PUSCH1 include  the symbols within and outside the resource indicated by the UL CI

Case 4:

Discussion point #4: If UE has to cancel a DG-PUSCH1 based on the detected UL CI, whether another DG-PUSCH can be scheduled on the resource indicated by the UL CI but not overlapping with cancelled symbol of DG-PUSCH1, if the 2nd UL grant is received no earlier than the UL CI?

  - Yes under some conditions

    - Condition 1 [1]: if the start of DG-PUSCH2 is 2*Tproc,2 after the end of the UL CI

    - Condition 2 [7]: The offset between the end of PDCCH carrying UL grant and the start of its scheduling transmission is no less than Tproc,2 +d1, where Tproc,2 is determined by UE processing time capability for the carrier, and d1 is the time duration corresponding to 0,1,2 symbols reported by UE capability.

    - Condition 3 [19]: If DG-PUSCH2 is of high priority

    - Condition 4 [11]: if the gap between the 2nd UL grant and the DG-PUSCH2 is at least maxTproc,2,T'proc,2+Tproc,2-Tdelta and Tdelta is the gap between the ending symbol of the UL CI and the ending symbol of the 2nd UL grant

      - Similar to condition#1

    - Condition 5 [15]: if the 2nd UL grant is at least X=1 slot after the ending symbol of UL CI, and the DG-PUSCH2 does not overlap with the cancelled symbols of DG-PUSCH1

  - Yes without condition

    - [2][6][10]

  - No: [5][12][13][14][15][17][20]

- FL proposal:

- To decide one between the following

  - Alt 1: If UE has to cancel a DG-PUSCH1 based on the detected UL CI, another DG-PUSCH2 CAN be scheduled on the resource indicated by the UL CI but not overlapping with cancelled symbol of DG-PUSCH1, if the 2nd UL grant is received no earlier than the UL CI and if a specified condition is satisfied

    - Condition to be decided

  - Alt 2: If UE has to cancel a DG-PUSCH1 based on the detected UL CI, another DG-PUSCH2 CANNOT be scheduled on the resource indicated by the UL CI but not overlapping with cancelled symbol of DG-PUSCH1, [if the 2nd UL grant is received no earlier than the UL CI]

- To discuss if the above only applies to UE behaviour #2 or UE behaviour#1 (HP PUSCH) or both

  - Note: UE behaviour#1 – UL CI only applicable to low priority transmissions, UE behaviour#2- UL CI applicable irrespective of transmission priority

Discussion point #5: From a UE configured with behaviour#2, if a PUCCH/SRS is cancelled by another PUSCH of higher priority, can the prioritized PUSCH be cancelled by UL CI?

Case 5:

- To be discussed during RAN1#101-e

Discussion point#6 processing order of UL cancellation by TDD configuration/SFI and other UL multiplexing/cancellation.

This issue was raised by [22] under UCI agenda item, but offloaded to inter-UE mux agenda item. The proponent would like to clarify the UE processing order of UL cancellation due to dynamic SFI/semi-static TDD configuration, and the intra-UE multiplexing/prioritization. Some discussion of the issue is also provided in R1-2002060

Note that in RAN1#100bis-e, we concluded the UE processing order between UL CI and intra-UE multiplexing/prioritization, with following agreement, which means the UL CI will be applied after intra-UE multiplexing/prioritization.

Agreements:

  UE behavior of handling intra-UE prioritization/multiplexing for overlapping UL transmissions is not affected by UL CI.

Therefore it seems natural to extend the agreement to the dynamic SFI case, which means the UL cancellation due to dynamic SFI will be applied after intra-UE multiplexing/prioritization. Therefore suggest to consider the following proposed conclusion

- Propose conclusion:

- UE behaviour of handling intra-UE prioritization/multiplexing for overlapping UL transmissions on semi-static flexible symbols is not affected by UL cancellation due to dynamic SFI or DL grant

As discussed in R1-2002060, a semi-static UL transmission (or PUSCH repetition) can be configured to be collided with DL symbols configured by semi-static TDD configuration, or SSB symbols and in such cases the semi-static UL transmission (or PUSCH repetition) is cancelled. It is proposed to clarify whether such cancellation due to semi-static DL symbols is processed before or after intra-UE prioritization/multiplexing. Suggest to consider the following alternatives

- Alt 1: UL cancellation due to collision with DL symbols configured by semi-static TDD configuration, or SSB symbols is processed before the intra-UE prioritization/multiplexing

- Alt 2: UL cancellation due to collision with DL symbols configured by semi-static TDD configuration, or SSB symbols is processed after the intra-UE prioritization/multiplexing

- Issue 2: Support of UL CI in the scenarios where processing capability #2 is not defined. [2[5][9][19]

[2][9][19] proposed that whenever UE processing capability 2 is not defined (for the frequency range, or SCS), use UE processing capability 1 to derive the UL CI processing time. Following TP is provided

TP for section 11.2A of TS38.213

------------------------------------------ Start of proposed change ------------------------------------------

An indication by a DCI format 2_4 for a serving cell is applicable to a PUSCH transmission or a SRS transmission on the serving cell. For the serving cell, the UE determines the first symbol of the TCI symbols to be the first symbol that is after Tproc,2+d from the end of a PDCCH reception where the UE detects the DCI format 2_4, where d is provided by XXX. Tproc,2 corresponds to the PUSCH processing capability 2 [6, TS 38.214] assuming d2,1=0 with μ being the smallest SCS configuration between the SCS configurations of the PDCCH and of a PUSCH transmission or of an SRS transmission on the serving cell. Processing capability 1 should be used if processing capability 2 is not defined for a pair of Frequency Range and SCS in [6, TS 38.214]. The UE does not expect to cancel the PUSCH transmission or the SRS transmission before a corresponding symbol that is Tproc,2 after a last symbol of a CORESET where the UE detects the DCI format 2_4.

------------------------------------------- end of proposed change   ------------------------------------------

[5] proposed that there is no need to support of UL CI in the scenarios where processing capability #2 is not defined.

- Issue 3: Bitmap re-arrangement due to excluding of DL and SSB symbols from reference time region [1][5][20]

[1] discussed the DCI bit waste issue when the number of partitions configured by timeGranularityforCI is larger than the number of remaining symbols after excluding DL and SSB symbols from the time region for UL CI, and proposed the following

Proposal: NR should support to take the minimum value between the number of partitions configured by timeGranularityforCI and the number of remaining symbols after excluding DL and SSB symbols from the time region for UL CI as the actual number of partitions.

--------------------------------------------Text Proposal for Section 11.2A in TS38.213[3]---------------------------------

11.2A Cancellation indication <---------------------------Other parts are omitted -------------------------------> For a serving cell having an associated field in a DCI format 2_4, for the field denote by - a number of bits provided by CI-PayloadSize - a number of PRBs provided by frequencyRegionforCI in timeFrequencyRegion - a number of symbols, excluding symbols for reception of SS/PBCH blocks and DL symbols indicated by tdd-UL-DL-ConfigurationCommon, from a number of symbols that - is provided by timeDurationforCI in timeFrequencyRegion, if the PDCCH monitoring periodicity for the search space set with the DCI format 2_4 is one slot and there are more than one PDCCH monitoring occasions in a slot, or - is equal to the PDCCH monitoring periodicity, otherwise. - a number of partitions for the symbols provided by timeGranularityforCI in timeFrequencyRegion - min {, } sets of bits from the MSB of the NCI bits have a one-to-one mapping with GCI groups of symbols where each of the first groups includes symbols and each of the remaining groups includes symbols. A UE determines a symbol duration with respect to a SCS configuration of an active DL BWP where the UE monitors PDCCH for DCI format 2_4 detection. For a group of symbols, NBI=NCIGCI bits from each set of bits have a one-to-one mapping with NBI groups of PRBs where each of the first groups includes PRBs and each of the remaining groups includes PRBs. A UE determines a first PRB index as NRFRstart=Ocarrier+RBstart and a number of contiguous RBs as NRFRsize=LRB from frequencyRegionforCI that indicates an offset and a length as RIV according to [6, TS 38.214], and from offsetToCarrier in FrequencyInfoUL-SIB that indicates for a SCS configuration of an active DL BWP where the UE monitors PDCCH for DCI format 2_4 detection. <---------------------------Other parts are omitted ------------------------------->

[5] Raised the issue of time region partitions caused by DL and SSB symbols and observed that a time partition (corresponding to one bit in the bitmap) may include symbols in different or even non-consecutive slots and having DL symbols/slots indicated by tdd-UL-DL-ConfigurationCommon in between. Thus the bitmap indication for such a time partition may cause unnecessary UL cancellations.

Figure 4 - UL CI time region splits into two regions

[5] proposed the following text proposal to address this issue.

Text Proposal for 38.213: GCI sets of bits from the NCI bits have a one-to-one mapping with GCI groups of symbols. A UE determines the number of symbols for each group as following: - Step 1: Each of the first GCI-TCI+TCIGCI⋅GCI groups includes TCIGCI symbols and each of the remaining TCI-TCIGCI⋅GCI groups includes TCIGCI symbols. - Step 2: When the ith group is across symbols for reception of SS/PBCH blocks and DL symbols indicated by tdd-UL-DL-ConfigurationCommon, it will be divided into two parts. When one part consists only one symbol, merge this symbol into the neighbor group if the neighbor group exists. A UE determines a symbol duration with respect to a SCS configuration of an active DL BWP where the UE monitors PDCCH for DCI format 2_4 detection.

[20] also proposed a method to reuse the “wasted” bits to indicate more frequency regions and provides the following text proposal

For a serving cell having an associated field in DCI format 2_4, for the field denote by - NCI a number of bits provided by CI-PayloadSize - BCI a number of PRBs provided by frequencyRegionforCI in timeFrequencyRegion - TCI a number of symbols, excluding symbols for reception of SS/PBCH blocks and DL symbols indicated by tdd-UL-DL-ConfigurationCommon, provided by timeDurationforCI in timeFrequencyRegion - GCI a number of partitions for the TCI symbols provided by timeGranularityforCI in timeFrequencyRegion If GCI≤TCI, - GCI sets of bits from the NCI bits have a one-to-one mapping with GCI groups of symbols where each of the first GCI-TCI+TCIGCI⋅GCI groups includes TCIGCI symbols and each of the remaining TCI-TCIGCI⋅GCI groups includes TCIGCI symbols. - For a group of symbols, NBI=NCIGCI bits from each set of bits have a one-to-one mapping with NBI groups of PRBs where each of the first NBI-BCI+BCINBI⋅NBI groups includes BCINBI PRBs and each of the remaining BCI-BCINBI⋅NBI groups includes BCINBI PRBs. If GCI>TCI, - TCI sets of bits from the NCI bits have a one-to-one mapping with the TCI symbols, where each of the first NCI-NCITCI⋅TBI set of bits includes NBI=NCITCI bits, and each of the remaining TCI-NCI+NCITCI⋅TBI sets include NBI=NCITCI bits - For a symbol, the corresponding set of bits has a one-to-one mapping with NBI RB groups (RBGs) of PRBs, where each of the first NBI-BCI+BCINBI⋅NBI groups includes BCINBI PRBs and each of the remaining BCI-BCINBI⋅NBI groups includes BCINBI PRBs. A UE determines a symbol duration with respect to a SCS configuration of an active DL BWP where the UE monitors PDCCH for DCI format 2_4 detection. A UE determines a first PRB index as NRFRstart=Ocarrier+RBstart and a number of contiguous RBs as NRFRsize=LRB from frequencyRegionforCI that indicates an offset RBstart and a length LRB as RIV according to [6, TS 38.214], and from offsetToCarrier that indicates Ocarrier for a SCS configuration of an active DL BWP where the UE monitors PDCCH for DCI format 2_4 detection.

- Issue 4: UE handling of intra-UE data-data collision and UL CI [2]

Discussion point#1

[2] Pointed out no only intra-UE handling of data-control collision, the intra-UE handling of data-data collision in UL should also not be affected by UL CI, according to the following agreement in last meeting.

Agreements:  UE behavior of handling intra-UE prioritization/multiplexing for overlapping UL transmissions is not affected by UL CI.

However, the current spec text in 38.213 does not cover the data-data collision case

Proposed text proposal for 38.213

---------------------------------Text proposal #2 starts for TS 38.213, Section 11.2A -------------------------------- =====omitted text ====== A UE that detects a DCI format 2_4 for a serving cell cancels a PUSCH transmission or a repetition of a PUSCH transmission [6, TS 38.214] if the PUSCH transmission is with repetitions, as determined in Clauses 9 and 9.2.5 or in Clause 6.1 in [6, TS 38.214], or an SRS transmission on the serving cell if, respectively, =====omitted text ====== ------------------------------- Text proposal #2 ends for TS 38.213, Section 11.2A -----------------------------------

Note there will be some parallel discussion in AI7.2.5.7 about the handling of CG-DG and CG-CG with same or different priority, which should be considered in this discussion. Therefore it is intended to discuss whether and how to clarify that data-data collision procedure is not affected by UL CI (limited to CG-DG collision case with Rel-15 timeline and potential Rel-16 timeline, if agreed in AI7.2.5.7)

Discussion point#2

It has been agreed in RAN1#98bis that for PUSCH repetition type B (i.e. Rel-16 PUSCH repetition), the UL is applied individually to each actual repetition.

Agreements:

- In case of PUSCH repetitions, UL CI is applied to each repetition individually (actual repetition in case of Rel-16 PUSCH repetition) that overlaps with the resource (in time and frequency) indicated by UL CI.

It was commented that the current spec is not clear enough about this aspect, it can be discussed whether any TP is needed.

- Issue 5 Independent configuration of RRC parameter “applicabilityforCI” for CG-PUSCH/DG-PUSCH/SRS [14]

[14] proposed to allow independent configuration of RRC parameter “applicabilityforCI” for CG-PUSCH/DG-PUSCH/SRS, with following proposals

Proposal 3: Cancellation based on priority level is configured for CG-PUSCH and DC-PUSCH separately.

Proposal 4: For SRS transmission, cancellation indication is always valid.

- Issue 6: Earlier cancellation relaxation for UL CI [20]

[20] As has been discussed for cancellation under intra-UE prioritization, enforcing an exact cancellation time incurs significant implementation complexity for the UE. Instead, if an uplink transmission needs to be cancelled, the UE should be allowed to start the cancellation even before the first indicated symbol.

Proposal: If a UE is indicated by DCI 2_4 to cancel a PUSCH transmission, the UE can cancel the transmission before the first indicated symbol group. In other words, the first indicated symbol using DCI format 2_4 for cancellation is the latest time that the UE should start the cancellation of PUSCH

- Issue 7: UL power control [7]

According to current spec, P0-PUSCH-Set is common to for different DCI formats while the OLPC configuration is DCI format specific, i.e. 1 bit OLPC for a DCI format with SRI and up to 2 OLPC bits for a DCI format without SRI. For DCI format not having SRI field but with 2bit OLPC indication, two values may be configured in P0-PUSCH-Set with lowest p0-PUSCH-SetID in P0-PUSCH-SetList. However, when a DCI format with SRI index (with 1bit OLPC) is used and indicates to the lowest p0-PUSCH-SetID in P0-PUSCH-SetList and the 1bit OLPC indicates '1', it is not clear which value in P0-PUSCH-Set with lowest p0-PUSCH-SetID is used in current specification. Hence, we have the following text proposal for TS 38.213.

The text proposals for open loop power control in TS 38.213 are shown as following including an editorial correction for RRC parameter name:

----------------------------------------- Start of text proposal for TS 38.213, Section 7.1.1 -------------------------------

<unchanged text omitted>

-	For , a  value, applicable for all , is provided by p0-NominalWithGrant, or  if p0-NominalWithGrant is not provided, for each carrier  of serving cell  and a set of values are provided by a set of p0 in P0-PUSCH-AlphaSet indicated by a respective set of p0-PUSCH-AlphaSetId for active UL BWP  of carrier  of serving cell

-	If the UE is provided by SRI-PUSCH-PowerControl more than one values of p0-PUSCH-AlphaSetId and if a DCI format scheduling the PUSCH transmission includes a SRI field, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the SRI field in the DCI format [5, TS 38.212] and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values and determines the value of  from the p0-PUSCH-AlphaSetId value that is mapped to the SRI field value. If the DCI format also includes a open-loop power control parameter set indication field and a value of the open-loop power control parameter set indication field is '1', the UE determines a value of  from a first value in P0-PUSCH-Set with a p0-PUSCH-SetId value mapped to the SRI field value.

-	If the PUSCH transmission is scheduled by a DCI format that does not include a SRI field, or if SRI-PUSCH-PowerControl is not provided to the UE, ,

-	If P0-PUSCH-Set is provided to the UE and the DCI format includes an open-loop power control parameter set indication field, the UE determines a value of  from

-	a first P0-PUSCH-AlphaSet in p0-AlphaSets if a value of the open-loop power control parameter set indication field is '0' or '00'

-     a first value in P0-PUSCH-Set with the lowest p0-PUSCH-SetId value if a value of the open-loop power control parameter set indication field is '1' or '01'

-     a second value in P0-PUSCH-Set with the lowest p0-PUSCH-SetId value if a value of the open-loop power control parameter set indication field is '10'

-	else, the UE determines  from the value of the first P0-PUSCH-AlphaSet in p0-AlphaSets

<unchanged text omitted>

----------------------------------------------------- End of text proposal ------------------------------------------------------

- Issue 8: On determination of RUR based on offsetToCarrier [9]

[9] proposed the following text proposal

Section 11.2A, TS 38.213 v16.1.0

**** Unchanged text omitted **** For a group of symbols, NBI=NCIGCI bits from each set of bits have a one-to-one mapping with NBI groups of PRBs where each of the first groups includes BCINBI PRBs and each of the remaining groups includes PRBs. A UE determines a first PRB index as NRFRstart=Ocarrier+RBstart and a number of contiguous RBs as NRFRsize=LRB from frequencyRegionforCI that indicates an offset RBstart and a length LRB as RIV according to [6, TS 38.214], and from offsetToCarrier in FrequencyInfoUL-SIB or FrequencyInfoUL that indicates Ocarrier for a SCS configuration of an active DL BWP where the UE monitors PDCCH for DCI format 2_4 detection. **** Unchanged text omitted ****

- Issue 9: UL cancellation for configured grant with repetitions when Configuredgrantconfig-StartingfromRV0 set to 'off’ [18]

[18] proposed to clarify the understanding of UL cancellation for configured grant with repetitions when Configuredgrantconfig-StartingfromRV0 set to 'off’, and proposed the following

Proposals: If a UE cancels an initial transmission of a TB in the first transmission occasion of the ‘K’ repetitions due to reception of an UL CI when Configuredgrantconfig-StartingfromRV0 set to 'off’, the UE does not transmit the TB in any of the transmission occasions of the ‘K’ repetitions.

- FL comment: there was following RAN1#98 agreement and the current spec is consistent with the agreement. No issue seems exist.

Agreements:

- In case of PUSCH repetitions, UL CI is applied to each repetition individually (actual repetition in case of Rel-16 PUSCH repetition) that overlaps with the resource (in time and frequency) indicated by UL CI.

- Issue 10: Correction on UL Cancelation indication [21]

[21]: d2,1 is not specified for Tproc,2 in the last sentence. Since the cancellation restriction is independent to delta_offset, there is no reason to assume d2,1=delta_offset⋅2-μUL2-μ used in T'proc,2 and thus it is straightforward to assume d2,1=0.

===========================The Start of Text Proposal of TS38.213============================ ## 11.2A Cancellation indication ===============================Unchanged part is omitted================================ An indication by a DCI format 2_4 for a serving cell is applicable to a PUSCH transmission or an SRS transmission on the serving cell. For the serving cell, the UE determines the first symbol of the TCI symbols to be the first symbol that is after T'proc,2 from the end of a PDCCH reception where the UE detects the DCI format 2_4, where T'proc,2 is obtained from Tproc,2 for PUSCH processing capability 2 [6, TS 38.214] assuming d2,1=delta_offset⋅2-μUL2-μ, μ being the smallest SCS configuration between the SCS configuration of the PDCCH and the smallest SCS configuration μUL provided in scs-SpecificCarrierList of FrequencyInfoUL or FrequencyInfoUL-SIB. The UE does not expect to cancel the PUSCH transmission or the SRS transmission before a corresponding symbol that is Tproc,2 assuming d2,1=0 after a last symbol of a CORESET where the UE detects the DCI format 2_4. ===============================Unchanged part is omitted================================ ===========================The END of Text Proposal of TS38.213============================

- Issue 11: Editor’s note in 38.213 section 11.2A

There is following editor’s note in 38.213 section 11.2A, there have been no concern raised in any contributions regarding this editor’s note, so suggest to confirm the relevant spec text and remove the editor’s note.

Previous agreements

- RAN1#96bis

Working assumption:

- PDCCH is used for UL cancelation indication

  - The Working assumption can be revisited if the DCI for cancelation indication only carry very small number of information bits, e.g. 1 bit.

Agreements:

- Upon detecting an UL cancelation indication, at least stop without resuming is supported

  - FFS whether and how to support stop with resume

Agreements:

- Further discuss which UL transmissions that can potentially be cancelled by the UL cancelation indication, including

  - Dynamic scheduled UL transmissions, including PUSCH, PUCCH, SRS

  - Semi-persistent UL transmissions, including PUSCH, PUCCH, SRS

  - Periodic UL transmissions, including configured grant PUSCH, PUCCH, SRS

  - PRACH

Agreements:

- Further discuss, aiming for down-selection, the group common DCI and UE-specific DCI for UL cancelation indication

  - For group common DCI (different from Rel-15 SFI)

    - UE is configured to monitor a group common DCI which indicates the time/frequency region on which the UL cancellation indication applies

  - For UE specific-DCI

    - When applicable, UE is configured to monitor a second UL grant for the same TB as an earlier PUSCH indicating UL cancellation before the end of the earlier PUSCH transmission. In this case, the UE follows the UL cancellation indication.

Conclusion:

- Further discuss the following power control enhancements

  - Increased TPC range

    - FFS details, e.g. supported value range, number of TPC bits, accumulated and/or absolute TPC, configurability of the TPC tables, applicability to SRS/PUCCH.

  - Indication of open-loop parameter sets based on scheduling DCI without using SRI

  - Indication of open-loop parameter sets based on GC-PDCCH

- RAN1#97

Agreements:

- Support at least group common DCI for cancelation indication

  - FFS whether or not to additionally support UE-specific DCI for cancelation indication

Conclusion:

To down-select from the following options for enhanced power control

- Option 1: Indication of open-loop parameter sets by DCI

- For DG-PUSCH, an open-loop parameter set indicated to the UE by scheduling DCI without using SRI is applied to the scheduled transmission

- FFS At least for single active CG-PUSCH, an open-loop parameter set is indicated to the UE by a UE-specific field in group common DCI

    - FFS for the case of multiple active CG-PUSCH

- FFS For a UE, the open-loop parameter sets for DG-PUSCH and CG-PUSCH may be same or different

- Option 2: Indication of TPC with increased range by DCI

- For DG-PUSCH, a TPC with increased range is indicated to the UE by the TPC field in scheduling DCI

- FFS At least for single active CG-PUSCH (and potentially also for DG-PUSCH), a TPC with increased range is indicated to the UE by a UE-specific TPC field in group common DCI

    - FFS for the case of multiple active CG-PUSCH

- At least for DG-PUSCH, for a UE, the number of TPC entries (4 or 8) and power adjustment value for each entry is higher layer configured

- FFS For a UE, the TPC configuration for DG-PUSCH and CG-PUSCH may be same or different

- Option 3:

- For DG-PUSCH, use either the solution from option 1 or option 2 for DG-PUSCH as above

    - To down-select from option 1 and 2

- FFS At least for single active CG-PUSCH, UE derives the transmissions power based on the time/frequency resource indicated by a group common DCI

    - If a CG-PUSCH transmission overlaps with the indicated time/frequency resource, UE use one open-loop parameter set with higher power for the transmission

    - If a CG-PUSCH transmission does NOT overlap with the indicated time/frequency resource, UE use another open-loop parameter set with lower power for the transmission

    - FFS for the case of multiple active CG-PUSCH

    - Note: some companies have concern that this was not captured in the TR as one potential solutions

- RAN1#98

Agreements:

- Reuse the existing methods for search space configuration to support UL CI monitoring

  - FFS possible restrictions

  - Note: this means both symbol level and slot level monitoring periodicities are possible from specification perspective

Agreements:

- The UE DCI size budget is not increased by UL CI monitoring

- Further discuss methods to reduce the UE monitoring for UL CI, e.g.

  - The number of aggregation levels and/or candidates for the UL CI monitoring should be limited

  - Conditions for eMBB UE UL CI monitoring:

    - For UL transmission with associated PDCCH,

      - Option 1: UE starts UL CI monitoring after the PDCCH is decoded

      - Option 2: UE monitors UL CI at least at the latest monitoring occasion ending no later than X symbols before the start of the UL transmission, and X is related to UL CI processing time.

    - For UL transmission without associated PDCCH, UE monitors UL CI at least at the latest monitoring occasion that ends no later than X symbols before the start of the UL transmission, and X is related to UL CI processing time.

    - Other conditions?

  - Others?

- FFS the enhancement of UE capability (number of non-overlapping CCE and/or blind decodes) for UL CI monitoring

Agreements:

- Upon detecting an UL cancelation indication, for the transmission of UL signal/channels, “stop with resuming” is not supported

  - Except:

    - SRS can still be transmitted on the non-cancelled symbols (conditioned on if SRS can be pre-empted)

    - FFS for the PUSCH repetition (Rel-15 & Rel-16) case

    - FFS for the PUCCH repetition case (conditioned on if PUCCH can be pre-empted)

  - FFS whether another PUSCH can be scheduled in non-pre-empted resource

  - FFS impact (e.g. phase continuity issue) to a different carrier due to UL cancelation

R1-1909774

Agreements:

- The following UL channel/signals can be cancelled by UL cancelation indication

  - PUSCH (including DG-, CG- and SP-)

  - FFS for SRS

  - FFS for PUCCH

    - Option 1: PUCCH (all types) can be cancelled

    - Option 2: Some PUCCH can be cancelled, e.g. PUCCH carrying CSI

    - Option 3: PUCCH cannot be cancelled

  - FFS for PRACH (preamble and/or MSG 3 PUSCH)

Agreements:

- The UE processing time requirement for UL cancelation indication based on N2 defined in Rel-15 UE cap#2 is supported

  - FFS whether the processing time requirement for UL cancelation indication larger than N2 as defined in Rel-15 UE cap#2 can also be supported as an UE capability

  - FFS whether the processing time requirement for UL cancelation indication shorter than N2 as defined in Rel-15 UE cap#2 as can also be supported an UE capability

Agreements:

- For a DG-PUSCH, an open-loop parameter set indicated to the UE by scheduling DCI using a separate field than SRI is supported.

  - FFS number of bits for the indication

- RAN1#98bis

Agreements:

- Regarding UL CI monitoring, support the following:

  - A new RNTI (e.g. CI-RNTI) is used for UL CI

  - FFS: Monitoring periodicity larger than [5] slot is not supported for UL CI

  - The aggregation level(s) and the number of PDCCH candidates configured by RRC

    - FFS possible restrictions, e.g., the ones associated with SFI

  - The DCI payload size for UL CI  is configured by RRC

    - FFS possible values

Agreements:

- SRS can be cancelled by UL CI

- PUCCH cannot be cancelled by UL CI

- RACH related UL transmissions cannot be cancelled by UL CI, including MSG 1/3 in case of 4-step RACH, MSG A in case of 2-step RACH.

Agreements:

- Cross-carrier UL cancelation indication is supported using the same way as Rel-15 SFI/DL PI

  - The indication field position in DCI for each cross-carrier indicated serving cell is configured by RRC

Agreements:

- Different UE processing time capability for UL CI (i.e. shorter or longer than T_proc2 for cap#2 UE) is not considered in Rel-16

  - d2,1=0 also when DMRS and UL-SCH (for the PUSCH to be cancelled) are multiplexed in the 1st symbol

Agreements:

- In case of PUSCH repetitions, UL CI is applied to each repetition individually (actual repetition in case of Rel-16 PUSCH repetition) that overlaps with the resource (in time and frequency) indicated by UL CI.

Agreements:

  - The reference time region where a detected UL CI is applicable is determined by the following:

      - The reference time region starts from X symbols after the ending symbol of the PDCCH CORESET carrying the UL CI, where X is at least equal to the minimum processing time for UL cancelation

        - FFS X can be configured to be larger than the minimum processing time for UL cancelation

      - The duration of the reference time region is configured by RRC

        - FFS Possible values (e.g. 2OS, 4OS, 7OS, 14OS, 28OS?)

      - FFS DL symbols are excluded from the reference time region

Agreements:

    - The reference frequency region where a detected UL CI is applicable is configured by RRC

Agreements:

Support the following for UL CI

  - Each UL cancelation indicator per serving cell has a RRC configurable field size of  X bits

      - One value of X is 14

      - FFS other values (e.g. X can be N (N>0) times of 7)

  - The time domain granularity for the reference time region is configured by RRC

      - FFS the possible values  (e.g. the time region can be divided into [1],[2],[4],[7],[14],…portions)

      - FFS valid configurations according to the duration of the time reference region

  - The frequency domain granularity is determined based on the configured time domain granularity and the configured bit field size of each indicator

  - The time and frequency resource for cancellation is jointly indicated by a 2D-bitmap (i.e. similar as DL PI) over the time and frequency partitions within the reference region

      - FFS dynamic 2D-bitmap

Agreements:

- For DG-PUSCH, one bit (separately from SRI) in UL grant is used to indicate the open loop power control parameter set

  - Introduce one new RRC parameter that contains one additional P0-PUSCH-Set per SRI

  - The one bit indication is present in the UL grant when the above new RRC parameter is configured

  - If present, the one bit in the DCI is used to switch between the P0 value from the existing P0-PUSCH-AlphaSet and the P0 value from the newly configured P0-PUSCH-Set

Conclusion:

No enhancement for CG-PUSCH power control in Rel-16 for inter-UE multiplexing

- RAN1#99

Agreements:

- There is no enhancement to PDCCH monitoring capability (number of BD and non-overlapping CCEs) specifically for UL CI monitoring purpose

Agreements:

- The maximum monitoring periodicity for UL CI is [5] slots

Agreements:

- Up to X BDs can be configured for UL CI

  - FFS per UL CI monitoring occasion or per span

  - The value of X is to be concluded during this week

  - Note: UE is not expected to be configured with search space configuration for UL CI with AL and number of candidates exceeding X BDs

Agreements:

- The maximum size for dci-PayloadSize-forCI is 126

Agreements:

- Possible values for RRC parameter timedurationforCI can be:

  - If the configured UL CI monitoring periodicity is >1 slot or 1-slot with only one monitoring occasion

    - At least the same as the configured UL CI monitoring periodicity

      - FFS whether or not to additionally support multiple of UL CI monitoring periodicity

  - Otherwise (i.e., >1 monitoring occasion within 1 slot when 1-slot is the configured UL CI monitoring periodicity)

    - {2, 4, 7, [14]} OS, which SCS is used when determine the time duration

      - SCS for the DL BWP carrying UL CI

    - FFS The UE is not expected to be configured with a time duration for CI less than the time different (in symbols) between any adjacent monitoring occasions in a slot

Agreements:

  - Possible values (16 values) for RRC parameter CI-PayloadSize are

    - {[1],2,4,[5],7,8,[10],14,16,[20],[25],28,32,[35],56,112}

  - timeGranularityforCI is defined as number of partitions within the time region, and possible values are

    - {1,2,4,7,14,28}

  - The configured value of CI-PayloadSize shall be a multiple integer of the configured value of timeGranularityforCI

Agreements:

  - The frequency region for UL CI is derived by the following

    - A RIV indication configured by RRC within value range of (0..37949) (i.e. the same way as IE “locationAndBandwidth” for BWP configuration ), the configuration is per serving cell specific

      - The reference point is derived based on the RRC parameter offsetToCarrier (existing parameter, same way as BWP configuration)

    - A reference SCS (no RRC configuration) for a serving cell (to handle the case where a UE is configured with multiple BWPs using different SCSs on the serving cell),

      - Use the SCS for the DL BWP carrying UL CI as the reference SCS

Agreements:

- Support per serving cell configuration for the following parameters

- CI-PayloadSize

- timedurationforCI

- timeGranularityforCI

- frequencyRegionforCI

Agreements:

- If a serving cell is configured with SUL, each UL carrier (SUL and non-SUL) can be configured with different positionInDCI.

Agreements:

- The DL symbols indicated by tdd-UL-DL-ConfigurationCommon are excluded from the reference time region for UL CI

  - The partition of reference time region is done after excluding the DL symbols

  - The symbols used for SSB are also excluded

Agreements:

- Clarification of 2D-bitmap

  - 2D-bitmap is to use X bits for bitmap indication over a time/frequency region with M partitions in time and N partitions in frequency, and X=M x N

Agreements:

Regarding “FFS whether or not to additionally support multiple of UL CI monitoring periodicity”

- If the configured UL CI monitoring periodicity is >1 slot or 1-slot with only one monitoring occasion, no additionally support that the time duration to be multiple of UL CI monitoring periodicity

Agreement

To determine the P0 value in case SRI is not configured in the DCI

- Option 1A: The open-loop power control parameter set indication field in the DCI can be configurable to be 1 or 2bits

  - P0-PUSCH-Set can provide up to two P0 values

    - UE uses the P0 values according to open loop power control indication field in DCI

    - UE use P0 from P0-PUSCH-AlphaSet when

      - open-loop power control parameter set indication field is 1bit and “0” is indicated, or

      - open-loop power control parameter set indication field is 2bits and “00” is indicated

  - Open-loop power control parameter set indication field can be separately configurable for DCI format 0_1 and DCI format 0_2

    - If open-loop power control parameter set indication field is not present for a DCI format, use P0 from P0-PUSCH-AlphaSet

  - A single configuration of P0-PUSCH-Set applies to both DCI format 0_1 and DCI format 0_2

- RAN1#100-e

Agreements:

- Confirm that 14OS can be configured for timedurationforCI (when 1-slot is the configured UL CI monitoring periodicity with more than one monitoring occasions within 1 slot)

- The possible values for CI-PayloadSize, are {1,2,4,5,7,8,10,14,16,20, 28,32,35,42,56,112}

The following TP is endorsed

------------------------------------ Start of TP for 38.213 --------------------------------------------

11.2A	Cancellation indication

< Unchanged parts are omitted >

For a group of symbols,  bits from each set of bits have a one-to-one mapping with  groups of PRBs where each of the first  groups includes  PRBs and each of the remaining  groups includes  PRBs. A UE determines a first PRB index as  and a number of contiguous RBs as  from frequencyRegionforCI that indicates an offset  and a length  as RIV according to [6, TS 38.214], and from offsetToCarrier in FrequencyInfoUL-SIB that indicates  for a SCS configuration of an active DL BWP where the UE monitors PDCCH for DCI format 2_4 detection.

< Unchanged parts are omitted >

------------------------------------ End of TP for 38.213 --------------------------------------------

Agreements:

- The maximum UL CI monitoring periodicity is 10 slots.

- Up to X BDs can be configured per UL CI monitoring occasion, X to be decided between X=1 or X=2 in RAN1#100bis.

No RAN1 spec impact - RRC parameter update only.

Agreements:

- UE derives the RUR start based on “logical time” (i.e. assuming DL timing difference is 0 and TA=0) and the actual cancellation symbol based on “actual time” (i.e. assuming actual DL timing difference, actual TA)

- A new RRC parameter delta_offset d having possible values {0, 1, 2} OFDM symbols is introduced, update the spec as the following

For the serving cell, the UE determines the first symbol of the symbols to be the first symbol that is after + d from the end of a PDCCH reception where the UE detects the DCI format 2_4.corresponds to the PUSCH processing capability 2 [6, TS 38.214] assuming with being the smallest SCS configuration between the SCS configurations of the PDCCH and of a PUSCH transmission or of an SRS transmission on the serving cell.

- Clarify the following by a RAN1 spec update (see below)

  - UE is not expected to cancel the transmission of SRS or PUSCH before the first symbol that is T_proc,2 after the end of the reception of the last symbol of the PDCCH carrying the ULCI including the effect of the timing advance.

38.213 Text proposal (maybe further refined by spec editor)

An indication by a DCI format 2_4 for a serving cell is applicable to PUSCH or SRS transmissions on the serving cell. For the serving cell, the UE determines the first symbol of the TCI symbols to be the first symbol that is after Tproc,2Tproc,2+d from the end of a PDCCH reception where the UE detects the DCI format 2_4, where d is provided by higher layer parameter [xxxx]. Tproc,2 corresponds to the PUSCH processing capability 2 [6, TS 38.214] assuming d2,1=0 with μ being the smallest SCS configuration between the SCS configurations of the PDCCH and of a PUSCH transmission or of an SRS transmission on the serving cell. UE is not expected to cancel the transmission of SRS or PUSCH before the first symbol that is Tproc,2 after the end of the reception of the last symbol of the PDCCH carrying the ULCI including the effect of the timing advance.

Agreements:

- UE performs the UL cancellation based on any detected UL CI, no additional specification for the case of overlapping reference time region for multiple UL CI occasions.

- A cancelled PUSCH transmission by a UE is counted towards the number of PUSCH that a UE can support per slot

Conclusion:

- It is possible for a UE to indicate both  pa-PhaseDiscontinuityImpacts  (i.e. 6-23) and the support of UL CI for intra-band UL CA

- For a UE indicates a capability to cancel overlapping PUSCHs on different intra-band serving cells (if any), and the capability of pa-PhaseDiscontinuityImpacts, and if the PUSCH on at least one serving cell is cancelled, the UE cancels the (repetition of the) PUSCHs transmission on all other intra-band serving cell(s). The cancellation of the (repetition of the) PUSCH transmission on a the set of intra-band serving cell(s) includes all symbols from the earliest symbol that is overlapping with the first cancelled symbol of the PUSCH on the serving cell for which the DCI format 2_4 is applicable to.

- RAN1#100bis-e

Agreements:

  UE behavior of handling intra-UE prioritization/multiplexing for overlapping UL transmissions is not affected by UL CI.

Agreeement:

- If both UL CI and intra-UE priority indicator are configured for a given UE, support a new RRC parameter to configure Behavior #1

  - Behaviour #1: UL CI is only applicable to the UL transmissions indicated/configured as low priority level

- When the RRC parameter is not provided to the UE, behaviour #2 is used

  - Behaviour #2: UL CI is applicable to UL transmission irrespective of its priority level

- Note: the RRC signaling details will be decided by RAN2

Agreement:

- Up to X BDs can be configured per UL CI monitoring occasion

  - For ULCI monitoring occasion determination, search space sets start at a same OFDM symbol correspond to a same monitoring occasion

  - X=1

Agreement:

- UE uses the smallest SCS configuration between the SCS configurations of the PDCCH for DCI format 2_4 detection and the SCS configurations in scs-SpecificCarrierList of UL carrier to determine the RUR starting symbol.

- UE uses the smallest SCS configurations in scs-SpecificCarrierList of UL carrier to determine offset d.

- Adopt the TP below for 38.213 section 11.2A

TP for 38.213 section 11.2A

11.2A Cancellation indication ---------------------------Other parts are omitted ------------------------------- An indication by a DCI format 2_4 for a serving cell is applicable to a PUSCH transmission or a SRS transmission on the serving cell. For the serving cell, the UE determines the first symbol of the symbols to be the first symbol that is after from the end of a PDCCH reception where the UE detects the DCI format 2_4, where is provided by XXX with the smallest SCS configuration between the SCS configurations of the PDCCH and the SCS configurations provided in scs-SpecificCarrierList of UL carrier. corresponds to the PUSCH processing capability 2 [6, TS 38.214] assuming with being the smallest SCS configuration between the SCS configurations of the PDCCH and the SCS configurations provided in scs-SpecificCarrierList of UL carrier of a PUSCH transmission or of an SRS transmission on the serving cell. The UE does not expect to cancel the PUSCH transmission or the SRS transmission before a corresponding symbol that is after a last symbol of a CORESET where the UE detects the DCI format 2_4. <---------------------------Other parts are omitted ------------------------------->

Agreements:

To adopt the following TP for 38.213

- 11.2A Cancellation indication =====omitted text ====== For a serving cell having an associated field in DCI format 2_4, for the field denote by - NCI a number of bits provided by CI-PayloadSize - BCI a number of PRBs provided by frequencyRegionforCI in timeFrequencyRegion - TCI a number of symbols, excluding symbols for reception of SS/PBCH blocks and DL symbols indicated bytdd-UL-DL-ConfigurationCommon, from the time duration provided by timeDurationforCI in timeFrequencyRegion if the configured UL CI monitoring periodicity is 1 slot with more than one monitoring occasions. Otherwise, the time duration is equal to the PDCCH monitoring periodicity provided by the value of monitoringSlotPeriodicityAndOffset, as described in Clause 10.1. - GCI a number of partitions for the TCI symbols provided by timeGranularityforCI in timeFrequencyRegion =====omitted text ======

Agreement: Adopt the following text proposal for TS38.213 section 11.2A

----------------------------- Text proposal starts for TS 38.213, v16.1.0, Section 11.2A ----------- A UE that detects a DCI format 2_4 for a serving cell cancels a PUSCH transmission, or a repetition of a PUSCH transmission [6, TS 38.214] if the PUSCH transmission is with repetitions, or an SRS transmission on the serving cell if, respectively, - a group of symbols, from the TCI symbols, has a corresponding at least one bit value of '1' in the corresponding set of NBI bits in the DCI format 2_4 and includes a symbol of the (repetition of the) PUSCH transmission or of the SRS transmission, and - a group of PRBs, from the BCI PRBs, has a corresponding bit value of '1' in the set of bits corresponding to the group of symbols in the DCI format 2_4 and includes a PRB of the (repetition of the) PUSCH transmission or of the SRS transmission, where - the cancellation of the (repetition of the) PUSCH transmission includes all symbols from the earliest symbol of the (repetition of the) PUSCH transmission that are is in one or more a groups of symbols having corresponding bit values of '1' in the DCI format 2_4; - the cancellation of the SRS transmission includes only symbols that are in one or more groups of symbols having corresponding bit values of '1' in the DCI format 2_4. ----------------------------- Text proposal ends for TS 38.213, v16.1.0, Section 11.2A -------------

Agreement

•          When UE is configured with both DCI format 0_1 and 0_2 with SRI presents in only one of the DCI formats, then for the DCI format without SRI field

  For 1 bit OLPC parameter indication, if OLPC parameter set indication in DCI is set to ‘1’

  P0-PUSCH-Set having the lowest p0-PUSCH-SetId is used.

  For 2 bit OLPC parameter indication, if OLPC parameter set indication in DCI is set to ‘01’ or ‘10’

  P0-PUSCH-Set having the lowest p0-PUSCH-SetId is used.

Agreement: Adopt the following text proposal for TS38.213 section 7.1.1

TP for 38.213 16.1.0 Section 7.1.1 <Unchanged text is omitted> - If the PUSCH transmission is scheduled by a DCI format that does not include a SRI field, or if SRI-PUSCHPowerControl is not provided to the UE, , - If P0-PUSCH-Set is provided to the UE and the DCI format includes an open-loop power control parameter set indication field, the UE determines a value of from - a first P0-PUSCH-AlphaSet in p0-AlphaSets if a value of the open-loop power control parameter set indication field is '0' or '00' - a first value in P0-PUSCH-Set with the lowest p0-PUSCH-SetId value if a value of the open-loop power control parameter set indication field is '1' or '01' - a second value in P0-PUSCH-Set with the lowest p0-PUSCH-SetId value if a value of the open-loop power control parameter set indication field is '10' - else, the UE determines from the value of the first P0-PUSCH-AlphaSet in p0-AlphaSets <Unchanged text is omitted>

Agreement: Adopt the following text proposal for 38.213 section 11.2A

- 11.2A Cancellation indication If a UE is provided UplinkCancellation, the UE is provided a CI-RNTI by ci-RNTI for monitoring PDCCH candidates for a DCI format 2_4 [5, TS 38.212]. UplinkCancellation additionally provides to the UE - a set of serving cells, by ci-ConfigurationPerServingCell, that includes a set of serving cell indexes and a corresponding set of locations for fields in DCI format 2_4 by positionInDCI - a number of fields in DCI format 2_4, by positionInDCI-forSUL, for each serving cell for a SUL carrier for a SUL carrier, if the serving cell is configured with a SUL carrier for SUL of a serving cell if the serving cell configured with SUL - an information payload size for DCI format 2_4 by dci-PayloadSize-forCI - an indication for time-frequency resources by timeFrequencyRegion For a serving cell having an associated field in DCI format 2_4, for the field denote by - NCI a number of bits provided by CI-PayloadSize - BCI a number of PRBs provided by frequencyRegionforCI in timeFrequencyRegion - TCI a number of symbols, excluding symbols for reception of SS/PBCH blocks and DL symbols indicated by tdd-UL-DL-ConfigurationCommon,provided by timeDurationforCI in timeFrequencyRegion - GCI a number of partitions for the TCI symbols provided by timeGranularityforCI in timeFrequencyRegion GCI sets of bits from the MSB of the NCI bits have a one-to-one mapping with GCI groups of symbols where each of the first GCI-TCI+TCIGCI⋅GCI groups includes TCIGCI symbols and each of the remaining TCI-TCIGCI⋅GCI groups includes TCIGCI symbols. A UE determines a symbol duration with respect to a SCS configuration of an active DL BWP where the UE monitors PDCCH for DCI format 2_4 detection. For a group of symbols, NBI=NCIGCI bits from MSB of each set of bits have a one-to-one mapping with NBI groups of PRBs where each of the first NBI-BCI+BCINBI⋅NBI groups includes BCINBI PRBs and each of the remaining BCI-BCINBI⋅NBI groups includes BCINBI PRBs. A UE determines a first PRB index as NRFRstart=Ocarrier+RBstart and a number of contiguous RBs as NRFRsize=LRB BCI=LRB from frequencyRegionforCI that indicates an offset RBstart and a length LRB as RIV according to [6, TS 38.214], and from offsetToCarrier in FrequencyInfoUL-SIB that indicates Ocarrier for a SCS configuration of an active DL BWP where the UE monitors PDCCH for DCI format 2_4 detection. An indication by a DCI format 2_4 for a serving cell is applicable to a PUSCH transmission or a SRS transmission on the serving cell. For the serving cell, the UE determines the first symbol of the TCI symbols to be the first symbol that is after Tproc,2+d from the end of a PDCCH reception where the UE detects the DCI format 2_4, where d is provided by XXX higher layer parameter delta_offset_d. Tproc,2 corresponds to the PUSCH processing capability 2 [6, TS 38.214] assuming d2,1=0 with μ being the smallest SCS configuration between the SCS configurations of the PDCCH and of a PUSCH transmission or of an SRS transmission on the serving cell. The UE does not expect to cancel the PUSCH transmission or the SRS transmission before a corresponding symbol that is Tproc,2 after a last symbol of a CORESET where the UE detects the DCI format 2_4. A UE that detects a DCI format 2_4 for a serving cell cancels a PUSCH transmission, or a repetition of a PUSCH transmission [6, TS 38.214] if the PUSCH transmission is with repetitions, or an SRS transmission on the serving cell if, respectively, - a group of symbols, from the TCI symbols, has a corresponding bit value of ‘1’ in the DCI format 2_4 and includes a symbol of the (repetition of the) PUSCH transmission or of the SRS transmission, and - a group of PRBs, from the BCI PRBs, has a corresponding bit value of ‘1’ in the DCI format 2_4 and includes a PRB of the (repetition of the) PUSCH transmission or of the SRS transmission, where - the cancellation of the (repetition of the) PUSCH transmission includes all symbols from the earliest symbol of the (repetition of the) PUSCH transmission that are in one or more groups of symbols having corresponding bit values of ‘1’ in the DCI format 2_4; - the cancellation of the SRS transmission includes only symbols that are in one or more groups of symbols having corresponding bit values of ‘1’ in the DCI format 2_4.

Agreement: Adopt the following text proposal for 38.212 section 7.3.1.3.5

- 7.3.1.3.5 Format 2_4 DCI format 2_4 is used for notifying the PRB(s) and OFDM symbol(s) where UE cancels the corresponding UL transmission from the UE according to Clause 11.2A5 of [5, TS 38.213]. The following information is transmitted by means of the DCI format 2_4 with CRC scrambled by CI-RNTI: - Cancellation indication 1, Cancellation indication 2, …, Cancellation indication indication N. The size of DCI format 2_4 is configurable by higher layers parameter dci-PayloadSize-forCI up to 126 bits, according to Clause 11.2A5 of [5, TS 38.213]. The number of bits for each cancellation indication is configurable by higher layer parameter CI-PayloadSize. For a UE, there is at most one cancellation indication for an UL carrier. <Unchanged text is omitted>

- TR 38.824

- 7.2 Potential enhancements In the following sub-sections, potential enhancements for UL inter UE Tx prioritization/multiplexing are presented. It is recommended to specify both UL cancelation scheme and enhanced UL power control scheme in the work item phase. - 7.2.1 UE UL cancelation mechanisms UE UL cancelation mechanisms are considered as one potential enhancement for UL inter-UE Tx prioritization/multiplexing and are studied from several aspects, including the potential mechanisms (e.g. UE UL cancelation/pausing indication, UL continuation indication, UL re-scheduling indication), physical channel/signal used for the UL cancelation indication, UE processing timeline for the UL cancelation indication, UE monitoring behaviours for the UL cancelation indication, UE PDCCH monitoring capability if the UL cancelation indication is by PDCCH, methods to ensure the reliability of the indication for UE UL cancelation. Either PDCCH or sequence can be considered as potential options for the UL cancelation indication. If PDCCH is used, either group common DCI or UE-specific DCI can be considered as potential options. If sequence is used, either group common sequence or UE-specific sequence can be considered. The monitoring periodicity for the UL cancelation indication should be configurable by the gNB and UE supporting UL cancelation indication should be able to support more than one monitoring occasions for the UL cancelation indication in a slot. If PDCCH is used, whether the UE PDCCH monitoring capability (number of CCEs/BDs per slot) should be increased is to be further investigated. The UE processing time for UL cancelation indication should be equal or shorter than N2 defined in Rel-15 UE capability#2. Upon detecting an UL cancelation indication, UE cancels the corresponding UL transmission. The corresponding UL transmission may include an on-going UL transmission, or an UL transmission that has not been started. After cancelation, the UE may resume the transmission afterwards as one option, or may not resume the transmission afterwards as another option. - 7.2.2 Enhanced UL power control Enhanced UL power control is considered as one potential enhancement for UL inter-UE Tx prioritization/multiplexing and the study mainly focuses on enhanced dynamic power boost for URLLC UE, including dynamic change of power control parameters (e.g. P0 and alpha without SRI configured) and enhanced TPC (e.g. increased TPC range and finer granularity). The need of URLLC UE power change during one transmission instance is not envisioned. It is assumed that there is no change of eMBB UE power control scheme in this study item. Enhanced dynamic power boost for URLLC UE are studied from several aspects, including feasibility of boosting UE power in power limited or interference limited scenarios, physical channel/signal used for the signalling, UE processing timeline for the signalling, UE monitoring behaviours for the signalling, UE PDCCH monitoring capability if the signalling is by PDCCH and methods to ensure the reliability of the signalling. It is concluded that the potential enhanced UL power control may include UE determining the power control parameter set (e.g. P0, alpha) based on scheduling DCI indication without using SRI, or based on group-common DCI indication. Increased TPC range compared to Rel-15 may also be considered. Power boosting is not applicable to power limited UEs.

List of contributions and proposals

| 1 | R1-2003321 | Remaining issues on UL inter-UE multiplexing between eMBB and URLLC | ZTE |
| --- | --- | --- | --- |
| 2 | R1-2003391 | UL inter UE Tx prioritization for URLLC | vivo |
| 3 | R1-2003394 | Summary of UL inter UE Tx prioritization | Moderator (vivo) |
| 4 | R1-2003443 | Remaining Issue of Inter-UE Prioritization and Multiplexing of UL Transmissions | Ericsson |
| 5 | R1-2003527 | Corrections on UL inter-UE multiplexing | Huawei, HiSilicon |
| 6 | R1-2003581 | Maintenance of Rel-16 URLLC Enhanced inter UE Tx prioritization/multiplexing | Nokia, Nokia Shanghai Bell |
| 7 | R1-2003623 | Remaining issues on inter-UE UL multiplexing | CATT |
| 8 | R1-2003686 | Remaining issues on enhanced inter-UE Tx prioritization/multiplexing | MediaTek Inc. |
| 9 | R1-2003709 | Remaining Issues on Enhanced Inter-UE Tx Prioritisaion / Multiplexing | NEC |
| 10 | R1-2003740 | Remaining issues on enhanced inter-UE multiplexing | Intel Corporation |
| 11 | R1-2003868 | Remaining issues for inter-UE multiplexing | Samsung |
| 12 | R1-2003981 | Remaining issues of enhanced inter UE Tx prioritization/multiplexing | Spreadtrum Communications |
| 13 | R1-2004032 | Remaining issues of UL inter UE Tx prioritization | LG Electronics |
| 14 | R1-2004118 | Inter UE Tx prioritization and multiplexing | OPPO |
| 15 | R1-2004185 | Remaining issues on Inter-UE Multiplexing for eURLLC | Sony |
| 16 | R1-2004225 | Remaining Issues on Inter-UE Cancellation for eURLLC | Apple |
| 17 | R1-2004272 | Inter-UE prioritization/multiplexing | InterDigital, Inc. |
| 18 | R1-2004371 | Remaining issues of enhanced inter UE Tx prioritization/multiplexing | Motorola Mobility, Lenovo |
| 19 | R1-2004393 | Remaining issue on inter-UE Tx multiplexing/prioritization | NTT DOCOMO, INC |
| 20 | R1-2004460 | Remaining issues on uplink Inter-UE Tx Multiplexing and Prioritization | Qualcomm Incorporated |
| 21 | R1-2004525 | Remaining issues on inter-UE multiplexing for NR URLLC | WILUS Inc. |
| 22 | R1-2004390 | Remaining issues for UCI enhancement for Rel-16 URLLC | NTT DOCOMO, INC |