3GPP TSG RAN WG1 #101-e			R1-2004801

e-Meeting, May 25th – June 5th, 2020

Agenda item:	6.2.1.2 Support for transmission in preconfigured UL resources

Source: 	Moderator (Sierra Wireless)

Title: 	Feature lead summary [101-e-LTE-eMTC5-PUR-02]

Document for:	Discussion

# Introduction

This contribution includes the summary for email discussion:

[101-e-LTE-eMTC5-PUR-02] PUR power control issues by 5/29

- Issue #2 in R1-2004691: PUCCH power control

- Issue #3 in R1-2004691: Power control Accumulation mechanism

# Issue #1 PUCCH Power Control Method

## Issue Description

There are terms in the “UE Transmit power” equation defined in TS 36.213 which are unclear when the UE is configured with PUR and whether PUCCH should use open loop or closed loop power control is an open issue.

## Agreement Alternatives

Companies should consider the following agreement alternatives:

Alt 1:  PUCCH uses open loop power control for CE mode A

Alt 2:  PUCCH uses closed loop power control for CE mode A

Ericsson: Alt 1, When L2/L3-signaling is received in response to a PUR transmission, PUCCH can be used as an acknowledgement for the eNodeB to know that the right entity has been reached, since PUSCH for PUR transmissions uses open-loop power control we believe that similarly PUCCH transmissions should also use open-loop power control.

## Text Proposals

### TP #1 for ALT1

Ericsson [4] provided the following TPs for TS 36.213 and TS 36.212:

--------------------------------------------------- 36.213 ---------------------------------------------------------

--------------------------------------------------- Text start ---------------------------------------------------------

5.1.2.1	UE behaviour

If serving cell is the primary cell, for PUCCH format 1/1a/1b/2/2a/2b/3, the setting of the UE Transmit powerfor the physical uplink control channel (PUCCH) transmission in subframe/slot/subslot i for serving cell  is defined by

------------------------------------------------- Text omitted -------------------------------------------------------

-	If  value is preconfigured or changed by higher layers,

-

Note: g(i) = 0 for i = 0,1,… up to the last subframe of the PUCCH transmission using preconfigured uplink resource.

----------------------------------------------- Text end ------------------------------------------------------------

--------------------------------------------------- 36.212 ---------------------------------------------------------

-	TPC command for PUCCH – 2 bits as defined in clause 5.1.2.1 of [3] . This field is not present when the format 6-1A CRC is scrambled with G-RNTI or PUR C-RNTI.

-	If the format 6-1A CRC is scrambled by RA-RNTI:

-	The most significant bit of the TPC command is reserved.

-	The least significant bit of the TPC command indicates column of the TBS table defined of [3].

-	If least significant bit is 0 then = 2 else = 3.

-	Else

-	The two bits including the most significant bit indicate the TPC command

----------------------------------------------- Text end ------------------------------------------------------------

### TP #2

CR Cover Sheet Information:

Reasons for change:  PUCCH power control is not defined when USS is given by PUR C-RNTI.

Summary of changes:   When USS is given by PUR C-RNTI,  PUCCH power control follows DCI and resets every  transmission on pre-configured resources.

Specs/Sections impacted: TS 36.213 section 5.1.2.1

Consequences if not approved:   Undefined PUCCH power control when USS is given by PUR C-RNTI.

--------------------------------------------------- Text start ---------------------------------------------------------

5.1.2.1	UE behaviour

< Unchanged parts are omitted>

-	For a BL/CE UE configured with CEModeA, the UE attempts to decode a MPDCCH of DCI format 3/3A with the UE's TPC-PUCCH-RNTI and MPDCCH of DCI format 6-1A with the UE's C-RNTI or SPS C-RNTI or PUR C-RNTI on every BL/CE downlink subframe except when in DRX.

-	If the UE decodes

-	a PDCCH with DCI format 1A/1B/1D/1/2A/2/2B/2C/2D/7-1A/1B/1C/1D/1E/1F/1G or

-	an EPDCCH with DCI format 1A/1B/1D/1/2A/2/2B/2C/2D or

-	an MPDCCH with DCI format 6-1A or

-	a SPDCCH with DCI format 7-1A/1B/1C/1D/1E/1F/1G

for the primary cell and the corresponding detected RNTI equals the C-RNTI or SPS C-RNTI or PUR C-RNTI of the UE and the TPC field in the DCI format is not used to determine the PUCCH resource as in Subclause 10.1, the UE shall use the  provided in that PDCCH/EPDCCH/MPDCCH/SPDCCH.

< Unchanged parts are omitted>

-	If  value is changed by higher layers, or if a BL/CE UE transmits using preconfigured uplink resources

-

< Unchanged parts are omitted>

-	UE shall reset accumulation

-	when  value is changed by higher layers

-	when the UE receives a random access response message for the primary cell

-	when the UE performs PUSCH transmission using preconfigured uplink resource

-	 if  is not an uplink subframe/slot in TDD or FDD-TDD and primary cell frame structure type 2.

< Unchanged parts are omitted>

----------------------------------------------- Text end ------------------------------------------------------------

## Company Views

Please indicate which alternative your company can agree too and comments:

| Company | Comments on Proposal |
| --- | --- |
| Ericsson | PUSCH for PUR transmissions uses open-loop power control, we believe that similarly PUCCH transmissions should also use open-loop power control, which is reflected by TP #1 for ALT1 in section 2.3 of this FLS. |
| Qualcomm | Maybe we are missing something, but is the statement “PUSCH for PUR transmissions uses open-loop power control” true? As far as we understand, during the PUR procedure the eNB can issue close loop power control commands for the PUSCH retransmissions. |
| Ericsson v003 | In our first comment we were strictly referring to the PUR transmission case. The PUR transmissions use open-loop power control, whereas the PUR re-transmissions in CE Mode A use closed-loop power control (for the PUR retransmissions in CE Mode B the UE will transmit at Max Power). |
| Sony | Is there supposed to be a TP#2? |
| ZTE,Sanechips | In our view PUCCH should be able to support close loop PC. Another two questions we have for TP#1 is : - Not sure we can say " value is preconfigured" - Not sure we can say "PUCCH transmission using preconfigured uplink resource" |
| Ericsson v006 | To ZTE, on “PUCCH should be able to support close loop PC” do you mean in the case L2/L3-signaling be used to acknowledge a retransmission? We thought about open-loop PUCCH when L2/L3-signaling is used to respond to a PUR transmission and closed-loop PUCCH when L2/L3-signaling is used to respond to a PUR retransmission, but given the following equation if our understanding is correct (see e.g., km), then to see a closed-loop several retransmission will be needed, and on top of that each of them would have to be assessed with a L2/L3-signaling as to keep transmitting PUCCH. That is why, we thought it would be reasonable to an open-loop for PUCCH for both transmission and retransmission case for PUR. On 1) We thought that it will work is similar to PUSCH where “ provided by higher layers” which can be re-configured if needed via RRC signalling. On 2) You are referring to the “Note”, we tried to re-use the wording agreed last time and the intention was for people to recognize it, but I agree that it has to be written slightly different for PUCCH. Perhaps: Note: g(i) = 0 for i = 0,1,… up to the last subframe of the PUCCH transmitted in response to a higher layer message used to respond a transmission using preconfigure uplink resource. |
| Qualcomm | I am a bit confused. Let’s say the UE gets a PDSCH carrying a DL message (it can be an RRC connection, a MAC CE with TA, etc.). That PDSCH is scheduled by a DL DCI, which has a TPC command for PUCCH. The UE would just follow that TPC command for the PUCCH. I think all we need to do is, similar to the reset of accumulation for PUSCH, state that g(i) is reset every time the UE restarts a PUR procedure. As a side note, we think it is a bad idea to say “in response to a higher layer message”, since when the UE is sending the PUCCH it doesn’t really know the content of the PDSCH (it just knows that CRC passed). |
| Ericsson v008 | On “That PDSCH is scheduled by a DL DCI, which has a TPC command for PUCCH. The UE would just follow that TPC command for the PUCCH.” But that would be under the assumption that the PUR PUCCH transmission power is related to the PUR PUSCH transmission power. On this matter, PUR PUSCH transmissions use open-loop power control. About the wording “in response to a higher layer message”, what we wanted to say is “in response to L2/L3-signaling” which is not easy to capture because as you said “it can be an RRC connection, a MAC CE with TA, etc.”. Perhaps the note can be better written as follows: Note: g(i) = 0 for i = 0,1,… up to the last subframe of the PUCCH transmitted in response to a PDSCH scheduled by DCI Format 6-1A, used to respond a transmission using preconfigure uplink resource. |
| Qualcomm | Sorry, I still don’t understand. On “the PUR PUCCH transmission power is related to the PUR PUSCH transmission power” – I think it is, no? Imagine you set alpha=1. In the first transmission, if the eNB detects that the power for PUSCH is low (e.g. due to fading in uplink or some bias in the pathloss calculation) it can issue power control commands for PUSCH, and same for PUCCH. The difference between the transmit power for PUCCH and PUSCH (except in the power limited case) is known by the eNB. I tried to take a more detailed look at what needs to be changed to enable this behavior – added in TP2 above. |
| Ericsson v010 | There is also a part that I do not understand, when you said “eNB … can issue power control commands for PUSCH”. That can not be done for transmissions using preconfigured uplink resources. The UL-Grant for what you used to call “initial transmission” is preconfigured, if such a transmission was successfully received by the eNodeB and this one responds e.g., with a L1-ACK its DCI doesn’t have a TPC command field. Moreover, the “initial transmissions” are separated in time by length of the PUR periodicity, that is why PUR transmissions use open-loop power control. |
| Qualcomm | Right, I meant for the retransmissions. In your example, the L1-ACK in DCI doesn’t have a corresponding PUCCH, so this should not be relevant for this discussion. If the eNB instead sends a PDSCH, then the DCI scheduling it will have a TPC command for PUCCH. |
| Ericsson v012 | Correct, the DCI used to schedule PDSCH has a TPC command for PUCCH. I guess the core of the discussion here is that in legacy both PUSCH (where the UL-Grant is dynamically provided via DCI) and PUCCH transmissions make use of closed-loop power control. Now for PUR, PUSCH transmissions are preconfigured and do not use closed-loop power control, which is different from the legacy case, now the question is whether PUCCH should still use a closed-loop power control scheme referenced to PUSCH that is no longer closed-loop power controlled, or if it also should use an open-loop power control scheme. It seems that when PUSCH (as in legacy) uses a closed-loop power controlled, giving TPC commands for PUCCH with respect to PUSCH is smoother. But if PUSCH uses an open-loop power control, providing TPC commands for PUCCH based on it probably won’t provide the same performance. In the case of PUR, it seems that both PUSCH and PUCCH will rely more on the UE-specific component of P0. |

# Issue #2 PUCCH Power Control missed PUR C-RNTI

## Issue Description

- PUCCH power control clause 5.1.2.1  missed to include the PUR C-RNTI for DCI format 6-1A.

## Text proposals

### TP #1

Ericsson [4] provided the following TPs for TS 36.213:

--------------------------------------------------- 36.213 ---------------------------------------------------------

--------------------------------------------------- Text start ---------------------------------------------------------

5.1.2.1	UE behaviour

If serving cell is the primary cell, for PUCCH format 1/1a/1b/2/2a/2b/3, the setting of the UE Transmit powerfor the physical uplink control channel (PUCCH) transmission in subframe/slot/subslot i for serving cell  is defined by

-------------------------------------------------- Text omitted -----------------------------------------------------

-	For a BL/CE UE configured with CEModeA, the UE attempts to decode a MPDCCH of DCI format 3/3A with the UE's TPC-PUCCH-RNTI and MPDCCH of DCI format 6-1A with the UE's C-RNTI or SPS C-RNTI or PUR C-RNTI on every BL/CE downlink subframe except when in DRX.-

--------------------------------------------------- Text End ---------------------------------------------------------

### TP #2

CR Cover Sheet Information:

Reasons for change:  Missing the case where UE should attempt to decode DCI format 6-0A when USS is given by PUR C-RNTI.

Summary of changes:   Added the case where UE should attempt to decode DCI format 6-0A when USS is given by PUR C-RNTI.

Specs/Sections impacted: TS 36.213 section 5.1.1.1

Consequences if not approved:   UE may not decode DCI format 6-0A when USS is given by PUR C-RNTI.

#### 5.1.1.1	UE behaviour

-------------------------------------------------- Text omitted -----------------------------------------------------

- For serving cell  and a BL/CE UE configured with CEModeA, the UE attempts to decode a MPDCCH of DCI format 6-0A with the UE's C-RNTI or SPS C-RNTI or PUR C-RNTI and a MPDCCH of DCI format 3/3A with this UE's TPC-PUSCH-RNTI in every BL/CE downlink subframe except when in DRX

--------------------------------------------------- Text End ---------------------------------------------------------

## Company Views

Please indicate which text proposals your company can agree too and comments:

| Company | Comments on Proposal |
| --- | --- |
| Ericsson | TP#1 in section 3.2 of this FLS is needed regardless of whether PUCCH for PUR is decided to follow an open-loop or closed-loop power control scheme. Simply, in clause 5.1.2.1 it has been missed to include the PUR C-RNTI for DCI format 6-1A. |
| Qualcomm | Probably we also need the following TP for 5.1.1 (added above too): - For serving cell and a BL/CE UE configured with CEModeA, the UE attempts to decode a MPDCCH of DCI format 6-0A with the UE's C-RNTI or SPS C-RNTI or PUR C-RNTI and a MPDCCH of DCI format 3/3A with this UE's TPC-PUSCH-RNTI in every BL/CE downlink subframe except when in DRX (Actually, strictly speaking these two sentences are not true, since there can be BL/CE DL subframes that do not belong to any search space, and thus the UE does not monitor DCI in those subframes – also, if the UE is receiving PDSCH in a different narrowband |
| Sony | Support TP#1. |
| ZTE,Sanechips | OK with TP#1 and QC's added modification for 5.1.1 |

# Issue #3 Power control accumulation mechanism

## Issue Description

Power control accumulation mechanism was agreed to be supported for re-transmission in PUR. In TS36.213 the accumulation is normally enabled based on the parameter Accumulation-enabled or accumulationEnabledsTTI provided by higher layers or if the TPC command is included in a MPDCCH with DCI format 6-0A for serving cell  where the CRC is scrambled by the Temporary C-RNTI.  However, there is no configuration specified for transmission in PUR.

## Agreement Alternatives

Companies should consider the following agreement alternatives:

Alt 1:  Power accumulation for PUR is enabled via an (new) RRC parameter in the PUR-config.

Alt 2:  Power accumulation for PUR is enabled via legacy RRC parameter Accumulation-enabled

Alt 3:  Power accumulation for PUR is enabled via existing RRC parameter in PUR-Config (e.g. pur-PUSCH-power-control-alpha)

Alt 4:    Power accumulation for PUR is always enabled

Ericsson: At first glance:

- Alt 1 is not desirable because ASN.1 freeze is very close.

- Alt 2 also impacts HL (Moreover, it seems that Accumulation-enabled is a parameter for connected mode that will need to be incorporated to PUR-Config).

- Alt 3 it seems that it would need to be associated to a power control related parameter in PUR-Config, e.g., alpha which seems to result in another way to achieve Alt 4, unless accumulation were specifically associated to e.g., some value(s) in the set of alpha which will be controversial.

- Alt 4 It can be implemented by re-using legacy text, with a minor RAN1 specification impact and without impacting HL. The accumulation will occur only when more than one retransmission is required, indeed from the second retransmission onwards (Recall the following Agreement from RAN1 #86bis “For PUR power control in CE mode A, the TPC accumulation mechanism is reset for every PUR transmission”).

In the case of re-transmissions, PUR uses an UL-Grant as in legacy including the following field “-	TPC command for scheduled PUSCH – 2 bits as defined in clause 5.1.1.1 of [3]”, therefore in our view the following legacy text “DCI format 6-0A” in clause 5.1.1.1 could be used to cover the Accumulation case for PUR without any need for adding an enabler impacting HL, we just need to append “PUR C-RNTI” as shown in the TP #2 below.

## Text proposals

### TP #1 for ALT1

ZTE[3] provide a TP for TS 36.213:

5.1.1.1	UE behaviour

<Unchanged parts are omitted>

-	 and  if accumulation is enabled based on the parameter Accumulation-enabled or accumulationEnabledsTTI or Accumulation-enabled-PUR provided by higher layers or if the TPC command  is included in a PDCCH/EPDCCH with DCI format 0 or in a MPDCCH with DCI format 6-0A for serving cell  where the CRC is scrambled by the Temporary C-RNTI

### TP #2

CR Cover Sheet Information:

Reasons for change:  Missing definition of how the PUSCH power control accumulation mechanism is enabled when USS is given by PUR C-RNTI.

Summary of changes:   Specified that PUSCH power control accumulation mechanism is enabled when the CRC is scrambled by PUR C-RNTI.

Specs/Sections impacted: TS 36.213 section 5.1.1.1

Consequences if not approved:   Unclear how PUSCH power control accumulation mechanism is enabled when USS is given by PUR C-RNTI.

<Start of Change >

5.1.1.1	UE behaviour

<Unchanged parts are omitted>

-	 and  if accumulation is enabled based on the parameter Accumulation-enabled or accumulationEnabledsTTI provided by higher layers or if the TPC command  is included in a PDCCH/EPDCCH with DCI format 0 or in a MPDCCH with DCI format 6-0A for serving cell  where the CRC is scrambled by the Temporary C-RNTI or PUR C-RNTI

<End of Change >

## Company Views

Please indicate which text proposals your company can agree too and comments:

| Company | Comments on Proposal |
| --- | --- |
| Ericsson | We propose TP#2 which implies a minor RAN1 specification impact without impacting HL (i.e., no ASN.1 impact). The accumulation in any case will occur only when more than one retransmission is required, indeed from the second retransmission onwards (Recall the following Agreement from RAN1 #86bis “For PUR power control in CE mode A, the TPC accumulation mechanism is reset for every PUR transmission”). TP#2 just appends “or PUR C-RNTI” to the legacy text associated with DCI format 6-0A. |
| Qualcomm | Thanks to Ericsson for proposing the TP, we support TP#2 since it doesn’t require adding a new RRC parameter. |
| Sony | We support TP#2 (no impact to RRC). |
| ZTE,Sanechips | If companies prefer changes not have RRC impact, we are OK with TP#2. |

# Summary

It is recommended that the following TPs are endorsed for the editor’s CR on TS 36.213

TP#2 in section 2.3.2

TP#2 in section 3.2.2

TP#2 in section 4.3.2

# References

R1-2003539	Corrections on transmission in preconfigured UL resources	Huawei, HiSilicon

R1-2003781	Support for transmission in preconfigured UL resources	Qualcomm Incorporated

R1-2003791	Remaining issues for transmission in preconfigured UL resources for MTC	ZTE

R1-2004655	Corrections for Preconfigured UL resources for LTE-MTC	Ericsson

R1-2003157,     Corrections to Additional MTC Enhancements for LTE,    Rel-16,      36.213,      16.1.0,      LTE_eMTC5-Core,         CR#1333

R1-2004691	LTE-M Preconfigured UL resources feature lead summary #2 RAN1 #101-e, Moderator (Sierra Wireless)