3GPP TSG RAN WG1 #101			R1-2004818

e-Meeting, May 25th – June 5th, 2020

Source:	Moderator (NTT DOCOMO, INC.)

Title:	Summary on [101-e-NR-UEFeatures-NRU-03]

Agenda Item:	7.2.11.2

Document for: 	Discussion and Decision

- Introduction

This contribution summarizes the following email discussion/approval regarding UE features for NR-U.

[101-e-NR-UEFeatures-NRU-03] Email discussion/approval on basic FGs for each NR-U operation/scenario (29th May – 4th June) – Hiroki (DCM)

- Discuss and down-select from Alt.1 or Alt.2 based on the working assumption

- Discuss and decide basic FGs for each NR-U operation/scenario based on finalized FG structure for NR-U

- Discussion on UE features for NR-U

Basic FGs

Following agreements and working assumption were made at RAN1#100bis-e meeting.

Agreements (RAN1#100bis-e): - Define new basic FGs with components that have tightly related functionality to replace current basic FGs - In “mandatory/optional” column for the possible basic FGs, it should be clarified that the FG may be a part of basic operation for a particular scenario - If the FG is decided as a basic FG, the note will be updated to clarify that the FG is “optional with capability signaling and is required to be supported for the scenario” - Note: each basic FG will have capability bit - [Working assumption] Take either one of following alternatives Alt.1: - Define a table to capture the basic FGs required for a certain NR-U deployment scenario in specification - Note: the table does not have impact on capability signaling - Note: the grouping of FGs in the table does not have impact on “prerequisite FGs” column in features list Alt.2: - Capture an association between the basic FGs required to be supported and a certain NR-U deployment scenario in the UE features list

- Confirm the working assumption on the association between the basic FGs and NR-U deployment scenarios

  - Alt.1 (Define a table to capture the basic FGs required for a certain NR-U deployment scenario in specification): [3]

  - Alt.2 (Capture an association between the basic FGs required to be supported and a certain NR-U deployment scenario in the UE features list): [2], [7]

- Define basic FG(s) for a certain NR-U deployment scenario (whether to keep the text “This FG may be a part of basic operation for a particular scenario”)

  - 10-1 (UL channel access for dynamic channel access mode): [3], [4], [5], [6], [7], [13]

  - 10-1a (UL channel access for semi-static channel access mode): [3], [4], [5], [6], [7], [13]

  - 10-2 (SSB-based RRM [for dynamic channel access mode]): [3], [4], [5], [7], [13]

    - Objected by: [6]

  - 10-2a (SSB-based RRM [for semi-static channel access mode]): [5], [7], [13]

    - Objected by: [6]

  - 10-2b (MIB reading on unlicensed cell): [3], [4], [5], [7], [13]

    - Objected by: [6]

  - 10-2c (SSB-based RLM with Q [for dynamic channel access mode]): [3], [4], [5], [7], [13]

    - Objected by: [6]

  - 10-2d (SSB-based RLM with Q [for semi-static channel access mode]): [5], [7], [13]

    - Objected by: [6]

  - 10-2e (SIB1 reception on unlicensed cell): [3], [4], [5], [7], [13]

    - Objected by: [6]

  - 10-2f (Support monitoring of extended RAR window): [4], [13]

    - Objected by: [5], [6], [7]

  - 10-3 (PRB interlace mapping for PUSCH): [4], [7]

  - 10-3a (PRB interlace mapping for PUCCH): [4], [7]

  - 10-7 (UL channel access for 10 MHz SCell): [4]

    - Objected by: [5], [6], [7]

  - 10-10 (RSSI and channel occupancy measurement and reporting):

    - Objected by: [4], [5], [6], [7]

  - 10-11 (SRS starting position at any OFDM symbol in a slot):

    - Objected by: [5], [6], [7]

  - 10-14 (Non-numerical PDSCH to HARQ-ACK timing): [4]

  - 10-15 (Enhanced dynamic HARQ codebook): [4]

  - 10-19a ([Support DL reception in a carrier with intra-cell guard-bands]): [13]

  - 10-19b ([Support UL transmission with subset of RB sets passing LBT]): [13]

  - 10-20 (Support search space set configuration with freqMonitorLocation-r16): [13]

    - Objected by: [5], [6], [7]

  - 10-20a (Support coreset configuration with rb-Offset):

    - Objected by: [5], [6], [7]

  - 10-23 (CGI reading on unlicensed cell [based on off-sync raster SSB] for ANR functionality):

    - Objected by: [5], [6], [7]

  - 10-25 (Enable configured UL transmissions when DCI 2_0 is configured but not detected):

    - Objected by: [5], [6], [7]

  - 10-27 (Wideband PRACH): [4]

    - Objected by: [5], [6], [7]

  - 10-29 (Support available RB set indicator field in DCI 2_0): [4], [13]

    - Objected by: [5], [6], [7]

  - 10-30 (Support channel occupancy duration indicator field in DCI 2_0): [4]

    - Objected by: [5], [6], [7]

Above remaining issues and proposals are identified based on following feedbacks provided in contributions for the RAN1#101-e meeting.

| [2] | For the capture of association between the FGs to a certain NR-U scenario, we think it is enough to clarify that the FG is a part of basic operation for a particular scenario and the association between the FGs and a certain NR-U deployment scenario is established. Note that, one FG could be clarified as a part of basic operation for multiple scenarios. For example, 10-1 is for NRU standalone scenario with dynamic channel access and CA DL+UL scenario with dynamic channel access. Proposal 3: Clarifying that the FG is a part of basic operation for a particular scenario in “mandatory/optional” column of UE feature list is enough to establish the association between the FGs to a certain NR-U scenario. |
| --- | --- |
| [3] | In our view, Alt.1 is clear as the table is defined with the intention that the basic FGs would be defined with tightly related functionality, and the basic FGs would try to avoid overlapping functionality as much as possible. It would be easier to build the relationship between the basic FGs and other optional FGs in terms of prerequisite condition. Proposal 1: - The following table is defined in specification to capture the basic FGs required for a certain NR-U deployment scenario. Basic FGs; Deployment scenarios. A-1 (SCell); A-2 (SCell); A-2 (SCell); C (Standalone), D (Standalone + SUL); C (Standalone), D (Standalone + SUL); B (EN-DC), E (NR-NR DC); B (EN-DC), E (NR-NR DC). DL only; DL+UL LBE; DL+UL FBE; DL+UL LBE; DL+UL FBE; DL+UL LBE; DL+UL FBE. 10-1: UL channel access for LBE; X; X; X. 10-1a: UL channel access for FBE; X; X; X. 10-2: SSB based RRM; X; X; X; X; X; X; X. 10-2b: MIB reading on unlicensed cell; X; X; X; X. 10-2c: SSB-based RLM; X; X; X; X. 10-2e: SIB1 reception; X; X. |
| [4] | MIB reading should basic FG for SA/DC deployment scenarios. RACH related FGs should be basic FGs for SA/DC deployment scenarios. RAR windows are extended to 40ms to cope with the transmission timing uncertainty caused by LBT. We think it should be supported by UE that plans to support NR-U SA and DC deployments. On the other hand, for UE that only supports NR-U CA operation, then it is not necessary, because RAR is transmitted in SpCell (PCell or PSCell). PRB interlaced mapping for PUSCH should be basic FGs for CA with UL and SA/DC deployment scenarios. PRB interlaced mapping for PUCCH should be basic FGs for SA/DC deployment scenarios. In our opinion, monitoring DCI 2_0 to acquire COT structure in both time (CO duration) and frequency (availableRB-Sets-r16) dimensions is beneficial to UE in terms of AGC adjustment, COT detection, skipping of PDCCH monitoring and CSI-RS reception, etc. Therefore, we suggest to merge FG10-29 and 10-30 as a basic FG for all NR-U deployment scenarios. In our opinion, FG10-11 (“SRS starting position at any OFDM symbol in a slot”) is an enhancement and should not be regarded as a basic feature group. FG10-14 (“Non-numerical PDSCH to HARQ-ACK timing”) is beneficial to unlicensed operation when channel occupancy time is limited by regulation. Specifically, gNB can tell UE to postpone its HARQ-ACK feedback to a PDSCH that is scheduled in the end of a COT. We hence think it should be a basic feature group for UE that supports NR-U SA and DC deployments. On the other hand, for a UE that only supports NR-U CA operation, then the support of this feature group should be optional. We think FG10-15 (“enhanced dynamic HARQ codebook”) should be regarded as a basic FG for UE that supports NR-U SA and DC operation. Similar to the motivation for introducing DRS windows to NR-U, enhanced HARQ procedures that have designed for NR-U can provide multiple transmission opportunities for HARQ-ACK feedbacks. As “dynamic HARQ-ACK codebook” is chosen to be the default codebook for Rel-15, we believe FG10-15 (“enhanced dynamic HARQ codebook”) rather than FG10-16 and 10-16a (one-shot codebook) should be the basic feature group for SA and DC deployment scenarios. Table 1: Proposed basic feature groups for NR-U deployment scenarios Scenario A with DL-only; Scenario A with both DL and UL in LBE; Scenario A with both DL and UL in FBE; Scenario B in LBE; Scenario B in FBE; Scenario C; Scenario D; Scenario E in LBE; Scenario E in FBE. 10-1 (LBT LBE); X; X; X; X. 10-1a (LBT FBE); X; X; X; X. 10-2 (SSB RRM); X; X; X; X; X; X; X; X; X. 10-2b (MIB); X; X; X; X; X; X. 10-2c (SSB RLM); X; X; X; X; X; X. 10-2e (SIB1); X; X; X; X; X; X. 10-2f (ext. RAR); X; X; X; X. 10-3 (interlaced PUSCH); X; X; X; X; X; X; X. 10-3a (interlaced PUCCH); X; X; X; X; X. 10-7 (10M UL); X. 10-14 (non-numerical HARQ); X; X; X; X; X. 10-15; X; X; X; X; X. 10-27 (wide PRACH); X; X; X; X; X. 10-29 (DCI 2_0); X; X; X; X; X; X; X; X; X. 10-30 (DCI 2_0); X; X; X; X; X; X; X; X; X. Proposal 2: Adopt the proposed basic feature groups in Table 1 for NR-U deployment scenarios. For FG10-10, remove “This FG may be a part of basic operation for a particular scenario” from the column of “Mandatory/Optional.” According to RAN2’s agreements, FG10-10 should be optional. Proposal 6: For FG10-10, remove “This FG may be a part of basic operation for a particular scenario” from the column of “Mandatory/Optional.” |
| [5] | In our view, the following FGs make sense to be part of a basic operation for a particular scenario as listed in the last column of the spreadsheet: - FG 10-1, -1a, -2, -2a, -2b, -2c, -2d, -2e However, the following features should be listed only as “Optional with capability signaling,” and not be part of basic operation for a particular scenario - FG 10-2f, -7, -10, -11, -20, -20a, -23, -25, -27, -29, -30 |
| [6] | In the lists, there are many FGs with the statement, ‘This FG may be a part of basic operation for a particular scenario’ in Mandatory/Optional column. Examples are listed as below. 10-10; RSSI and channel occupancy measurement and reporting; 1. RSSI measurement 2. Channel occupancy reporting; FFS: Per band or Per UE; Optional with capability signaling This FG may be a part of basic operation for a particular scenario. 10-11; SRS starting position at any OFDM symbol in a slot; 1. Support transmitting SRS starting in all symbols (0,…,13) of a slot; FFS: Per band or Per UE; Optional with capability signaling This FG may be a part of basic operation for a particular scenario. 10-20; Support search space set configuration with freqMonitorLocation-r16; 1. Support search space set configuration with freqMonitorLocations-r16; Per band; Optional with capability signaling This FG may be a part of basic operation for a particular scenario. …; …; …; …; …. However, in our view, all of those FGs are not essential features to operate NR-U in a particular scenario. For example, FG 10-10 and FG 10-11, it is quite sure that those feature groups should not necessary regardless of deployment scenarios. Therefore, it is proposed to remove (or put FFS) the statements in Mandatory/Optional column or put FFS for the further discussion. Proposal 2: Remove (or put FFS) ‘This FG may be a part of basic operation for a particular scenario’ for FGs other than FG 10-1 and FG 10-1a. |
| [7] | In RAN1#100bis-e meeting, it was agreed to define BFGs with working assumption for two alternatives on how to capture the relationship between BFG and the corresponding deployment scenario. Between two alternatives, Alt 2 is preferred since it seems sufficient to capture the association relationship between BFG and the corresponding deployment scenario in mandatory/optional column in UE features list, and capturing and managing the association in separate specification as in Alt 1 could be redundant. Proposal #1: Confirm the working assumption by adopting Alt 2 (Capture an association between the basic FGs required to be supported and a certain NR-U deployment scenario in the UE features list). Considering the deployment scenarios identified in NR-U WID [2], in our view, the following feature groups can be considered for BFGs applicable to a specific deployment scenario, with the principle that the minimum set of BFGs should be defined for each deployment scenario. - FG 10-1 (UL channel access for dynamic channel access mode): This FG is optional with capability signaling and is required to be supported for the scenarios A (except for DL-only), B, C, D, and E - FG 10-1a (UL channel access for semi-static channel access mode): This FG is optional with capability signaling and is required to be supported for the scenarios A (except for DL-only), B, C, D, and E - FG 10-2 (SSB-based RRM [for dynamic channel access mode]): This FG is optional with capability signaling and is required to be supported for the scenarios A, B, C, D, and E - FG 10-2a (SSB-based RRM [for semi-static channel access mode]): This FG is optional with capability signaling and is required to be supported for the scenarios A, B, C, D, and E - FG 10-2b (MIB reading on unlicensed cell): This FG is optional with capability signaling and is required to be supported for the scenarios C and D - FG 10-2c (SSB-based RLM [for dynamic channel access mode]): This FG is optional with capability signaling and is required to be supported for the scenarios B, C, D, and E - FG 10-2d (SSB-based RLM [for semi-static channel access mode]): This FG is optional with capability signaling and is required to be supported for the scenarios B, C, D, and E - FG 10-2e (SIB1 reception on unlicensed cell): This FG is optional with capability signaling and is required to be supported for the scenarios C and D - FG 10-3 (PRB interlace mapping for PUSCH): This FG is optional with capability signaling and is required to be supported for the scenarios A (except for DL-only), B, C, D, and E - FG 10-3a (PRB interlace mapping for PUCCH): This FG is optional with capability signaling and is required to be supported for the scenarios B, C, D, and E For all of feature groups other than above BFGs, we propose to remove “This FG may be a part of basic operation for a particular scenario” in mandatory/optional column. Proposal #2: For FG 10-1/1a/2/2a/2b/2c/2d/2e, update “This FG may be a part of basic operation for a particular scenario” in mandatory/optional column as follows, where each scenario is given by RP-191575: - FG 10-1 (UL channel access for dynamic channel access mode): This FG is optional with capability signaling and is required to be supported for the scenarios A (except for DL-only), B, C, D, and E - FG 10-1a (UL channel access for semi-static channel access mode): This FG is optional with capability signaling and is required to be supported for the scenarios A (except for DL-only), B, C, D, and E - FG 10-2 (SSB-based RRM [for dynamic channel access mode]): This FG is optional with capability signaling and is required to be supported for the scenarios A, B, C, D, and E - FG 10-2a (SSB-based RRM [for semi-static channel access mode]): This FG is optional with capability signaling and is required to be supported for the scenarios A, B, C, D, and E - FG 10-2b (MIB reading on unlicensed cell): This FG is optional with capability signaling and is required to be supported for the scenarios C and D - FG 10-2c (SSB-based RLM [for dynamic channel access mode]): This FG is optional with capability signaling and is required to be supported for the scenarios B, C, D, and E - FG 10-2d (SSB-based RLM [for semi-static channel access mode]): This FG is optional with capability signaling and is required to be supported for the scenarios B, C, D, and E - FG 10-2e (SIB1 reception on unlicensed cell): This FG is optional with capability signaling and is required to be supported for the scenarios C and D Proposal #3: Remove “This FG may be a part of basic operation for a particular scenario” in mandatory/optional column for all FGs except for FG 10-1/1a/2/2a/2b/2c/2d/2e. Proposal #4: For FG 10-3/3a, add the following statement in mandatory/optional column, where each scenario is given by RP-191575: - FG 10-3 (PRB interlace mapping for PUSCH): This FG is optional with capability signaling and is required to be supported for the scenarios A (except for DL-only), B, C, D, and E - FG 10-3a (PRB interlace mapping for PUCCH): This FG is optional with capability signaling and is required to be supported for the scenarios B, C, D, and E |
| [13] | Table 1 and 2 below provides our proposed mapping for the basic FGs to the NR-U deployment scenarios for dynamic and semi-static channel access modes, respectively. In the tables, ‘O’ indicates the FG can be optional for the corresponding scenario, while ‘M’ indicates the FG should be mandatory for the corresponding scenario. Table 1: Proposed mapping for the basic FGs to the NR-U deployment scenarios assuming dynamic channel access mode Index; FG; Scen. A (DL-only); Scen. A (UL+DL); Scen. B; Scen. C; Scen. D; Scen. E; Comment. 10-1; UL channel access for dynamic channel access mode; N/A; M; M; M; N/A; M; Implies support of transmission of relevant UL channels/signals as well. 10-2; SSB-based RRM [for dynamic channel access mode]; M; M; M; M; M; M. 10-2b; MIB reading on unlicensed cell; N/A; N/A; M; M; M; M. 10-2c; SSB-based RLM [for dynamic channel access mode]; N/A; N/A; M; M; M; M. 10-2e; SIB1 reception on unlicensed cell; N/A; N/A; M; M; M; M. 10-2f; Support monitoring of extended RAR window; N/A; N/A; M; M; M; M. 10-30; Support channel occupancy duration indicator field in DCI 2_0; [O/M]; [O/M]; [O/M]; [O/M]; [O/M]; [O/M]; Depends on whether SFI is mandatory or optional in DCI 2_0. Table 2: Proposed mapping for the basic FGs to the NR-U deployment scenarios assuming semi-static channel access mode Index; FG; Scen. A; Scen. A (UL+DL); Scen. B; Scen. C; Scen. D; Scen. E; Comment. 10-1a; UL channel access for semi-static channel access mode; M (DL+UL); M; M; M; N/A; M; Not applicable to Scenario A with DL-only operation.. 10-2a; SSB-based RRM [for semi-static channel access mode]; M; M; M; M; M; M. 10-2b; MIB reading on unlicensed cell; N/A; N/A; M; M; M; M. 10-2d; SSB-based RLM [for semi-static channel access mode]; N/A; N/A; M; M; M; M. 10-2e; SIB1 reception on unlicensed cell; N/A; N/A; M; M; M; M. 10-2f; Support monitoring of extended RAR window; N/A; N/A; M; M; M; M. 10-30; Support channel occupancy duration indicator field in DCI 2_0; [O/M]; [O/M]; [O/M]; [O/M]; [O/M]; [O/M]; This depends on whether “end of FFP is treated as end of COT in the specification”. Other aspects can be considered when mapping basic feature groups to scenarios. One such aspect is wideband operation modes 2/3, where such a mapping is proposed in Table 3 below. Given the simpler mapping in this case, it might be straightforward to handle the relationships by mean of pre-requisites in the FG table as well, depending on the final decisions regarding the FGs themselves and applicability to deployment scenarios. Table 3: Basic FGs for wideband operation mode 2/3 Index; FG; Comment. 10-20; Support search space set configuration with freqMonitorLocation-r16; Applies for scenarios A, B, C, D, E. 10-29; Support available RB set indicator field in DCI 2_0; Applies for scenarios A, B, C, D, E. [10-19a]; [Support DL reception in a carrier with intra-cell guard-bands]; If FG is confirmed. [10-19b]; [Support UL transmission with subset of RB sets passing LBT]; If FG is confirmed. |

Based on above, following FL proposals are made.

FL proposal 1

- Confirm the working assumption on the association between the basic FGs and NR-U deployment scenarios, and take one of following alternatives

  - Alt.1 (Define a table to capture the basic FGs required for a certain NR-U deployment scenario in specification)

  - Alt.2 (Capture an association between the basic FGs required to be supported and a certain NR-U deployment scenario in the UE features list)

Companies are encouraged to check above FL proposals and to provide feedback if any in below. If you cannot accept the FL proposals, please put your company name after “Cannot accept the proposals” below and please provide your alternative proposal (in your comment) which could be acceptable to all in your consideration.

Cannot accept the proposals:

Cannot accept Alt.1:

Cannot accept Alt.2:

| Company | Comment |
| --- | --- |
| Ericsson | We support Alt-1. It can be further discussed what is the appropriate place to capture the table. |
| Nokia, NSB | We support Alt-1 as it makes the relationship between FGs and scenarios more clear. |
| LG Electronics | Alt 2 is preferred as capturing the association in UE features list seems sufficient. |
| MediaTek | We prefer Alt-1 which provides more clear association between basic feature groups and deployment scenarios. A question for clarification: In Alt-2, what we mean by “the UE feature list” is the one to be captured in TS38.306 or the one in TR38.822? If it is to be captured in a TR, then we have concerns with Alt-2. |
| Moderator (NTT DOCOMO) | Based on feedbacks so far, we can continue working on defining “table”. |

FL proposal 2

- Decide whether to keep the text “This FG may be a part of basic operation for a particular scenario” in mandatory/optional column

  - Keep the text for folloing FGs

    - 10-1 (UL channel access for dynamic channel access mode)

    - 10-1a (UL channel access for semi-static channel access mode)

    - 10-2 (SSB-based RRM [for dynamic channel access mode])

    - 10-2a (SSB-based RRM [for semi-static channel access mode])

    - 10-2b (MIB reading on unlicensed cell)

    - 10-2c (SSB-based RLM with Q [for dynamic channel access mode])

    - 10-2d (SSB-based RLM with Q [for semi-static channel access mode])

    - 10-2e (SIB1 reception on unlicensed cell)

    - 10-3 (PRB interlace mapping for PUSCH)

    - 10-3a (PRB interlace mapping for PUCCH)

    - 10-14 (Non-numerical PDSCH to HARQ-ACK timing)

    - 10-15 (Enhanced dynamic HARQ codebook)

    - 10-19a ([Support DL reception in a carrier with intra-cell guard-bands])

    - 10-19b ([Support UL transmission with subset of RB sets passing LBT])

  - Remove the text for following FGs

    - 10-2f (Support monitoring of extended RAR window)

    - 10-7 (UL channel access for 10 MHz SCell)

    - 10-10 (RSSI and channel occupancy measurement and reporting)

    - 10-11 (SRS starting position at any OFDM symbol in a slot)

    - 10-20 (Support search space set configuration with freqMonitorLocation-r16)

    - 10-20a (Support coreset configuration with rb-Offset)

    - 10-23 (CGI reading on unlicensed cell [based on off-sync raster SSB] for ANR functionality)

    - 10-25 (Enable configured UL transmissions when DCI 2_0 is configured but not detected)

    - 10-27 (Wideband PRACH)

    - 10-29 (Support available RB set indicator field in DCI 2_0)

    - 10-30 (Support channel occupancy duration indicator field in DCI 2_0)

Companies are encouraged to check above FL proposals and to provide feedback if any in below. If you cannot accept the FL proposals, please put your company name after “Cannot accept the proposals” below and please provide your alternative proposal (in your comment) which could be acceptable to all in your consideration.

Cannot accept the proposals:

| Company | Comment |
| --- | --- |
| Ericsson | We support to keep the text “This FG may be a part of basic operation for a particular scenario” only for the following FGs. These ones are truly basic functionality, required for a working NR-U system. The remainder of the features are of course useful; however, not essential to define as basic functionaly. - 10-1, -1a, -2, -2a, -2b, -2c, -2d, -2e |
| Nokia, NSB | We support to change the text to “This FG is a part of basic operation for a particular scenario”, and apply it only for the following FGs, which are essential for at least one scenario: 10-1, 10-1a, 10-2, 10-2a, 10-2b, 10-2c, 10-2d, 10-2e, 10-2f. |
| LG Electronics | We support to change the text to “This FG is optional with capability signaling and is required to be supported for the scenario X” only for the following FGs, which are essential for at least one scenario. - 10-1, 10-1a, 10-2, 10-2a, 10-2b, 10-2c, 10-2d, 10-2e, 10-3, 10-3a We support to remove the text “This FG may be a part of basic operation for a particular scenario” for all FGs other than the above FGs. |
| MediaTek | We support to keep the text “This FG may be a part of basic operation for a particular scenario” only for the following FGs. - 10-1, 10-1a, 10-2, 10-2b, 10-2c, 10-2e, 10-2f, 10-3, 10-3a, 10-14, 10-15, 10-27, 10-29, 10-30 |
| Moderator (NTT DOCOMO) | Based on feedbacks so far, at least removal of text from following FGs would be acceptable.  10-7 (UL channel access for 10 MHz SCell)  10-10 (RSSI and channel occupancy measurement and reporting)  10-11 (SRS starting position at any OFDM symbol in a slot)  10-20 (Support search space set configuration with freqMonitorLocation-r16)  10-20a (Support coreset configuration with rb-Offset)  10-23 (CGI reading on unlicensed cell [based on off-sync raster SSB] for ANR functionality)  10-25 (Enable configured UL transmissions when DCI 2_0 is configured but not detected) |

Updated FL proposal 2

- Remove the text “This FG may be a part of basic operation for a particular scenario” in mandatory/optional column from following FGs

  - 10-7 (UL channel access for 10 MHz SCell)

  - 10-10 (RSSI and channel occupancy measurement and reporting)

  - 10-11 (SRS starting position at any OFDM symbol in a slot)

  - 10-20 (Support search space set configuration with freqMonitorLocation-r16)

  - 10-20a (Support coreset configuration with rb-Offset)

  - 10-23 (CGI reading on unlicensed cell [based on off-sync raster SSB] for ANR functionality)

  - 10-25 (Enable configured UL transmissions when DCI 2_0 is configured but not detected)

FL proposal 3

- Decide classification of NR-U deployment scenarios for the purpose of defining basic FGs as below

  - Scenario A with DL-only

  - Scenario A with both DL and UL in LBE

  - Scenario A with both DL and UL in FBE

  - Scenario B in LBE

  - Scenario B in FBE

  - Scenario C in LBE

  - Scenario C in FBE

  - Scenario D in LBE

  - Scenario D in FBE

  - Scenario E in LBE

  - Scenario E in FBE

Companies are encouraged to check above FL proposals and to provide feedback if any in below. If you cannot accept the FL proposals, please put your company name after “Cannot accept the proposals” below and please provide your alternative proposal (in your comment) which could be acceptable to all in your consideration.

Cannot accept the proposals:

| Company | Comment |
| --- | --- |
| Ericsson | We think that it is better to define the essential aspect that differentiates between the different deployment scenarios, hence our suggestion would be the following: - SCell (DL-Only) in band for shared spectrum channel access (maps to Scenario A) - SCell (DL + UL) in band for shared spectrum channel access (maps to Scenario A) - For dynamic channel access mode - For semi-static channel access mode - PCell (DL + UL) in band for shared spectrum channel access (maps to Scenario C,D) - For dynamic channel access mode - For semi-static channel access mode - PSCell (DL + UL) in band for shared spectrum channel access (maps to Scenarios B,E) - For dynamic channel access mode - For semi-static channel access mode |
| Nokia, NSB | We think the deployment scenarios in the FL proposal are OK. The classification given by Ericsson above is interesting too, however it does not work for all the cases. For example, the mapping of 3 to scenarios C and D is not really feasible because in scenario D the UL is in licensed band, and hence some UL features do not apply there. However it should be possible to consider scenarios B and E together. Regarding the table below, it is not complete given that it is not based on the features listed in FL proposal 2, and it even uses outdated FG numbers. We propose to delete it from this document to avoid confusion. If anything it shows that the grouping of scenarios C and D is not feasible as the UL channel access features do not apply to scenario D at all. |
| LG Electronics | Support FL proposal. |
| MediaTek | We are fine with FL’s proposal. |
| Ericsson2 | Just to clarify, the only reason for including the table below was as an example. I acknowledge that the FG#'s were not accurate. To avoid confusion, I removed, the table. To address Nokia's comments about avoiding grouping scenarios C and D, the following would be a viable alternative from our perspective (preserves grouping of B,E): - SCell (DL-Only) in band for shared spectrum channel access (maps to Scenario A) - SCell (DL + UL) in band for shared spectrum channel access (maps to Scenario A) - For dynamic channel access mode - For semi-static channel access mode - PCell (DL + UL) in band for shared spectrum channel access (maps to Scenario C) - For dynamic channel access mode - For semi-static channel access mode - PCell (DL + UL) in band for shared spectrum channel access + SUL in licensed band (maps to Scenario D) - For dynamic channel access mode - For semi-static channel access mode - PSCell (DL + UL) in band for shared spectrum channel access (maps to Scenarios B,E) - For dynamic channel access mode - For semi-static channel access mode With this proposal, the size of the table is reduced to 9 from 11, and cryptic scenario names are not needed. |
| Moderator (NTT DOCOMO) | Based on the feedbacks so far, we can start listing up basic FGs according to classification in FL proposal. Merging some scearios as suggested by Ericsson can also be discussed in parallel (based on how basic FGs in scenarios are common). |

# 3. Conclusion

Agreements:

- Remove the text “This FG may be a part of basic operation for a particular scenario” in mandatory/optional column from following FGs

  - 10-7 (UL channel access for 10 MHz SCell)

  - 10-10 (RSSI and channel occupancy measurement and reporting)

  - 10-11 (SRS starting position at any OFDM symbol in a slot)

  - 10-20 (Support search space set configuration with freqMonitorLocation-r16)

  - 10-20a (Support coreset configuration with rb-Offset)

  - 10-23 (CGI reading on unlicensed cell [based on off-sync raster SSB] for ANR functionality)

  - 10-25 (Enable configured UL transmissions when DCI 2_0 is configured but not detected)

# Reference

[1]	R1-2003198	Summary on email discussion [100b-e-NR-UEFeatures-Remaining] NR-unlicensed	Moderator (NTT DOCOMO, INC.)

[2]	R1-2003416	Discussion on UE features for NRU	vivo

[3]	R1-2003460	Discussion on the remaining issues of the UE features for NR-U	ZTE, Sanechips

[4]	R1-2003694	Views on Rel-16 UE features for NR-U	MediaTek Inc.

[5]	R1-2003848	UE features for NR-U	Ericsson

[6]	R1-2003894	UE features for NR-U	Samsung

[7]	R1-2004019	Discussion on UE features for NR-U	LG Electronics

[8]	R1-2004091	Discussion on UE feature for NRU	OPPO

[9]	R1-2004152	Rel-16 UE features for NR-U	Huawei, HiSilicon

[10]	R1-2004241	Discussions on NR-U UE features	Apple

[11]	R1-2004402	UE features for NR-U	NTT DOCOMO, INC

[12]	R1-2004477	Discussion on NR-U UE features	Qualcomm Incorporated

[13]	R1-2004560	On UE features NR Unlicensed	Nokia, Nokia Shanghai Bell

[14]	R1-2004062	Discussion on the support of SRS transmission in all symbols of a slot	OPPO

# Appendix: latest version of UE features list for NR-U [1]

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type ( 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10. NR-unlicensed | 10-1 | UL channel access for dynamic channel access mode | 1. Type 1 channel access 2. Type 2A channel access 3. Type 2B channel access 4. Type 2C channel access 5. 20MHz LBT bandwidth 6. CP extension up to 1 symbol for PUSCH/PUCCH transmission |  | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-1a | UL channel access for semi-static channel access mode | 1. Type 2C channel access 2. Single sensing slot of 9us channel access 3. 20MHz LBT bandwidth | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-2 | SSB-based RRM [for dynamic channel access mode] | 1. SSB-based RRM with Q [for dynamic channel access mode] | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A | Q indicates the value of RAN1 parameter NSSBQCL | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-2a | SSB-based RRM [for semi-static channel access mode] | 1. SSB-based RRM with Q [for semi-static channel access mode] | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A | Q indicates the value of RAN1 parameter NSSBQCL | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-2b | MIB reading on unlicensed cell | 1. MIB reading on unlicensed cell | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-2c | SSB-based RLM [for dynamic channel access mode] | 1. SSB-based RLM with Q [for dynamic channel access mode] | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A | Q indicates the value of RAN1 parameter NSSBQCL | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-2d | SSB-based RLM [for semi-static channel access mode] | 1. SSB-based RLM with Q [for semi-static channel access mode] | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A | Q indicates the value of RAN1 parameter NSSBQCL | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-2e | SIB1 reception on unlicensed cell | 1. SIB1 reception on unlicensed cell | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-2f | Support monitoring of extended RAR window | 1. Support of RAR extension from 10ms to [40ms] by decoding of the 2-bit SFN indication in DCI 1_0 | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-7 | UL channel access for 10 MHz SCell | - 10 MHz LBT bandwidth | one of {10-1, 10-1a} | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-10 | RSSI and channel occupancy measurement and reporting | - RSSI measurement - Channel occupancy reporting | TBD | Yes | N/A |  | FFS: Per band or Per UE | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-11 | SRS starting position at any OFDM symbol in a slot | - Support transmitting SRS starting in all symbols (0,…,13) of a slot | TBD | Yes | N/A |  | FFS: Per band or Per UE | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-20 | Support search space set configuration with freqMonitorLocation-r16 | 1. Support search space set configuration with freqMonitorLocations-r16 | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-20a | Support coreset configuration with rb-Offset | 1. Support coreset configuration with rb-Offset | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-23 | CGI reading on unlicensed cell [based on off-sync raster SSB] for ANR functionality | 1. Support acquisition of relevant information from a neighbouring NR unlicensed cell in an unlicensed carrier by reading the RMSI of the neighbouring unlicensed cell and reporting the acquired information to the network | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A | Support reading RMSI from an unlicensed cell [with an off-sync raster SSB] for ANR | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-25 | Enable configured UL transmissions when DCI 2_0 is configured but not detected | 1. Support configuration of enableConfiguredUL-r16 and enable transmission of higher-layer configured UL *SRS, PUCCH, CG-PUSCH etc) when DCI 2_0 is configured but not detected | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-27 | Wideband PRACH | - Enhanced PRACH design for NR-U by adopting a single long ZC sequence, with ZC sequence = 1151 for 15kHz and ZC sequence = 571 for 30kHz | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-29 | Support available RB set indicator field in DCI 2_0 | - Support monitoring DCI 2_0 to read availableRB-Sets-r16 | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-30 | Support channel occupancy duration indicator field in DCI 2_0 | - Support monitoring DCI 2_0 to read COT duration | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-8 | Type B PDSCH length {3, 5, 6, 8, [9, 10,] 11, 12, 13} without DMRS shift due to CRS collision | - Type B PDSCH length {3, 5, 6, 8, [9, 10,] 11, 12, 13} without DMRS shift due to CRS collision | 5-6a | Yes | N/A |  | FFS: Per band or Per UE | N/A | N/A | N/A | Note length 9/10 with DMRS shift due to CRS collision are already covered by 14-3 | Optional with capability signalling |
| 10. NR-unlicensed | 10-9 | Search space set group switching with explicit DCI 2_0 bit field trigger or with implicit PDCCH decoding with DCI 2_0 monitoring | 1. Two groups of search space sets 2. Monitor DCI 2_0 with a search space set switching field 3. Support switching the search space set group with PDCCH decoding in group 1 4. Support a timer to switch back to original search space set group 5. Monitor DCI 2_0 for channel occupancy time and use the end of channel occupancy time to switch back to the original search space set group | TBD | Yes | N/A |  | FFS: Per UE or per band or per BC | N/A | N/A | N/A | Being configured with two groups of search spaces, and switch between them. Some search space sets can be configured in both groups. | Optional with capability signalling |
| 10. NR-unlicensed | 10-9b | Search space set group switching with implicit PDCCH decoding without DCI 2_0 monitoring | 1. Two groups of search space sets 2. Support switching the search space set group with PDCCH decoding in group 1 3. Support a timer to switch back to original search space set group | TBD | Yes | N/A |  | FFS: Per UE or per band | N/A | N/A | N/A | Being configured with two groups of search spaces, and switch between them. Some search space sets can be configured in both groups. | Optional with capability signalling |
| 10. NR-unlicensed | 10-9c | Joint search space group switching across multiple cells | - Configured with a group of cells and switch search space set group jointly over these cells | one of {10-9, 10-9b} | Yes | N/A |  | FFS: Per UE or per band or per BC | N/A | N/A | N/A | Without this capability, the UE will switch search space set groups for different cells independently | Optional with capability signalling |
| 10. NR-unlicensed | 10-9d | Support Search space set group switching capability 2 | - Search space set group switching Capability-2: P=10/12/22 symbols for µ = 0/1/2 SCS | one of {10-9, 10-9b} | Yes | N/A |  | FFS: Per UE or per band | N/A | N/A | N/A | Without this capability, the UE supports search space set group switching capability-1: P=25/25/25 symbols for µ=0/1/2 | Optional with capability signalling |
| 10. NR-unlicensed | 10-14 | Non-numerical PDSCH to HARQ-ACK timing | - Support configuration of a value for dl-DataToUL-ACK indicating an inapplicable time to report HARQ ACK | TBD | Yes | N/A |  | FFS: Per UE or per band | N/A | N/A | N/A | If non-numerical K1 value is supported | Optional with capability signalling |
| 10. NR-unlicensed | 10-15 | Enhanced dynamic HARQ codebook | 1. Support of bit fields signalling PDSCH HARQ group index and NFI in DCI 1_1 (configuration of nfi-TotalDAI-Included) 2. Support of bit field in DCI 0_1 for other group total DAI if configured. (configuration of ul-TotalDAI-Included) 3. Support the retransmission of HARQ ACK (pdsch-HARQ-ACK-Codebook = enhancedDynamic-r16) | TBD | Yes | N/A |  | FFS: Per UE or per band | N/A | N/A | N/A | Enhanced dynamic HARQ codebook supporting grouping of HARQ ACK and triggering the retransmission of HARQ ACK in each groups | Optional with capability signalling |
| 10. NR-unlicensed | 10-16 | One-shot HARQ ACK feedback | - Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_1 scheduling a PDSCH - Support feedback of type 3 HARQ-ACK codebook , triggered by a DCI 1_1 without scheduling a PDSCH using a reserved FDRA value | TBD | Yes | N/A |  | FFS: Per band or Per UE | N/A | N/A | N/A | Upon triggering, UE reports A/N for all HARQ processes and all CCs in a PUCCH group. | Optional with capability signalling |
| 10. NR-unlicensed | 10-17 | Multi-PUSCH UL grant | 1. Support of scheduling up to 8 PUSCH with a single DCI 0_1 | TBD | Yes | N/A |  | FFS: Per band or Per UE | N/A | N/A | N/A |  | Optional with capability signalling |
| 10. NR-unlicensed | [10-19a] | [Support DL reception in a carrier with intra-cell guard-bands] | 1. [When DL BWP has multiple RB sets, support using the available RB set bitmap in DCI 2_0 to validate the periodic CSI-RS transmission if the CSI-RS is over multiple RB-sets] | TBD | Yes | N/A |  | Per band | N/A | N/A |  | Without this capability, UE will assume all RB sets in the DL BWP are all transmitted or none of them are transmitted | Optional with capability signalling |
| 10. NR-unlicensed | [10-19b] | [Support UL transmission with subset of RB sets passing LBT] | 1. [When UL BWP has multiple RB sets, support transmission of UL signal or channels when LBT passes for only the RB sets the UL signals or channels are located] | TBD | Yes | N/A |  | Per band | N/A | N/A |  | Without this capability, UE will transmit UL when all RB sets in the UL BWP pass LBT | Optional with capability signalling |
| 10. NR-unlicensed | 10-26 | CSI-RS based RLM for NR-U | [CSI-RS based RLM for NR-U] | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 10. NR-unlicensed | 10-26a | CSI-RS based RRM for NR-U | [CSI-RS based RRM for NR-U] | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 10. NR-unlicensed | [10-31] | [Support of CSI-RS measurements for CSI reporting and tracking without COT duration from DCI 2_0] | [· Perform CSI measurements for reporting and tracking using CSI-RS resources that are not within a COT duration indicated by DCI 2_0 · Note: This includes the cases when DCI 2_0 is not configured and when DCI 2_0 is configured but COT duration is not provided by either CO duration field or SFI.] | TBD | Yes | N/A |  | [Per band] | N/A | N/A |  |  | Optional with capability signaling |
| 10. NR-unlicensed | 10-3 | PRB interlace mapping for PUSCH | 1. PRB interlace frequency domain resource allocation for PUSCH | TBD One of {10-1, 10-1a} | Yes | N/A |  | Per band | N/A | N/A | N/A | Support of PRB interlace PUSCH | Optional with capability signalling |
| 10. NR-unlicensed | 10-3a | PRB interlace mapping for PUCCH | - PRB interlace frequency domain resource allocation for PUCCH format 0 and format 1 - PRB interlace frequency domain resource allocation for PUCCH format 2 - PRB interlace frequency domain resource allocation for PUCCH format 3 | TBD One of {10-1, 10-1a} | Yes | N/A |  | Per band | N/A | N/A | N/A | Support of PRB interlace PUCCH format 0/1 | Optional with capability signalling |
| 10. NR-unlicensed | 10-12 | OCC for PRB interlace mapping for PF2 and PF3 | 1. OCC2 2. OCC4 | 10-3a | Yes | N/A |  | Per band | N/A | N/A | N/A | UE OCC capability for EPF2/EFP3 | Optional with capability signalling |
| 10. NR-unlicensed | 10-13a | Extended CP range of more than one symbol for CG-PUSCH | - UE supports generating a CP extension of length longer than 1 symbol for Configured Grant PUSCH transmission | TBD One of {5-19, 5-20} | Yes | N/A |  | Per band | N/A | N/A | N/A | How long a UE can generate the CP extension beyond 1 symbol for CG-PUSCH | Optional with capability signalling |
| 10. NR-unlicensed | 10-18 | Configured grant with retransmission in CG resources | 1. Support retransmission in CG resources 2. Support configured grant retransmission timer 3. Support DFI monitoring 4. Support CG-UCI in CG-PUSCH | TBD One of {5-19, 5-20} | Yes | N/A |  | Per band | N/A | N/A | N/A | Support configured grant with retransmission in configured grant resource | Optional with capability signalling |
| 10. NR-unlicensed | 10-21a | Support using ED threshold given by gNB for UL to DL COT sharing | 1. Use ULtoDL-CO-SharingED-Threshold-r16 for cat 4 LBT for scheduled UL to share COT with gNB for DL 2. Use ULtoDL-CO-SharingED-Threshold-r16 for cat 4 LBT for CG-PUSCH to share COT with gNB for DL 3. Indicate in CG-UCI the COT sharing information | TBD 10-1 | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 10. NR-unlicensed | 10-24 | CG-UCI multiplexing with HARQ ACK | 1. Support multiplexing CG-UCI with HARQ ACK | 10-18 | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 10. NR-unlicensed | 10-28 | Configured grant with Rel-16 enhanced resource configuration | 1. Support configuration of resources with cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16, | TBD One of {5-19, 5-20} | Yes | N/A |  | FFS: Per UE or per band | N/A | N/A | N/A |  | Optional with capability signalling |