3GPP TSG-WG1 Meeting #101-E	R1-2004990

# E-meeting, May 25 – June 05, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | DRAFT | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on correction on power sharing mode for DAPS HO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_Enh-Core |  |  |  |  |  | Date: |  |  | 06-04-2020 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RRC parameter referenced for the DAPS HO power sharing mode configuration is wrong. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace UplinkPowersharingDAPS-HO with NR-DC-PC-mode for when referencing the MR-DC power control for DAPS HO and clarify the capability support for power sharing modes supported by the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated Impact Analysis: UE that has implemented this CR connected gNB that has not implemented this CR: - gNB expectation of UE behavior for DAPS HO is ambiguos due to incorrect specification. UE that has not implemented this CR connected to gNB that has implemented this CR: - UE behavior for DAPS HO is ambiguos due to incorrect specification. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 15	Dual active protocol stack based handover

If a UE indicates a capability for dual active protocol stack based handover (DAPS HO), the UE can be provided with a source MCG and a target MCG.

If a UE is configured with a target MCG and a source MCG using NR radio access in FR1 and/or in FR2, the UE is configured a maximum power PMCG for transmissions on the target MCG by p-DAPS-FR1 and/or by p-DAPS-FR2 and a maximum power PSCG for transmissions on the source MCG by p-DAPS-FR1 and/or by p-DAPS-FR2 and with an inter-CG power sharing mode by UplinkPowerSharingDAPS-HO-mode for FR1 and/or by UplinkPowerSharingDAPS-HO-mode for FR2. The UE determines a transmission power on the target MCG and a transmission power on the source MCG per frequency range.

If the UE indicates support for semi-static power sharing mode1 and is provided UplinkPowerSharingDAPS-HO-mode = Semi-static-mode1, the UE determines a transmission power for the target MCG or for the source MCG as described in Clause 7.6.2 for NR-DC-PC-mode = Semi-static-mode1 by considering the target MCG as the MCG and the source MCG as the SCG.

If the UE indicates support for semi-static power sharing mode2  and is provided UplinkPowerSharingDAPS-HO-mode = Semi-static-mode2, the UE determines a transmission power for the target MCG or for the source SCG as described in Clause 7.6.2 for NR-DC-PC-mode = Semi-static-mode2 by considering the target MCG as the MCG and the source MCG as the SCG.

If the UE indicates support for dynamic power sharing and is provided UplinkPowerSharingDAPS-HO-mode = Dynamic, the UE determines a transmission power for the target MCG or for the source MCG as described in Clause 7.6.2 for NR-DC-PC-mode = Dynamic by considering the target MCG as the MCG and the source MCG as the SCG.

If

-	the UE does not provide UplinkPowerSharingDAPS-HO, and

-	UE transmissions on the target cell and the source cell overlap

the UE transmits only on the target cell

UE transmissions on the target cell and the source cell overlap if they are in

-	overlapping time resources if the carrier frequencies for the target MCG and the source MCG are intra-frequency and intra-band

-	overlapping time resources and overlapping frequency resources if the carrier frequencies for the target MCG and the source MCG are not intra-frequency and intra-band

For intra-frequency DAPS HO operation, the UE expects that an active DL BWP and an active UL BWP on the target cell are within an active DL BWP and an active UL BWP on the source cell, respectively.

The UE can provide pdcch-BlindDetectionMCG1-UE to indicate a capability to monitor a maximum number of PDCCH candidates per slot that corresponds to Ncellstarget downlink cells for the target MCG and pdcch-BlindDetectionMCG2-UE to indicate a capability monitor a maximum number of PDCCH candidates per slot that corresponds to Ncellssource downlink cells for the source MCG. If the UE is provided search space sets on both the target MCG and the source MCG, the UE does not expect to have in any slot any USS set without allocated PDCCH candidates for monitoring on both the target MCG and the source MCG.

For DAPS operation in a same frequency band, a UE does not transmit PUSCH/PUCCH/SRS to the source MCG in a slot when the transmission would overlap in time with a PRACH transmission to the target MCG or when a gap between a first or last symbol of a PRACH transmission to the target MCG in a first slot would be separated by less than N symbols from a last or first symbol, respectively, of the PUSCH/PUCCH/SRS transmission to the source MCG in a second slot. N=2 for μ=0 or μ=1, N=4 for μ=2 or μ=3, and μ is the SCS configuration of the active UL BWP for the PUSCH/PUCCH/SRS transmission to source MCG.