3GPP TSG-WG1 Meeting #101-E	R1-2004991

# E-meeting, May 25 – June 05, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | DRAFT | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on correction on overlapping uplink transmissions for DAPS HO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_Enh-Core |  |  |  |  |  | Date: |  |  | 06-04-2020 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1. During DAPS HO, if the source and target cell transmission collide (based on collision definition defined in TS38.213 Section 15), UE transmit only in the target cell and source cell transmission. However, UE may not be able to processing the transmission cancellation if the target cell transmission is known to late. This issue is currently not addressed in the specification. 2. Definition of overlapping slots for PRACH and PUSCH/PUCCH/SRS could be ambiguous when target and source cell are synchronized 3. UE behaviour for when the UE needs to transmit PRACH in the source cell and PUSCH/PUCCH/SRS is missing from specification |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. UE cancels the transmission in the source cell if the scheduling of the target cell transmission is known specified time offset before source cell transmission. Otherwise UE is not expected to perform transmission cancellation. The changes are based on TP#1-16 from R1-2004957. Note this changes supercede (i.e. are an update of the) text agreed in RAN1 #100bis-e for the same paragraph. 2. Clarify the slot overlap for the synchronous case by deleting ‘when the transmision woud overlap’ and replace with ‘overlapping’. 3. Clarify that when PRACH transmission in source cell would be scheduled to occur in same slot or be separated by less than N symbols from PUSCH/PUCCH/SRS in target cell, UE is not required to transmit PRACH in source UL BWP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. UE may not be able to conform to specification, as due to transceiver processing latency, there could be some cases where collision of source and target cell transmission is only known after source cell transmission has already occurred. Therefore, the expected behavior by the gNB will not match practical UE implementation. 2. Specification ambiguity for synchronous DAPS HO cases. 3. Missing UE behavior for when UE trasnmits PRACH in source cell during DAPS HO. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated Impact Analysis: UE that has implemented this CR connected gNB that has not implemented this CR: - gNB may perform scheduling that the UE may drop or not drop source or target cell transmission in such collision event. If source cell transmission are dropped by the UE, no network and device impact is expected. In the worst case UE may drop the target cell transmission, which may result in some uplink performance degradation to the target cell. - UE may not be able to transmit PRACH to source cell when there is conflict but gNB may not be aware of this issue. This may result in some unexpected behavior by the UE in the network. UE that has not implemented this CR connected to gNB that has implemented this CR: - gNBs will not perform aggressive scheduling such that UE will need to handle unforseeable UL transmission collision cases. Therefore no significant impact to network and device is expected. - UE may be required to transmit PRACH in source cell and PUSCH/PUCCH/SRS in target cell simultaneously, which may lead to signal degradation of the transmitted signal. This may result in some performance loss of the transmited signal/channel. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 15	Dual active protocol stack based handover

If a UE indicates a capability for dual active protocol stack based handover (DAPS HO), the UE can be provided with a source MCG and a target MCG.

If a UE is configured with a target MCG and a source MCG using NR radio access in FR1 and/or in FR2, the UE is configured a maximum power PMCG for transmissions on the target MCG by p-DAPS-FR1 and/or by p-DAPS-FR2 and a maximum power PSCG for transmissions on the source MCG by p-DAPS-FR1 and/or by p-DAPS-FR2 and with an inter-CG power sharing mode by UplinkPowerSharingDAPS-HO-mode for FR1 and/or by UplinkPowerSharingDAPS-HO-mode for FR2. The UE determines a transmission power on the target MCG and a transmission power on the source MCG per frequency range.

If the UE indicates UplinkPowerSharingDAPS-HO = Semistatic-mode1 and is provided UplinkPowerSharingDAPS-HO-mode = Semi-static-mode1, the UE determines a transmission power for the target MCG or for the source MCG as described in Clause 7.6.2 for UplinkPowerSharingDAPS-HO = Semi-static-mode1 by considering the target MCG as the MCG and the source MCG as the SCG.

If the UE indicates UplinkPowerSharingDAPS-HO = Semistatic-mode2 and is provided UplinkPowerSharingDAPS-HO-mode = Semi-static-mode2, the UE determines a transmission power for the target MCG or for the source SCG as described in Clause 7.6.2 for UplinkPowerSharingDAPS-HO = Semi-static-mode2 by considering the target MCG as the MCG and the source MCG as the SCG.

If the UE indicates UplinkPowerSharingDAPS-HO = Dynamic and is provided UplinkPowerSharingDAPS-HO-mode = Dynamic, the UE determines a transmission power for the target MCG or for the source MCG as described in Clause 7.6.2 for UplinkPowerSharingDAPS-HO = Dynamic by considering the target MCG as the MCG and the source MCG as the SCG.

If

-	the UE does not provide UplinkPowerSharingDAPS-HO, and

-	UE transmissions on the target cell and the source cell overlap

the UE transmits only on the target cell, and cancels the transmission to source cell if the first symbol of source cell transmission is after Tproc,2+d. The UE does not expect to cancel the transmission on the source cell with first symbol that occurs, relative to a last symbol of a CORESET where the UE detects a DCI format scheduling a transmission on the target cell, after a number of symbols that is smaller than Tproc,2+d. Tproc,2 is the PUSCH preparation time for the corresponding PUSCH processing capability [6, TS 38.214] assuming d2,1 = 1 after a last symbol of a CORESET where the UE detects a DCI format scheduling the transmission on the target cell, d is the time duration of 2 symbols with SCS based on SCS configuration μ, and μ corresponds to the smallest SCS configuration among the SCS configuration of the PDCCH carrying the DCI format and the SCS configuration of the UE transmission on the source cell. If the UE transmits PRACH using 1.25 kHz or 5 kHz SCS on the source cell, the UE determines Tproc,2 assuming SCS configuration μ=0.

UE transmissions on the target cell and the source cell overlap if they are in

-	overlapping time resources if the carrier frequencies for the target MCG and the source MCG are intra-frequency and intra-band

-	overlapping time resources and overlapping frequency resources if the carrier frequencies for the target MCG and the source MCG are not intra-frequency and intra-band

For intra-frequency DAPS HO operation, the UE expects that an active DL BWP and an active UL BWP on the target cell are within an active DL BWP and an active UL BWP on the source cell, respectively.

The UE can provide pdcch-BlindDetectionMCG1-UE to indicate a capability to monitor a maximum number of PDCCH candidates per slot that corresponds to Ncellstarget downlink cells for the target MCG and pdcch-BlindDetectionMCG2-UE to indicate a capability monitor a maximum number of PDCCH candidates per slot that corresponds to Ncellssource downlink cells for the source MCG. If the UE is provided search space sets on both the target MCG and the source MCG, the UE does not expect to have in any slot any USS set without allocated PDCCH candidates for monitoring on both the target MCG and the source MCG.

For DAPS operation in a same frequency band, a UE does not transmit PUSCH/PUCCH/SRS to the source MCG in a slot overlapping in time with a PRACH transmission to the target MCG or when a gap between a first or last symbol of a PRACH transmission to the target MCG in a first slot would be separated by less than N symbols from a last or first symbol, respectively, of the PUSCH/PUCCH/SRS transmission to the source MCG in a second slot. For DAPS operation in a same frequency band, a UE does not transmit PRACH to the MCG in a slot overlapping in time with a PUSCH/PUCCH/SRS transmission to the target MCG or when a gap between the first or last symbol of a PUSCH/PUCCH/SRS transmission to the target MCG in a first slot is separated by less than N symbols from the last or first symbol, respectively, of a PRACH transmission to the source MCG in a second slot.  N=2 for μ=0 or μ=1, N=4 for μ=2 or μ=3, and μ is the SCS configuration of the active UL BWP for the PUSCH/PUCCH/SRS transmission to source MCG.