3GPP TSG RAN WG1 #101	R1-2005073

e-Meeting, May 25th – June 5th, 2020

Source:	moderator (vivo)

Title:	Outcome of [101-e-NR-unlic-NRU-CG-01]

Agenda Item:	7.2.2.2.4

Document for:	Discussion and Decision

Introduction

Agreement:

Adopt TP#1 in R1-2005002 (FL summary) for TS 38.212

Agreement:

Adopt TP#2 in R1-2005002 (FL summary) for TS 38.213

Agreement:

Adopt TP#3 in R1-2005002 (FL summary) for TS 37.213

TP proposals

TP for TS 38.212

Reason for change: in the case of multiple CG-PUSCH within the UE initiated COT, how does UE indicate the COT sharing information in CG-UCI is not clear

Summary of change: it is clarified that the UE should provide consistent COT sharing information in multiple CG-PUSCH within the UE initiated COT.

Consequences if not approved: UE behavior on indicating COT sharing information is unclear

Clauses affected: 38.212, section 6.3.2.1.3

-------------------------------------start---------------------------------------------

6.3.2.1.3	CG-UCI

For CG-UCI bits transmitted on a CG PUSCH, the CG-UCI bit sequence a0, a1, a2, a3, …,aA-1  is determined as follows:

-	set ai=oiCG-UCI  for i=0,1, …, OCG-UCI-1 and A=OCG-UCI, where the CG-UCI bit sequence o0CG-UCI, o1CG-UCI, …, oOCG-UCI-1CG-UCI is given by Table 6.3.2.1.3-1, mapped in the order from upper part to lower part.

Table 6.3.2.1.3-1: Mapping order of CG-UCI fields

| Field | Bitwidth |
| --- | --- |
| HARQ process number | 4 |
| Redundancy version | 2 |
| New data indicator | 1 |
| Channel Occupancy Time (COT) sharing information | log2C if both higher layer parameter ULtoDL-CO-SharingED-Threshold-r16 and higher layer parameter cg-COT-SharingList-r16 are configured, where C is the number of combinations configured in cg-COT-SharingList-r16; 1 if higher layer parameter ULtoDL-CO-SharingED-Threshold-r16 is not configured and higher layer parameter cg-COT-SharingOffset-r16 is configured; 0 otherwise; If a UE indicates COT sharing other than “no sharing” in a CG PUSCH within the UE's initiated COT, the UE should provide consistent COT sharing information in all the subsequent CG PUSCHs, if any, occurring within the same UE’s initiated COT such that the same DL starting point and duration are maintained. |

-------------------------------------end---------------------------------------------

TP for TS 38.213 section

Reason for change: according to agreement as shown below HARQ-ACK feedback for CG-PUSCH in DFI is per TB. Current spec doesn’t reflect the agreement

Agreement:

- For DFI design for configured grants, support at least the following

- DFI including at least TB level HARQ-ACK bitmap for all UL HARQ processes

- Note: Total number of HARQ processes is as defined in Rel-15

- FFS: CBG level HARQ-ACK feedback, if supported

Summary of change: It is clarified in the spec that for initial transmission of CG-PUSCH HARQ-ACK feedback in DFI is per TB

Consequences if not approved: interpretation of HARQ-ACK feedback in DFI is ambiguous.

Clauses affected: 38.213, section 10.5

------------------------------------------------start----------------------------------------------------

10.5	HARQ-ACK information for PUSCH transmissions

A UE can be configured a number of search space sets to monitor PDCCH for detecting a DCI format 0_1 with CRC scrambled with a CS-RNTI provided by cs-RNTI. The UE determines that the DCI format provides HARQ-ACK information for PUSCH transmissions based on an XYZ field value, as described in [5, TS 38.212], if a PUSCH transmission is configured by ConfiguredGrantConfig or is an activated PUSCH transmission configured by semiPersistentOnPUSCH.

The HARQ-ACK information corresponds to transport blocks in PUSCH transmissions for a number of HARQ processes provided by nrofHARQ-Processes for a serving cell of a PDCCH reception that provides DCI format 1_0 or, if DCI format 1_0 includes a carrier indicator field, for a serving cell indicated by a value of the carrier indicator field.

For a PUSCH transmission configured by ConfiguredGrantConfig or for activated PUSCH transmissions configured by semiPersistentOnPUSCH, HARQ-ACK information for a transport block of a corresponding HARQ process number is valid if a first symbol of the PDCCH reception is after a last symbol of the PUSCH transmission, or of any repetition of the PUSCH transmission, by a number of symbols provided by cg-minDFIDelay-r16.

For an initial PUSCH transmission configured by ConfiguredGrantConfig, if a UE is provided PUSCH-CodeBlockGroupTransmission for a serving cell, a value of HARQ-ACK information for a transport block of a corresponding HARQ process number in CG-DFI is ACK if HARQ-ACK for the transport block is ACK; otherwise, a value of HARQ-ACK information is NACK.

For a PUSCH transmission scheduled by a DCI format, HARQ-ACK information for a transport block of a corresponding HARQ process number is valid if a first symbol of the PDCCH reception is after a last symbol of the PUSCH transmission or, if the PUSCH transmission is over multiple slots,

-	after a last symbol of the PUSCH transmission in a first slot from the multiple slots by a number of symbols provided by cg-minDFIDelay-r16, if a value of the HARQ-ACK information is ACK.

-	after a last symbol of the PUSCH transmission in a last slot from the multiple slots by a number of symbols provided by cg-minDFIDelay-r16, if a value of the HARQ-ACK information is NACK.

----------------------------------------------end-------------------------------------------------------

TP for TS 37.213 section

Reason for change:  editorial correction, HARQ-ACK is feedback not transmission

Summary of change: the word “transmission” is changed to “feedback”

Consequences if not approved:

Clauses affected: 37.213, section 4.2.2.2

----------------------------------------- Start -------------------------------------------------------------

*** Unchanged text is omitted ***

4.2.2.2 	Contention window adjustment procedures for UL transmissions scheduled/configured by gNB

If a UE transmits transmissions using Type 1 channel access procedures that are associated with channel access priority class p on a channel, the UE maintains the contention window value CWp and adjusts CWp for those transmissions before step 1 of the procedure described in subclause 4.2.1.1, using the following steps:

1)	For every priority class p∈1,2,3,4, set CWp=CWmin,p;

2)	If HARQ-ACK feedback is available after the last update of CWp,  go to step 3. Otherwise, if the UE transmission after procedure described in subclause 4.2.1.1 does not include a retransmission or is transmitted within a duration Tw from the end of the reference duration corresponding to the earliest UL transmission burst after the last update of CWp transmitted after the procedures described in subclause 4.1.1, go to step 5; otherwise go to step 4.

3)	The HARQ-ACK feedback(s) corresponding to PUSCH(s) in the reference duration for the latest UL transmission burst for which HARQ-ACK feedback is available is used as follows:

a.	If at least one HARQ-ACK feedback is 'ACK' for PUSCH(s) with transport block (TB) based feedback transmissions or at least 10% of HARQ-ACK feedbacks is 'ACK' for PUSCH(s) with code block group (CBG) based feedback transmissions go to step 1; otherwise go to step 4.

4)	Increase CWp for every priority class p∈1,2,3,4 to the next higher allowed value;

5)	For every priority class p∈1,2,3,4, maintain CWp as it is; go to step 2.

*** Unchanged text is omitted ***

-------------------------------------end-----------------------------------------

- Reference

[1] R1-2005002, “Feature lead summary#3 on [101-e-NR-unlic-NRU-CG-01]”, vivo, RAN1#101-e