3GPP TSG RAN WG1 #101-e						   	                R1-2005077

May 25th – June 5th, 2020

Agenda item:	7.2.5.4

Source: 	Moderator (Qualcomm)

Title: 	Endorsed TPs of email discussion [101-e-NR-L1enh-URLLC-HARQ&Scheduling-03]

Document for:	Discussion and Decision

# Introduction

This document provides text proposals for the list of issues identified during the preparation pertaining to the HARQ and scheduling aspects (AI 7.2.5.4) of NR eURLLC.

Email Discussion #3 by 5/29 and corresponding TP (if any) by 6/5 – Kianoush (Qualcomm):

- Issue #1: CPU release with uplink interruption

- Issue #2: priority of A-SRS

# Text Proposal to Capture CPU Occupancy Time

| Reason for change: | Define the occupancy time duration for CPUs |
| --- | --- |
| Summary of change: | The occupied CPUs are remained occupied until the last symbol of the scheduled/configured PUCCH/PUSCH reporting CSI |
| Consequences if not approved: | Ambiguity between a UE and a gNB for reporting CSI |

Modified clause (Section 5.2.1.6 of TS 38.214). For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity not set to 'none', the CPU(s) are occupied for a number of OFDM symbols as follows: - A periodic or semi-persistent CSI report (excluding an initial semi-persistent CSI report on PUSCH after the PDCCH triggering the report) occupies CPU(s) from the first symbol of the earliest one of each CSI-RS/CSI-IM/SSB resource for channel or interference measurement, respective latest CSI-RS/CSI-IM/SSB occasion no later than the corresponding CSI reference resource, until the last symbol of the configured PUSCH/PUCCH carrying the report. - An aperiodic CSI report occupies CPU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the report. - An initial semi-persistent CSI report on PUSCH after the PDCCH trigger occupies CPU(s) from the first symbol after the PDCCH until the last symbol of the scheduled PUSCH carrying the report. End.

# Text Proposal to Capture A-SRS Priority

| Reason for change: | Assigning a low priority to aperiodic SRS |
| --- | --- |
| Summary of change: | A-SRS is of low priority |
| Consequences if not approved: | Ambiguity between a UE and a gNB for intra-UE cancellation involving an A-SRS |

Modified clause (Section 6.2.1 of TS 38.214). If a UE is not configured with [intraUEPrioritization] and a PUSCH with a priority index 0 and SRS are transmitted in the same slot on a serving cell, the UE may only be configured to transmit SRS after the transmission of the PUSCH and the corresponding DM-RS. If a UE is configured with [intraUEPrioritization] and a PUSCH transmission with a priority index 1 or a PUCCH transmission with a priority index 1 would overlap in time with an SRS transmission on a serving cell, the UE does not transmit the SRS in the overlapping symbol(s). End.