3GPP TSG RAN WG1 Meeting #101-e	R1-200xxxx

E-meeting, May 25 – June 5, 2020

Agenda Item:	6.2.2.4

Source:	Moderator(Huawei)

Title:	Feature lead summary on 101-e-LTE-NB_IoTenh3-Coex-NR-01

Document for:	Discussion and Decision

Introduction

This documents provides the proposals and summary of discussions of the following second phase email discussion for identified issues of NB-IoT co-existence with NR [5].

[101-e-LTE-NB_IoTenh3-Coex-NR-01] DCI size misalignment by 5/29 – Yubo (Huawei)

- Issues #1 of R1-2004704

Discussion

## Issue #1: DCI size misalignment

For resource reservation, [2] finds that the DCI N0 size will be one bit less than that of DCI N1 if only downlink reserved resource is configured and uplink reserved resource is not configured. So [2] proposes to append zeros to DCI N0 to align the size of N0 and N1 in the same search space. And [2] also proposes to correct the parameter names in RAN1 spec. One TP is proposed to correct it.

In [4], it is also observed that since the resource reservation and multi-TB scheduling are introduced in Rel-16, if these two features are configured separately for UL and DL, the DCI size of format N1 will be larger than that of format N0. One TP is proposed to correct it.

The corresponding TP proposed by [2] is below.

-----------------------------------------------------Start of Text Proposal for 212-----------------------------------

#### 6.4.3.1	DCI Format N0

Unchanged part is omitted

-	Resource reservation – 1 bit as defined in x.x of [3]. This field is only present if higher layer parameter valid-subframe-config-UL or slot-reserved-resource-config-UL is configured and the CRC of the DCI is scrambled by C-RNTI (except during random access) or SPS C-RNTI.

If the number of information bits in format N0 in the UE specific search space given by the C-RNTI is less than that of format N1 in the same search space, zeros shall be appended to format N0 until the payload size equals that of format N1 in the same search space.

------------------------------------------------------End of Text Proposal for 212------------------------------------

The corresponding TP proposed by [4] is below.

TP to TS 36.212, section 6.4.3.2:

<Unchanged parts are omitted>

If the number of information bits in format N1 is less than that of format N0 and the format N1 CRC is not scrambled by G-RNTI, zeros shall be appended to format N1 until the payload size equals that of format N0. If the number of information bits in format N1 is larger than that of format N0 and the format N1 CRC is not scrambled by G-RNTI, zeros shall be appended to format N0 until the payload size equals that of format N1.

<Unchanged parts are omitted>

| Companies | Comments |
| --- | --- |
| Ericsson | We prefer the TP from [2]. It seems most reasonable to add the new sentence in 6.4.3.1, where DCI format N0 is defined. The same approach is used for e.g. DCI formats 0 and 1. |
| ZTE | We think both TPs can work. If same approach as DCI format 0 is considered, adding the new sentence in 6.4.3.1 is better. |

Summary

- References

- R1-1913595, “RAN1 agreements for Rel-16 Additional Enhancements for NB-IoT”, Futurewei, Reno, USA, November 2019.

- R1-2003538	Corrections on coexistence of NB-IoT with NR,	Huawei, HiSilicon

- R1-2003798	Remaining issues on NB-IoT resource reservation,	ZTE

- R1-2003797, Remaining issues on scheduling enhancement for NB-IoT, ZTE

- R1-2004704 Feature lead summary on coexistence of NB-IoT with NR, Moderator (Huawei), E-meeting, May 2020.