3GPP TSG RAN WG1 Meeting #101-e	  R1-20xxxxx

E-meeting, 25 May – 5 June, 2020

Agenda Item:	7.2.5.1

Source:	Moderator (Huawei)

Title:	Email discussion/approval #3 on remaining issues and miscellaneous corrections on enhanced PDCCH monitoring capability

Document for:	Discussion and Decision

Introduction

The email discussion is to discuss the remaining issues and miscellaneous corrections on enhanced PDCCH monitoring capability.

[101-e-NR-L1enh-URLLC-PDCCH enhancements-03] Email discussion/approval on remaining issues and miscellaneous corrections on enhanced PDCCH monitoring capability by 5/29 and corresponding TP (if any) by 6/5 – Chengyan (Huawei) including

- Issue B-1: Minimum value of (pdcch-BlindDetectionCA-R15 + pdcch-BlindDetectionCA-R16) for case 3

- Issue D-1: Correction on determination of a combination (X, Y) for PDCCH monitoring in section 10.1 in TS 38.213

- Issue D-2: PDCCH candidates allocation for primary cell in a slot or a span

- Issue C-6: Text proposal for Rel-16 PDCCH monitoring capability for NR-DC in section 10 in TS 38.213

This document summarizes the above issue and provide some initial proposals for discussion. Companies are encouraged to provide the first round views by 5/26, then we can adjust the proposals and prepare the TPs for the next step discussions.

Enhanced PDCCH monitoring capability

This section summarize the issues on enhanced PDCCH monitoring capability for email discussion #3.

Remaining issues on reporting enhanced PDCCH monitoring capability

- Issue B-1: Minimum value of (pdcch-BlindDetectionCA-R15 + pdcch-BlindDetectionCA-R16) for case 3

In the RAN1#100bis-e meeting, the following agreement was made:

Agreements: For one reported combination of (pdcch-BlindDetectionCA-R15, pdcch-BlindDetectionCA-R16) for CA:  The minimum value of pdcch-BlindDetectionCA-R15 is 1 and the minimum value of pdcch-BlindDetectionCA-R16 is 1  [3]<=pdcch-BlindDetectionCA-R15 + pdcch-BlindDetectionCA-R16 <=16  Candidate values for pdcch-BlindDetectionCA-R15 is 1 to 15  Candidate values for pdcch-BlindDetectionCA-R16 is 1 to 15

Some companies provide views on the above FFS and the position is summarized as below:

Option 1: Update the agreement from RAN1#100b-e as below by removing the bracket.

Agreements: For one reported combination of (pdcch-BlindDetectionCA-R15, pdcch-BlindDetectionCA-R16) for CA:  The minimum value of pdcch-BlindDetectionCA-R15 is 1 and the minimum value of pdcch-BlindDetectionCA-R16 is 1  [3]<=pdcch-BlindDetectionCA-R15 + pdcch-BlindDetectionCA-R16 <=16  Candidate values for pdcch-BlindDetectionCA-R15 is 1 to 15  Candidate values for pdcch-BlindDetectionCA-R16 is 1 to 15

- Support: Huawei/HiSilicon, Ericsson, Intel, NTT DOCOMO

- Reasons:

  - A UE supporting 4 Rel-15 CCs should be able to support 1 Rel-16 CC and 2 Rel-15 CC.

  - The BD/CCE limits are gated by the span durations and span gaps, implying that the peak loading from Rel-16 monitoring can be managed much better than that for Rel-15 monitoring wherein all the BD/CCEs may be located in a single span in a slot.

Feature lead view: The reasons given by the proponents of option 1 look reasonable. In addition, based on the discussion in RAN1#100bis-e, almost all companies support option 1 also.

Proposal 2.1-1: Update the agreement below by removing the bracket:

Agreements: For one reported combination of (pdcch-BlindDetectionCA-R15, pdcch-BlindDetectionCA-R16) for CA:  The minimum value of pdcch-BlindDetectionCA-R15 is 1 and the minimum value of pdcch-BlindDetectionCA-R16 is 1  [3]<=pdcch-BlindDetectionCA-R15 + pdcch-BlindDetectionCA-R16 <=16  Candidate values for pdcch-BlindDetectionCA-R15 is 1 to 15  Candidate values for pdcch-BlindDetectionCA-R16 is 1 to 15

Please comment if you have strong concern on the above proposal 2.1-1.

| Company | View |
| --- | --- |

Remaining issues on scaling PDCCH monitoring capability if the number of CCs configured is larger than the reported capability

- Issue C-6: Text proposal for Rel-16 PDCCH monitoring capability for NR-DC in section 10 in TS 38.213

In Rel-15, the PDCCH monitoring capability for CA was extended to Rel-15 NR-DC. In the RAN1#100bis-e meeting, it was agreed for carrier aggregation that the minimum number of cells to be supported with Rel-16 PDCCH monitoring capability is 2. For Case 3 (the mixed capability monitoring) the UE can report one or more combinations of Rel-15 #CC(s) and Rel-16 #CC(s). In the current section 10 in TS 38.213, the UE behavior for Rel-16 PDCCH monitoring in case of NR-DC is still missing. Some TPs are needed for section 10 in TS 38.213.

Feature lead: This issue needs to be solved. Since there is no any input on this issue in the papers, I made the following questions for further discussion.

Question C-6-1: Whether/how to introduce new RRC parameters to indicate the reference number of cells for PDCCH blind detection for a CG for NR-DC operation with the following cases in Rel-16?

    Case 1: A UE configured with all serving cell(s) using Rel-15 PDCCH monitoring capability

    Case 2: A UE configured with all serving cell(s) using Rel-16 PDCCH monitoring capability

    Case 3: A UE configured with Rel-15 monitoring capability and Rel-16 monitoring capability on different serving cell

In Rel-15, as shown in the following text in section 10 in TS 38.213, pdcch-BlindDetection was introduced to indicate the reference number of cells for PDCCH blind detection for the CG.

Section 10 in TS 38.213 When a UE is configured for NR-DC operation, the UE determines a capability to monitor a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs per slot that corresponds to downlink cells for the MCG where is provided by pdcch-BlindDetection for the MCG and determines a capability to monitor a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs per slot that corresponds to downlink cells for the SCG where is provided by pdcch-BlindDetection for the SCG. When the UE is configured for carrier aggregation operation over more than 4 cells, or for a cell group when the UE is configured for NR-DC operation, the UE does not expect to monitor per slot a number of PDCCH candidates or a number of non-overlapped CCEs that is larger than the maximum number as derived from the corresponding value of . When a UE is configured for NR-DC operation with a total of downlink cells on both the MCG and the SCG, the UE expects to be provided pdcch-BlindDetection for the MCG and pdcch-BlindDetection for the SCG with values that satisfy - pdcch-BlindDetection for the MCG + pdcch-BlindDetection for the SCG <= pdcch-BlindDetectionCA, if the UE reports pdcch-BlindDetectionCA, or - pdcch-BlindDetection for the MCG + pdcch-BlindDetection for the SCG <= , if the UE does not report pdcch-BlindDetectionCA.

Feature lead:

    For case 1, the Rel-15 RRC parameter and the corresponding constraints can be reused.

    For case 2, in theory we can reuse the R15 RRC parameter pdcch-BlindDetection also, since the candidate value keeps the same. However, considering for case 3 it seems reasonable to indicate a new RRC parameter for cells with Rel-16 PDCCH monitoring capability, it seems more reasonable to reuse the new RRC parameter for case 3. That is a new RRC parameter pdcch-BlindDetection-r16 can be introduced for case 2.

    For case 3, since a CG may be configured with Rel-15 monitoring capability and Rel-16 monitoring capability on different serving cells, and we agreed to have different CC limits for the group with Rel-15 monitoring capability and the group with Rel-16 monitoring capability, it seems reasonable to introduce new RRC parameter.

.

Proposal 2.2-1: For a UE configured with NR-DC operation with a total of downlink cells on both the MCG and the SCG, including at least one downlink cell using Rel-16 PDCCH monitoring capability,

    Introduce a new RRC parameter pdcch-BlindDetection-r16 to indicate the reference number of cells for PDCCH blind detection for a group of serving cells using Rel-16 PDCCH monitoring capability in a CG.

    Introduce a new RRC parameter pdcch-BlindDetection-r15 to indicate the reference number of cells for PDCCH blind detection for a group of serving cells using Rel-15 PDCCH monitoring capability in a CG.

   pdcch-BlindDetection-r16 for the MCG + pdcch-BlindDetection-r16 for the SCG <= pdcch-BlindDetectionCA-r16, if the UE reports pdcch-BlindDetectionCA-r16, or pdcch-BlindDetection-r16 for the MCG + pdcch-BlindDetection-r16 for the SCG <= NNR-DCDL,cells, if the UE does not report pdcch-BlindDetectionCA-r16

   pdcch-BlindDetection-r15 for the MCG + pdcch-BlindDetection-r15 for the SCG <= pdcch-BlindDetectionCA-r15, if the UE reports pdcch-BlindDetectionCA-r15, or pdcch-BlindDetection-r15 for the MCG + pdcch-BlindDetection-r15 for the SCG <= NNR-DCDL,cells, if the UE does not report pdcch-BlindDetectionCA-r15

Please provide your views on the above proposal 2.2-1.

| Company | View |
| --- | --- |

Question C-6-2: Whether/how to report the capability of CC limits for NR-DC operation separately from CA operation for the following cases?

    Case 1: A UE configured with all serving cell(s) using Rel-15 PDCCH monitoring capability

    Case 2: A UE configured with all serving cell(s) using Rel-16 PDCCH monitoring capability

    Case 3: A UE configured with Rel-15 monitoring capability and Rel-16 monitoring capability on different serving cell

In Rel-15, as shown in the following text in section 10 in TS 38.213, UE may report the CC limit for MCG and SCG separately.

Section 10 in TS 38.213 For NR-DC operation, the UE may indicate, through pdcch-BlindDetectionMCG-UE and pdcch-BlindDetectionSCG-UE, respective maximum values for pdcch-BlindDetection for the MCG and pdcch-BlindDetection for the SCG. If the UE reports pdcch-BlindDetectionCA, - the value range of pdcch-BlindDetectionMCG-UE or of pdcch-BlindDetectionSCG-UE is [1, …, pdcch-BlindDetectionCA-1], and - pdcch-BlindDetectionMCG-UE + pdcch-BlindDetectionSCG-UE >= pdcch-BlindDetectionCA. Otherwise, if is a maximum total number of downlink cells that the UE can be configured on both the MCG and the SCG as described in [10, TS 38.133], - the value range of pdcch-BlindDetectionMCG-UE or of pdcch-BlindDetectionSCG-UE is [1, 2, 3], - pdcch-BlindDetectionMCG-UE + pdcch-BlindDetectionSCG-UE >= .

Companies are encouraged to provide your views on the above question.

| Company | View |
| --- | --- |

Miscellaneous corrections

- Issue D-1: Correction on determination of a combination (X, Y) for PDCCH monitoring in section 10.1 in TS 38.213

Samsung (R1-2002131) proposes to adopt the following TP, to cover the case that a configuration of search space sets for PDCCH monitoring results to a separation of every two consecutive PDCCH monitoring spans that is equal to or larger than the value of X for only one of the multiple combinations X,Y.

If the UE indicates a capability to monitor PDCCH according to multiple X,Y combinations and a configuration of search space sets to the UE for PDCCH monitoring on a cell results to a separation of every two consecutive PDCCH monitoring spans that is equal to or larger than the value of X for one or more combinations X,Y of the multiple combinations X,Y, the UE is expected to monitor PDCCH on the cell according to the combination X,Y from the one or more combinations X,Y associated with the largest maximum number of CPDCCHmax,X,Y,μ and MPDCCHmax,X,Y,μ.

Feature lead view: the current spec should be clear even without the changes, but no harm to make it clear. If time permits, we can discuss and endorse the TP.

Proposal 2.3-1: Adopt the following TP on determination of a combination (X, Y) for PDCCH monitoring in section 10.1 in TS 38.213.

If the UE indicates a capability to monitor PDCCH according to multiple X,Y combinations and a configuration of search space sets to the UE for PDCCH monitoring on a cell results to a separation of every two consecutive PDCCH monitoring spans that is equal to or larger than the value of X for one or more combinations X,Y of the multiple combinations X,Y, the UE is expected to monitor PDCCH on the cell according to the combination X,Y from the one or more combinations X,Y associated with the largest maximum number of CPDCCHmax,X,Y,μ and MPDCCHmax,X,Y,μ.

Please provide your views on the above proposal 4.3-1.

| Company | View |
| --- | --- |

- Issue D-2: PDCCH candidates allocation for primary cell in a slot or a span

Sharp R1-2004331 - Issue 3: PDCCH candidates allocation for primary cell in a slot or a span Given the PDCCH overbooking would happen in a primary cell, the eURLLC UEs need to allocate PDCCH candidates for monitoring to USS sets for the primary cell in a slot level or a span level according to the PDCCH monitoring capability configuration. The relevant description in TS 38.213[4] has been captured as below. - 10.1 UE procedure for determining physical downlink control channel assignment The UE allocates PDCCH candidates for monitoring to USS sets for the primary cell having an active DL BWP with SCS configuration in a slot if the UE is not provided PDCCHMonitoringCapabilityConfig for the primary cell or if the UE is provided PDCCHMonitoringCapabilityConfig = R15 PDCCH monitoring capability for all serving cells, or in a span if the UE is provided PDCCHMonitoringCapabilityConfig = R16 PDCCH monitoring capability for the primary cell, according to the following pseudocode. If for the USS sets for scheduling on the primary cell the UE is not provided CORESETPoolIndex for first CORESETs, or is provided CORESETPoolIndex with value 0 for first CORESETs, and is provided CORESETPoolIndex with value 1 for second CORESETs, and if minγ∙MPDCCHmax,slot,μ,MPDCCHtotal,slot,μ>minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ or minγ∙CPDCCHmax,slot,μ,CPDCCHtotal,slot,μ>minCPDCCHmax,slot,μ,CPDCCHtotal,slot,μ, the following pseudocode applies only to USS sets associated with the first CORESETs. A UE does not expect to monitor PDCCH in a USS set without allocated PDCCH candidates for monitoring.. The above cases that the UE allocates PDCCH candidates for monitoring to USS sets for the primary cell in a slot, correspond to the Rel-15 UE and the eURLLC UE being configured all serving cells with Rel-15 monitoring capability. However, for an eURLLC UE, who is configured a number of serving cells with Rel-15 monitoring capability and Rel-16 monitoring capability on different serving cells, there is a case that the primary cell is configured with Rel-15 monitoring capability. The current description on the TS38.213 seems to miss capturing the case. Proposal 3: Adopt the following TP3 for section 10.1 in TS 38.213 to compensate for a missing case of a UE being configured with Rel-15 monitoring capability and Rel-16 monitoring capability on different serving sells and the primary cell is configured with Rel-15 monitoring capability. TP3 TS 38.213 V16.1.0 (2020-03) - 10.1 UE procedure for determining physical downlink control channel assignment < Unchanged parts are omitted > The UE allocates PDCCH candidates for monitoring to USS sets for the primary cell having an active DL BWP with SCS configuration in a slot if the UE is not provided PDCCHMonitoringCapabilityConfig for the primary cell or if the UE is provided PDCCHMonitoringCapabilityConfig = R15 PDCCH monitoring capability for all serving cells the primary cell, or in a span if the UE is provided PDCCHMonitoringCapabilityConfig = R16 PDCCH monitoring capability for the primary cell, according to the following pseudocode. If for the USS sets for scheduling on the primary cell the UE is not provided CORESETPoolIndex for first CORESETs, or is provided CORESETPoolIndex with value 0 for first CORESETs, and is provided CORESETPoolIndex with value 1 for second CORESETs, and if minγ∙MPDCCHmax,slot,μ,MPDCCHtotal,slot,μ>minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ or minγ∙CPDCCHmax,slot,μ,CPDCCHtotal,slot,μ>minCPDCCHmax,slot,μ,CPDCCHtotal,slot,μ, the following pseudocode applies only to USS sets associated with the first CORESETs. A UE does not expect to monitor PDCCH in a USS set without allocated PDCCH candidates for monitoring. < Unchanged parts are omitted >.

Feature lead view: The correction is needed. If time permits, we can discuss it.

Proposal 2.3-2: Adopt the following TP for section 10.1 in TS 38.213.

- 10.1 UE procedure for determining physical downlink control channel assignment < Unchanged parts are omitted > The UE allocates PDCCH candidates for monitoring to USS sets for the primary cell having an active DL BWP with SCS configuration in a slot if the UE is not provided PDCCHMonitoringCapabilityConfig for the primary cell or if the UE is provided PDCCHMonitoringCapabilityConfig = R15 PDCCH monitoring capability for all serving cells the primary cell, or in a span if the UE is provided PDCCHMonitoringCapabilityConfig = R16 PDCCH monitoring capability for the primary cell, according to the following pseudocode. If for the USS sets for scheduling on the primary cell the UE is not provided CORESETPoolIndex for first CORESETs, or is provided CORESETPoolIndex with value 0 for first CORESETs, and is provided CORESETPoolIndex with value 1 for second CORESETs, and if minγ∙MPDCCHmax,slot,μ,MPDCCHtotal,slot,μ>minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ or minγ∙CPDCCHmax,slot,μ,CPDCCHtotal,slot,μ>minCPDCCHmax,slot,μ,CPDCCHtotal,slot,μ, the following pseudocode applies only to USS sets associated with the first CORESETs. A UE does not expect to monitor PDCCH in a USS set without allocated PDCCH candidates for monitoring. < Unchanged parts are omitted >

Please provide your views on the above proposal 2.3-2.

| Company | View |
| --- | --- |