3GPP TSG RAN WG1 #101			R1-2004713

e-Meeting, May 25th – June 5th, 2020

Agenda item:	7.2.6.1

Source: 	Moderator (Samsung)

Title: 	Feature lead summary for MU-MIMO CSI thread #2

Document for:	Discussion and Decision

- Introduction

# Based on the discussion during the preparation phase summarized in [1] and the Phase-2 instruction from the MIMO chairman, the following issue is to be discussed in thread #2 for MU CSI:

Discuss the following issue: - Examine when the UE reports useless information (bitmap and reference amplitude are reported even if LCC is absent) for 3 out of 8 supported parameter combinations and whether this optimization is significantly beneficial and should be addressed by specification - If so, modify the definition of K0=maxx,β2LM1 to ensure that a UE can report one NZC per polarization for rank ν≤4. Two alternatives to be discussed are: - Alt 1: x=max2,ν - Alt 2: x=4 - Possible conclusion - There is no consensus in supporting the proposals in N.1, N.2, N.3, N.4-1a/2a/2b, N.5, N.6, and N.7 as summarized in Table 2 [of [1]]

# Issue 3 (“possible conclusion”) has been discussed at length in Phase-1 and the content has been agreeable to the group. Therefore, the group is to focus on issues 1 and 2 which are pertinent to N.4-1b.

# It is quite apparent that issue 1 and 2 are inter-dependent. Whether issue 2 needs to be discussed depends on whether the problem raised in issue 1 requires spec-based solution(s). Conversely, the need for spec-based solution(s) in issue 1 should be based on the assessment of the two alternatives given in issue 2 (Alt1 and Alt2 for the value of x) since only such alternatives are to be considered if a spec-based solution is to be adopted. Therefore, issue 1 and 2 cannot be decoupled and should be decided jointly.

# In addition, given that proposal N.4-1b is non-essential and an optimization, more stringent assessment based on a properly identified core issue should be made. In this case, the core issue is precisely the absence of LCC with the presence of bitmap and reference amplitude in an instant of eType-II CSI reporting. To assess the seriousness of this core issue, each interested company is encouraged to:

- state the scenarios in which the core issue occurs,

- assess the importance of each of those scenarios,

- assess the benefit of the two alternatives, and

- state whether a spec-based solution is justified and, if so, which one

- Summary

Interested companies are to provide their abridged views in the following table.

- Table 1 Abridged views

| Company | Position |  |  |
| --- | --- | --- | --- |
|  | Spec-based solution not needed | Spec-based solution needed: Alt 1 | Spec-based solution needed: Alt 2 |

In the table below, interested companies are to provide more detailed views based on the four points in section 1.

- Table 2 More detailed views

| Company | Comment |
| --- | --- |

...

- Conclusion

Based on the above inputs and discussion, the following FL proposals are made:

- The following is to be captured as a conclusion: - There is no consensus in supporting the proposals in N.1, N.2, N.3, N.4-1a/2a/2b, N.5, N.6, and N.7 as summarized in Table 2 of [1] (R1-2003880) - …

# References

- R1-2003880	Feature lead summary for MU-MIMO CSI	Moderator (Samsung)