3GPP TSG RAN WG1 Meeting #102-e	                                                  	                   R1-2005247

E-meeting, August 17 – 28, 2020

Agenda Item:	8.1.3

Source:	Huawei, HiSilicon

Title:	Enhancements on SRS for Rel-17

Document for:	Discussion and Decision

- Introduction

During the discussion before and during RAN# 86, the need to further enhance SRS was proposed and the following WID was approved:

- Enhancement on SRS, targeting both FR1 and FR2:

  - Identify and specify enhancements on aperiodic SRS triggering to facilitate more flexible triggering and/or DCI overhead/usage reduction

  - Specify SRS switching for up to 8 antennas (e.g., xTyR, x = {1, 2, 4} and y = {6, 8})

  - Evaluate and, if needed, specify the following mechanism(s) to enhance SRS capacity and/or coverage: SRS time bundling, increased SRS repetition, partial sounding across frequency

The enhancements include flexibility of aperiodic SR triggering, supporting more types of antenna switching, and enhancements on SRS capacity/coverage. We will share some thoughts on those aspects and propose some potential directions for further discussion.

- Enhancement on aperiodic SRS triggering

In NR Rel-15, aperiodic SRS is specified to introduce flexibility of acquiring in-time UL/DL CSI. It can be, e.g., triggered between two periodic SRS transmission in case the periodicity is too long, so as to maintain in-time CSI measurement. The mechanism is that RRC configures a slotOffset for a SRS resource set, and when UE receives a DCI at slot n triggering this SRS, the UE will transmit the SRS at slot n+ slotOffset which should be a slot with uplink symbols.

Figure 1 Example of Rel-15 aperiodic SRS triggering mechanism

However, this mechanism is not flexible enough with only one slotOffset. For example, as shown in Figure 1, if the slotOffset = 5, and gNB intend to trigger the SRS transmission at the first special slot, then the gNB need to send a UL/DL grant at the 3rd downlink slot. If traffic arrives at 6th slot, gNB can not trigger aperiodic SRS transmission in the nearest S or U slots due to the slotOffset is RRC configured with 5. The only chance to triggering SRS at 13th slot and transmit SRS  at 18th slot, which will cause a long delay. Since it is hard to predict when traffic arrives, the RRC configured slotOffset may not be proper. Another disadvantage of current mechanism is that it may cause PDCCH congestion. For example, if gNB intend to trigger the SRS transmission at the special slot for multiple UEs whose slotOffset is set to 5, it need to send multiple DCIs in 3rd slot, which will cause PDCCH congestion. Thus, flexibility need to be further enhanced to avoid the above problems.

Observation 1: The aperiodic SRS triggering mechanism in NR Rel-15 is not flexible enough with one slotOffset per SRS resource set.

One straightforward way is to use a mechanism similar to aperiodic CSI-RS triggering: RRC configures multiple slotOffset, and DCI selects one while triggering the SRS transmission. However, to support sufficient flexibility, slotOffset from 0 to 7 need to be configured for just one SRS resource set, considering 8:2 DL-UL slot configuration in TDD as shown in Figure-1. The SRS filed is only with 2bits (3bits for SUL case) in current spec for multiple usages. So, if introduce the multiple slotOffset for aperiodic SRS triggering, the SRS triggering filed need much more bits. To simply increase the aperiodic SRS triggering bits to support flexible configuration is not our preference, which introduce other issues such as DCI overhead.

To achieve the flexibility without increasing the DCI overhead, one potential solution is to re-define the aperiodic SRS triggering offset, i.e., slotOffset = k-1 is re-defined as the k-th nearest available slot that can be used for SRS transmission (the values of slotOffset is from 0), while D slots (not available for SRS transmission) are not counted in slot-offset. For example, if slotOffset = 0,  the DCI for aperiodic SRS triggering can be occurs in any DL slot, the uplink SRS transmission will be happen in S slot as shown in Figure 2, while gap between triggering and SRS transmission are satisfied for UE capability. In this way, there is no DCI overhead increasing and gNB can trigger a UE to transmit SRS conveniently in the slot when DL traffic arrived and PDCCH is not congested.

Figure 2 Redefine of slotOffset for aperiodic SRS triggering

Proposal 1: Redefine the slotOffset=k-1 as the k-th nearest available slot for SRS transmission, i.e., Downlink slots are not counted in slot offset.

In NR Rel-16, AP-SRS can be triggered by DCI format 1_1 or DCI format 0_1. But for DCI format 0_1, gNB can’t trigger SRS without data scheduling and CSI request. In the practical network, such restriction may cause UL resource wasted, since gNB may trigger SRS before UL or DL scheduling. Therefore, we propose:

Proposal 2: Allow triggering SRS for DCI format 0_1 without data scheduling and CSI request.

- On SRS antenna switching

In the WID, up to 8 antennas will be specified for SRS antenna switching (e.g., xTyR, x = {1, 2, 4} and y = {6, 8}). The first question is which xTyR combinations are necessary to be supported. In our understanding, the case for 1T6R and 1T8R are not typical, while the UE capability on DL is too much more than UL transmission.

Proposal 3: Prioritize to support 2T6R, 2T8R and 4T8R SRS antenna switching in Rel-17.

For 2T8R, switching mechanism can follow 1T4R, i.e., SRS resource set configured with four SRS resources transmitted in different symbols, but each SRS resource in the set consisting of two SRS ports. Similarly, 4T8R can follow the switching mechanism of 2T4R, but each SRS resource consisting of for SRS ports. And for 2T6R, SRS resource set consisting of three SRS resources transmitted in different symbols should be configured, and each SRS resource consisting of two SRS ports.

Similar to 1T4R SRS switching, the 2T8R switching requires at least 7 symbols if sounding all ports in one slots. Currently, only up to six symbols can be allocated for SRS transmission, which means at least 2 slots are needed for NW to obtain full CSI. In the case that periodic SRS is used to maintain in-time CSI measurement, inter-slot SRS antenna switching may introduce extra delay. One straightforward solution is to extend the number of SRS symbols in a slot, at least more than 6 symbols for SRS antenna switching.

Proposal 4: Support more than six OFDM symbols for SRS transmission in a slot.

- On SRS capacity/coverage enhancement

  - SRS time bundling

SRS time bundling may be beneficial for SRS coverage enhancement by performing joint channel estimation on adjacent SRS slots. It is worth noting that there are at least two basic constrains on UE to guarantee SRS time bundling performance.

The first constraint is related to the start of the corresponding downlink frame of timing advance (TA). R15/R16 only specifies that ‘Uplink frame number i for transmission from the UE shall start TA before the start of the corresponding downlink frame i at the UE’. It implies that the start of the corresponding downlink frame of TA can be adjusted by UE and the quantity of adjustment is unknown to NW. When such adjustment occurs during SRS transmission, it would cause joint channel estimation error due to the uncertainty of the time gap between two adjacent SRS slots. Therefore, to guarantee the TA alignment, it is necessary for UE to report adjustment quantity, which will be considered in joint channel estimation as shown in Fig.3.

Figure 3 Definition of start of corresponding downlink frame for TA to support SRS time bundling

The second constraint is that the correspondence between SRS ports and UE physical ports should be the same for different slots for SRS time bundling. It assures joint channel estimation between SRS slots is performed on the same UE physical ports.

Proposal 5: To guarantee the performance of SRS time bundling, at least TA for SRS transmission for the bundled slots should be aligned, and the SRS mapping between ports and physic antennas should be the same across slots.

Furthermore, SRS time bundling can be in the same SRS resource but different slots (named as intra-SRS-resource bundling) or between different SRS resources (inter-SRS-resource bundling). The difference between intra-SRS-resource time bundling and inter-SRS-resource bundling is that the former perform bundling between different slots in a same SRS resource and the latter perform bundling between different SRS resources in different slots. The inter-SRS-reource bundling is more flexible for SRS periodicity. For example, based on measurement requirement of SRS periodicity, the second semi-persistent SRS resource can be activated/deactivated to double/halve SRS periodicity. For both intra-SRS-resource bundling and inter-SRS-resource bundling, the aforementioned constraints need to be satisfied.

  - SRS repetitions

Due to the limited transmit power of UE, SRS can be coverage limited especially for UEs at cell edge. To improve SRS coverage, SRS repetition and configurable SRS hopping bandwidth are supported from Rel-15. These two methods can be used in conjunction with each other to achieve a trade-off between SRS periodicity and SRS coverage.

In Rel-15, SRS can be repeated twice or four times, which can greatly increase the received power. Except increased repetitions, reduced hopping bandwidth also can improve SRS SNR. In Figure 5(a), we show the different SRS repetition-hopping patterns, where pattern-1 is for 4-times repetition for SRS transmission, Pattern-2 is for 8-times SRS repetition (not supported yet), and Pattern-3 is for 4-times repetition and half bandwidth hopping. Then, in Figure 5(b), we compared the three SRS repetition-hopping patterns. In the evaluation, the bandwidth is assumed as 48 RBs, where half bandwidth is 24 RBs per hop, and DL SINR is assumed as 9 dB higher than SRS SINR. Since the transmit bandwidth of pattern-3 is smaller than others, the power boost is also considered in the simulation. More assumptions for the evaluation is shown in Table 1 in Appendix.

From Figure 5(b), it can be observed that to increase repetitions (i.e., pattern 2) has the same performance with reducing hopping bandwidth (i.e., pattern 3). However, to increase repetitions result in SRS multiplexing capability reduction, while reduced  hopping bandwidth don’t effect SRS multiplexing capability.

(a)

(b)

Figure 5 Performance comparison of increased repetitions and reduced hopping bandwidth

Observation 2: Increasing SRS repetitions has the similar performance with reducing hopping bandwidth, but SRS multiplexing capacity will decrease by increasing SRS repetitions.

During email discussion, TD-OCC is proposed for SRS repetition, to maintain the SRS multiplexing capacity while SRS repetition. However, the benefit of TD-OCC is not clear. Firstly, it requires the multiplexed UEs are configured with the same SRS repetition times, to keep the same multiplexing capability, which is not flexible enough. In addition, according to the current spec, when SRS partially overlap with PUCCH, then the overlapped symbols of SRS may be dropped. Then the orthogonality of such truncated TD-OCC cannot be maintained. Moreover, the coverage enhancement can equivalently be enhanced by reducing the SRS sounding bandwidth, which does not reduce the SRS multiplexing capability at all. Therefore, whether to support TD-OCC for SRS repletion needs further discussion.

Observation 3: The benefit of introducing TD-OCC over SRS repetition is not clear yet.

Figure 6 shows the CDF curve of received SNR of SRS with 48 RB hopping bandwidth, 23dBm transmission power, comb 2 structure. The noise figure is -169 dBm/Hz and interference over thermal (IoT) is 2dB in the evaluation. It shows that even for 500m ISD, the SRS SNR of most UEs is above -15 dB, which would not cause significant performance loss as shown in Figure 5(b). And for the UEs with SNR lower than -15 dB,  reducing hopping bandwidth can be used to ensure the coverage.

Figure 6 CDF of received SNR of SRS

It seems purely increasing the number of repetition of SRS transmission is not efficient way to improve channel estimation accuracy. One reason is that SNR for SRS is high enough for channel estimation as shown in Figure 6. Another reason is that the interference power (from e.g., neighbouring cell) increase as well as received SRS power, if interfering UE also transmit SRS with repetition. Thus, reducing the inter-cell interference is more critical to guarantee the channel estimation performance during SRS repetition. One potential way to reduce such interference is that randomizing the inter-cell interference for SRS transmissions.

In current spec, SRS group/root-sequence hopping supported by configuring groupOrSequenceHopping as 'groupHopping' or 'sequenceHopping', which aim to randomize interference. However, for 'groupHopping', it requires all gNBs to configure 'groupHopping' for all UEs, or otherwise same sequence group may be used in neighbouring cell which cause serious interference. For 'sequenceHopping', at most two sequence can be used for hopping which can’t randomize the interference well, especially for case where repetition factor is 4.

Therefore, how to randomize interference when 'groupOrSequenceHopping' is ‘neither’ needs to be considered. One way is to support cyclic shift hopping during SRS repetition, i.e., root sequence remains the same but cyclic shift hops. One example is shown in Figure 7. As long as the shift hopping patterns are different for different root sequences, the interference among different sequences can be randomized. The exact hopping pattern can be further discussed.

Figure 7 Illustration of SRS cyclic shift hopping during SRS transmission

Figure 8 shows channel estimation performance of CS hopping/no CS hopping case, where there is cyclic shift (CS) hopping between 2 repeated SRS symbols for case1 and there is no CS hopping between 2 repeated SRS symbols for case2. In the simulation, 2 UEs whose SRS occupies the same time frequency resource and uses different root sequences are assumed to emulate the inter cell interference. Transmit power is same for 2 UEs. And for the CS hopping case, CS hopping patterns are different for 2 UEs. The SRS SNR is 0dB and 5dB separately for the left side of Figure 7 and right side of Figure 7. More assumptions for the evaluation is shown in Table 2 in Appendix. The results of MSE between ideal channel and estimated channel in Figure 7, shows that CS hopping case can obtain about 1.6dB~2dB channel estimation performance gain over no CS hopping case.

Figure 8 Chanel estimation performance of CS hopping/No CS hopping case

As seen from Figure 8, due to the interference among different sequences can be randomized, CS hopping for SRS transmission can improve the channel estimation accuracy.  Therefore, we propose:

Proposal 6: For SRS repetition transmission, inter-cell interference randomization should be supported, such as cyclic shift hopping for SRS transmission.

  - Partial sounding across frequency

Current SRS frequency hopping aim to sound the whole bandwidth. However, it is observed in real network that some parts of the configured BWP suffer from serious UL and/or DL interference from other wireless system, i.e., video backhaul system. Therefore, DL scheduling may not be granted on these frequency parts, and thus SRS on this part is not needed as well. Since this kind of interference is relatively stable during a period of time, the actual frequency parts for SRS transmission can be higher-layer configured.

In the case of high correlation in frequency domain, the only partial band on each SRS hop should be transmitted. The no SRS transmission bandwidth can be reconstructed by interpolation. The partial SRS transmission is with the two main benefits: One is the power density will be increased, since power boost will be on the partial SRS bandwidth. The other one is to increase the multiplexing capacity, since the no transmitted SRS band can be used for another UE.

There are two examples shown in the figures. As shown in Figure-9(a), the part of SRS transmission in a hop is consecutive in RBs, otherwise the low PAPR for ZC sequence will be not guaranteed. As shown in Fugure-9(b), only partial SRS hops will be happen, compared to the left sub-figure for regular whole band SRS hopping, the right sub-figure only with part bandwidth is for SRS transmission, so that only half hopping period is needed in such cases.

(a)

(b)

Figure 9 two examples of partial sounding

Proposal 7: Both partial sounding on each hopping bandwidth or on some hopping bandwidth can be studied.

It is worth noting that partial sounding can work together with SRS time bundling to increase SRS capacity and/or SRS coverage. Regular whole band SRS transmission will help to measure the whole band information, and the partial band SRS transmission as analysed above is beneficial for power density and multiplexing capacity. Time bundling the regular whole band SRS transmission and partial band SRS transmission will provide more accurate on channel estimation, and larger multiplexing capacity.

Figure 10 shows an example of time bundling between legacy R15/R16 whole band SRS transmission and SRS for partial sounding. Furthermore, as shown in the example in Figure-4, if the SRS for partial sounding in a slot is composed of multiple SRS segments discontinuous in frequency domain (e.g., two discontinuous SRS segments in slot 10 and slot 30). These SRS segments can be located in different OFDM symbols in a slot to avoid PAPR issue. Joint channel estimation can be performed to help recover channel estimate over RB groups where SRS for partial sounding has no occupation.

Figure 10 Time bundling between legacy R15/R16 SRS and SRS for partial sounding

Observation 4: Time bundling between whole band SRS transmission and partial band SRS transmission is beneficial on channel estimation and capacity enhancement.

  - Other enhancements

The target of the SRS WID 3c is to increase the SRS capacity and coverage. However, only partial sound across frequency may help to achieve an enhanced capacity, but at the risk of compromising the DL performance on some RBs if frequency domain is uncorrelated.

As we analysed in our previous RAN-P tdoc [2][3], extending the number of SRS root sequence is an important way to increase SRS capacity. With well-selected root sequence, a gNB can configure two groups of UE to transmit SRS on the same t-f resources, where different groups of UE using different root sequences, and the interference from different root sequences can be kept low. Extending root sequence was discussed in Rel15, and thus the consensus about the necessity of extending root sequence has been reached. However, no solution is settled. It is beneficial to consider this as one candidate to enhance SRS capacity in Rel-17.

Proposal 8: Consider extending SRS root sequence as one candidate for SRS capacity enhancement.

- Evaluation methodology and assumption

In phase 2 offline discussion on SRS evaluation methodology, most companies agree that CDL channel should be used for link level simulation, but there is some discussion on how to use CDL in MU-MIMO. In our understanding, CDL is more proper for simulation related to MIMO. And for the evaluation of MU-MIMO, CDL can also be used by setting different angle values for different UEs according to the locations of UEs, which is similar as UE-dropping in SLS. As long as the locations of UEs is not too close, MU pairing can work well.

- Conclusion

In this contribution, we discussed the enhancements on SRS for Rel-17 and provide following observation and proposal:

Observation 1: The aperiodic SRS triggering mechanism in NR Rel-15 is not flexible enough with one slotOffset per SRS resource set.

Proposal 1: Redefine the slotOffset=k-1 as the k-th nearest available slot for SRS transmission, i.e., Downlink slots are not counted in slot offset.

Proposal 2: Allow triggering SRS for DCI format 0_1 without data scheduling and CSI request.

Proposal 3: Prioritize to support 2T6R, 2T8R and 4T8R SRS antenna switching in Rel-17.

Proposal 4: Support more than six OFDM symbols for SRS transmission in a slot.

Proposal 5: To guarantee the performance of SRS time bundling, at least TA for SRS transmission for the bundled slots should be aligned, and the SRS mapping between ports and physic antennas should be the same across slots.

Observation 2: Increasing SRS repetitions has the similar performance with reducing hopping bandwidth, but SRS multiplexing capacity will decrease by increasing SRS repetitions.

Observation 3: The benefit of introducing TD-OCC over SRS repetition is not clear yet.

Proposal 6: For SRS repetition transmission, inter-cell interference randomization should be supported, such as cyclic shift hopping for SRS transmission.

Proposal 7: Both partial sounding on each hopping bandwidth or on some hopping bandwidth can be studied.

Observation 4: Time bundling between whole band SRS transmission and partial band SRS transmission is beneficial on channel estimation and capacity enhancement.

Proposal 8: Consider extending SRS root sequence as one candidate for SRS capacity enhancement.

References

- [1] 3GPP, “RAN1#99 Meeting Chairman’s Notes”, Reno, USA, November 18th – 22nd, 2019.

[2] RP-191762, “Rel-17 work scope on NR MIMO and sub-3 GHz FDD enhancements”, Newport Beach, USA, September 16-20, 2019, Huawei, HiSilicon.

[3] RP-191486, “Overview of Rel-17 work areas for NR and LTE”, Newport Beach, USA, June 3-6, 2019, Huawei, HiSilicon.

Appendix

Table 1 LLS assumptions for Increased SRS repetitions evaluation

| Setting Items | Parameter Values |
| --- | --- |
| System bandwidth | 48RB |
| SRS comb | 2 |
| Carrier Freq | 3.5GHz |
| SCS | 30K |
| slot configuration | DSUDD |
| SRS period | 5ms |
| channel | CDL-B 300ns |
| BS antenna | 64T64R (8,8,2,1,1,4,8), (dH,dV) = (0.5, 0.8)λ, +45°, -45° polarization |
| UE antenna | 2T4R(1,2,2,1,1,1,2) with antenna switching, (dH,dV) = (0.5, 0.5)λ, +0°, 90° polarization |
| UE speed | 3km/h |
| MCS | Adaptive |
| Rank and precoder | Adaptive |
| PRG | 2RB |

Table 2 LLS assumptions for CS hopping evaluation

| Setting Items | Parameter Values |
| --- | --- |
| SRS hopping bandwidth | 24 RB |
| Repetition factor | 2 |
| comb | 2 |
| Carrier Freq | 3.5GHz |
| SCS | 30K |
| channel | CDL-B 100ns |
| BS antenna | 64T64R (8,8,2,1,1,4,8), (dH,dV) = (0.5, 0.8)λ, +45°, -45° polarization |
| UE antenna | 2T4R(1,2,2,1,1,1,2) with antenna switching, (dH,dV) = (0.5, 0.5)λ, +0°, 90° polarization |
| UE speed | 3km/h |