3GPP TSG RAN WG1 Meeting #102-e	R1-2005262

E-meeting, August 17-28, 2020

Agenda Item:	8.7.1.1

Source:	Huawei, HiSilicon

Title:	Paging enhancement(s) for UE power saving in IDLE/inactive mode

Document for:	Discussion and Decision

Introduction

In RAN#86, it is agreed to further specify enhancements for UE power saving in Rel-17 on the top of Rel-16 schemes [1]. For IDLE/INACTIVE mode, as illustrated in [2], unnecessary paging reception would consumes much power due to additional state transmission and PDSCH reception.

- Specify enhancements for idle/inactive-mode UE power saving, considering system performance aspects [RAN2, RAN1] - Study and specify paging enhancement(s) to reduce unnecessary UE paging receptions, subject to no impact to legacy UEs [RAN2, RAN1] - NOTE: RAN1 to check and update, if needed, evaluation methodology in RAN1 #100 meeting

The contribution firstly analyzes the issue of power consumption waste when UE monitors paging but is not paged, based on some statistics data from the field test. Second, we discuss the evaluation methodology for paging enhancement. Finally, the potential schemes to reduce paging reception are discussed and initial evaluation results are also provided.

Power consumption analysis for IDLE/INACTIVE mode UE

As analyzed in our companion paper [2], in order to receive the PDSCH scheduled by paging DCI to receive potential paging message, a UE needs to pre-wake before the start of the paging occasion (PO) to obtain finer time/frequency tracking and AGC. For good coverage UEs, one SSB burst is needed and for bad coverage UEs, at least two SSB bursts would be needed. In consequence, additional state transitions to/from ‘light sleep’ are introduced and contribute a big portion of the power consumption for IDLE mode UEs.

For the DRX cycle where a paging message is actually carried by the paging PDSCH, the pre-wake-up and additional state transitions from/to ‘light sleep’ state would be essential to guarantee the reception performance of the PDSCH. However, the power consumption is wasted in case when the UE is not paged in the received PDSCH, where the pre-wake-up before the PO and additional state transitions from/to ‘Light Sleep’ state are considered to be not necessary.

Observation 1: The power consumption due to pre-wake-up before the PO and additional state transitions to/from ‘light sleep’ on a UE is wasted in DRX cycles where the paging message is not carried by the paging PDSCH.

Figure 1 UE procedures for Rel-15/16 in good/bad coverage.

Furthermore, according to the calculation formula of POs, a group of UEs would share the same PO, and therefore a UE in the group may often detect a DCI masked by P-RNTI which schedules a PDSCH, however without a paging message corresponding to the UE in the scheduled PDSCH, i.e. its higher layer ID is not included in the scheduled PDSCH. The following statistical values are from LTE UEs in field test, which can illustrate the probability of DRX cycles without actually paging message for the UE carried by the PDSCH. The paging cycle in the field test is 1.28s.

Table 1: An illustration of high probability of unpaged DRX cycles from LTE field test log

|  | Staring time | Ending time | Idle time(s) | # of PDCCH with P-RNTI | # of Paging with UE ID | Probability of unpaged DRX cycles |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 5:56:46 | 6:21:58 | 1277 | 220 | 3 | 98.64% |
| 2 | 6:22:07 | 6:39:49 | 855 | 179 | 5 | 97.21% |
| 3 | 6:39:49 | 6:49:52 | 500 | 121 | 3 | 97.52% |
| 4 | 15:07:41 | 15:17:42 | 443 | 181 | 3 | 98.34% |
| 5 | 15:17:43 | 15:27:30 | 364 | 141 | 6 | 95.74% |
| 6 | 15:27:31 | 15:38:20 | 492 | 179 | 2 | 98.88% |
| 7 | 20:37:41 | 20:44:13 | 250 | 52 | 2 | 96.2% |
| 8 | 21:42:10 | 21:56:04 | 634 | 317 | 7 | 97.79% |
| 9 | 21:56:30 | 22:17:10 | 75 | 347 | 6 | 98.27% |
| 10 | 22:18:41 | 22:38:32 | 990 | 287 | 6 | 97.91% |

From the statistical results, it can be observed the average probability of unpaged DRX cycles for the observed UE is 97.65%, which is significantly high. Therefore, the power consumption due to a lot of unnecessary ‘state transition’ and ‘light sleep’ in the unpaged DRX cycles are wasted which can be further reduced for power saving.

Observation 2: The high probability of unpaged DRX cycles leads to the waste of power consumption due to unnecessary ‘state transitions’ and ‘light sleep’, which can be further reduced for power saving.

Evaluation methodology

According to the discussion in RAN plenary, both the eMBB UEs and RedCap UEs have the motivation to specify mechanisms to reduce the power consumption in IDLE/INACTIVE Mode. Considering the eMBB UE in IDLE mode has the same reception bandwidth, i.e. 20MHz, as that of REDCAP UE, it is proposed to consider power saving for IDLE/ INACTIVE mode for both eMBB UEs and REDCAP UEs.

Proposal 1: Power saving mechanisms for IDLE/ INACTIVE mode for both eMBB and REDCAP UEs are specified by power saving WI.

As per Rel-16 UE power saving, power saving gain can be used to evaluate different schemes. In [3], the power model for RS reception, PDCCH-only and PDCCH+PDSCH has already been determined, which can be used to calculate the power consumption of IDLE/INACTIVE UE behaviors. Since in IDLE/INACTIVE mode a UE stays in the initial BWP, the bandwidth of which is typically 20MHz. Therefore, scaling is needed to get the power consumption of IDLE/INACTIVE UE. According to [3], the scaling factor of X MHz is 0.4+0.6∙(X-20)/80, then the power consumption of ‘PDCCH-only’ is 100∙0.4=40 power units for IDLE/INACTIVE UE. However, it should be noted that the power consumption of ‘micro-sleep’ is 45 power units. It is not reasonable that the power consumption of ‘PDCCH-only’ is smaller than that of ‘micro-sleep’. Therefore, the power consumption of ‘PDCCH-only’ can be 50 power units. Following the same principle, the power consumption of ‘SSB’ or ‘CSI-RS’ are also 50 power units. As for ‘PDCCH+PDSCH’, the power consumption is 300∙0.4=120. If the UE bandwidth is smaller than 20MHz, the same scaling and bounding method can be used. As for the power consumption of ‘sleep’, it should stay unchanged since scaling is only applied to non-sleep power states.

Proposal 2: For evaluation, the power model in the following table is used, which follows the scaling rule and power model in TR38.840.

| Power state | Relative power units |
| --- | --- |
| Deep sleep | 1 |
| Light sleep | 20 |
| Micro sleep | 45 |
| PDCCH-only | 50 |
| PDCCH+PDSCH | 120 |
| SSB or CSI-RS proc. | 50 |

Paging traffic model needs to be defined for the evaluation. Considering we need to model the unnecessary pre-wake-up and paging reception in the evaluation, the ‘traffic model’ in IDLE/INACTIVE mode should consider the following two aspects:

- The probability when a UE is actually paged by network (i.e. the paging rate per UE), which is defined as the paging rate of a UE: p.

- The number of UEs sharing the same PO: N.

With the above two parameters, the probability of sending a paging DCI and PDSCH on a PO, i.e. the paging rate per PO, can be calculated by the following equation

P=1-1-pN

Typically the value of p can be 1%. For the value of N, two scenarios could be considered: light load and high load paging scenarios. For light load, N is set to 10, which corresponds to a paging rate per PO of 9.56% (P=1-1-pN=9.56% ). For high load, i.e. there would be a large number of UEs within the tracking area (TA), N can be set to [50], which corresponds to a paging rate per PO of 39.50%. In deployment, the number of UEs camping on the same cell could be 600. According to current spec, there are at most 64 POs in an I-DRX cycle, where the maximum number of PF in an I-DRX is 16 and the maximum number of PO in a PF is 4. Since the network doesn’t know clearly which cell a IDLE/INACTIVE mode UE is camping on, the paging message is usually transmitted within a tracking area (TA), which covers several cells. If we assume that there are 5~6 cells in a TA, then about 50 UEs sharing a PO can be a reasonable assumption.

Proposal 3: For evaluation, the paging traffic in IDLE/INACTIVE mode is modeled by the paging rate of a UE and the number of UEs sharing a PO(N), which are defined by the following table.

| Parameter | Low load | High load |
| --- | --- | --- |
| Paging rate of a UE(p) | 1% | 1% |
| N (the number of UEs sharing a PO) | 10 | [50] |
| P(paging rate of a PO, P=1-(1-p)^N) | 9.56% | [39.50%] |

To align the evaluation results from different companies, other evaluation assumptions should also be made. Some reference configuration can be as follows.

  - I-DRX cycle length is 1.28s

  - SCS = 30kHz

  - SSB burst set periodicity is 20ms

  - When UE is in good coverage, i.e. SINR is high, one SSB is used for pre-sync before PO (including time/frequency synchronization, AGC and etc); while UE is in bad coverage, i.e. SINR is low, two or three SSBs are used.

  - When UE is in good coverage, S-criterion should be considered, then UE doesn’t perform neighbour cell measurement

  - When UE is in bad coverage, the neighbour cell measurement relaxation scheme, which is specified in Rel-16, should also be considered, and one frequency layer with 5ms SMTC window is assumed

Proposal 4: Take the following as evaluation assumption for IDLE/INACTIVE mode UE

| Parameter | Good coverage | Bad coverage |
| --- | --- | --- |
| #SSB burst (Number of SSB burst to be received before the PO) | 1 | 2 (eMBB UE) or 3(RedCap UE) |
| SSB burst set periodicity | 20ms | 20ms |
| SCS | 30kHz | 30kHz |
| Neighbor cell measurement | Not modeled for good coverage UE (considering S-criterion and RRM relaxation in Rel-16) | Only monitor one frequency layer in one DRX cycle (considering RRM relaxation in Rel-16) SMTC = 5ms |

Potential schemes

This section provides some initial considerations on schemes to reduce power consumption in IDLE/INACTIVE mode. In order to reduce the power consumption of state transition, some early transmitted paging information can be sent before POs. Furthermore, sub-grouping indication for a PO could be carried by the early transmitted paging information to further reduce the rate of unnecessary state transitions and paging reception.  The availability of assistance RS can be also carried by the early transmitted power information for further power reduction.

Early transmitted paging information

As observed in Observation 1 and 2, the power consumption due to unnecessary ‘state transition’ and ‘light sleep’ in the unpaged DRX cycles are wasted which can be further reduced for power saving. If the network could transmit the paging information carried by the paging DCI earlier before the PO, which could be near the transmission of the first SSB burst that UE needs to receive, the UE could know whether a PO reception is needed so that unnecessary ‘state transition’ and ‘light sleep’ in the unpaged DRX cycles can be avoided.

In summary, early transmitted paging information before PO can provide the benefit to reduce state transition and extend the time duration in deep sleep. As illustrated in Figure 2, the UE can go to deep sleep immediately after the early transmitted paging information if there is no paging message for the UE.

To save more power, the early transmitted paging information can be near the SSB transmission. If the early transmitted paging information is close to SSB, then UE can receive the information right after it performs pre-sync. The closer the information is to the SSB, the less time the UE needs to wait, which leads to more power saving gain.

Figure 2 Early transmitted paging information to reduce state transition and extend the time in deep sleep

Observation 3: Early transmitted paging information before PO provides the benefit to reduce state transition from/to additional light sleep and extend the time in deep sleep.

The paging information is carried by the paging DCI, i.e. DCI format 1_0, which schedules a PDSCH in legacy paging mechanism. To minimize the implementation and specification impact, as a starting point, a paging DCI can be simply transmitted earlier than the PO, and near the transmission of the first SSB burst that UE needs to receive. Some initial evaluation results are shown in Table 2. Therein, we assume that the early transmitted paging DCI is in the same slot of SSB and other evaluation consumption can be found in our companion paper [2].

Table 2: Evaluation results for paging enhancement schemes

| Power saving gain vs NR R16 | Good coverage | Bad coverage |
| --- | --- | --- |
| 10 UEs per PO | 11.9% | 18.2% |
| 50 UEs per PO | 7.9% | 12.1% |

Based on the above analysis and evaluation results, it is proposed to introduce early transmitted paging DCI which is close to the SSB transmission.

Proposal 5: Introduce early transmitted paging DCI to resolve the issue of unnecessary pre-wake-up and state transitions from/to light sleep causing power consumption waste.

Besides the information of paging PDSCH reception, the short message to inform SI change and/or ETWS information should be  also informed in the early transmitted paging information. It can be further discussed later to decide the detailed information in paging DCI needs to be transmitted earlier. According to [5], different POs in one PF are successively located, it would be possible to merge multiple Paging DCIs transmitted on different POs into a single DCI to reduce the resource overhead. Furthermore, as we analyzed in the previous sections, for a single UE, it is the best choice to transmit the early transmitted paging DCI near the first SSB where the UE performs pre-sync. When considering multiple UEs, since the UEs in different coverage condition require different number of SSBs to perform pre-sync, the best occasion for different UE to transmit the early transmitted paging DCI could also be different. To help UEs in different coverage condition to save power, the information for the same PO could be indicated in multiple DCIs in different time occasions.

Proposal 6: Further discuss which information in paging DCI needs to be informed in an early transmitted paging DCI before the PO.

Further paging sub-grouping indication

To reduce the probability of unpaged DRX cycle, the UEs that shares the same PO can be further sub-grouped, and sub-grouping information can be carried by the legacy paging DCI. For each paging procedure, only the UEs in the sub-group(s) being paged need to monitor paging message. Then the UE procedure is 1) wake up before its PO to get everything ready, 2) detect paging DCI to check whether the sub-group it belongs to is being paged or not, 3) If not, the UE can sleep directly and skip corresponding PDSCH reception, otherwise, the UE needs to decode the message on PDSCH. In this way, power consumption for PDSCH reception can be saved based on UE sub-grouping. Potentially, there are several ways to indicate the UE sub-grouping information, e.g. by different P-RNTIs for different sub-groups, by different CORESETs/search spaces of paging DCIs for different sub-groups, or by indicators in paging DCI indicating sub-group information explicitly, etc.

Besides, the UE sub-group information can be carried in the early transmitted paging information. According to the analysis in Figure 2, the power saving gain of early transmitted paging DCI comes from the improved UE procedure for the unpaged DRX cycles. However, a PO is shared by a group of UEs, and the UE would still use the regular paging reception procedure with a probability that equals to paging rate per PO. If this probability could be further reduced, the probability for a UE to use the improved procedure for power saving shall be increased. For example, if the probability to use regular paging reception procedure can be reduced from 39% to 1%, a significant power saving gain would be obtained.

Considering the overhead and coverage issue, the number of bits in the early transmitted paging information may be limited so that the sub-group information may not be fully carried by the early transmitted paging DCI. It would be helpful to carry part of the sub-group information in the early transmitted paging information and the remaining part in the paging DCI.

Some evaluation results are shown in Table 3.

Table 3: Evaluation results for sub-grouping indication with 2UEs per sub-group

| Power saving gain vs NR R16 |  | Good coverage | Bad coverage |
| --- | --- | --- | --- |
| 10 UEs per PO | Sub-grouping indication in paging DCI | 0.13% | 0.08% |
|  | Sub-grouping indication in Early Transmitted Paging (2UE per sub-group) | 13.0% | 19.8% |
| 50 UEs per PO | Sub-grouping indication in paging DCI | 0.62% | 0.41% |
|  | Sub-grouping indication in Early Transmitted Paging (2UE per sub-group) | 13.4% | 20.0% |

Proposal 7: Support UE sub-grouping to increase the power saving gain, which can be indicated in early transmitted paging information and/or paging DCI.

Indication of assistance RS

To reduce unnecessary power consumption due to state transition and light sleep, one efficient solution is to provide UE the potential TRS/CSI-RS, which can be a TRS/CSI-RS used by other connected mode UEs. As the RS resource is not always transmitted, the RS availability indication mechanism should also be specified. If the early transmitted paging is introduced, it can be also used to indicate whether the assistance TRS/CSI-RS is transmitted before the incoming paging occasion(s). One of the benefit to use early transmitted paging indicating assistance RS is that if a UE is paged in a DRX cycle, the UE can acquire the finer T/F tracking, AGC tracking by using SSB burst and assistance RS continuously, and can reduce unnecessary power state transition and light sleep. The paging reception procedures are shown in Figure 3, and the power saving gain is shown in Table 4. Some more detailed analysis on the assistance RS can be found in our companion paper [4]

Figure 3 Early transmitted paging information to reduce state transition and extend the time in deep.

Table 4: Evaluation results for Early Transmitted Paging indicating assistance RS

| Power saving gain vs NR 16 |  | Good coverage | Bad coverage |
| --- | --- | --- | --- |
| 10 UEs per PO | no sub-group indication | 11.9% | 19.9% |
| 50 UEs per PO | no sub-group indication | 7.9% | 18.7% |

Proposal 8: Support early transmitted paging to indicate the availability of assistance TRS.

Conclusions

In this contribution, we discuss the paging procedure enhancement. Based on the analysis, we have the following observations and proposals:

Observation 1: The power consumption due to pre-wake-up before the PO and additional state transitions to/from ‘light sleep’ on a UE is wasted in DRX cycles where the paging message is not carried by the paging PDSCH.

Observation 2: The high probability of unpaged DRX cycles leads to the waste of power consumption due to unnecessary ‘state transitions’ and ‘light sleep’, which can be further reduced for power saving.

Observation 3: Early transmitted paging information before PO provides the benefit to reduce state transition from/to additional light sleep and extend the time in deep sleep.

Proposal 1: Power saving mechanisms for IDLE/ INACTIVE mode for both eMBB and REDCAP UEs are specified by power saving WI.

Proposal 2: For evaluation, the power model in the following table is used, which follows the scaling rule and power model in TR38.840.

| Power state | Relative power units |
| --- | --- |
| Deep sleep | 1 |
| Light sleep | 20 |
| Micro sleep | 45 |
| PDCCH-only | 50 |
| PDCCH+PDSCH | 120 |
| SSB or CSI-RS proc. | 50 |

Proposal 3: For evaluation, the paging traffic in IDLE/INACTIVE mode is modeled by the paging rate of a UE and the number of UEs sharing a PO(N), which are defined by the following table.

| Parameter | Low load | High load |
| --- | --- | --- |
| Paging rate of a UE(p) | 1% | 1% |
| N (the number of UEs sharing a PO) | 10 | [50] |
| P(paging rate of a PO, P=1-(1-p)^N) | 9.56% | [39.50%] |

Proposal 4: Take the following as evaluation assumption for IDLE/INACTIVE mode UE

| Parameter | Good coverage | Bad coverage |
| --- | --- | --- |
| #SSB burst (Number of SSB burst to be received before the PO) | 1 | 2 (eMBB UE) or 3(RedCap UE) |
| SSB burst set periodicity | 20ms | 20ms |
| SCS | 30kHz | 30kHz |
| Neighbor cell measurement | Not modeled for good coverage UE (considering S-criterion and RRM relaxation in Rel-16) | Only monitor one frequency layer in one DRX cycle (considering RRM relaxation in Rel-16) SMTC = 5ms |

Proposal 5: Introduce early transmitted paging DCI to resolve the issue of unnecessary pre-wake-up and state transitions from/to light sleep causing power consumption waste.

Proposal 6: Further discuss which information in paging DCI needs to be informed in an early transmitted paging DCI before the PO.

Proposal 7: Support UE sub-grouping to increase the power saving gain, which can be indicated in early transmitted paging information and/or paging DCI.

Proposal 8: Support early transmitted paging to indicate the availability of assistance TRS.

- References

RP-193239, “New WID: UE Power Saving Enhancements”, MediaTek

R1-2006412, “Initial analysis on power consumption for IDLE mode UE”, Huawei, HiSilicon

TR 38.840, Study on UE power saving.

R1-2005263, “Assistance RS occasions for IDLE/inactive mode”, Huawei, HiSilicon

TS 38.304, User Equipment (UE) procedures in Idle mode and RRC Inactive state.