3GPP TSG RAN WG1 Meeting #102-e	R1-2005284

e-Meeting, August 17-28, 2020

Agenda Item:	8.5.3

Source:	Futurewei

Title:	Positioning Enhancements

Document for:	Discussion and decision

# Introduction

In 3GPP RAN#86, a new Study Item on NR Positioning Enhancements was approved in RP-200928 and in RAN1#101-e, the scenarios and evaluation parameters have been finalized [1].  In this contribution, we provide our views on some of the potential enhancements that have been proposed [2]-[6].

# Proposed Enhancements

## Reference Signals

One of the main objectives of Rel-17 in addition to the focus on IIOT use cases, is the reduction of latency in acquiring positioning. This is an aspect that has not been considered sufficiently during Rel-16, hence it needs to be addressed now in Rel-17. Various enhancements have been proposed [2]-[6] and in the following sections, some of the proposed enhancements are discussed.

### PRS/SRS Transmissions

Rel.16 currently supports multiplexing of positioning signals in time (across different symbols/slots), frequency (across different RE patterns) and code domains (different sequences) to support multiple TRPs or gNBs transmissions of Rel.16 DL PRS. Within a multiplexing domain, muting patterns are used to ensure that these PRS transmissions do not collide. The tradeoff is the added latency due to the fewer number of PRS transmissions per TRP/gNB that are sent within a transmission period and hence multiple periods are needed to transmit sufficient number of PRSs for the UE to measure.

One solution is to configure the PRS transmissions only when it is needed for positioning measurements at the UE.  The PRS transmissions can be additional PRS transmissions on top of a sparse PRS transmissions such in the case when positioning calculated at the UE (UE-based positioning) is not achieving the required accuracy or when the calculated position varies greatly and additional measurements are needed to enhance the positioning.

These additional DL PRS transmissions would be triggered by the UE through UL signaling e.g. uplink control signaling, dedicated pre-allocated UL SRS resources, or with the less dynamic RRC signaling. These options and possibly various others should be studied further.

Proposal 1: Aperiodic DL PRS requests and transmission should be supported for its benefits in reducing PRS transmissions overhead and improving latency. Details on the specifications support and impact are for further study.

For UL SRS, it is already supported in current specifications semi-persistent and aperiodic SRS transmissions for RRC connected UEs in addition to periodic transmissions. Given that UL SRS is used for positioning calculated at the network, and its transmissions are also configured by the network, the tradeoff between overhead and latency can be fully managed by the network. On the other hand, enabling autonomous UE transmissions of UL SRS without network coordination may lead to increased uplink interference that needs to be studied to consider the gain of autonomous UE transmissions of SRS on the positioning latency versus the possible increased interference in the uplink.

Observation 1: The UL interference due to autonomous UL SRS transmissions would need to be studied further.

## NLOS Effects Mitigation

The degradation on the positioning accuracy due to NLOS paths is a major concern that have been raised by many companies [2] [3] [4] [5].  To correctly model the NLOS paths propagation delay, it was agreed to incorporate Δτ (from Table 7.6.9-1 of TR38.901) as the mean positive bias in the time of arrival for the NLOS Channel Impulse Response (CIR) due to the additional reflections resulting in additional  propagation delays of all arriving  paths.

Table 7.6.9-1: Parameters for the absolute time of arrival model

| Scenarios |  | InF-SL, InF-DL | InF-SH, InF-DH |
| --- | --- | --- | --- |
| lgΔτ=log10Δτ1s | μlgΔτ | -7.5 | -7.5 |
|  | σlgΔτ | 0.4 | 0.4 |
| Correlation distance in the horizontal plane [m] |  | 6 | 11 |

Various companies studied this effect by simulations in the last meeting, and have confirmed the degradation in localization accuracy when there is presence of NLOS CIRs during the localization. In such NLOS deployments, the effectiveness and accuracy of localization algorithms depend on if there is any prior information regarding the amount and severity of these NLOS offsets. In other words, the properties and distribution of the NLOS paths would need to be estimated in the localization algorithms for appropriate compensation of its effects.

Another approach to mitigate the NLOS effects is to identify the NLOS paths and discard them during localization. The estimation of the UE location is then performed using the various existing LOS based techniques. In the following, we describe an approach that reuse the existing R16 PRS framework by transmitting the PRS resources with different polarizations, which when measured at the UE can be used to determine if the received path is with high likelihood a NLOS or LOS path. The decision can be made at the UE or at the network if the measurements are fed back to the LMF.

At the gNB, with the support of two ports, each Antenna array is steered with fixed linear polarization direction and the two polarization orientations are orthogonal. We will denote here Pol1 for Vertical orientation at TXRU port 1 and Pol2 for a Horizontal orientation at TXRU port 2 w.r.t to the Local Coordinate System (LCS). The rotation of the PRSs due to the channel is then calculated using the following steps:

- Two PRS signals  PPRSV and PPRSH, which are two short identical impulses in time (assume unit power for simplicity), are transmitted by the BS in the DL. Here, PPRSV=1,0T is transmitted via TXRU1 selecting s=V with vertical polarization and PPRSH=0,1T  via TXRU2 selecting s=H with horizontal polarization.

- The 3D antenna power pattern is given by A'θ',ϕ' =Fθ''(θ',ϕ')2 +Fϕ''(θ',ϕ')2 , where  Fθ''(θ',ϕ') is the field component in vertical direction and Fϕ''(θ',ϕ') in horizontal direction. Hence, the two transmitted orthogonal field-vectors (in the V and H with respect to the LCS of the BS antennas) are given by:

FtxV'=Fθ''(θ',ϕ')  PPRSV  =FtxV,θ''    0  and  FtxH'=Fϕ''(θ',ϕ') PPRSH =0    FtxH,ϕ''

- The magnitude of the CIR’s first path (FP), as per [7], via a single ray is given by:

|H u,s |=Hu,s,1 t=Frx,u,θθ1,ZOA,φ1,AOAFrx,u,φθ1,ZOA,φ1,AOATSFtx,s,θθ1,ZOD,φ1,AODFtx,s,φθ1,ZOD,φ1,AOD                 (1)

Where S denotes the complex scattering matrix, which will be in general not orthogonal. Here we also assume that the receiver has access to two orthogonal linear polarizations denoted by u=V and u=H.

- The transformation of the field vector to the Global Coordinate System (GCS) is given by an orthonormal matrix

Ψ=cos ψ-sinψsinψcosψ

which is defined by the field rotation angle ψ =ψα ,β ,γ ,θ,ϕ, given in [7] by the three rotation angles α ,β ,γ  which rotate the LCS to the GCS and the spherical direction ϕ,θ .

- Let Ψrx and Ψtx be the rotation matrix for the UE (rx) and gNB (tx), respectively. Then (1) can be rewritten by:

Frx,u,θ''''θ''1,ZOA,φ''1,AOAFrx,u,φ''''θ''1,ZOA,φ''1,AOATΨrxTSΨtxFtx,s,θ''θ'1,ZOD,φ'1,AODFtx,s,φ''θ'1,ZOD,φ'1,AOD

Using the above steps, for the case when the path corresponds to a LOS path, then  S=SLOS=100-1 is an orthonormal matrix, and the multiplication of orthogonal matrices is again orthogonal.

In a NLOS path, S will introduce a non-orthogonal transformation of the electric field of the waveform. The transmitted field-vector is rotated by some unknown angle δ which can be determined in the received LCS of the UE. At the UE, the received PRS signal is then projected to the vertical and horizontal space, which also have to be orthogonal to each other:

HtxV,rxH  , HtxH  ,rxH  , HtxV,rxV    , HtxH  ,rxV     		                             (2)

From these projections in (2), the relative rotation of the received vertical and horizontal PRS signals w.r.t to the vertical and horizontal orientation of the UE LCS can be obtained:

δV=atan HtxV  ,rxH   HtxV  ,rxV  ,   atan HtxH  ,rxH   HtxH  ,rxV = δH

The relative rotation of the received paths calculated from two orthogonal linear polarized transmitted PRSs can be a useful and beneficial metric to assist the existing positioning algorithm by identifying paths that are of high likelihood to be NLOS paths. The optimal angle threshold for the relative rotations would be subjected to further optimization and studies taking into considerations factor such as SNR, time-resolution (bandwidth),  potentially channel statistic (how much non-orthogonal scattering is present for the first path in NLOS links), antenna pattern etc. to ensure minimum performance requirements.

Rel-16 PRS transmissions framework already supports configurations of multiple PRS resources/resource sets for the UE measurements. Hence, these PRS resources can be sent with different polarizations. At the UE, it needs to be able to recognize such transmissions for the correct measurements. Informing the UE of such transmissions as part of the PRS configurations can be done as part of the PRS configurations enhancement in Rel-17.

Proposal 2: For the purpose of improving accuracy, methods to identify NLOS and the corresponding mitigation methods should be studied including the usage of polarization transmissions and measurements.

## Multipath Measurements

Current TS 38.215 defines the DL PRS-RSRP, DL RSTD and the UE Rx-Tx time difference measurements. For the time difference-based approaches such as TDOA and RTT, the measurements are based on first detected path in time. However, for positioning methods relying on PRS RSRP measurements such as AoD that relies on the total received power of the PRS, first detected may or may not be the LOS path. When later paths are received with higher power than the first path, the angle of departure estimates would be biased by these latter paths.

One approach is to be able to identify the NLOS path as described in the previous section above. For power-based methods such as AoD, other approaches such additional measurements should be studied. They may include the power of the first arrival path measurement and its K-factor w.r.t to other paths. Defining measurements relative to the power of the first detected should be further investigated.

Proposal 3: Additional measurement relative to the first detected path should be studied including its feasibility to improve AoD positioning accuracy.

## Inactive and Idle UE Positioning

For Rel-16, the positioning support is limited to UE in connected mode. For Rel-17, extending it to Inactive and Idle UEs should be supported. This would improve the positioning latency for the UE since transitioning to Connected state would not be needed prior to obtaining its position. As such, existing positioning methods: UL, DL, DL+UL approaches should be extended to both Inactive and Idle UEs. Studies should focus on the transmissions and receptions of the PRS and SRS reference signals in these modes.

Given that the studies and specifications work is expected to be greater for UL than DL, such as the considerations the transmission of SRS during RRC_INACTIVE or reusing 2-step RACH procedures,  efforts should focus on at least completing the support for DL within the Rel-17 time frame.

Proposal 4: Extend the support of Rel-16 positioning methods to Inactive and Idle UEs, at least for the DL positioning.

# Conclusions

# In this contribution, some of the proposed enhancements are discussed and our views provided. Below are the observations and proposals.

Observation 1: The UL interference due to autonomous UL SRS transmissions would need to be studied further.

Proposal 1: Aperiodic DL PRS requests and transmission should be supported for its benefits in reducing PRS transmissions overhead and improving latency. Details on the specifications support and impact are for further study.

Proposal 2: For the purpose of improving accuracy, methods to identify NLOS and the corresponding mitigation methods should be studied including the usage of polarization transmissions and measurements.

Proposal 3: Additional measurement relative to the first detected path should be studied including its feasibility to improve AoD positioning accuracy.

Proposal 4: Extend the support of Rel-16 positioning methods to Inactive and Idle UEs, at least for the DL positioning.

# References

TR38.857, “Study on NR Positioning Enhancements.”

R1-2003769, “Potential NR positioning enhancements,” Intel Corporation.

R1-2003721, “Initial views on potential positioning enhancements,” Nokia, Nokia Shanghai Bell.

R1-2003297, “Discussion of positioning enhancement,” Huawei, HiSilicon.

R1-2003285, “IIOT Positioning techniques Consideration,” Futurewei.

R1-2003642, “Discussion of NR positioning enhancements,” CATT.

TR38.901, “Study on channel model for frequencies from 0.5 to 100 GHz”