3GPP TSG RAN WG1 Meeting #102-e                                       R1-2005463

e-Meeting,August 17th – August 28th, 2020

Source:         ZTE

Title:            Evaluation results based on NR Rel-16 positioning

Agenda item:    8.5.2

Document for:	Discussion and Decision

Introduction

In 3GPP TSG RAN Meeting #86, a new study item was agreed [1]. The study item is intended to study enhancements and solutions necessary to support the high accuracy (horizontal and vertical), low latency, network efficiency (scalability, RS overhead, etc.), and device efficiency (power consumption, complexity, etc.) requirements. In RAN1#101-e meeting [2], the following agreements related to evaluation assumptions were reached.

Agreement:

- InF-SH and InF-DH models in TR 38.901 are adopted as the baseline scenarios for defining the channel models, parameters and modelling techniques for performance evaluations in the Rel. 17 positioning enhancements at least for IIoT use cases.

Agreement:

- Optional: For evaluating vertical positioning performance, UE antenna height can be uniformly distributed within [0.5, X2]m, where X2 = 2m for InF-SH and X2=hc for InF-DH defined in TR 38.901.

Agreement:

Optional: For evaluating vertical positioning performance, gNB antenna height can also be set to two fixed heights, which is either {4, 8} m, or {max(4,), 8}.

Agreement:

Clutter parameters {density r, height hc,size dclutter } for high clutter density are set as follows:

- (Baseline): {40%, 2m, 2m} for fixed UE antenna height and gNB antenna height

- (Optional): {40%, 3m, 5m}

- (Optional): {60%, 6m, 2m}

Agreement:

CDF values for positioning accuracy for IIoT scenarios are derived based on:

- Case 1 (Required): The UEs inside convex hull of the horizontal BS deployment area.

- Case 2 (Optional): All the UEs

Agreement:

- In Rel-17 target positioning requirements for IIoT use cases are defined as follows:

  - Horizontal position accuracy (< X m) for [90%] of UEs

    - X = [0.2 or 0.5] m

  - Vertical position accuracy (< Y m) for [90%] of UEs

    - Y = [0.2 or 1] m

  - End-to-end latency for position estimation of UE (< [10ms, 20ms, or 100ms])

  - FFS: Physical layer latency for position estimation of UE (< [10ms])

In this contribution, we provide some evaluation results on InF-SH and InF-DH scenarios.

Evaluation results of InF scenarios

In this section, we provide some simulation results on InF-SH and InF-DH scenarios. Some basic simulation assumptions are discussed in our companion contribution [3], and other detailed configurations and algorithms are listed in appendix table I and table II. In addition, table III and table IV in the appendix reveal some controlled variables of all simulation cases.

Firstly, the horizontal positioning simulation results of FR1 and FR2 for InF-SH are shown in table 1and table 2 respectively, and the corresponding CDF curves are attached in appendix figure 1. As it can be shown in table 1 and table 2, when there is no network synchronization error and all UEs are inside convex hull, the horizontal positioning accuracy of 90% UEs are less than 0.450 m in FR1, while the value is 0.044 m in FR2. Regarding the cases with 50 ns synchronization error, the horizontal positioning error is larger than 40 m for most of cases at the percentile of 90% UEs. In addition, all simulation cases in table 1 and table 2, the positioning accuracy of UE inside convex hull is slightly better than the cases where UEs are uniformly distributed over the factory, because the high quality links for positioning are always enough under high LOS probability scenario.

Table 1 CDF of horizontal positioning accuracy in InF-SH scenario,FR1

| Simulation case | UE horizontal drop procedure | Network synchronization | Horizontal positioning accuracy (m) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50% | 67% | 80% | 90% |
| Case 1 | Inside convex hull | Without sync error | 0.206 | 0.279 | 0.345 | 0.450 |
| Case 2 | Uniformly distributed | Without sync error | 0.205 | 0.267 | 0.345 | 0.439 |
| Case 3 | Inside convex hull | 50 ns sync error | 9.816 | 12.745 | 21.583 | 41.545 |
| Case 4 | Uniformly distributed | 50 ns sync error | 11.503 | 14.866 | 22.346 | 39.099 |

Table 2 CDF of horizontal positioning accuracy in InF-SH scenario,FR2

| Simulation case | UE horizontal drop procedure | Network synchronization | Horizontal positioning accuracy (m) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50% | 67% | 80% | 90% |
| Case 5 | Inside convex hull | Without sync error | 0.025 | 0.030 | 0.037 | 0.044 |
| Case 6 | Uniformly distributed | Without sync error | 0.029 | 0.035 | 0.039 | 0.046 |
| Case 7 | Inside convex hull | 50 ns sync error | 15.676 | 23.461 | 31.224 | 48.395 |
| Case 8 | Uniformly distributed | 50 ns sync error | 16.033 | 20.875 | 32.123 | 50.512 |

Observation 1: For InF-SH scenario,

- When there is no network synchronization error and all UEs are inside convex hull, the horizontal positioning accuracy of 90% UEs are less than 0.450 m in FR1, while the value is 0.044 m in FR2.

- The horizontal location error is larger than 40 m for most of cases with 50 ns synchronization error at the percentile of 90% UEs.

- The positioning accuracy of UE inside convex hull is slightly better than the cases where all UEs are uniformly distributed over the factory.

Secondly, the horizontal positioning simulation results of FR1 and FR2 for InF-DH are shown in table 3 and table 4 respectively, the corresponding CDF curves are attached in appendix figure 2. As it can be seen from table 3 and table 4, the sub-meter level requirement can be met in the following cases,

- at the percentile of 50% UEs in simulation case 11.

- at the percentile of 47% UEs in simulation case 12.

- at the percentile of 67% UEs in simulation case 15.

- at the percentile of 50% UEs in simulation case 16.

When 50 ns synchronization error exists, the horizontal positioning error of all simulation cases exceeds 40 m for at the percentile of 50% UEs. Better positioning accuracy can be observed in the cases that all UEs are inside convex hull, e.g. at the percentile of 50% UEs, simulation case 11 has 1.535 m performance gain than simulation case 12, and simulation case 15 has 0.158 m performance gain than simulation case 16.

Table 3 CDF of horizontal positioning accuracy InF-DH in scenario, FR1

| Simulation case | UE horizontal drop procedure | Network synchronization | Horizontal positioning accuracy (m) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50% | 67% | 80% | 90% |
| Case 9 | Inside convex hull | 50 ns sync error | 40.398 | 45.682 | 55.171 | 69.263 |
| Case 10 | Uniformly distributed | 50 ns sync error | 40.733 | 48.420 | 57.524 | 84.079 |
| Case 11 | Inside convex hull | Without sync error | 0.472 | 30.612 | 49.329 | 66.177 |
| Case 12 | Uniformly distributed | Without sync error | 2.007 | 44.059 | 62.029 | 82.323 |

Table 4 CDF of horizontal positioning accuracy in InF-DHscenario, FR2

| Simulation case | UE horizontal drop procedure | Network synchronization | Horizontal positioning accuracy (m) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50% | 67% | 80% | 90% |
| Case 13 | Inside convex hull | 50 ns sync error | 45.340 | 50.837 | 59.986 | 77.910 |
| Case 14 | Uniformly distributed | 50 ns sync error | 45.799 | 53.545 | 64.869 | 87.815 |
| Case 15 | Inside convex hull | Without sync error | 0.351 | 0.619 | 16.254 | 47.801 |
| Case 16 | Uniformly distributed | Without sync error | 0.509 | 18.840 | 41.870 | 70.570 |

Observation 2: For InF-DH scenario, at least the following cases with clutter settings {40%, 2m, 2m} can meet sub-meter level requirement,

- at the percentile of 50% UEs when all UEs are inside convex hull in FR1.

- at the percentile of 47% UEs when all UEs are uniformly distributed in FR1.

- at the percentile of 67% UEs when all UEs are inside convex hull in FR2.

- at the percentile of 50% UEs when all UEs are uniformly distributed in FR2.

Thirdly, the vertical positioning simulation results for InF-SH and InF-DH are shown in table 5, where the corresponding CDF curves can be found in appendix figure 3. What needs illustration is that estimated vertical locations of all UEs for InF-SH and InF-DH are restricted to 0.5-2 m and 0.5-3 m respectively. The result shows that only some cases ( i.e. simulation case 17, 18 and 20) can meet the loose vertical accuracy requirement (i.e. 1 m for 90% of UEs) based on current assumptions and Rel-16 positioning method under perfect synchronization condition.

Table 5 CDF of vertical positioning accuracy in scenario InF-SH

| Simulation case | FR1/FR2 | Scenario | Vertical positioning accuracy (m) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50% | 67% | 80% | 90% |
| Case 17 | FR1 | InF-SH | 0.387 | 0.582 | 0.741 | 0.931 |
| Case 18 | FR2 | InF-SH | 0.155 | 0.210 | 0.272 | 0.381 |
| Case 19 | FR1 | InF-DH | 0.502 | 0.731 | 1.043 | 1.245 |
| Case 20 | FR2 | InF-DH | 0.225 | 0.370 | 0.572 | 0.951 |

Observation 3: Regarding vertical positioning accuracy, only some simulation cases ( i.e. simulation case 17, 18 and 20) can meet the loose vertical accuracy requirement (i.e. 1 m for 90% of UEs) based on current assumptions and Rel-16 positioning method under perfect synchronization condition.

Conclusions

In this contribution, we provide evaluation results of InF-SH and InF-DH scenarios based on Rel-16 positioning approaches. Based on the evaluation, we have the following observations.

Observation 1: For InF-SH scenario,

- When there is no network synchronization error and all UEs are inside convex hull, the horizontal positioning accuracy of 90% UEs are less than 0.450 m in FR1, while the value is 0.044 m in FR2.

- The horizontal location error is larger than 40 m for most of cases with 50 ns synchronization error at the percentile of 90% UEs.

- The positioning accuracy of UE inside convex hull is slightly better than the cases where all UEs are uniformly distributed over the factory.

Observation 2: For InF-DH scenario, at least the following cases with clutter settings {40%, 2m, 2m} can meet sub-meter level requirement,

- at the percentile of 50% UEs when all UEs are inside convex hull in FR1.

- at the percentile of 47% UEs when all UEs are uniformly distributed in FR1.

- at the percentile of 67% UEs when all UEs are inside convex hull in FR2.

- at the percentile of 50% UEs when all UEs are uniformly distributed in FR2.

Observation 3: Regarding vertical positioning accuracy, only some simulation cases ( i.e. simulation case 17, 18 and 20) can meet the loose vertical accuracy requirement (i.e. 1 m for 90% of UEs) based on current assumptions and Rel-16 positioning method under perfect synchronization condition.

References

- RP-193237, New SID on NR Positioning Enhancements, Qualcomm Incorporated

- Draft Report of 3GPP TSG RAN WG1 #101-e v0.2.0

- R1-2003480 Evaluation results of additional scenarios for positioning, ZTE

- Appendix

## Table I Common scenario parameters

| Parameter | FR1, 100 MHz | FR2, 400 MHz |
| --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline Channel Model based on common assumptions defined related to the channel models of 3GPP TRs 38.901 / 38.802 / 37.857. |  |
| Carrier frequency | 3.5 GHz | 28 GHz |
| Subcarrier spacing | 30 KHz | 120 KHz |
| Reference Signal Transmission Bandwidth | 100 MHz | 400 MHz |
| gNB noise figure, dB | 5dB | 7dB |
| UE noise figure, dB | 9dB | 13dB |
| UE max. TX power, dBm | 23dBm | 23dBm EIRP should not exceed 43 dBm. |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | DL-PRS-CombSizeN = 6 DL-PRS-ReOffset = {0,3,1,4,2,5} DL-PRS-NumSymbols = 6 |  |
| Number of sites | 18 |  |
| UE number per site | 10 |  |
| number of occasions used per positioning estimate | 4 |  |
| Power-boosting level | 7.8dB |  |
| Uplink power control (applied/not applied) | Not applied |  |
| interference modelling (ideal muting, or other) | Ideal muting |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | MUSIC algorithm |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA, Guass-Newton algorithm |  |
| Network synchronization assumptions | Without sync error，50 ns sync error |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | DFT codebook |  |
| Additional notes, if any | - Only LOS path to calculate TOA - The absolute time of arrival is applied according to TR 38.901 |  |

## Table II Parameters common to InF scenario(s)

|  |  | FR1 Specific Values |  | FR2 Specific Values |
| --- | --- | --- | --- | --- |
| Channel model |  | InF-SH, InF-DH |  | InF-SH, InF-DH |
| Layout | Hall size | InF-SH: (baseline) 300x150 m InF-DH: (baseline) 120x60 m |  |  |
|  | BS locations | 18 BSs on a square lattice with spacing D, located D/2 from the walls. - for the small hall (L=120m x W=60m): D=20m - for the big hall (L=300m x W=150m): D=50m |  |  |
|  | Room height | 10m |  |  |
| Total gNB TX power, dBm |  | 24dBm | 24dBm EIRP should not exceed 58 dBm |  |
| gNB antenna configuration |  | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ – Note 1 | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH=dV=0.5λ – Note 1 One TXRU per polarization per panel is assumed |  |
| gNB antenna radiation pattern |  | Single sector – Note 1 | 3-sector antenna configuration – Note 1 |  |
| Peneteration loss |  | 0dB |  |  |
| Number of floors |  | 1 |  |  |
| UE horizontal drop procedure |  | Uniformly distributed over the horizontal evaluation area for obtaining the CDF values for positioning accuracy, The evaluation area should be at least the convex hull of the horizontal BS deployment. It can also be the whole hall area if the CDF values for positioning accuracy is obtained from whole hall area. |  |  |
| UE antenna height |  | horizontal evaluation: 1.5m vertical evaluation: uniformly distributed within [0.5, X2]m, where X2 = 2m for InF-SH and X2=hc for InF-DH |  |  |
| UE mobility |  | 3km/h (Optional): FFS |  |  |
| Min gNB-UE distance (2D), m |  | 0m |  |  |
| gNB antenna height |  | horizontal evaluation: 8m vertical evaluation: {8,4} |  |  |
| Clutter parameters: {density , height ,size } |  | Low clutter density: {20%, 2m, 10m} High clutter density: {40%, 2m, 2m} {40%, 3m, 5m} {60%, 6m, 2m} |  |  |
| Note 1: According to Table A.2.1-7 in 3GPP TR 38.802 |  |  |  |  |

## Table III Simulation cases for horizontal accuracy

| Simulation Case | Scenario | FR1/FR2 | UE horizontal drop procedure | Network synchronization | Clutter parameters |
| --- | --- | --- | --- | --- | --- |
| Case 1 | InF-SH | FR1 | Inside convex hull | Without sync error | {20%, 2m, 10m} |
| Case 2 | InF-SH | FR1 | Uniformly distributed | Without sync error | {20%, 2m, 10m} |
| Case 3 | InF-SH | FR1 | Inside convex hull | 50 ns sync error | {20%, 2m, 10m} |
| Case 4 | InF-SH | FR1 | Uniformly distributed | 50 ns sync error | {20%, 2m, 10m} |
| Case 5 | InF-SH | FR2 | Inside convex hull | Without sync error | {20%, 2m, 10m} |
| Case 6 | InF-SH | FR2 | Uniformly distributed | Without sync error | {20%, 2m, 10m} |
| Case 7 | InF-SH | FR2 | Inside convex hull | 50 ns sync error | {20%, 2m, 10m} |
| Case 8 | InF-SH | FR2 | Uniformly distributed | 50 ns sync error | {20%, 2m, 10m} |
| Case 9 | InF-DH | FR1 | Inside convex hull | Without sync error | {60%, 6m, 2m} |
| Case 10 | InF-DH | FR1 | Uniformly distributed | Without sync error | {60%, 6m, 2m} |
| Case 11 | InF-DH | FR1 | Inside convex hull | Without sync error | {40%, 2m, 2m} |
| Case 12 | InF-DH | FR1 | Uniformly distributed | Without sync error | {40%, 2m, 2m} |
| Case 13 | InF-DH | FR2 | Inside convex hull | Without sync error | {60%, 6m, 2m} |
| Case 14 | InF-DH | FR2 | Uniformly distributed | Without sync error | {60%, 6m, 2m} |
| Case 15 | InF-DH | FR2 | Inside convex hull | Without sync error | {40%, 2m, 2m} |
| Case 16 | InF-DH | FR2 | Uniformly distributed | Without sync error | {40%, 2m, 2m} |

## Table IV Simulation cases for vertical accuracy

| Simulation Case | Scenario | FR1/FR2 | UE horizontal drop procedure | Network synchronization | Clutter parameters |
| --- | --- | --- | --- | --- | --- |
| Case 17 | InF-SH | FR1 | Inside convex hull | Without sync error | {20%, 2m, 10m} |
| Case18 | InF-SH | FR2 | Inside convex hull | Without sync error | {20%, 2m, 10m} |
| Case19 | InF-DH | FR1 | Inside convex hull | Without sync error | {40%, 3m, 5m} |
| Case 20 | InF-DH | FR2 | Inside convex hull | Without sync error | {40%, 3m, 5m} |

## Figure 1  CDF curves of horizontal positioning error for InF-SH

Figure 1-1 InF-SH, FR1, without sync error           Figure 1-2 InF-SH, FR1, 50ns sync error

Figure 1-3 FR2, without sync error                   Figure 1-4 InF-SH, FR2, 50ns sync error

## Figure 2  CDF curves of horizontal positioning error for InF-DH

Figure 2-1 InF-DH, FR1, 50 ns sync error            Figure 2-2 InF-DH, FR1, without sync error

Figure 2-3 InF-DH, FR2, 50 ns sync error            Figure 2-4 InF-DH FR2, without sync error

## Figure 3  CDF curves of vetical positioning error for InF-SH and InF-DH

Figure 3-1 InF-SH, vertical accuracy                  Figure 3-2 InF-DH, vertical accuracy