3GPP TSG RAN WG1 Meeting #102                               R1-2005470

e-Meeting, Aug 17th – Aug 28th, 2020

Source:           ZTE

Title:              Remaining issues on scheduling enhancement for MTC

Agenda item:      6.2.1

Document for:     Discussion and Decision

Introduction

For multi-TB scheduling in Rel-16 MTC enhancement, some remaining issues about scheduling gap, early termination, TDD design and legacy features in unicast have not been discussed due to the limited time. In this document, the remaining issues are discussed.

Remaining issues for multi-TBs scheduling

Early termination

In RAN1 #101-e meeting, the discussion about early termination has not been decided. There are mainly two options -under discussion.

Option1: terminate all TBs

Option2: terminate individual TB(s)

In this meeting, we may need to make a decision from above two options. Some companies thought option1 is consistent with the current spec. However, the current spec is just for single TB and does not show the compatibility with multi-TBs case. For example in 36.213, the UE behavior is described as following:

A BL/CE UE configured with mpdcch-UL-HARQ-ACK-FeedbackConfig shall upon detection on a given serving cell of an MPDCCH with DCI format 6-0A/6-0B intended for the UE in the UE-specific search space indicating HARQ-ACK corresponding to a transport block associated to a HARQ process

Obviously, it is just for single TB associated to a HARQ process, instead of multiple TBs.

Observation 1: The current spec for early termination is used for a transport block, instead of multiple TBs.

It is noticed that some propose the early termination to be used for freeing up resources for high priority transmissions. Actually, the intention of introducing early termination is used for UE power saving, and the agreement in 88b meeting is shown as following:

RAN1#88bis discussed uplink HARQ-ACK feedback and made the following agreement:

- For the purpose of evaluations of HARQ-ACK feedback for UE power saving: - At least the following two options are considered for HARQ-ACK feedback channel: - Option 1: One HARQ-ACK feedback channel for multiple UE PUSCHs in one transmission - Option 2: One HARQ-ACK feedback channel for a single UE PUSCH in one transmission - Detailed design on HARQ-feedback channel FFS among at least the following options: - PHICH-like channel - MPDCCH with reduced DCI size(s) - MPDCCH with existing DCI size(s) - Early termination of PUSCH transmission and/or MPDCCH monitoring are considered. - The detailed solutions can be different for HD-FDD and FD-FDD/TDD.

Observation 2: The purpose of introducing early termination in R15 is not for freeing up resources for high priority transmissions, but instead for UE power saving.

For the non-interleaving case with multi-TBs scheduling, if the uplink ACK feedback is sent by the eNB, option 2, save more power in most cases and the power saving for option 1 is extraordinarily limited, since only the power for last TB can be saved. The illustration is shown as following:

Figure 1. Case1: difference of power saving between option2 and option1 when all the TBs are decoded successfully.

Figure 2.  Case2: option1 would not be used when one of the TBs is decoded with failure.

For case1, when all the TBs are decoded successfully, option1 can save some transmission power for last TB. For case2, when one of the TBs is decoded with failure, option 1 cannot be used to terminate all the TBs, which is conflicting with the consensus about ACK (that the eNB send the ACK after eNB decode the TB successfully).

Observation 3: Option 1 cannot be used for non-interleaving case without changing the definition of ACK.

Last, some companies thought option1 can be used to terminate the on-going PUSCH transmission, even it has not been decoded successfully. For option2, each ACK means that the corresponding TB successfully decoded, and the detection error of ACK to NACK is the same with legacy. For option1, if the ACK is sent and the corresponding TB is not decoded, it actually increases the detection error of ACK to NACK. Based on this, detection error probability maybe much higher than the predefined value and also results in more RLC retransmission and delay. Moreover, this is not a rather exceptional situation, for example, for 8 TBs scheduling, the probability of at least one of them is decoded with failure is 0.57 based on the BLER equal to 0.1. The more TBs scheduled, the more serious the influence.

Observation 4: Option 1 would cause higher ACK detection error probability, RLC retransmission and delay than legacy, if uplink HARQ-ACK feedback can be used to terminate the failed TB.

Based on the above analysis, we have the following proposal:

Proposal 1:  Individual TB feedback for early termination should be adopted.

Moreover, here is the text proposal in 36.212:

<Unchanged parts are omitted>

If multi-TB-UL-config is not enabled and the Resource block assignment in format 6-0A is set to all ones, or multi-TB-UL-config is enabled and the 6 MSB bits of the Scheduling TBs for Unicast Field are set to '110111', format 6-0A is used for the indication of ACK feedback, 8 bits including the 6 LSB bits of the Scheduling TBs for Unicast Field and 2 MSB bits of repetition number are used to indicate 8 HARQ-ACK by bitmap corresponding to HARQ processes 1-8 if multi-TB-UL-config is enabled, and all the remaining bits except Flag format 6-0A/format 6-1A differentiation and DCI subframe repetition number are set to zero.

<Unchanged parts are omitted>

<Unchanged parts are omitted>

If multi-TB-UL-config is not enabled and the Modulation and coding scheme in format 6-0B is 4 bits and set to all ones, or multi-TB-UL-config is enabled and the 6 MSB bits of the Scheduling TBs for Unicast Field are set to '111111', format 6-0B is used for the indication of ACK feedback, 4 LSB bits of the Scheduling TBs for Unicast Field are used to indicate 4 HARQ-ACK by bitmap corresponding to HARQ processes 1-4 by bitmap if multi-TB-UL-config is enabled, and all the remaining bits except Flag for format 6-0B/format 6-1B differentiation and DCI subframe repetition number are set to zero.

<Unchanged parts are omitted>

<TP 1, TS 36.212, 5.3.3.1 >

Proposal 2: TP1 should be endorsed.

HARQ processes for TDD CE mode B

According to the agreement [2], it is agreed that the maximum TB number is 4 for CE mode B, which can be applicable for TDD and FDD. However, in the current spec, for TDD a BL/CE UE configured with CEModeB, the maximum TB number is 2. Therefore, the current spec is not consistent with the agreement.

Additionally, if the maximum HARQ processes for CE mode B in TDD is only 2, the DCI design should be reconsidered. Obviously, the current DCI design for 4 HARQ processes will cause lot of resource waste, since the 3 TBs and 4 TBs scheduling is not necessary any more. Also, the corresponding spec for 3 HARQ processes and 4 HARQ process scheduling in 36.212 for CE mode B should be removed. Considering the original intention when RAN1 agreed to have 4 TB for CE mode B, it is suggested to increase the maximum HARQ process number from 2 to 4 for TDD CE mode B.

Therefore, to keep consistence with the agreement and minimize the specification impact, we have the following text proposal:

  - 8.0	UE procedure for transmitting the physical uplink shared channel

<Unchanged parts are omitted>

For a BL/CE UE configured with CEModeA and for TDD, the maximum number of HARQ processes per serving cell shall be determined by the UL/DL configuration (Table 4.2-2 of [3]) according to the normal HARQ operation in Table 8-1. For TDD a BL/CE UE configured with CEModeB is not expected to support more than 4 uplink HARQ processes per serving cell if the UE is configured with higher layer parameter multi-TB-UL-config, 2 uplink HARQ processes per serving cell otherwise.

<Unchanged parts are omitted>

<TP 2, TS 36.213, 8.0>

Proposal 3: TP2 should be endorsed.

TDD grouping

Agreement in #99 meeting

For DCI 6-1A

  - For TDD, apply grouping of HARQ processes with consecutive HARQ PIDs: two groups with 8 HARQ process per each group and 1 bit indication of the group.

Based on the agreement, two key words should be noted: consecutive HARQ PID and 8 HARQ process per each group. It is clear that when the maximum HARQ process number is larger than 8, 2 groups should be provided, each group contains 8 HARQ processes and the HARQ PIDS are consecutive.

According to the current spec description,  if ‘Multi-TB HARQ processes group’ field is present and set to '1' , then we have . If the number of HARQ processes is 10 and k=1, one group contains 8 HARQ process and another group contains 2 HARQ process, which is contradictory to the agreement that each group contains 8 HARQ processes.

Observation 5: The current description about HARQ grouping(e.g., 11 HARQ processes in total, includes first 8 HARQ processes in the first group and last 3 HARQ processes in the second group) is contradictory to the agreement that each group contains 8 HARQ processes.

Moreover, in this case, if the second group containing 3 HARQ processes, there is no chance to schedule the other HARQ processes in the first group within one DCI. Therefore, the mixed scheduling is limited within one group.

Additionally, the DCI overhead for second group would always be less than that for first group, which cause the DCI bits wasting. Especially for the TDD UL/DL configuration 2 in downlink, the number of HARQ processes is 9 and the second group just contains 1 HARQ process. It would waste a lot of DCI bits. Therefore,

Observation 6: The current method about HARQ grouping may

- Limit the mixed scheduling within one group, which does not help the PDCCH saving.

- Cause a lot of DCI bits wasting.

Therefore, in order to avoid the contradiction, obtain larger scheduling flexibility, reduce specification efforts and make the best of DCI bits, the grouping method can be described as following: 1 bit is used to indicate 2 groups, 1 group contains the first 8 HARQ processes and the other group contains last 8 HARQ processes. Compared with the method described in the above text, it makes the best of DCI bits which avoids the DCI bits wasting, it has higher scheduling flexibility which provides more HARQ ID combinations, and it has the lowest specification efforts for the TDD case with maximum HARQ process number larger than 8.

Additionally, there are two editorial errors for this section in the current specification. The first is the equation  should be changed to .  Also, for the second ‘corresponding’, a symbol ‘n’ is used inside the word for some unknown reason. It is suggested these two editorial errors should also be fixed.

<Unchanged parts are omitted>

For a BL/CE UE, if the UE is configured with higher layer parameter multi-TB-DL-config and multiple TB, , are scheduled in the corresponding DCI with CRC scrambled by C-RNTI, the HARQ process ID  for each of the scheduled  TBs are determined from the value of the HARQ index field in the corresponding DCI which is a combinatorial index r defined as , where

-	the set , () contains the  sorted HARQ process IDs and  is the extended binomial coefficient, resulting in unique label ,

-	 is the number of scheduled TB, and

-	 if UE is configured with CEModeA, and  if UE is configured with CEModeB,

-	 and  is the maximum number of HARQ processes for the given TDD operation, if UE is configured with CEModeA, and ‘Multi-TB HARQ processes group’ field is present and set to '1' in the corresponding DCI,  otherwise.

<Unchanged parts are omitted>

<TP 3, TS 36.213, 7.1.7.2>

From TP3, we can see that a typo and a formula error have been corrected. Therefore, we have the proposal

Proposal 4: If the number of HARQ processes is larger than 8 for TDD, one group contains the first 8 HARQ processes and the other group contains the last 8 HARQ processes (some HARQ processes can be referenced by both groups).

Proposal 5: Adopt TP3 which includes TDD grouping change and two editorial corrections.

Conclusion

In this contribution, we have discussed the scheduling enhancement for MTC. We make the following observations and proposals:

Observations:

Observation 1: The current spec for early termination is used for a transport block, instead of multiple TBs.

Observation 2: The purpose of introducing early termination in R15 is not for freeing up resources for high priority transmissions, but instead for UE power saving.

Observation 3: Option 1 cannot be used for non-interleaving case without changing the definition of ACK.

Observation 4: Option 1 would cause higher ACK detection error probability, RLC retransmission and delay than legacy, if uplink HARQ-ACK feedback can be used to terminate the failed TB.

Observation 5: The current description about HARQ grouping(e.g., 11 HARQ processes in total, includes first 8 HARQ processes in the first group and last 3 HARQ processes in the second group) is contradictory to the agreement that each group contains 8 HARQ processes.

Observation 6: The current method about HARQ grouping may

- Limit the mixed scheduling within one group, which does not help the PDCCH saving.

- Cause a lot of DCI bits wasting.

Proposals:

Proposal 1: For uplink ACK feedback with multi-TBs scheduling, option 2 with individual feedback should be supported.

Proposal 2: TP1 should be endorsed.

Proposal 3: TP2 should be endorsed.

Proposal 4: If the number of HARQ processes is larger than 8 for TDD, one group contains the first 8 HARQ processes and the other group contains the last 8 HARQ processes (some HARQ processes can be referenced by both groups).

Proposal 5: Adopt TP3 which includes TDD grouping change and two editorial corrections.

- References

- 3GPP,RAN1 Chairman’s Notes final, May 25th – June 5th, 2020 3GPP TSG RAN1 Meeting #101e.

- 3GPP,RAN1 Chairman’s Notes final, November 12th – November 16th, 2018 3GPP TSG RAN1 Meeting #95.