3GPP TSG-RAN WG1 Meeting #102-e	R1-2005507

e-Meeting, 17th-28th August 2020

Agenda Item:	7.2.5.2

Source:	Ericsson

Title:	Remaining Issues of UCI Enhancements for NR URLLC

Document for:	Discussion, Decision

# 1	Introduction

In this contribution, we address the remaining issues and propose solutions with respect to Rel-16 specifications of NR URLLC.

# 2	Discussion

## 2.1 	Configuration of two PUCCH-Configs

At RAN1#100bis-e meeting, the following was agreed:

Agreements:

- SR priority comes from phy-PriorityIndex-r16 in SchedulingRequestResourceConfig. If not configured, SR is treated as low priority (index 0).

- In Rel-16, if a UE is configured with one HARQ codebook, the HARQ-ACK codebook is considered as low priority.

- If a PUCCH-Config is provided with a subslotLengthFor PUCCH-r16, the PUCCH resource corresponding to any SR or CSI configuration with the same priority as the PUCCH-Config, should be confined within the sub-slot associated to the PUCCH-Config.

The third bullet in the above agreement has not been captured yet in Rel-16 NR specifications. When the agreement was made, RAN1 assumed that the 3rd bullet could be captured in TS38.331 for example as part of stating the restriction on the starting symbol and duration of a PUCCH resource for SR or CSI configurations. However, after further consideration, it seems to us that TS38.213 is better suited for capturing the missing agreement.

The reason is that the agreed constraint describes a dependency between the properties of different IEs. Our preference is to avoid introducing dependency between different IEs in TS 38.331 if possible and keep an IE description intact. When the corresponding IEs are provided in TS38.213 due to the corresponding procedures, we can describe the constraint that would affect these IEs together.

Moreover, on placement of the corresponding text proposal, our view is that since the agreement would have an impact on the overlapping resolution procedures between PUCCH/PUSCH resources of the same or different priorities, the corresponding TP is better suited in Clause 9 in TS38.213 as proposed in the following:

Adopt the following TP1 for Subclause 9 of TS38.213.

--------------------------------- Start of Proposed Text 1 for TS 38.213 -------------------------------

9		UE procedure for reporting control information

A PUSCH or a PUCCH transmission, including repetitions if any, can be of priority index 0 or of priority index 1. For a configured grant PUSCH transmission, a UE determines a priority index from priority, if provided. For a PUCCH transmission with HARQ-ACK information corresponding to a SPS PDSCH reception or a SPS PDSCH release, a UE determines a priority index from harq-CodebookID, if provided. For a PUCCH transmission with SR, a UE determines the corresponding priority as described in subclause 9.2.4. If a priority index is not provided to a UE for a PUSCH or a PUCCH transmission, the priority index is 0.

If a PUCCH-Config is provided with a subslotLengthFor PUCCH, the PUCCH resource corresponding to any SR or CSI configuration with the same priority as the PUCCH-Config, should be confined within the sub-slot associated to the PUCCH-Config.

--------------------------------- End of Proposed Text 1 for TS 38.213 -------------------------------

## 2.2	Inclusion of DCI formats 0_2/1_2 in HARQ-ACK codebook generation

The following proposal suggests a set of changes across few Subclauses in TS 38.213 that have to be implemented for specification to correctly incorporate the contribution of new DCI formats in HARQ-ACK codebook generation and corresponding procedures.

Adopt the following TP2 for Subclauses 9.1.1 to 9.1.2.2 and 9.1.3.2 of TS 38.213.

--------------------------------- Start of Proposed Text 2 to TS 38.213 -------------------------------

9.1.1	CBG-based HARQ-ACK codebook determination

If a UE is provided PDSCH-CodeBlockGroupTransmission for a serving cell, the UE receives a PDSCH scheduled by DCI format 1_1 or DCI format 1_2, that includes code block groups (CBGs) of a transport block. The UE is also provided maxCodeBlockGroupsPerTransportBlock indicating a maximum number  of CBGs for generating respective HARQ-ACK information bits for a transport block reception for the serving cell.

*** Unchanged text is omitted ***

9.1.2	Type-1 HARQ-ACK codebook determination

This clause applies if the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static.

A UE reports HARQ-ACK information for a corresponding PDSCH reception or SPS PDSCH release only in a HARQ-ACK codebook that the UE transmits in a slot indicated by a value of a PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format 1_0 or DCI format 1_1DCI format. The UE reports NACK value(s) for HARQ-ACK information bit(s) in a HARQ-ACK codebook that the UE transmits in a slot not indicated by a value of a PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format 1_0 or DCI format 1_1.

*** Unchanged text is omitted ***

9.1.2.1            Type-1 HARQ-ACK codebook in physical uplink control channel

*** Unchanged text is omitted ***

If an occasion for a candidate PDSCH reception can be in response to a PDCCH with DCI format 1_1 or DCI format 1_2 and if maxNrofCodeWordsScheduledByDCI indicates reception of two transport blocks, when the UE receives a PDSCH with one transport block, the HARQ-ACK information is associated with the first transport block and the UE generates a NACK for the second transport block if harq-ACK-SpatialBundlingPUCCH is not provided and generates HARQ-ACK information with value of ACK for the second transport block if harq-ACK-SpatialBundlingPUCCH is provided.

A UE determines   HARQ-ACK information bits, for a total number of  HARQ-ACK information bits, of a HARQ-ACK codebook for transmission in a PUCCH according to the following pseudo-code. In the following pseudo-code, if the UE does not receive a transport block or a CBG, due to the UE not detecting a corresponding DCI format 1_0, or DCI format 1_1 or DCI format 1_2, the UE generates a NACK value for the transport block or the CBG. The cardinality of the set  defines a total number  of occasions for PDSCH reception or SPS PDSCH release for serving cell  corresponding to the HARQ-ACK information bits.

*** Unchanged text is omitted ***

If , the UE determines a number of HARQ-ACK information bits  for obtaining a transmission power for a PUCCH, as described in Clause 7.2.1, as  where

-    is the number of transport blocks the UE receives in PDSCH reception occasion  for serving cell  if harq-ACK-SpatialBundlingPUCCH and PDSCH-CodeBlockGroupTransmission are not provided, or the number of transport blocks the UE receives in PDSCH reception occasion  for serving cell  if PDSCH-CodeBlockGroupTransmission is provided and the PDSCH reception is scheduled by a DCI format 1_0 or DCI format 1_2, or the number of PDSCH receptions if harq-ACK-SpatialBundlingPUCCH is provided or SPS PDSCH release in PDSCH reception occasion  for serving cell  and the UE reports corresponding HARQ-ACK information in the PUCCH.

*** Unchanged text is omitted ***

9.1.2.2	Type-1 HARQ-ACK codebook in physical uplink shared channel

If a UE would multiplex HARQ-ACK information in a PUSCH transmission that is not scheduled by a DCI format or is scheduled by DCI format 0_0, then

-	if the UE has not received any PDSCH or SPS PDSCH release that the UE transmits corresponding HARQ-ACK information in the PUSCH, based on a value of a respective PDSCH-to-HARQ_feedback timing indicator field in a DCI format scheduling the PDSCH reception or the SPS PDSCH release or on the value of dl-DataToUL-ACK or dl-DataToUL-ACK-ForDCI-Format1_2-r16 if the PDSCH-to-HARQ_feedback timing indicator field is not present in the DCI format 1_1 or DCI format 1_2, respectively, in any of the  occasions for candidate PDSCH receptions by DCI format 1_0 or DCI format 1_1 DCI format or SPS PDSCH on any serving cell , as described in Clause 9.1.2.1, the UE does not multiplex HARQ-ACK information in the PUSCH transmission;

-	else the UE generates the HARQ-ACK codebook as described in Clause 9.1.2.1, except that harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH, unless the UE receives only a SPS PDSCH release, or only SPS PDSCH reception, or only a PDSCH that is scheduled by DCI format 1_0 with a counter DAI field value of 1 on the PCell in the  occasions for candidate PDSCH receptions in which case the UE generates HARQ-ACK information only for the SPS PDSCH release or only for the PDSCH reception as described in Clause 9.1.2.

A UE sets to NACK value in the HARQ-ACK codebook any HARQ-ACK information corresponding to PDSCH reception or SPS PDSCH release that the UE detects in a PDCCH monitoring occasion that starts after a PDCCH monitoring occasion where the UE detects a DCI format 0_0 or a DCI format 0_1 scheduling the PUSCH transmission.

A UE does not expect to detect a DCI format switching a DL BWP within  symbols prior to a first symbol of a PUSCH transmission where the UE multiplexes HARQ-ACK information, where  is defined in [6, TS 38.214].

If a UE multiplexes HARQ-ACK information in a PUSCH transmission that is scheduled by DCI format 0_1 or DCI format 0_2, the UE generates the HARQ-ACK codebook as described in Clause 9.1.2.1 when a value of the DAI field in DCI format 0_1 or DCI format 0_2 is  except that harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH. The UE does not generate a HARQ-ACK codebook for multiplexing in the PUSCH transmission when  unless the UE receives only a SPS PDSCH release, or only SPS PDSCH(s), or only a PDSCH that is scheduled by DCI format 1_0 with a counter DAI field value of 1 on the PCell in the  occasions for candidate PDSCH receptions in which case the UE generates HARQ-ACK information only for the SPS PDSCH release or only for the PDSCH reception as described in Clause 9.1.2.  if the DAI field in DCI format 0_1 or DCI format 0_2 is set to '0'; otherwise, .

*** Unchanged text is omitted ***

9.1.3.2	Type-2 HARQ-ACK codebook in physical uplink shared channel

If a UE would multiplex HARQ-ACK information in a PUSCH transmission that is not scheduled by a DCI format or is scheduled by a DCI format that does not include a DAI field, then

-	if the UE has not received any PDCCH within the monitoring occasions for DCI formats scheduling PDSCH receptions or SPS PDSCH release on any serving cell  and the UE does not have HARQ-ACK information in response to a SPS PDSCH reception to multiplex in the PUSCH, as described in Clause 9.1.3.1, the UE does not multiplex HARQ-ACK information in the PUSCH transmission;

-	else, the UE generates the HARQ-ACK codebook as described in Clause 9.1.3.1, except that harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH.

If a UE multiplexes HARQ-ACK information in a PUSCH transmission that is scheduled by DCI format 0_1 or DCI format 0_2, the UE generates the HARQ-ACK codebook as described in Clause 9.1.3.1, with the following modifications:

-	For the pseudo-code for the HARQ-ACK codebook generation in Clause 9.1.3.1, after the completion of the  and  loops, the UE sets  where  is the value of the DAI field in DCI format 0_1 or DCI format 0_2 according to Table 9.1.3-2

-	For the case of first and second HARQ-ACK sub-codebooks, DCI format 0_1 or DCI format 0_2 includes a first DAI field corresponding to the first HARQ-ACK sub-codebook and a second DAI field corresponding to the second HARQ-ACK sub-codebook

-	harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH.

If a UE is not provided PDSCH-CodeBlockGroupTransmission and the UE is scheduled for a PUSCH transmission by DCI format 0_1 or DCI format 0_2 with DAI field value  and the UE has not received any PDCCH within the monitoring occasions for PDCCH with DCI format 1_0 or DCI format 1_1 DCI format for scheduling PDSCH receptions or SPS PDSCH release on any serving cell  and the UE does not have HARQ-ACK information in response to a SPS PDSCH reception to multiplex in the PUSCH, as described in Clause 9.1.3.1, the UE does not multiplex HARQ-ACK information in the PUSCH transmission.

If a UE is provided PDSCH-CodeBlockGroupTransmission and the UE is scheduled for a PUSCH transmission by DCI format 0_1 or DCI format 0_2 with first DAI field value  or with second DAI field value  and the UE has not received any PDCCH within the monitoring occasions for PDCCH with DCI format 1_0 or with DCI format 1_1 or DCI format 1_2, respectively, for scheduling PDSCH receptions or SPS PDSCH release on any serving cell  and the UE does not have HARQ-ACK information in response to a SPS PDSCH reception to multiplex in the PUSCH, as described in Clause 9.1.3.1, the UE does not multiplex HARQ-ACK information for the first sub-codebook or for the second sub-codebook, respectively, in the PUSCH transmission.

Table 9.1.3-2: Value of DAI in DCI format 0_1

| DAI MSB, LSB |  | Number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH transmission(s) associated with PDCCH or PDCCH indicating SPS PDSCH release is present, denoted as and |
| --- | --- | --- |
| 0,0 | 1 |  |
| 0,1 | 2 |  |
| 1,0 | 3 |  |
| 1,1 | 4 |  |

---------------------------------------- End of Text Proposal 2 to TS 38.213 ---------------------------------------

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Adopt the following TP1 for Subclause 9 of TS38.213.

Proposal 2	Adopt the following TP2 for Subclauses 9.1.1 to 9.1.2.2 and 9.1.3.2 of TS 38.213.