3GPP TSG RAN WG1#102e	        	R1-2005560

E-meeting, August 17th-28th, 2020

Agenda Item:					    8.1.1

Source:	Sony

Title:	Considerations on the enhancement of multi-beam operation

Document for:	Discussion

# Introduction

In RANP#86 meeting, the WID on multi-beam enhancement of Rel.17 was established as below

- Enhancement on multi-beam operation, mainly targeting FR2 while also applicable to FR1: - Identify and specify features to facilitate more efficient (lower latency and overhead) DL/UL beam management to support higher intra- and L1/L2-centric inter-cell mobility and/or a larger number of configured TCI states: - Common beam for data and control transmission/reception for DL and UL, especially for intra-band CA - Unified TCI framework for DL and UL beam indication - Enhancement on signaling mechanisms for the above features to improve latency and efficiency with more usage of dynamic control signaling (as opposed to RRC) - Identify and specify features to facilitate UL beam selection for UEs equipped with multiple panels, considering UL coverage loss mitigation due to MPE, based on UL beam indication with the unified TCI framework for UL fast panel selection

In this contribution, we present our considerations on the enhancement of multi-beam operation in Rel.17.

# L1/L2 inter-cell mobility

In Rel.15/16 NR, the intra-cell mobility was supported and enhanced on a per-beam basis. In the WID of Rel.17, RAN plans to support the inter-cell mobility with as less latency as possible, i.e. by using the L1/L2 signaling, not relying on conventional handover procedure which basically involves RRC signaling and suffers from large latency. The general handover procedure for UE in RRC_CONNECTED in TS 38.300 can be found in Figure 1.

- : RRC involved inter-cell mobility

The motivation for introducing this L1/L2-centric inter-cell mobility is to address an issue associated with a moderate- to high-speed UE that intends to cross the border of its serving cell and enter the coverage of a non-serving cell. The benefit comes from avoiding the undesirably large latency of RRC signaling before the UE establishes beam-level connection with a target cell. Instead, by reusing the initial RRC configuration (no need to re-configure), L1/L2 signaling can be applied to conduct beam management with the target cell as early as possible.

Conventionally, the procedure of beam management may include the following steps: beam measurement, beam reporting and beam indication. Without the involvement of L3 signaling during handover procedure, let’s reconsider these aspects.

Beam measurement

Without QCL assumption or TCI states configuration from RRCReconfiguration, the measurement setting of RRM can be leveraged by a UE to find out proper DL beam(s) from non-serving cell(s). More specifically during initial RRC configuration, the UE can be pre-configured to measure SSB and/or CSI-RS resources from non-serving cell(s) with pre-configured L3 filtering. Moreover, if these RS resources associated with RRM or detected by UE can be configured into TCI states or applied as QCL assumptions in DL and/or spatial relation in UL, the L1/L2 method can then be enabled accordingly.

Beam reporting

When any of triggering conditions of inter-cell handover has been satisfied, a UE may report the L3-filtered RSRP to NW. In addition, the results of L1-related measurement can be reported as well. From the perspective of latency, L3-RSRP reporting normally takes longer time than L1-RSRP reporting. But on the other hand, the stability of L3-RSRP is better than that of L1-RSRP in making handover decision. It seems no harm to allow both L3-RSRP and L1-RSRP reporting and it is up to NW on selecting which results for inter-cell mobility purpose.

Beam indication

Based on the Rel.15/16 framework of beam indication, one simple modification can be implemented to support beam indication from non-serving cell(s). Taking the TCI state in DL as example, it can be done by optionally allowing PCI (either from serving cell or non-serving cell) configured into QCL-info during initial RRC configuration. Afterwards, NW in serving cell is able to apply the L2 approach, i.e. MAC CE to activate the particular TCI state(s) which contains the target cell’s PCI for a UE’s reception of PDCCH and/or PDSCH from target cell(s). Moreover, one DCI in target cell may indicate the Tx beam(s) for PDSCH from target cell. That’s the so called L1 approach. As a consequence, the UE then can receive PDCCH/PDSCH in advance from target cell with aligned Rx beam(s). Since the UE can be configured with TCI states including any PCI in the network, this kind of TCI state can be given a name, such as “universal” TCI state which is just for discussion purpose, not to be included into Spec.

For UL in non-serving cell, it is also reasonable to add PCI of one target cell into spatial relation, e.g. PUCCH-SpatialRelationInfo and SRS-SpatialRelationInfo. With such configuration and activation if needed, the UE can access the target cell in UL too with as less latency as possible.

Based on our considerations on both DL and UL, we have following

- : Study and specify (if necessary) the L1/L2-centric approach of both DL and UL beam management considering the use cases of inter-cell mobility

- Incorporate PCI into TCI state for DL beam management

- Incorporate PCI into Spatial Relation for UL beam management

Even though we state L1/L2 approach should be specified to assist inter-cell mobility, the L3 (RRC) signaling is still necessary in some cases. For instance, the initial RRC configuration which adds DL beams from target cell(s) to TCI states is surely necessary. In addition, we have to keep in mind the fact that the RRC configuration between serving cell(s) and target cell(s) can be greatly different. It seems too restrictive to have all cells configured in the exactly same format. So after beam-level connection established between UE and target cell, RRC reconfiguration might be needed sooner or later depending on the system aspects other than beam management. Hence we would like to share

- : Beam management from non-serving cell can be carried out before RRCReconfiguration which doesn’t mean that RRCReconfiguration is completely unnecessary.

# Common beam management

## Common beam management among BWPs/CCs (intra-band)

In the case of multiple BWP/CC for intra-band CA, each BWP/CC may require at least one beam management procedure respectively. The UE needs to monitor multiple beam sweeping procedures, which cause heavy UE burden. For DL beam management, SSB and/or CSI-RS resource sets can be used. In the current specification a QCL-TypeD relation has been defined among RS resources, but not among RS resource sets. In order to reduce the number of beam management procedures, a QCL assumption among RS resource sets on different BWPs/CCs needs to be specified to reduce UE burden.

- : Specify the QCL assumption among SSB/CSI-RS resource sets on different BWPs/CCs (intra band).

## Optimizing TCI states (inter-band)

For the case of multiple CCs in different bands, i.e., inter-band CCs, there are, as well, opportunities to reduce the burden of signaling overhead. With no optimizations, multiple beam management procedures may be required in the inter-band case to cater for multiple CCs, BWPs and UE panels, as illustrated for the intra-band case. This has several adverse effects:

- Time-frequency resources need to be reserved for initial beam establishment and beam refinement on multiple CCs.

- The burden of signaling overhead, e.g., to configure suitable TCI states, increases with the number of CCs and UE panels.

- The UE burden and energy consumption associated with additional beam management procedures scales up with the number of CCs, UE panels and BWPs.

It is possible, however, to reuse beam management procedures across bands. For instance, when TRPs of different bands (e.g., 28 GHz and 39 GHz bands) are co-located, signals between NW and UE tend to propagate through the same physical directions. In this case, a beam sweep procedure common to multiple CCs may be feasible, including the reuse of TCI states. To harbor opportunities for optimizations, we make the following proposal:

- : For inter-band CCs, a common beam management procedure, including TCI states across bands, should be considered for data/control, DL/UL, multiple BWPs, multiple TRPs and multiple UE panels.

The feasibility of the above proposal depends, of course, on the UE ability to simultaneously produce beams in different bands which roughly point in the same directions. It is apparent that the ability to do so will depend on the UE implementation. For example, some UEs might not be able to simultaneously control directions of beams in different bands, or they might be able to do so for certain band combinations, but not for others. Therefore, the achievable savings depend on certain UE capabilities. RAN4 has been discussing a potential new UE capability on independent beam management (IBM) and/or common beam management (CBM) for inter band CA operation. Note that if such capability would be eventually agreed, Proposal 3 supporting a common beam management procedure states across bands would then become necessary.

- : The reporting of UE capabilities related to the ability of the UE to simultaneously steer in the same direction beams belonging to CCs in different bands should be considered.

Furthermore, RAN4 is discussing possible test setups for inter-band CCs wherein the beams of different CCs may be configured to use different polarization directions. In addition, some UEs may only be able to have independent beam control on different CCs with different polarizations due to limitations from the RF implementation. Therefore, when optimizing TCI states across bands, polarization needs to be taken into account. In particular, the polarization property of beams shall not impair optimizations of the TCI states while, at the same time, it should be possible to select orthogonal polarizations for beams in different bands pointing in the same/different directions.

- : For the optimization of TCI states across bands, the polarization property of beams should be considered.

# Unified TCI states

In Rel.15/16, the DL beam management depends on TCI states which can be configured in PDSCH-Config per BWP and viewed as a resource pool for DL Tx beams. The essential function of TCI state is to conduct QCL relation between two DL RSs, e.g. between CSI-RS and PDSCH DMRS.

Apart from that of DL, the beam management of UL is built on spatial relations which are separately configured and/or activated for UL channels and signals in a per resource level. Moreover, the spatial relation may contain an SRS resource besides DL RS as a QCL source, whereas a TCI state could only contain a DL RS (either SSB or CSI-RS) as a QCL source.

Due to the fundamental difference between UL and DL beam management mechanisms in the NR system, at least two sets of beam management parameters, i.e. TCI states for DL and spatial relations for UL, should be configured, (de)activated by NW, and maintained by UEs. Therefore, the WID of Rel.17 plans to have a unified TCI state framework which applies to both DL and UL beam management with less overhead and more flexibility.

In our view, the Rel.17 unified TCI state framework can be built on that of Rel.16 TCI state as illustrated in Figure 2. To make unified TCI states compatible with legacy UL beam management, the first thing that come to mind is to allow the option of an SRS resource in the unified TCI state, in addition to DL RS. Secondly, it is needed to allow UL beam indication by using the unified TCI states along with the spatial relation. Therefore, one arising question would be how to align these two parallel beam indication mechanisms for UL with as less standard impact as possible. We believe RAN1 may need to comprehensively discuss this issue to have an aligned solution that takes back compatibility, overhead, latency, etc. into consideration. Moreover, with the unified TCI states, one may wonder whether DL and UL share a common TCI state pool, e.g. tci-StatesToAddModList configured in PDSCH-Config, or whether NW configures a similar unified TCI state pool in PUSCH-Config for UL separately. Our thought prefers to maintain the flexibility of current DL and UL beam management. In our view, only if single unified TCI states pool can provide additional benefits, it should be discussed. Hence, we have following

- : Unified TCI state framework for both DL and UL

- : Study and specify (if necessary) a unified TCI state framework which can be applied to UL beam management in addition to DL beam management.

# Uplink panel-specific beam management

Regarding the UL panel-specific beam selection, good progress was made until RAN1#98 and then the conclusion came suddenly that this feature may be postponed to Rel.17 due to no consensus. With a new start in Rel.17, RAN1 may continue the discussion, and reconsider to design/specify the panel-specific beam selection in UL. Moreover, UL panel-specific beam indication can be even facilitated by the adoption of the unified TCI state framework discussed in Section 4.

Agreement Select one of the following alternatives in RAN1#98. Companies should take into account the maturity, forward compatibility to future releases, efficient use of SRS resource usage, and extension to simultaneous transmission across multiple panels of each alternatives for completion within the intended Rel-16 schedule. If there is no consensus in RAN1#98, UL multi-panel enhancement will not be specified in Rel-16. gNB can configure/indicate panel-specific transmission for UL transmission, via - Alt.2: Introduce a UL-TCI framework in Rel-16 and support UL-TCI based signaling analogous to DL beam indication supported in Rel-15, e.g., as illustrated below. - A new panel ID may or may not be introduced. - A panel specific signaling is performed using UL-TCI state - Alt.3: a new panel-ID is introduced, which can be implicitly/explicitly applied to the transmission for a target RS resource or resource set, for PUCCH resource, for SRS resource, FFS for PRACH - A panel specific signaling is performed using the new panel-ID implicitly (e.g., by DL beam reporting enhancement) or explicitly. - If explicitly signaled, the ID can be configured in the target RS/channel or reference RS (e.g., in the DL RS resource configuration or in spatial relation info). - No new MAC CE is specified for the purpose of introducing the ID.

## A new ID for panel-specific beam selection

In Rel.17, RAN1 can further narrow down between Alt.2 and Alt.3 listed in above agreement for panel-specific beam selection on UE antenna panels. Since the usage of SRS resource set as a panel ID is no longer an option, allow us to present our views on Alt.2 and Alt.3 in what follows. Regarding Alt.2, from WID perspective, it aligns well with the unified TCI state. However the TCI state is not directly linked to a UE panel in current specification. So to enable TCI state based panel-specific beam selection, the association between unified TCI state and UL panel should be specified either explicitly or implicitly.

As for Alt.3, a new ID either implicitly or explicitly introduced can be used for panel-specific beam selection. Given the current spatial relation based UL beam indication, it is reasonable to explicitly configure the ID in spatial relation for simplicity. Therefore, we believe Alt.3 is a reasonable choice to provide an ID for specifically controlling UE’s antenna panel(s).

- : An ID explicitly configured in spatialRelationInfo can be used for UL panel-specific beam selection.

## Panel-specific beam selection for other UL channels/signals

In RAN1#96bis, RAN1 agreed the MPUE-Assumption3 that multiple panels at UE can be simultaneously activated, but only one panel can be used for UL transmission. In addition, the discussion of panel-specific transmission has been extended to UL channels/signals other than PUSCH, i.e. PUCCH, SRS and even PRACH.

Agreement In Rel-16, only introduce specification enhancement for MPUE-Assumption3 - MPUE-Assumption3: Multiple panels are implemented on a UE and multiple panels can be activated at a time but only one panel can be used for transmission. - Note that this does not require a UE to always activate multi-panels simultaneously - Note: UE can control the panel activation/deactivation - Possible use cases at least include - (General) UL coverage enhancement for FR2 considering the UE power consumption - Discussion topics in Rel-16 include: - Details on the identification for a panel and corresponding panel definition - Any enhancement introduced in Rel-16 should take further enhancement of simultaneous transmission across multiple panels for future releases into account. This is a UE optional feature Working Assumption The agreed ID (not excluding to reuse existing ID) for a panel can be used for panel-selection-based transmission of PUSCH, PUCCH and SRS, among multiple activated panels. - FFS details, including an explicit/implicit indication of the panel, also considering beam correspondence at UE. - FFS on whether the ID can be used for panel-specific PRACH transmission, if supported.

From our understanding, it is natural to extend panel-specific beam selection from PUSCH to PUCCH/SRS/PRACH. Considering different beam indication mechanisms for PUSCH and PUCCH/SRS/etc, RAN1 needs to further look into the details of panel-specific beam selection for each channel/signal. For instance, in Rel.15, SRI in DCI (if present) indicates the Tx beam of PUSCH, but PUCCH and SRS rely on higher layer parameters PUCCH-SpatialRelationInfo and SRS-SpatialRelationInfo, respectively. Moreover, in RRC idle state, a UE transmits PRACH depending on the measurement of SSBs; in RRC connected state, the UE may transmit PRACH for different purposes, e.g. PCell BFR. During initial access procedure, it seems like the chance for NW to control UE’s panel is small. However, after RRC connection, the PRACH can be transmitted by UE in a panel-specific way.

- : Confirm the working assumption on explicit panel-specific indication on PUSCH/PUCCH/SRS and include PRACH with conditions FFS.

# Uplink coverage issue due to MPE

In Rel.16, RAN1 received an LS from RAN4 to handle the MPE issue so as to comply with regulatory RF exposure limits. In RAN4, two approaches to handle the MPE issue have been agreed upon. One is directly based on backing off from the configured maximum transmit power by a quantity P-MPR. The other one is to honor the maxUplinkDutyCycle reported in the signaling of UE capabilities.

Though aforementioned RAN4 solutions can address the MPE issue, they may trigger an UL coverage issue at FR2, at least for some of the beams. For instance, when an UL power back off is applied by the UE when transmitting UL channels/signals, the UE may lower its maximum power by, e.g. 10dB to satisfy RF exposure limits. This can possibly result in dissatisfactory measurements of an UL Tx beam at gNB side, though the corresponding DL beam selection may still function well. Hence, we would like to present our view

- : Due to the impact of power back off on UL Tx beam(s), Rel.15/16 UL beam management mechanism may yield suboptimal Tx beam selection from UL coverage perspective.

In RAN1#97, one agreement was achieved to enhance beam management when taking MPE issue into account

Agreement - Down-select in RAN1#98 from the following options for beam management enhancements: - Alt1. Support UE to report CRI/SSBRI where the CRI/SSBRI refers to a preferred spatial relation RS for UL transmission - FFS: Whether to support SRI in addition to CRI/SSBRI - FFS on details of the reporting configuration (e.g. separate or joint reporting with existing DL beam reporting, introduction of new information from UE such as MPR) - Alt2. Support SRI field in the DCI can be used to indicate multiple SRS resources and UE’s autonomous selection of one SRS resource for PUSCH beam determination out of the multiple - Alt3: Reuse Rel-15 beam specific PHR reporting to determine beam-specific MPE impact transparently, i.e., by difference value between Pc,max (which is calculated based on P-MPR) and the required transmission power. - FFS: Enhancement on UL beam configuration for virtual PHR. - Alt4: No enhancements considering MPE issues in Rel-16 RAN1 specifications. It is up to UE implementation in conjunction to RAN4 specification support. - If no consensus in RAN1#98, no further discussion in RAN1.

## MPE-aware beam reporting

Due to lack of consensus in RAN1#98, the discussion on MPE was prematurely concluded. In Rel.17, these alternatives (not limited necessarily to the above agreement) can be further discussed. Specifically, for UEs supporting beamCorrespondenceWithoutUL-BeamSweeping, the UL beam selection is established on DL beam sweeping and UE reporting. However, the power back-off impact was not taken into account in the current beam reporting mechanism. Hence, we have following

- : For UE supporting beam correspondence without UL beam sweeping, RAN1 should study and, if necessary, specify MPE-aware beam reporting.

For UEs supporting beam correspondence with UL beam sweeping, the SRS based beam sweeping is necessary. When a UE carries out this UL beam sweeping procedure, the UE may apply the power back off on a per Tx beam basis. More specifically, different SRS resources within the SRS resource set with usage set to ‘beam management’ may have different P-MPR values. SRS resources with their Tx beams experiencing the MPE issue should apply a larger power back off value, e.g. 10dB, when compared to other SRS resources not experiencing this issue. Therefore, to enable fair SRS resource selection among SRS resources with different P-MPR values, UEs should provide power back values per SRS resource (UL Tx beam) to NW. Hence we have

- : For UE supporting beam correspondence with the aid of UL beam sweeping, RAN1 specifies UE behavior on MPE-aware UL beam sweeping.

## Beam/panel-specific P-MPR reporting

The UL transmit power is strictly limited by regulators’ limitations on MPE. Therefore, a power back off (P-MPR) can be used by UEs to meet the MPE limitation defined in the Rel-15 RAN4 specification. However, since the P-MPR value can be significant and unpredictable in real life, it will severely reduce the UL coverage and potentially result in radio link failures (RLFs). In Rel-16, enhancements of the MPE solution target avoiding RLFs and connection releases caused by FR2 UE RF exposure compliance. In RAN4#93, it was agreed that the P-MPR shall be indicated to the network through MAC-CE [2].

For Rel-17 multiple-beam operation with MPE impact, it is reasonable for RAN1 to consider supporting the existing RAN4 solutions. As we mentioned earlier, RAN4 has agreed in RAN4#93 to indicate the P-MPR value to the network, so that the network can estimate a reasonable UL duty cycle for the UE to avoid potential radio link failure and UL coverage loss. From the RAN1 aspect, it can further associate the P-MPR value with spatial relation, e.g., reporting P-MPR of multiple candidate UL beams, so that the gNB can select the optimal beam pair for the uplink transmission.

- : Associate the P-MPR values with the spatial relation for UL transmission to mitigate the coverage loss due to the MPE.

Some possible ways to implement the above proposals have been discussed in RAN1 during Rel-16. However, to our understanding, no consensus was reached due to the limited time of the discussions. We believe those methods are still valid and need to be further discussed in Rel-17 to support further agreements reached in RAN4.

We reiterate one possible solution here which addresses our previous concern: the UE can report the preferred UL beam (via reference RS index) in the UL report which includes estimated P-MPR value. The reference RS index could include CSI-RS/SSB as well as SRS for gNB to select the optimal beam pair.

For multi UE-panel operation, a more straight forward way is to let the UE report a P-MPR value associated to a UE panel ID. Please note here that the MPE level is related to the physical antenna panels. Therefore, a panel ID likely needs to be related to a single physical antenna panel. If the P-MPR of each UE-panel is indicated to the network, the gNB can specify which panel to use and further optimize of the UL scheduling, resource allocation, etc.

- : Introducing a panel-specific P-MPR associated to a panel ID.

## UL beam/panel switching to avoid MPE

Enhancements from RAN4 are limited to transmitted power control and uplink scheduling. However, even for the same antenna panel, the exposure level can be different between different beams, and even larger differences can be observed between different panels. An example from [3] is shown below in Figure 3. The free space power density of three different array designs averaged over 4 cm2 is plotted: For the same array, the power density level varies for each spatial filter (beam). For different array designs, the difference is even more pronounced. As a result, one can expect the actual max UL duty cycle, and P-MPR to be different for each beam in a real implementation. Therefore, we believe that beam/panel switching can be an effective solution and enhancement to mitigating the UL coverage loss due to MPE.

(a)

(b)

- : The view of simulation models of 28 GHz UE mock-ups of three different 4×1 arrays, and phase excitation scheme with a progressive phase shift βi (b) maximum 4 cm 2-averaged incident power density when d (distance to the antenna array)  and βi vary for 28 GHz 4×1 array.

- : Beam/panel switching can be an effective solution and enhancement to mitigate the UL coverage loss due to MPE.

In addition to the efforts on enhancing MPE related beam reporting and P-MPR reporting, another way to avoid MPE is to switch from a beam which exceeds the maximum exposure limit. For instance, when a UE is served by two TRPs, if one UL Tx beam with one TRP shall back off Tx power due to MPE, this UE could choose the other TRP for UL transmission. In such case it would be beneficial to study the UE triggered UL Tx beam change. Hence, we have

- : To avoid UL MPE issue, it would be beneficial for RAN1 to study and, if necessary, specify the mechanism of UE triggered UL beam/panel switch.

As a further observation, we notice that the MPE issue only affects UL. Therefore, it is possible to only switch the UL beam or panel to overcome the UL coverage limitation caused by the MPE restriction. Depending on whether beam correspondence holds or not at the UE side, due to UL MPE issue, the UE may be scheduled with an UL Tx beam which is not the corresponding DL Rx beam. From Rel.15/16 spec, the difference between UL Tx beam and DL Rx beam mentioned above is surely not restricted, thanks to two parallel beam management mechanisms, i.e. TCI states in DL and SpatialRelationInfo in UL. So we have following observation

- : Depending on whether beam correspondence holds or not at UE side, the UE can be scheduled with an UL Tx beam which is not the corresponding DL Rx beam due to UL MPE issue.

With the observation above, it is possible to use different beam pairs or even different antenna panels for DL and UL. A UE can switch the UL beam or panel towards a new direction or even another TRP but still keep the DL beam if the current beam pair runs into a low UL coverage caused by the MPE restriction.

# Definition on UE antennas

## Antenna panel definition

During offline discussions of RAN1#96bis, it was pointed out that RAN1 still does not have a clear definition of UE’s antenna panel(s). Because different UE vendors may implement UE antenna panels in different ways, the conventional panel definition, a physical antenna array with either single or dual polarization, doesn’t cover all aspects for panel-specific beam selection. The following conclusion was reached for informative purpose and companies have been given enough time to digest the consequences.

Conclusion - From RAN1 point of view, a “UE panel” would be a logical entity and how to map physical UE antennas to the logical entity is up to UE implementation. - (Informative) For certain condition(s), gNB can assume the mapping between UE’s physical antennas to the logical entity “UE panel” activated for transmission will not be changed - FFS: Whether “UE panel” is transparent to gNB - FFS: UE capability includes at least the number of “UE panels”. - FFS: Whether/how to define the certain condition(s) - For example: The duration of time over which the gNB assumes there will be no change - For example: Until next update or report from UE - (Informative) Depending on UE’s own implementation, a “UE panel” can have at least the following functionality as an operational role of - Unit of antenna group to control its Tx beam independently

Furthermore, in the WID on multi-beam enhancement of Rel.17, features facilitating UL beam selection for UEs equipped with multiple panels, considering UL coverage loss mitigation due to MPE, based on UL beam indication with the unified TCI framework for UL fast panel selection are targeted. To achieve those goals, we have following

- : To enable an efficient implementation of features for Rel.17 UL multi-panel operation, a definition of “UE panel” is a prerequisite.

The definition of antenna panel may vary from implementation to implementation. However, to enable an effective multi-panel operation, we suggest that the definition of a UE panel shall be formulated in such a way that it can be associated with one or more of the properties listed below.

UE panel properties

- A UE panel supports Y beams

- Whether a UE panel supports beam correspondence (BC)

- A UE panel supports YRX independent Rx-beams (from the Y set of beams)

  - coherent

  - non-coherent

- A UE panel supports YTX independent Tx beams (from the Y set of beams)

  - coherent

  - non-coherent

- Each beam from a UE panel may support one or two polarizations when receiving

  - Single layer (coherent)

  - Dual layer (coherent or non-coherent)

- Each beam from a UE panel may support one or two polarizations when transmitting

  - Single layer (coherent)

  - Dual layer (coherent or non-coherent)

- A UE panel has an associated P-MPR

To enable the multi-panel operation and let UE select the optimal panel, a panel ID in addition to the spatial relation is necessary. This ID enables the UE to report the panel specific capability as listed above. In addition, some other UE panel properties are not listed above, e.g. supported beam widths, has not been discussed so much in RAN1 so far. However, we identify them as critical for further enhancement on multiple beam operation and coverage extension, which shall not be precluded in the discussion. Consequently, for a UE that supports multi-panel operation, the following capabilities or properties may also need to be further defined and have following proposal

UE panel related properties

- A UE may have up to X1 UE panels

- A UE panel has an associated UE panel ID

- A UE supports NRx simultaneous Rx layers (coherent or non-coherent)*

- A UE supports NTx simultaneous Tx layers (coherent or non-coherent)*

- A panel has an activation-delay when in standby

- Unit of UE panel group to control UL timing

- Unit of UE panel group to control transmit power

*Partly defined in TS 38.214, section 6.2.6.2.

- : We suggest that definition of a UE panel and the related properties are addressed prior to further advancing discussions related to UE panel enhancements.

## Analog beam definition

Besides UE antenna panel definition, we also observe that there is no definition of what a beam is. Though analog beamforming is highly up to UE implementation, but in our understanding, a beam can be defined as

- : A beam can be defined as a spatial filtering associated with one or two antenna ports carrying one or two layers separated in the polarization domain.

With the beam definition in hand, next one may consider whether to signal the UE capability on whether single or dual layer(s) are supported by a panel. In RAN4#90bis main session’s chairman notes, it was agreed to use 8 beams for an UL beam sweep in beam-correspondence test. In addition it was also agree to further study whether there is an advantage to specify the polarization aspect in the UL beam sweep. This may have system benefits both from a MU perspective as well as from a link level capacity perspective. Hence we have following

- : RAN1 needs to study and specify (if necessary) whether additional signaling is necessary when a beam can support up to two independent layers separated by polarization.

# Conclusions

Finally, allow us to repeat our proposals to draw attention.

- : Study and specify (if necessary) the L1/L2-centric approach of both DL and UL beam management considering the use cases of inter-cell mobility

- Incorporate PCI into TCI state for DL beam management

- Incorporate PCI into Spatial Relation for UL beam management

- : Specify the QCL assumption among SSB/CSI-RS resource sets on different BWPs/CCs (intra band).

- : For inter-band CCs, a common beam management procedure, including TCI states across bands, should be considered for data/control, DL/UL, multiple BWPs, multiple TRPs and multiple UE panels.

- : The reporting of UE capabilities related to the ability of the UE to simultaneously steer in the same direction beams belonging to CCs in different bands should be considered.

- : For the optimization of TCI states across bands, the polarization property of beams should be considered.

- : Study and specify (if necessary) a unified TCI state framework which can be applied to UL beam management in addition to DL beam management.

- : An ID explicitly configured in spatialRelationInfo can be used for UL panel-specific beam selection.

- : Confirm the working assumption on explicit panel-specific indication on PUSCH/PUCCH/SRS and include PRACH with conditions FFS.

- : For UE supporting beam correspondence without UL beam sweeping, RAN1 should study and, if necessary, specify MPE-aware beam reporting.

- : For UE supporting beam correspondence with the aid of UL beam sweeping, RAN1 specifies UE behavior on MPE-aware UL beam sweeping.

- : Associate the P-MPR values with the spatial relation for UL transmission to mitigate the coverage loss due to the MPE.

- : Introducing a panel-specific P-MPR associated to a panel ID.

- : To avoid UL MPE issue, it would be beneficial for RAN1 to study and, if necessary, specify the mechanism of UE triggered UL beam/panel switch.

- : We suggest that definition of a UE panel and the related properties are addressed prior to further advancing discussions related to UE panel enhancements.

- : A beam can be defined as a spatial filtering associated with one or two antenna ports carrying one or two layers separated in the polarization domain.

- : RAN1 needs to study and specify (if necessary) whether additional signaling is necessary when a beam can support up to two independent layers separated by polarization.

# Reference

- RP-193133, “New WID: Further enhancements on MIMO for NR”, Samsung

- 3GPP TS 38.321, “Medium Access Control (MAC) protocol specification”

- B. Xu, K. Zhao, Z. Ying, D. Sjöberg, W. He and S. He, "Analysis of Impacts of Expected RF EMF Exposure Restrictions on Peak EIRP of 5G User Equipment at 28 GHz and 39 GHz Bands," in IEEE Access, vol. 7, pp. 20996-21005, 2019.