3GPP TSG RAN WG1 #102-e			R1-2005579

e-Meeting, August 17th – 28th, 2020

Agenda Item: 	8.5.3

Source:	Sony

Title:	Discussion on Positioning Enhancements

Document for:	Discussion & Decision

# Introduction

A study item on NR positioning enhancements in Rel-17 has been approved with the main motivation to address the higher accuracy location requirements resulting from new applications and industry verticals [1]. One of the study item objectives is to identify and evaluate positioning techniques, DL/UL positioning reference signals, signalling and procedures for improved accuracy, reduced latency, network efficiency, and device efficiency. Enhancements to Rel-16 positioning techniques, if they meet the requirements, will be prioritized, and new techniques will not be considered in this case.

In RAN1 #101-e, the following agreements related to positioning requirements were made [2]:

Agreement:

- In Rel-17 target positioning requirements for commercial use cases are defined as follows:

  - Horizontal position accuracy (< 1 m) for [90%] of UEs

  - Vertical position accuracy (< [2 or 3] m) for [90%] of UEs

  - End-to-end latency for position estimation of UE (< [100 ms])

  - FFS: Physical layer latency for position estimation of UE (< [10 ms])

- In Rel-17 target positioning requirements for IIoT use cases are defined as follows:

  - Horizontal position accuracy (< X m) for [90%] of UEs

    - X = [0.2 or 0.5] m

  - Vertical position accuracy (< Y m) for [90%] of UEs

    - Y = [0.2 or 1] m

  - End-to-end latency for position estimation of UE (< [10ms, 20ms, or 100ms])

  - FFS: Physical layer latency for position estimation of UE (< [10ms])

- Note: Target positioning requirements may not necessarily be reached for all scenarios

In this contribution, we discuss our views on potential techniques which are basically the enhancements to Rel-16 positioning techniques in order to meet NR positioning commercial requirements in Rel-17.

# Discussions

During NR-Rel16, these RAT dependent positioning techniques have been identified:

- Downlink Time Difference of Arrival (DL-TDOA),

- Uplink Time Difference of Arrival (UL-TDOA),

- Multi-Cell Round Trip Time (Multi-RTT),

- Downlink Angle of Departure (DL-AoD),

- Uplink Angle of Arrival (UL-AoA), and

- Enhanced Cell ID (E-CID).

We consider possible positioning techniques that are targeting to fulfil NR positioning commercial requirement in Rel-17 shall be based on the aforementioned techniques. The motivation is to minimize any potential hardware impacts on both gNB and UE and also to minimize the signalling between UE to gNB or Location Server (LS). The following are some of the potential techniques.

## Enhancements of Reference Signal Transmission

### Dynamic Positioning Reference Signal Resource Allocation

Various positioning requirement parameters and the associated requirement values have been defined for various use-cases. The positioning requirements for general 5G positioning services can be found in [4] and requirements specific to industrial factory automation can be found in [5]. There may be a case that some UE requires higher positioning accuracy and the other UEs do not require such high accuracy. Higher accuracy typically requires large amount of positioning reference signal resource allocation. We consider the resource for positioning reference signal should support dynamic allocation. This can also be called as on-demand resource allocation depending on the required accuracy of the UE. Hence, the overall resource allocation can be used wisely.

Furthermore, dynamic positioning reference signal transmission can also be used to further reduce latency. The periodicity of PRS transmission can be relatively large. It can be larger than the required latency of IIoT application which is around [10] ms. Hence, the transmission of dynamic PRS in a form of aperiodic PRS triggered for the UE in a need of low latency positioning can solve this issue. The detailed operation can be further studied during study item phase.

Observation 1: Aperiodic PRS can be used to improve positioning accuracy and/or reduce positioning latency.

Proposal 1: Support the study on dynamic PRS allocation / Aperiodic PRS transmission to improve positioning accuracy and/or reduce positioning latency.

### Enhanced Beam Operation & Network Efficiency

In Rel.16 NR, downlink-based positioning, including DL-TDOA, has been supported. In principle, the base-stations transmit PRS and the UE calculates the time of arrival (DL-TDoA) from each base-station. In NR system, a PRS transmission with a beam direction (instead of omni-directional and sectorized as in LTE) is supported. To support a good coverage (i.e. in multiple directions), the base-station may transmit a beam sweeping operation to cover all directions. The base-station may also transmit ultra-narrow-beam to all directions. However, this may not be preferred solution as it will significantly increase the reserved resources for the PRS transmission (i.e. high signal overhead).

Considering the above aspects (i.e. coverage and overhead), the PRS may be transmitted with a relatively wide-beam. The UE can still obtain a reasonable positioning accuracy. Commercial requirements have various levels, including centi-meter level of accuracy. To support better accuracy and coverage with low enough overhead, methods of finding the bests beams pairs efficiently from multiple base-stations are needed, particularly when more narrow beams can be considered. We also consider a narrow beam operation can be formed by transmitting PRS from more than one antenna port as it is today.

Proposal 2: Support the study on enhanced beam operation for positioning, including PRS transmission with more than one antenna port (i.e. 2 antenna ports) and the usage of legacy reference signal for positioning (TRS, CSI-RS).

### 1 Symbol PRS transmission

In Rel.16 NR positioning, the smallest number of PRS resource is with 2 OFDM symbols and with Comb-2 and 12 OFDM symbols as the largest one. In order to reduce latency, we propose to support the operation with 1 OFDM symbol. The corresponding Comb structure can be further investigated.

Proposal 3: Support PRS configuration with 1 symbol PRS transmission.

### Interference Mitigation

Part of the effort to improve positioning accuracy is to deploy a densified network in which the network deploys many reference points (e.g. Transmission/Reception Point (TRP)) in a rather small area. The reference points, with known positions, transmit reference signals and the UE performs positioning measurements based on those reference signals. However, if not handled properly, the huge number of reference points that simultaneously transmit positioning reference signal (PRS) may create signal interference, which will degrade the received reference signal quality at the UE side.

PRS transmissions shall be coordinated and adapted according to the needs of IIoT applications. By controlling the PRS transmission in a dense network, the interference can be mitigated and hence one can improve the received reference signal quality at the UE side. The interference of PRS can be mitigated by introduction a new PRS resource pattern that is especially designed for dense network deployment. The usage of aperiodic PRS as proposed above can also be considered. Hence, only a few selected gNBs transmit PRS.

Proposal 4: A mechanism to control or to mitigate interference of PRS transmission in a densified network shall be studied.

## Enhancements on Measurements & Reporting

### LOS & NLOS Detection

The main issue in indoor factory scenario is the existence of multipath components. The non-light of sight (NLOS) components can be more dominant than the Line of Sight (LoS) component. The usage of NLOS components can compromise the performance of the positioning measurements.

DL-Time Different of Arrival (DL-TDoA) is one of the positioning techniques in Rel-16. The gNB transmits positioning reference signals, the UE performs time of arrival (ToA) measurements (positioning measurement) and reports them to the Location Server (LS). Then, LS performs the positioning estimation. In order for the LS to perform the positioning estimation, multiple positioning measurements (e.g. RSTD) based on the received PRS from multiple gNBs (multi-lateration) are required. In practice, there could be timing error in each DL-TDoA measurement.

There are many factors which may introduce timing errors. If the measurement is purely based on the LoS component, then the error is relatively small. The error can be relatively large if the measurement is based on NLoS component as the signal is propagated and reflected. Hence, it does not represent the true propagation time of the LoS between the BS and the UE. The timing error may become severe in indoor factory environments.

Our simulation result as shown in Figure 1 below indicates that providing NLOS indication/information as part of the positioning measurement report can improve the positioning accuracy significantly. The details of the simulation assumptions and parameters can be found in our companion contribution in evaluation of positioning accuracy agenda item.

Figure 1: Simulation results in Indoor Factory Sparse High (InF-SH) using FR2 with BW 400 MHz.

If the UE can detect whether the received signal from a gNB is dominantly LOS or NLOS then this information can be used to minimize the timing error. The UE can indicate or report this information to the LS. Hence, LS can use this information for the positioning estimation purpose. The detailed UE detection mechanism and signalling procedure can be studied during study item phase.

Proposal 5: Support the study on LOS & NLOS detection mechanism at the UE and the associated signalling procedure.

### Fast Measurement Report

The signaling of positioning in the legacy radio access network can be illustrated in Figure 2. LS sends positioning measurement request to the UE via LPP protocol (and transparent to gNB). In case of inter-frequency measurement, the UE performs measurement within a measurement gap, particularly when PRS from multiple gNB(s) are scheduled. Once the UE obtained the positioning measurement data, the UE sends uplink scheduling request (SR) to the serving gNB. The serving gNB sends the corresponding UL grant in downlink control channel (PDCCH). The UE then finally can transmit the positioning measurement reports in uplink data channel (PUSCH) to the LS. This legacy positioning reporting procedure needs multiple consecutive signaling in which it results in high latency. This can be improved by using a method so that the UE can report immediately once the positioning report has been obtained. Using uplink configured grant uplink allocation for positioning measurement report can be one of the methods to support fast measurement report. Alternatively, in order to provide more flexibility for the gNB to allocate the uplink resource, the UE can be configured to monitor downlink control channel (uplink grant) once it has obtained positioning measurement.

PRS configuration

PRS configuration

SR request

Meas GAP

PRS transmission

Positioning meas request

UE

gNB

LS

POS meas

UL grant

POS meas_rep

Figure 2: Legacy signalling mechanism in DL-TDOA positioning technique

Proposal 6: Support the operation of fast positioning measurement report once the UE has obtained positioning measurement result (e.g. using uplink configured grant for positioning, UE to directly monitor control channel for uplink grant)

## Enhancements of NR Positioning Procedures and Signalling

### Positioning in RRC Idle/Inactive

NR positioning operation can so far be supported when the UE is in RRC connected mode. Supporting the positioning operation in RRC idle/inactive mode can be beneficial, particularly in an attempt to reduce latency, and reduce UE power consumption. We can consider DL-PRS is available as the signal is periodically broadcasted by gNBs. The UE can perform RSTD measurements while the UE is in idle/inactive mode. Furthermore, the UE can transmit the measurement report in preconfigured uplink resource (PUR) or with early data transmission (EDT) mechanism. The detailed signalling and procedure to support positioning measurement and positioning reporting in idle/inactive can be studied during study item phase.

Proposal 7: Support the operation of NR positioning when the UE is in RRC idle/inactive mode

### Two Steps Positioning

NR positioning requires high positioning accuracy and one of the challenging requirements is to achieve 20 cm horizontal positioning accuracy. High accuracy typically requires a large number of PRS resources. For example, the baseline PRS bandwidth considered in study item phase for FR1 and FR2 are 100 MHz and 400 MHz, respectively. We consider not all UEs in a cell require high accuracy positioning. gNB can transmit periodic PRS with relatively small bandwidth (e.g. 20 MHz) so that a coarse UE position can be obtained. A fine UE position can be facilitated by the transmission of aperiodic PRS transmission with larger bandwidth. The impact of PRS bandwidth configuration to positioning estimation accuracy is shown in Figure 3. The details of simulation assumption and configuration can be found in our companion contribution [6]. Here, we can observe that high positioning accuracy require large bandwidth.

Figure 3: Simulation results in Indoor Factory Sparse High (InF-SH) using FR2 with various BW size.

Proposal 8: Support positioning procedure for the operation of two steps positioning (i.e. coarse and fine positioning)

# Summary

# In this contribution, we provide our initial views on potential positioning techniques in Rel-17. Our observation and proposals are listed below:

Observation 1: Aperiodic PRS can be used to improve positioning accuracy and/or reduce positioning latency.

Proposal 1: Support the study on dynamic PRS allocation / Aperiodic PRS transmission to improve positioning accuracy and/or reduce positioning latency.

Proposal 2: Support the study on enhanced beam operation for positioning, including PRS transmission with more than one antenna port (i.e. 2 antenna ports) and the usage of legacy reference signal for positioning (TRS, CSI-RS).

Proposal 3: Support PRS configuration with 1 symbol PRS transmission.

Proposal 4: A mechanism to control or to mitigate interference of PRS transmission in a densified network shall be studied.

Proposal 5: Support the study on LOS & NLOS detection mechanism at the UE and the associated signalling procedure.

Proposal 6: Support the operation of fast positioning measurement report once the UE has obtained positioning measurement result (e.g. using uplink configured grant for positioning, UE to directly monitor control channel for uplink grant)

Proposal 7: Support the operation of NR positioning when the UE is in RRC idle/inactive mode

Proposal 8: Support positioning procedure for the operation of two steps positioning (i.e. coarse and fine positioning)

# References

- RP-193237 SID: NR Positioning Enhancements

- TR 38.901 Study on channel model for frequencies from 0.5 to 100 GHz v16.0

- TR 38.855 Study on NR positioning support

- TS 22.261 “Service requirements for the 5G system”

- TR 22.804 “Study on Communication for Automation in Vertical Domains”

- R1-2005578, “Initial Views on Evaluation of Positioning Accuracy and Latency”, Sony , RAN1#102e, August 2020.